WHY DID ADDING MY MODEM

AT COM3 OR COM4

HANG THE

MODEM?

OR

WHY

EVEN IF YOU DON'T "SHARE INTERRUPTS"

YOU

GET CONFLICTS

ON THE COM1-4 PORTS

by Dr. W. Curtiss Priest

Center for Information, Technology and Society

Lexington, MA  02173

Internet: bmslib@mitvma.mit.edu

Voice: 617-862-7763

_______________________________________________________________________

***********************************************************************

Most of us quickly have learned that while there are four COM

ports, there are only (usually) 2 IRQ's.  [In AT's you can also

use IRQ's 5,7, and 9, if your serial board supports those.]

First -- many uses of serial ports do not use interrupts and so you

can in those cases just forget about them.  Example:  you print

out a serial port using XON/XOFF hand-shaking using a redirection

program like LPTCOM1.

Second -- if you use only one IRQ, there is still no potential conflict

in terms of IRQ assignments.

Third -- if you want to use 2 IRQ's [3 and 4], you want one port to use

one of those and the other to use the other one.  Typically IRQ 4

is hardwired for use with COM1 and COM3, and IRQ 3 is hardwired for

use with COM2 and COM4.

So -- a typical configuration is:  your serial card is set for COM1

and COM2.  You use COM2 for your mouse (usually a DB 9 connector).

The mouse always uses interrupt servicing, and so your mouse driver

"hooks interrupt vector 3" and when an interrupt occurs the driver

services the request.

But now you add an internal modem.  Most telecommunications programs

like Procomm Plus use interrupt servicing of the modem.  So --

you must use the other IRQ, IRQ 4.  So, you set the modem to COM3, since

this usually sets the IRQ to 4 [newer boards allow mixing and matching

of IRQ's but the problem is always the same -- don't let two ports use

the same IRQ ! !

______________________________________________________________________

********************

BUT

You find things still don't work.  Why?

The answer is in hardware, not in software.

When IBM designed the PC, they used an 8259 chip to service the

interrupt request.  On the bus there are separate lines for each IRQ.

On the XT there are 8 interrupts.  On the AT there are 16 (actually

15, since IRQ 9 is redirected to IRQ2)

An informative shareware program called IRQR produces the following

on my machine:

IRQR -- Report Status of 8259 Hardware Interrupt Channels

Copyright 1988 by Lawrence R. Steeger

8259  IRQ  Status   IBM Assignments  OEM Assignments

----  ---  ------   ---------------  ---------------

0x21    0    used   Timer

1    used   Keyboard

2    used   2nd 8259         Autofax Imager

3   >FREE   COM2

4   >FREE   COM1

5   >FREE   LPT2             MS Bus Mouse

6    used   Floppy Disk

7   >FREE   LPT1

0xA1    8    used   Clock

9    used   Redirected IRQ2

10   >FREE   (reserved)

11   >FREE   (reserved)

12   >FREE   (reserved)

13    used   Coprocessor

14    used   Hard Disk

15   >FREE   (reserved)

If you go into Procomm and shell to DOS and do this again, and if your

modem is on IRQ 4, you'll find the report changed to show IRQ 4

now used.  When you leave Procomm, Procomm releases the interrupt and

the report reverts back to showing FREE.

If you take a circuit diagram of a PC, you'll find each of the IRQ's

to correspond to actual bus pins [yes, the ones you plug cards into].

For example, you'll find IRQ 4 on Pins B24 and A24 [the only slot in

an XT and the slot nearest the back on an AT].  On an AT you'll find

for example, IRQ 14 (Hard Disk) on Pin D7.  This is in the second

slot from the back in the AT, and not present on the XT.

********************

THE ANSWER

The 8279 watches all the IRQ lines (8 in an XT and 16 in an AT).

It recognizes an IRQ when the line goes from 0 volts to 5 volts.

It is also "edge triggered" -- the line only needs to go high long

enough for the 8279 to register the event on the rise of the square

wave.

In hardware things often are held high and pulled low.  This is commonly

done with a resistor to an output going to +5 volts and a transistor from

the output connecting the anode to the output and the emitter to ground.

A signal on the base causes the transistor to conduct, the output is

then pulled low.  This is very common in TTL (Transistor-Transistor Logic).

Boards sharing the same line will typically not "electrically" interfere

since they the line is high across all of them using the pull-up resistor

and so any one board can pull the line low, regardless of the presence of

extra pull-up resistors (these resistors are typically ten thousand ohms

and a transistor can pull dozens of these in parallel low).

But remember, IBM uses a chip that wants an output that goes low to high.

To achieve this the serial board designer uses a "totem pole driver."

Here there are two transistors. One to pull the line up, the other to pull

the line down.

Returning to software, you might think that if you don't ask the UART (the

Universal Asynchronous Receiver/Transmitter) chip to enable the interrupt

that the output on the IRQ line will be not present.  This would be

achieved with a "tri-state" output.  Tri-state is either low, high, or not

there.

But this just isn't so.  Even if you haven't enabled the interrupt in

software, it still exists in the hardware.  So on that

serial port, a transistor is still trying to pull that IRQ line low.

So, if another board tries to pull it high using its transistor to +5,

there is a battle between the transistor on one board pulling the line

low and the other trying to pull it high.

The result is confusion.  Sometimes it works because the IRQ you want is

able to beat out the other port's transistor, and sometimes it doesn't

because it just can't compete.

********************

THE SOLUTION

The solution is easy.  On boards that you are not going to use the

offending IRQ line, you disconnect that line from the bus.

There are two ways:

1.)  For boards that are hardwired, you take a narrow strip of

scotch tape and place it over the gold contact that slide into

the board.  To disable IRQ4 you wrap B24/A24 and to disable

IRQ3 you wrap B25/A25.

2.)  For boards that have jumpers for IRQ assignments, just

take the jumper for the offending IRQ and remove it, or let

it dangle from one pin into mid air.

Is this unbelievable?  Absolutely ! !