__________________________________________________________________________

STACKER NOTE                        Stac Electronics Technical Note

Subject: Configuring a CD-ROM on a Stacked System.

Tec036 - 6/18/92

__________________________________________________________________________

The Stacker device driver is assigned a drive letter by DOS when it loads.

The CD ROM drive letter is assigned in the Autoexec.bat.  Stacker will

often be given the letter currently being used by the CD-ROM, so its

introduction to the system configuration will require that the CD-ROM

letter change.

Configuring the CD-ROM:

1. CONFIG.SYS

Here are is a sample Config.sys.  It is recommended that the CD-ROM

device be loaded after SSWAP.COM as shown.

DEVICE=C:\DOS\SETVER.EXE  (this is required if running under DOS 5.0)

DEVICE=C:\STACKER\STACKER.COM  C:\STACVOL.DSK .....etc.

DEVICE=C:\STACKER\SSWAP.COM C:\STACVOL.DSK /SYNC

DEVICE=SONYCDU.SYS or DEVICE=PCD650S.SYS   [/D:name /B:xxx /M:mode]

2. AUTOEXEC.BAT:

The CD-ROM letter is assigned here with the /L: parameter on the

MSCDEX command line.  Take note of the Stacker drive letter assignments

at boot time, or use the Swapmap command to see how the drives have been

assigned.  Set the CD-ROM to the next available letter.

MSCDEX /D:name /M:x /V /L:letter /E

After reassignment, it may be necessary to re-setup any applications that

are drive letter specific.

--------------------------------------------------------------------------

1992 Stac Electronics