SHADOW RAM MEMORY CONTROLLERS

supported by

THE LAST BYTE MEMORY MANAGER (tm)

Version 2.20

Copyright (C) 1990-92

All Rights Reserved

by

KEY SOFTWARE PRODUCTS

440 Ninth Avenue

Menlo Park, California 94025

415-364-9847

The Last Byte Memory Manager is a trademark of Key Software Products.

TABLE OF CONTENTS

CHAPTER 1 - INTRODUCTION        ...........................    1

1.1  Hardware Requirements       .......................    1

1.2  Supported Chipsets       ..........................    1

1.2.1  ACC Microelectronics       ...................    1

1.2.2  AddTech Research (PCChip)      ...............    2

1.2.3  Appian Technology       ......................    2

1.2.4  Atmel Corporation       ......................    2

1.2.5  Chips and Technologies      ..................    3

1.2.6  EFAR Microelectronics       ..................    3

1.2.7  ETEQ Microelectronics       ..................    4

1.2.8  Forex Computer Corporation      ..............    4

1.2.9  Headland Technology (G2)      ................    5

1.2.10  Intel Corporation       .....................    5

1.2.11  Mosel Corporation       .....................    5

1.2.12  Oak Technology       ........................    5

1.2.13  OPTi Electronics       ......................    6

1.2.14  PicoPower Technology       ..................    7

1.2.15  Silicon Integrated Systems (SiS)     ........    7

1.2.16  Sun Electronics (SUNTAC)      ...............    7

1.2.17  Symphony Laboratories       .................    7

1.2.18  Texas Instruments       .....................    8

1.2.19  UNIchip Incorporated       ..................    8

1.2.20  United Microelectronics Corporation (UMC)        8

1.2.21  VIA Technologies       ......................    8

1.2.22  VLSI Technology       .......................    9

1.3  Unsupported Chipsets       ........................   10

1.3.1  No Shadow Ram Support     ....................   10

1.3.2  Shadow Ram is not Read/Write    ..............   10

1.3.3  Overly Complex Control      ..................   10

1.3.4  Locked Configuration Registers      ..........   11

1.3.5  Configuration I/O Ports Reassigned     .......   11

1.4  Checking Compatibility Using CHIPSET     ..........   11

Jan 31, 1993      SHADOW RAM MEMORY CONTROLLERS                 1

CHAPTER 1 - INTRODUCTION

1.1 Hardware Requirements

To  install The Last Byte Memory Manager, your computer must have

a  means  of providing read-write memory in the Upper Memory Area

between  640k  and  1 megabyte.  This is possible if you have any

of the following:

o  Fixed  RAM memory already resident in the upper area

(such  as  the 32-bit "Built In Memory" installed in

some Compaq 386 machines), or

o  An  expanded memory board (EEMS, LIM 3.2 EMS, or LIM

4.0 EMS), or

�������������������������������������������������Ŀ

� NOTE: Using a LIM 3.2 EMS board is not advised  �

� as a method of providing upper memory.  It pro- �

� vides only 64k of upper memory and prevents ap- �

� plication software from using expanded memory.  �

���������������������������������������������������

o  A  386sx,  386,  486,  or  486sx  processor  and the

Microsoft  Windows  3.0/3.1  or  DOS  5  versions of

EMM386, or

o  One  of  the  shadow  ram  memory  controller  chips

listed in the next section.

1.2 Supported Chipsets

Key  Software  Products  is  continually  adding  support for new

chipsets  as  they  become  known to us and technical information

can  be  obtained.  Those that are currently supported in version

2.20  are  listed in the paragraphs below.  Some that we know of,

but which cannot be supported are listed in the next section.

1.2.1 ACC Microelectronics

The  following  chips  all provide a single block of 64k by using

the chip set EMS page frame.

ACC2036 - Single Chip Solution for the 286 and 386SX

ACC2121 - From the Turbo PC/AT Chip Set for the 286 and 386SX

The  following  chip  provides  a  single  block  of 128k between

E000-FFFF:

Copyright (C) 1990-92, Key Software Products. All Rights Reserved

Jan 31, 1993      SHADOW RAM MEMORY CONTROLLERS                 2

ACC2300 - From the 82300 386 AT Chip Set

1.2.2 AddTech Research (PCChip)

AddTech  chipsets  are  usually  labelled  "PCChip"  or "Hedaka".

AddTech  designs  their  chips for use on their own motherboards,

and   they  identify  their  chipsets  by  the  motherboard  part

numbers.   There  is at least one AddTech chipset that is used on

two   different  motherboards  with  different  motherboard  part

numbers.   To  avoid  confusion,  The  Last  Byte  Memory Manager

refers  to  these  chips  using  "PCCHIP"  followed  by  a unique

number.

PCCHIP1 - Used on 286/386SX AT motherboards M205 and M209

PCCHIP2 - Used on 386 AT motherboard M321

PCCHIP1  provides  256k  of  memory  in twenty 16k blocks between

C0000-FFFFF.

PCCHIP4  provides  192k  of  memory  in twelve 16k blocks between

C0000-EFFFF.   If  you  have  a  VGA  or EGA adapter, you may get

additional  memory  with  these  chips  by using the MOVE=OVERLAY

option of LASTBYTE.SYS.

PCCHIP4  is functionally identical to the OPTi Electronics 82C391

and  the  Atmel AT40391; their corresponding PHYSICAL options are

equivalent.

1.2.3 Appian Technology

The  following chip provides 256k of memory in sixteen 16k blocks

between C0000-FFFFF.

P915 - 486/386DX System Controller

1.2.4 Atmel Corporation

The  following  chips  all  provide  192k of memory in twelve 16k

blocks  between  C0000-EFFFF.   If you have a VGA or EGA adapter,

you  may  get  additional memory by using the MOVE=OVERLAY option

of LASTBYTE.SYS.

Copyright (C) 1990-92, Key Software Products. All Rights Reserved

Jan 31, 1993      SHADOW RAM MEMORY CONTROLLERS                 3

AT40281 - 386SX Core Logic Controller w/Cache

AT40283 - 386SX Core Logic Controller w/o Cache

AT40391 - From the 80386DX PC/AT Chip Set

AT40493 - From the 80486DX PC/AT Chip Set

All  of  the above chips are functionally identical to chips with

similar  part  numbers  made  by OPTi Electronics: the AT40281 to

the  OPTi  82C281, the AT40283 to the OPTi 82C283, the AT40391 to

the  OPTi  82C391  and  to  AddTech  Research's  PCCHIP4, and the

AT40493  to the OPTi 82C493; these corresponding PHYSICAL options

are equivalent.

1.2.5 Chips and Technologies

The  following  chips  all  provide 384k of memory in twenty-four

16k blocks between A0000-FFFFF.

82C212 - From the CS8221 NEAT (tm) chipset

82C222 - From the CHIPS/250 PS/2 50/60 chipset

82C230 - The PS/2 30 Single Chip Controller

82C235 - The SCAT (tm) Single Chip AT VLSI chip

82C241 - From the CS8223 LeAPset (tm) chipset

82C302 - From the CS8230 386/AT CHIPSet (tm)

82C307 - From the CS8231 386/AT CHIPSet (tm)

82C311 - From the CS8233 PEAK 386/AT CHIPSet (tm)

82C351 - From the CS82310 PEAK DM/386 AT CHIPSet (tm)

82C812 - From the CS8281 NEATsx (tm) chipset

82C836 - The SCATsx (tm) Single Chip AT VLSI chip

82C841 - From the CS8283 LeAPset-sx (tm) chipset

The    82C212   is   functionally   identical   to   the   United

Microelectronics  UMC82C212; their corresponding PHYSICAL options

are equivalent.

The  following chip provides 384k of memory in sixteen 16k blocks

between A0000-DFFFF and one 128k block between E0000-FFFFF.

82C322 - From the CHIPS/280 PS/2 70/80 chipset

The  following  chip  provides  a maximum of 384k of memory using

two  64k  blocks  between  A0000-BFFFF,  four  16k blocks between

C0000-CFFFF,  and three 64k blocks between D0000-FFFFF.  However,

any  of  the  first two and last two 64k blocks might be "locked"

and  unavailable due to the design of your particular bios.  This

may reduce the amount of memory from 384k to as little as 128k.

4021 - From the ISA/486 chipset.

Copyright (C) 1990-92, Key Software Products. All Rights Reserved

Jan 31, 1993      SHADOW RAM MEMORY CONTROLLERS                 4

1.2.6 EFAR Microelectronics

The  following  chips  all  provide  192k of memory in twelve 16k

blocks  between  C0000-EFFFF.   If you have a VGA or EGA adapter,

you  may  get  additional memory by using the MOVE=OVERLAY option

of LASTBYTE.SYS.

82EC495 - From the EF8290WB 486/386 PC/AT ChipSet

82EC798 - From the EC798 386/486 PC/AT Single Chip

1.2.7 ETEQ Microelectronics

The  following  chips  all  provide  192k  of memory in three 64k

blocks  between  C0000-EFFFF.   If you have a VGA or EGA adapter,

you  may  get  additional memory by using the MOVE=OVERLAY option

of LASTBYTE.SYS.

ET6000 - From the Cheetah 486DX/SX Non-Cache System

ET9000 - From the Jaguar 486DX/SX Cache System

ET9800/ET391SX - From the Firefox 386SX Write Back chipset

�������������������������������������������������Ŀ

� If you have a ET9800/ET391SX chipset and an     �

� adapter that occupies non-shadow ram address    �

� space between C800-CFFF, you will need to re-   �

� configure that adapter to use another location. �

���������������������������������������������������

The  following  chips  all  provide  192k of memory in twelve 16k

blocks  between  C0000-EFFFF.   If you have a VGA or EGA adapter,

you  may  get  additional memory by using the MOVE=OVERLAY option

of LASTBYTE.SYS.

ET82C391  - From the Bobcat 386DX/486DX chipset

ET82C491  - From the Cougar 386DX/486DX chipset

ET82C4901 - From the Bengal 386DX/486DX chipset

The   CHIPSET   program   cannot  distinguish  the  ET82C391  and

ET82C491; their corresponding PHYSICAL options are equivalent.

1.2.8 Forex Computer Corporation

The  following  chip  provides  256k  of memory in six 32k blocks

between C0000-EFFFF and one 64k block between F0000-FFFFF.

FRX36C300 - From the FRX36C300/200 Cache chipset

FRX46C411 - From the FRX46C411/412 Cache chipset

FRX46C421 - From the FRX46C421/422 Cache chipset

Copyright (C) 1990-92, Key Software Products. All Rights Reserved

Jan 31, 1993      SHADOW RAM MEMORY CONTROLLERS                 5

1.2.9 Headland Technology (G2)

The  following  chip provides 64k of memory by using the four 16k

blocks of the EMS page frame.

HT12 - Single 286 AT Chip

The  following  chips all provide 128k of memory by using the EMS

mapping area between C0000-DFFFF.

HT18  - 286/386SX Advanced Memory Manager Chip

HT21  - 286/386SX Advanced Memory Manager Chip

HT22  - 286/386SX Advanced Memory Manager Chip

HT103 - 286/386SX Advanced Memory Manager Chip

HT113 - 286/386SX Advanced Memory Manager Chip

The  CHIPSET  program cannot distinguish the HT18 and HT22; their

corresponding PHYSICAL options are equivalent.

1.2.10 Intel Corporation

The  following  chips provide 192k of memory in twelve 16k blocks

between  C0000-EFFFF.   If you have a VGA or EGA adapter, you may

get  additional  memory  by  using  the  MOVE=OVERLAY  option  of

LASTBYTE.SYS.

82343 - From the 82340SX Chip Set

82346 - From the 82340DX Chip Set

The  82343  is  functionally  identical  to  the  VLSI Technology

VL82C320,  and  the  82346 to the VLSI Technology VL82C330; these

corresponding PHYSICAL options are equivalent.

The  following  chip  provides  384k of memory in twenty-four 16k

blocks between A0000-FFFFF.

82359 - From the 82350DT EISA Chip Set

1.2.11 Mosel Corporation

The  following  chip  provides  256k of memory in four 16k blocks

between C0000-CFFFF and three 64k blocks between D0000-FFFFF.

MS400 - From the 486SX/DX Single Chip AT

Copyright (C) 1990-92, Key Software Products. All Rights Reserved

Jan 31, 1993      SHADOW RAM MEMORY CONTROLLERS                 6

1.2.12 Oak Technology

The  following  chips  all  provide 256k of memory, consisting of

four  32k  blocks between C0000-DFFFF, and two 64k blocks between

E0000-FFFFF.

OTI-021 - From the OTI-020 Desktop PC Chip Set

OTI-041 - From the OTI-040 OakNote Chip Set

The  CHIPSET  program cannot distinguish the OTI-021 and OTI-041;

their corresponding PHYSICAL options are equivalent.

1.2.13 OPTi Electronics

The  following  chips  all  provide  192k of memory in twelve 16k

blocks  between  C0000-EFFFF.   If you have a VGA or EGA adapter,

you  may  get  additional memory by using the MOVE=OVERLAY option

of LASTBYTE.SYS.

82C281 - From the Cache SX/AT chipset (w/Cache)

82C282 - From the Cache SX/AT chipset (w/o Cache)

82C283 - From the Block Interleave SX/AT chipset

82C382 - From the HiD/386 AT chipset

82C391 - From the 386WB PC/AT chipset

82C461 - From the Notebook PC/AT chipset

82C482 - From the HiB/486 AT chipset

82C491 - From the 486SXWB PC/AT chipset

82C493 - From the 486SXWB PC/AT chipset

82C495 - From the 486SXWB PC/AT chipset

82C495SX - From the LCWB PC/AT chipset

82C496A - From the DXBB PC/AT chipset (Rev. A)

82C498 - From the DXWB PC/AT chipset

82C499 - From the DXSC PC/AT Single Chip

The  CHIPSET  program  cannot  distinguish the 82C281 and 82C282,

and  the  82C281  is functionally identical to the Atmel AT40281;

these corresponding PHYSICAL options are equivalent.

The  82C283 is functionally identical to the Atmel AT40283; their

corresponding PHYSICAL options are equivalent.

The  82C391  is  functionally  identical to the Atmel AT40391 and

AddTech  Research's PCCHIP4; their corresponding PHYSICAL options

are equivalent.

The  CHIPSET  program  cannot distinguish the 82C491, 82C493, and

82C495,  and  the  82C493  is functionally identical to the Atmel

AT40493; these corresponding PHYSICAL options are equivalent.

The  following  chips provide 256k of memory in twelve 16k blocks

Copyright (C) 1990-92, Key Software Products. All Rights Reserved

Jan 31, 1993      SHADOW RAM MEMORY CONTROLLERS                 7

between C0000-EFFFF and one 64k block between F0000-FFFFF.

82C291 - From the SXWB PC/AT chipset

82C496B - From the DXBB PC/AT chipset (Rev. B)

The  following  chip  provides 256k of memory in eight 16k blocks

between C0000-DFFFF and two 64k blocks between E0000-FFFFF.

82C682 - From the 386/486WB EISA chipset

82C683 - From the 386/486AWB EISA chipset

1.2.14 PicoPower Technology

The  following  chips  all  provide  256k of memory in twelve 16k

blocks   between   C0000-EFFFF   and   one   64k   block  between

F0000-FFFFF.

PT86C168 - Evergreen Portable Computer Core Logic Chip

PT86C268 - Evergreen HV Core Logic Chip (Hybrid 3.3/5.0 volts)

1.2.15 Silicon Integrated Systems (SiS)

The  following chips all provide 192k of memory in six 32k blocks

between  C0000-EFFFF.   If you have a VGA or EGA adapter, you may

get  additional  memory  by  using  the  MOVE=OVERLAY  option  of

LASTBYTE.SYS.

85C360 - ISA 386DX Single Chip

85C401 - ISA 486DX/SX Chipset

85C411 - EISA 386/486 Chipset

85C460 - ISA 486DX/SX Single Chip

1.2.16 Sun Electronics (SUNTAC)

All  SunTac  chips  provide  128k  of  memory  in four 32k blocks

between  E0000-FFFFF.   An  additional  128k  in eight 16k blocks

between  C0000-DFFFF  is available at the expense of all Expanded

Memory  by adding the word NOEMS to the SUNTAC Physical option as

in: Physical=ST62C202,NOEMS.

ST62C202 - From the chipset 20 for 286

ST62C241 - From the chipset 24 for 386

ST62C251 - From the chipset 25 for 286 and 386sx

Copyright (C) 1990-92, Key Software Products. All Rights Reserved

Jan 31, 1993      SHADOW RAM MEMORY CONTROLLERS                 8

1.2.17 Symphony Laboratories

The  following  chips  provide 256k of memory in eight 16k blocks

between   C0000-DFFFF   and   another   two  64k  blocks  between

E0000-FFFFF.

SL82C361 - From the Haydn SL82C360 80386DX/SX chipset

SL82C461 - From the Haydn SL82C460 80486 chipset

SL82C471 - From the Mozart SL82C470 486/386 EISA chipset

1.2.18 Texas Instruments

The  following  chips  provide  384k of memory in twenty-four 16k

blocks between A0000-FFFFF.

TACT82411  - The Snake Single-Chip AT Controller

TACT82S411 - The Snake+ Single-Chip AT Controller

1.2.19 UNIchip Incorporated

The  following  chip  provides 192k of memory in three 64k blocks

between  C0000-EFFFF.   If you have a VGA or EGA adapter, you may

get  additional  memory  by  using  the  MOVE=OVERLAY  option  of

LASTBYTE.SYS.

U4800 - From the 386/486 AT Chip Set

1.2.20 United Microelectronics Corporation (UMC)

The  following  chip  provides  384k of memory in twenty-four 16k

blocks between A0000-FFFFF.

UM82C212 - From the UM82C210 386SX/286 AT Chip Set

The   UMC82C212  is  funcitonally  identical  to  the  Chips  and

Technologies  82C212;  their  corresponding  PHYSICAL options are

equivalent.

The  following  chip  provides 256k of memory in eight 16k blocks

between C0000-DFFFF and two 64k blocks between E0000-FFFFF.

UM82C481 - From the UM82C480 50Mhz 80486 Chip Set

Copyright (C) 1990-92, Key Software Products. All Rights Reserved

Jan 31, 1993      SHADOW RAM MEMORY CONTROLLERS                 9

1.2.21 VIA Technologies

The  following  chips  provide 256k of memory consisting of eight

16k  blocks  between  C0000-DFFFF  and  two  64k  blocks  between

E0000-FFFFF.

VT82C470 - From the Jupiter Chip Set (w/o cache)

VT82C475 - From the Jupiter Chip Set (w/cache)

VT82C495 - From the Venus Chip Set

1.2.22 VLSI Technology

The  following  chip  provides  384k  of memory in six 64k blocks

between A0000-FFFFF.

VL82C202 - PC/AT-Compatible High Speed Memory Controller

�������������������������������������������������Ŀ

� Pin 69 on th VL82C202 controls whether its      �

� shadow ram is enabled or relocated.  This pin   �

� is usually tied to a jumper on the motherboard  �

� which must be set properly to work.             �

���������������������������������������������������

The  following  chips  provide  384k of memory in four 32k blocks

between  A0000-BFFFF,  eight  16k blocks between C0000-DFFFF, and

four 32k blocks between E0000-FFFFF.

VL82C310 - From the SCAMP 286/386sx Chip Set

VL82C311 - From the SCAMP 286/386sx Chip Set

VL82C315 - The SCAMP II PC/AT-Compatible System Controller

VL82C316 - The SCAMP II Chip Set for Power-Managed Portables

The   CHIPSET   program   cannot  distinguish  the  VL82C315  and

VL82C316; their corresponding PHYSICAL options are equivalent.

The  following  chips provide 192k of memory in twelve 16k blocks

between  C0000-EFFFF.   If you have a VGA or EGA adapter, you may

get  additional  memory  by  using  the  MOVE=OVERLAY  option  of

LASTBYTE.SYS.

VL82C320 - From the TOPCAT 286/386sx Chip Set

VL82C330 - From the TOPCAT 386 DX Chip Set

The  VL82C320  is  functionally identical to the Intel 82343, and

the  VL82C330  to  the Intel 82346 ; their corresponding PHYSICAL

options are equivalent.

The  following  chip  provides  384k of memory in twenty-four 16k

blocks between A0000-FFFFF.

Copyright (C) 1990-92, Key Software Products. All Rights Reserved

Jan 31, 1993      SHADOW RAM MEMORY CONTROLLERS                10

VL82C486 - Single Chip, High Performance 486 PC/AT Controller

1.3 Unsupported Chipsets

If  your computer has shadow ram capability, then it probably has

a  shadow ram memory controller.  However, some of these chipsets

cannot  be  supported by The Last Byte Memory Manager for reasons

shown below.

Some  of  these  chips, however, do have hardware EMS support and

thus  many  of  their motherboards come with an EMS device driver

designed  specifically  to take advantage of this capability.  So

in  many  cases,  you can install this driver first, and then The

Last  Byte  Memory  Manager can use it to manage the upper memory

area.

1.3.1 No Shadow Ram Support

Some   of   the   older   chipsets  do  not  provide  shadow  ram

capability.

Examples: VLSI Technologies 82C100 series

Western Digital chipsets

Texas Instruments TACT82300 series

Early Suntac chipsets (ST62BC001-6)

United Microelectronics Corp. UM82C231

1.3.2 Shadow Ram is not Read/Write

Some  chipsets  have  shadow ram, but can only put it into either

write-only  mode  (for copying rom contents to ram), or read-only

mode  (to  prevent accidental writes into what's supposed to be a

copy  of  rom).  The hardware EMS in some such chipsets sometimes

provides  a limited amount (e.g., 64k) of Hi-DOS memory, but only

if the user doesn't need expanded memory.

Examples: CONTAQ 82C591 (no hardware EMS support)

Headland/G2 (many do have hardware EMS however)

ACC Microelectronics (many do have hardware EMS)

USA Integration (USAi) (no hardware EMS)

1.3.3 Overly Complex Control

The  Last Byte Memory Manager software is table-driven, making it

relatively   easy   to   adapt  to  different  memory  controller

architectures, but there are limits!

Copyright (C) 1990-92, Key Software Products. All Rights Reserved

Jan 31, 1993      SHADOW RAM MEMORY CONTROLLERS                11

Examples: Zymos/Appian System 90 chipset

United Microelectronics Corp UM82C384, '388, '389

Forex Computer Corp. FRX36C100

Silicon Integrated System (SiS) 85C310.

1.3.4 Locked Configuration Registers

Some  memory  controllers  are  designed  so  that  they  can  be

"locked"  at the end of the main bios initialization, requiring a

hardware  reset  to  unlock,  and  thus  preventing The Last Byte

Memory Manager from accessing the controller.

Examples: Intel 82335, 82335SX

1.3.5 Configuration I/O Ports Reassigned

Some  memory  controllers  are  designed so that the "index/data"

pair  of I/O ports used to access the configuration registers can

be  reassigned  to  different  I/O  addresses.   This  makes  the

configuration  registers  effectively  inaccessible  to  The Last

Byte  Memory  Manager since it has no way of knowing what new I/O

port addresses may have been established within the main bios.

Examples: Texas Instruments TACT84500

GS Technology GS62CS03

1.4 Checking Compatibility Using CHIPSET

CHIPSET  is a program that attempts to determine if your computer

has  one  of the memory controllers required to use The Last Byte

Memory  Manager.   To  run  CHIPSET, simply enter its name on the

command line:

A>CHIPSET

and  follow  the  prompts.   CHIPSET  will  try  to  identify the

particular  memory controller you have, if any.  If it can't find

a  memory  controller, it will look for fixed memory in the upper

area  or  expanded  memory as a means of installing The Last Byte

Memory  Manager.   If  it  is  successful,  it  will give you the

option  of  automatically linking to the INSTALL program to setup

for use of The Last Byte Memory Manager.

CHIPSET   automatically  senses  whether  you  have  a  color  or

monochrome  display  to  determine  how  to  present  data on the

screen.   However,  you  may  force  the mode by using one of the

command  line  options:  /MONO  or  /COLOR (for monochrome liquid

Copyright (C) 1990-92, Key Software Products. All Rights Reserved

Jan 31, 1993      SHADOW RAM MEMORY CONTROLLERS                12

crystal displays).

If  your  computer  has  one of the listed devices, then The Last

Byte Memory Manager should work on your computer.

Copyright (C) 1990-92, Key Software Products. All Rights Reserved