Messaging Protocol for Amateur Radio MailBox Systems.

December 2, 1992

Version 0.1

A.  Overall message structure.

1.  A message consists of a header followed by a body.	The body is separated

from the header by a single blank line.  The header contains no blank lines.

2.  Each system that relays a message appends it's header line to the existing

header lines within the message header.  The header line takes the form

R:yymmdd/hhmm nnn@bbs.hloc

yymmdd/hhmm is the GMT time that system received the message.

nnn is the message number at that system, from 1 to 65535 inclusive.

bbs is the callsign of the system, six characters maximum.

hloc is the hierarchical location of the system, 31 characters maximum,

as defined by the document "INTERNATIONAL ROUTING DESIGNATORS" ARRL Networking

Conference Procedings xxx.

3.  A message body consists of any number of body lines.  The message body

is terminated by either; a) a line containing the text "/EX" in any case,

or b) control-Z.  If the control-Z is not the first character of the line,

it is replaced by end-of-line.

4.  A body line consists of any number of allowed characters, terminated by

end-of-line.

5.  End-of-line is any of; a) CR (0x0d), b) LF (0x0a), or c) CRLF (0x0a0d).

6.  Allowed characters are all 8 bit characters except NUL (0x00),

control-C (0x03), control-Z (0x1a), and end-of-line.

B.  Message address.

1.  The message address takes the form to@bbs.hloc where:

to contains up to six allowed characters,

bbs contains up to six allowed characters,

hloc contains up to 31 allowed characters,

as defined by the document "INTERNATIONAL ROUTING DESIGNATORS" ARRL Networking

Conference Procedings xxx.

The use and meaning of the message address differs by message type.

C.  Message subject.

1.  The message subject consists of less than 80 allowed characters.

2.  No system may modify the message subject.

D.  Message types.

1.  Personal.

2.  NTS traffic.

3.  Bulletins.

E.  Message identification.

1.  A message may reside on more than one system as it is forwarded through

the network or as it is distributed.  Each copy of a given message is known

as a message instance.	Messages are identified in two ways: the initial

instance of the message at the system of entry, and the particular instance

of that same message at any other system.  Messages are uniquely identified by

the message number and callsign of the system into which they are originally

entered.  Each instance of the same message is identified by the message number

and callsign at the system of interest, where the instance of the message

resides.

F.  Bulletin Identifier (BID).

1. A "BID" (Bulletin IDentifier) must be associated with each Bulletin, and

may be associated with with any Personal message.

2. If a BID is not given explicitly when the message is entered into the system,

and the message type requires a BID, one is created automatically from the

message number and callsign of the system into which the message was initially

entered.  An automatically created BID has the form nnn_bbs, where nnn is the

message number at the originating system, and bbs is the callsign of the

originating system.

3. The BID must always be sent with the mesage, and may not be changed

by any system.

4. For Bulletins, a the message always has a BID.

5. For Personal messages, the message has a BID, only if the sender specifies one,

or the message has a distribution list.

6. For NTS messages, there is never a BID.

7. If the user enters a message with BID specified, the message is assigned

that BID.  If specified BID has been seen before, the message is rejected and

the text "NO - Duplicate BID" is displayed.

8. If the user enteres the message with BID specified as "$", the message is

assigned a unique system generated BID.  This BID has the form nnn_bbs where

nnn is the message number and bbs is the callsign of the system.

9. If the message is addressed to a distribution list, and the user does not

specify a BID at message entry, a unique system generated BID is assigned, as

above.

10. Implications of the BID rules:

10a. A Personal message entered into the system without a user supplied BID,

and without a distribution list may loop within the network.

10b. A Personal message which was entered with a user supplied BID will be

rejected when it is forwarded back to any system it previously passed through.

10c. Messages of type Bulletin or Personal may have a distribution list,

messages of type NTS may not.

10d. There will be no attempt to pass a Personal message which has a BID

back to the station that sent it to you.

G.  Forwarding.

1.  Connecting to the target system.

2.  System Identifier.

3.  Reverse forwarding.

4.  Refusal of messages by the target system.