SLOW SCAN TV

using the

SOUND BLASTER

This program is designed to display 8 second BLACK & WHITE slow scan

pictures that you can copy off the ham radio bands. The frequencies

that you will normally find slow scan are:

3.845  MHZ

7.171  MHZ

14.230 MHZ

21.340 MHZ

28.680 MHZ

The most popular frequency is on 20 meters at 14.230 MHZ, with 14.233 MHZ

being used on weekends when there are too many on 14.230. The audio from

your reciever needs to be connected to the microphone input on the Sound

Blaster. I did the development on the Sound Blaster Pro, but I kept the

sample rate at 12 KHZ (the top speed for the Sound Blaster) so all Sound

Blasters could be used. It seems to give acceptable results at this speed,

however, a faster sample rate would give even better pictures. In a future

version, if there is interest, I may make a version with extra options

for the Pro version.

Copy the program to your hard disk (I would make a seperate subdirectory

called ss). You need all the BGI files for the program to run. Your

computer should have several meg of free space as the .VOC files will

take up about 100k for every 8 seconds of recording. Also you will want to

save some of your favorite pictures in .PCX files so you can recall them

later. The reason I chose .PCX format to save is that it generally takes

a lot less space than .VOC files. The display on your computer needs to

be a VGA display that will work in 640 x 480 - 256 color mode. Some

VGA cards do not have enough memory to do this. If it doesn't work on

your system, I need to know so I can make the necessary changes. When

reporting problems, please make sure to include as much information

about your computer system as possible, so I can be of help. To start the

program after it is loaded type "SS".

Once you have the audio connected, and a station ready to transmit, pressing

F2 will start the recieving process. The sound is stored in a disk file

named "temp.voc". This file is overwritten every time you press F2. You

will not see anything happen until you press ESC which should be pressed

as soon as the audio for the picture stops. The "temp.voc" file will be

a minimum of 100k in size for one 8 second period, so you will need to

make sure that you have at least 500k free on your hard disk where ss.exe

is loaded (several megabytes would be even better). You will learn after

using the program that it takes a little bit before it starts recording,

so you need to anticipate the start of the picture just a little.

If you anticipate too much, you can do a couple of different things:

1. Press ESC will stop the recording & start to display whatever there

was to that point - Pressing ESC again will stop the display -

Pressing F2 will restart the recording.

2. If, after you are displaying the picture, you see that the top of

the picture is part way down on the screen,,pressing the space bar

will make the rest of the picture restart from the top so you will

see a complete frame.

Note: I know that it is pretty slow by not copying the picture to the

screen as it is comming in, but that can be changed in a future version.

Once you have a picture on the screen, if you Press F2 again you will

loose the one you have. Menu item F7 allows you to save the picture

as a .PCX file, or you can rename the "temp.voc" file to another name

so when you press F2 again, you do not loose the picture. I have, just

for the fun of it, looked at the .VOC file & edited them in VEDIT which

comes with the Sound Blaster.

Even though I wrote the program code for 8 second Black & White, I have

been able to recieve 36 second, 72 second, & SCOTTY 1 in Black & White.

A portion of the picture gets thrown away (you only see the left hand

upper corner) but, even I was quite suprised when I saw how well it did.

Help is built into the program and accessed by pressing F1. This will

give you a second menu of help items. Just press the F key that will give

you the information that you want.

Test images are built in, but, other than nice to look at, until there

is a send program written, are of little use.

The CONTRAST, SYNC, and BLACK can be adjusted if desired. This is left

over from my development stages & was left in. It may give you more

insight into how a Slow Scan picture works by playing with these. Once

you have recorded & have a "temp.voc" file, you can use this file under

the F7 option to redisplay as many times as you like using the SYNC,

CONTRAST, and BLACK. For those not familiar with Slow Scan, there are

many different type & modes. An excellant discussion is in the January

1993 issue of QST.

I hope this program will give you some enjoyment. If it does, I would

also hope that you will register the program for $20.00. I have spent

over 2 years writing this (not every day mind you) & it was an exersize

for me in learning to program in "C". Well, I guess something must

have worked! Anyway, if I get enough response, I will get to work

& finish the program with the other modes & a send routine. Also

I am interested in any ideas you may have to enhance the program or

ideas for other programs for the Sound Blaster. I bought their

development kit, & might as well put it to work. So please send

your $20.00 & ideas for enhancement & other programs you would

like to see (how about copying RTTY or PACKET RADIO with the Sound

Blaster ???). Also, anyone sending in the registration with $20.00

will recieve one free upgrade.

Comments or questions can be left for me on Compuserve. Make sure

that you address to Gene Harlan  76645,2455.

Questions can be answered evenings at (815) 398-2683. I'm usually

home by 6:00 pm unless I have meetings, ect. Compuserve is better!

Please send the following registration form to:

Gene Harlan - WB9MMM

5931 Alma Dr.

Rockford, Illinois 61108

Registration Information

______________________________________________________________________

I have enclosed $20.00 & would like to register Slow Scan program.

Name___________________________________ Callsign______________________

Street_________________________________

City___________________________________ State __________ Zip__________

Type/speed of computer __________________________________

Size of hard drive ______________________________________

Type of display card ____________________________________

Ram - how much do you have ______________________________

Sound Blaster _____ Sound Blaster Pro _____ SB20 _____

3 1/2 disk _____  5 1/4 disk _____

Other information - ideas - suggestions

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________

______________________________________________________________________