Voice              B.J. Guillot            Data/Fax

713.893.9320      2611 Rushwood Circle      713.893.9124

Houston Texas 77067-1941

Copyright (C) 1993 B.J. Guillot.  All Rights Reserved.

=============================================================

BGFAX 1.0 Beta 2                                THU 22 JUL 93

=============================================================

Dedicated to Gene Roddenberry

1921-1991

-------------------------------------------------------------

ABSTRACT

-------------------------------------------------------------

BGFAX allows SYSOPs to accept faxes on their BBS line.

BGFAX is a tool designed for use with *ANY* Class 2 compliant fax

modem.  The fax modem must also support fully functional ADAPTIVE

ANSWERING.  Common Class 2 fax modems include the Supra, Practical

Peripherials, Zoom, ZyXEL, etc.

BGFAX will *NOT* function with Class 1 *ONLY* fax modems.  Common

Class 1 *ONLY* fax modems include *ALL* USR fax modems, Digicom

Scouts, and the AT&T Dataport.

BGFAX can be used in two distinct modes.

FRONTEND MODE ... BGFAX will answer the telephone line ITSELF,

handle events, pass data calls to a front end

compatible BBS, and take fax calls.

REAR END MODE ... BGFAX will be called from another program when

an incoming fax is detected.  FidoNet sysops

using front end mailers such as Front Door,

Intermail, D'Bridge, Dutchie, etc. are required

to use this mode.  The mailer must support the

ability to exit to BGFAX when a fax is coming.

-------------------------------------------------------------

INCLUDED FILES

-------------------------------------------------------------

1.    BGFAX.DOC -- general documentation

2.    BGFAX.TXT -- beta version release notes

3.    BGFAX.HLP -- specialized help for various setups

4.    BGFAX.EXE -- the main executable

5. REGISTER.FRM -- registration form

-------------------------------------------------------------

DISCLAIMER

-------------------------------------------------------------

BGFAX and accompanying files are provided AS IS without any

warranty, expressed or implied.  This includes without limitation

the fitfulness for a particular purpose or application and any

warranties of merchantability.

While I tried to be as through as possible in preparing BGFAX,

B.J. Guillot shall not be liable for any damages, whether direct,

indirect, special, incidental or consequential arising from a

failure of this program or accompanying files to operate in a

manner desired by the user.  B.J. Guillot shall not be liable for

any damage to data or property which may by caused directly or

indirectly by use of this program.

In no event will B.J. Guillot be liable to you for any damages,

including any lost profits, lost savings or other incidental or

consequential damages arising out of your use or inability to use

the program, or for any claim by any other party.  Individuals using

the software bear all risk as to its quality and performance.

-------------------------------------------------------------

CONTACTING ME

-------------------------------------------------------------

FidoNet:  1:106/400                   Voice: +1 713.893.9320

InterNet:  st1r8@jetson.uh.edu      Data/Fax: +1 713.893.9124

The latest versions of BGFAX can be obtained by Fido FREQ at the

above Fido node number with the magic name "BGFAX" and can be

obtained by anonymous FTP for InterNet users at the site

"uhdux2.dt.uh.edu" in the "/incoming" directory.

-------------------------------------------------------------

MISCELLANEOUS NOTES

-------------------------------------------------------------

BGFAX takes the fax call itself.  It does *NOT* require any special

TSR drivers to be used and does *NOT* pass control to another fax

program to receive the fax.

...HOWEVER... You *WILL* need some type of external fax program to

VIEW and PRINT the received fax.  BGFAX currently supports two

popular fax formats.  Smith Micro Software's Quick Link II, which

is included with several faxmodems, and ZyXEL's ZFAX software.

(ZFAX will not function on a normal Class 2 faxmodem, but it will

allow you to view and print BGFAX faxes stored in ZFAX format.)

If anyone can get me the formats for other popular packages, I

will gladly add them to BGFAX if its not too much trouble.  A

number of people have requested WinFax format, but I can't find

any information on Delrina's WinFax file format.

PLEASE NOTE: BGFAX does not need nor use a FOSSIL driver.  BGFAX

has internal communication support for all UART types including

the 16550A FIFO chip.  BGFAX assumes ...

COM1=3F8x4     COM2=2F8x3     COM3=3E8x4     COM4=2E8x3

If you are using a nonstandard port configuration, whenever BGFAX

requires a port number to be entered (such as po=2), instead of

entering the port number, enter the base address and IRQ seperated

by an 'x' character (such as po=2E8x5).  BGFAX requires the base

address be given in hexidecimal notation.

BGFAX supports Desqview's alternate video buffer zone, if used, and

will return ticks to other tasks.

This program was compiled under Borland's Turbo Pascal 7.0.

I do all testing with a Supra Fax Modem v.32bis external with

V1.7-B04 firmware and Frontdoor 2.02/NC running on a 386DX/40.

I also routinely test BGFAX with a generic no-namer 2400 data,

9600 send/receive fax modem from Computer City running on the

ever-so-slow 286/8.

-------------------------------------------------------------

QUICK TUTORIAL ON SOME FAX CLASS 2 COMMANDS

-------------------------------------------------------------

AT+FCLASS=?

===========

This command will allow you to see if your fax modem supports

the "unofficial" Fax Class 2 standard.  The modem should report

back a string similar to:

0,1,2

This indicates the modes the modem is capable of opering in.  0

means data, 1 means "official" Class 1, and 2 means "unofficial"

Class 2.  (3 may indicate "official" Class 2.0, which is not

necessiarly compatible with BGFAX.)  If the modem response if

"ERROR" this means your modem does not have any CCITT standard fax

capabilities.

The above command is explained only for people's curiosity.  BGFAX

should always be used in data mode.  If you issue an "AT+FCLASS=2"

command, you may disable adaptive answer and cause your modem to

attempt a fax connect on *ALL* incoming calls.

AGAIN, do *NOT* use the "AT+FCLASS=2" command.

GROUP III and CLASS 2

=====================

Many people are often confused between the terminology "GROUP" and

"CLASS" regarding fax modems.

ALL FAX EQUIPMENT is "GROUP 3" compatible.  This includes fax

machines, fax modems, standalone PC fax cards, etc.  Group 3 just

defines the fax specifications, e.g., black-and-white, the

resolutions, etc.  When Group 4 fax machines are available to

consumers, they will offer color fax capabilities, but are fax

modems will still only do black-and-white.

"CLASS" refers only to fax modems.  CLASS has NOTHING AT ALL to do

with fax machines.  Class 1, Class 2 and Class 2.0 are different

methods of sending commands to a fax modem from a computer serial

port.  Many fax modems offer the Class 2 command set.

AT+FAA=1

========

This command will put your modem in ADAPTIVE ANSWERING mode.  This

means your modem will properly mate with either a standard modem

data call or a fax machine/fax modem.  Some modems, such as the

Supra, will report a "FAX" response the instant a fax CNG tone is

heard from the remote end.  Some modems are brain dead and will

report "FAX" whenever an "ATA" command is issued to the modem

regardless of whether the incoming fax is data or fax.

All modems, on true fax calls, will report a "+FCON" response code

when the fax connection is truly established.  (This is similar to

a "CONNECT 14400" on a data call.)

Many modems are slightly brain dead and will forget the +FAA

setting when another command is issued to the modem.  To ensure

you always answer in adaptive mode, rather than using "ATA" to

answer the phone, use the following: "AT+FAA=1;A".

The ";", semicolon, is used as a seperator for fax commands.  Some

modems, such as the Supra, do not require it.  Others, such as the

PPI, will "ERROR" out if the semicolons are not used.

A final note about adaptive answering: IT IS NOT 100% ACCURATE.

Every now and then, your modem may get confused and think a data

call is a fax call.  There is nothing I can do about this as it is

most likely a firmware problem with the modem.  Some modems are

very brain dead and allow low speed data callers to call the BBS,

but all high speed callers are treated like they are fax machines.

(That is NOT GOOD.)  The only way to cure this is get a ROM chip

upgrade to the modem, if a newer version exists.  You'll just have

to experiment and make sure your adaptive answer works about

95% of the time, or whatever you decide is good.

The Supra Fax Modem, for example, is pretty good concerning

adaptive answering.  (Note that very earlier versions of the

Supra, did not offer adaptive answering until the 1.2-H and 1.2-J

ROMs appeared on the scene.)  However, many older fax machines do

not send out a CNG tone when they are engaged.  Many fax modems,

including the Supra, won't know what to do, and will assume the

old fax machine is really a data caller.  Again, this is not good,

but its not that much of a problem.  Tell the operator of the old

fax machine to push their "START" button *IMMEDIATELY* after dialing

the last digit of the telephone number instead of waiting to hear

the fax tones on our end like many office people do.

AT+FCR=1

========

As silly as this command may sound, it gives permission to your

fax modem the ability to accept fax calls.  The fax modems default

to *NOT* allow incoming fax calls (EVEN IF ADAPTIVE ANSWERING IS

ENABLED!)  So, just make sure +FCR is always equal to 1.

AT+FLID="TRANQUILITY BASE"

==========================

The above command sets your modems fax ID string.  This is the

string that is send to the remote end on fax connects.  (It's the

deal that's printed on the little 'confirmation reports' that many

fax machines make.)  The fax ID can ONLY BE A MAXIMUM OF *20*

CHARACTERS.  If you try to stuff more than 20 characters in a fax

ID string, the modem will respond with an "ERROR".  DO NOT USE

HIGH-BIT ASCII CHARACTERS IN THIS STRING.  Use only simple

letters, numbers, and punctuation marks.  (High-bit ASCII

characters include foreign accented letters, inverted question

marks, etc.)

AT+FDCC=1,5,0,2,0,0,0,0

=======================

The above command is what the power on defaults to a 14400 fax

modem are, with one exception.  The first number, 1, allows your

fax modem to accept high resolution fax documents.  The power on

default to most modems are to accept only low resolution

documents.  The second number, 5, allows your fax modem to use fax

speeds up to 14400 bps.

If you issue the above command on a fax modem with an upper limit

of 9600 on fax tranmissions, you will receive an "ERROR" or cause

the modem to behave odd.  Use this string on a 9600 fax modem:

AT+FDCC=1,3,0,2,0,0,0,0

The remaining numbers are not that important.  Changing them

around, though, may cause BGFAX to fail.  The command can also be

abbreviated to "AT+FDCC=1,5" or "AT+FDCC=1,3".

STACKING

========

A single command string can be made using what we need ...

AT+FAA=1;+FCR=1;+FLID="BGFAX IS GREAT";+FDCC=1,5,0,2,0,0,0,0

However, if you are using BGFAX in rear end mode with a Fido

mailer, you may have a hard time of making an initialization

string that long, so you may be required to split the line into a

few pieces.  Remeber to make your ATA command "AT+FAA=1;A" if it

is possible to change in your mailer.

FAX MODE

========

Probably 95% of you will be using BGFAX with a high speed modem

(v.32 or v.32bis).  Many of you may have started off with slower

300, 1200, or 2400 modems from long ago.  If you can remember back

that far, when an incoming data call came in, you would see a

string like "CONNECT 1200".  If your port was set at 2400, you

would garbage on the screen unless you shifted your bps rate down

to 1200 to match that of the caller.  Many newer terminal programs

refer to this speed shift as "Autobaud".

Modern modems require your serial port be locked at a specific

speed (such as 19200, 38400, 57600, etc.) so that you can achieve

higher thruput via compression engines such as MNP and v.42bis.

The modem-to-modem speed may only be 14400, but the modem-to-

computer speed stays "locked" at 38400.  Autobaud is not required,

and if used, would result is garbage on the screen.

Why am I telling you all this?  Well, I don't know who's silly

idea it was, but the powers that be decided that fax modems should

use the ancient method of changing the port rate when a fax is

incoming or outgoing.

After your modem reports the result code "+FCON" all further

communications *MUST* be done at 19200 bps, even if you are locked

at 38400 or 57600.  If the port speed is not relocked at 19200,

nothing but garbage will come on the screen.

I'm telling you this because I frequently get mail asking me why

I'm changing the port speed to 19200.  It's not my fault!  That's

the way is has to be done!  The only modem that I know that does

not require this idiotic speed shift is the ZyXEL modem.  In fact,

a special option must be used to tell BGFAX you are using a ZyXEL

or BGFAX will fail as it will try talking to the ZyXEL modem at

19200 instead of 57600 or whatever rate it's locked at.

-------------------------------------------------------------

FRONTEND MODE

-------------------------------------------------------------

If you plan on using BGFAX in Frontend mode, you need to do a

couple of things first.

1. An evironment variable called BGFAX needs to be inserted in

your AUTOEXEC.BAT or someother batch file.  Example:

SET BGFAX=C:\BGFAX

This environment variable points to the location of your BGFAX

configuration file and the directory where faxes will be

received into.

2. Make sure the BGFAX.CNF configuration file is in the directory

pointed to by the BGFAX environment variable and make any

changes you think necessary.  The included configuration file

is self explantory.  I won't repeat the format here.

3. BGFAX can exit with 4 possible errorlevels:

4 - fax was received

3 - local logon

2 - data call

1 - restart batch file

If no errorlevel is encountered, it means the sysop aborted

BGFAX by pressing the [Esc] key or a serious error occurred.

4. When BGFAX is waiting for a call, pressing Ctrl-R will reset

the on screen counters.  Ctrl-A will force BGFAX to send the

answer string to the modem.

5. When a data call comes in, BGFAX creates a DOBBS.BAT file that

looks like so:

C:\BGFAX\EXEBBS.BAT 9600 1 528 /ARQ

The first parameter is the DCE data speed, then the com port,

number of minutes until the next event, and then the error

correcting string, if any.  The EXEBBS.BAT file must be created

by you so that your bulletin board software is loaded up in

frontend mode (ie., the data connection is already hot.)  When

you speed to specify the DCE rate in your EXEBBS.BAT file use

the variable %1 and the com port is %2, etc.

-------------------------------------------------------------

REAR END MODE (FIDO MAILERS, ETC.)

-------------------------------------------------------------

Rear end mode is useful for those of us who are FidoNet members

where a Fido mailer is required to be running 24 hours a day.  It

will be your job to figure out how to get your mailer to exit to

BGFAX when your modem informs the mailer a fax call is coming

through.  (See the BGFAX.HLP file for hints on specific mailers.)

First of all, though, you need to discover what string your modem

sends when it has reliably mated with an incoming fax.  My Supra

Fax Modem for example returns "FAX".  All modems eventually report

"+FCON".  In my opinion, it is better to trigger your mailer to

exit on "FAX" *IF* your faxmodem supports it.  I've heard about

some brain dead modems to report "FAX" whenever an "ATA" is sent

to the modem.  If your modem does that, do *NOT* use "FAX" as the

trigger string in your mailer.

Why is it better to use "FAX" as a trigger string?  My Supra

reports "FAX" as soon as it hears the CNG tone from the remote fax

device.  It takes about 1.5 seconds or so for the fax machine and

my Supra to mate, which gives BGFAX plenty and plenty of time to

load up and take the fax, even on slow computers.  (It should be

noted that your mailer must exit *IMMEDIATELY* upon receipt of the

fax trigger string, because after those 1.5 seconds, if the remote

fax machine cannot mate, it will hangup!)

For those whose modems do not report "FAX", you will have to use

the "+FCON" string as the trigger string.  One downside to this is

that it makes timing much more critical and you will probably miss

your chance of obtaining the remote fax ID string.

Your mailer should run BGFAX as soon as the trigger string comes

in the port.  It should call BGFAX in the following manner:

BGFAX /how faxpath port faxtype

FAXPATH will be the directory where incoming faxes are to be

placed.  PORT is a valid communications port from 1 to 4, or,

optionally, a hexidecial base address and IRQ formatted as

followed "3F8x5".  Also, optionally, if the port has a bang, !,

following it, it will force the port to stay locked at the current

speed rather than shifting to 19200 on fax connects.  (THIS '!' IS

TO BE USED ONLY FOR ZYXEL MODEMS, OR FOR PEOPLE THAT REALLY KNOW

WHAT THEY ARE DOING.)  FAXTYPE is a single character 'Z' or 'Q'.

'Z' for ZyXEL ZFAX format, 'Q' for Smith Micro Software's Quick

Link II fax format.

The /how deal is the most critical switch.  There are three

possible settings.

/FAX   ... Use this on modems like the Supra where the "FAX"

response is generated by the modem, otherwise, you

will have to use one of the following:

/FCON  ... If your modem reports +FCON only, try this option

first.  If it doesn't seem to work, try ...

/FAST  ... what is the difference between these last two?

If you use the /FCON switch, BGFAX will assume the fax machine

has already fully mated with your fax modem, and will BLINDLY tell

the remote fax machine to start sending a page.  HOWEVER, if the

machines have not fully mated, this will distrub the mating

process and make both ends hangup.

If you use the /FAST switch, BGFAX will assume the fax machine is

still in the process of mating with your modem and will wait for

the "OK" signal before telling it to start sending the page.  The

/FAST switch may be necessary on very fast computers, or just

depending upon what type of modem is being used.

Consider these situations ... (My Supra exits with the "FAX"

result, but for testing purposes, I ignored it to show you an

example of how the /FCON and /FAST switches work.  In reality, I

*SHOULD* use the /FAX option as it works more reliable, but this

is for example purposes only) ...

a. Running FD 2.02/NC with a 386DX/40 and my external modem, I

have to use the /FAST option or BGFAX will hang up right away

when it is called.

b. Running FD 2.02/NC with a 286/8 and my external modem, I have

to use the /FCON option or BGFAX will sit there and eventually

time out after 40 seconds.

c. Running FD 2.02/NC with the same 286/8 and my INTERNAL 2400

data, 9600 send/receive fax modem, I must use the /FAST option

or BGFAX will sit there and timeout.

-------------------------------------------------------------

AFTER YOU RECEIVE THE FAX

-------------------------------------------------------------

NAMING CONVENTION

=================

The name of the fax file will be FAXnnnn.xFX where "nnnn" is a

number that is padded with zeros and "x" is the letter "Z" or "Q"

depending upon which fax format you save the file in.  If you ever

see a BGFAX.TMP file in your directory, it should *NOT* be there,

as it should have been renamed in the FAXnnnn.xFX format.  If it

is there, it means some kind of problem happened during the fax

reception.

FAXIN.LOG

=========

BGFAX creates a BGFAX.LOG file that contains a bunch of

information that is mainly only for debug purposes.  A much

cleaner log file is called FAXIN.LOG and looks like this:

Date  Time  ET    Bytes   Rate  Filename     Remote Fax ID        Pgs  Notes

-------------------------------------------------------------------------------

07-22 11:52 00:42   18183  9600*FAX0001.ZFX  Not Available          2  Finished

07-22 18:04 00:38   18182  9600 FAX0002.ZFX  Not Available          2  Not Done

07-22 21:30 00:29   17793 14400*FAX0003.ZFX  7138939124             1  Finished

07-22 21:42 00:29   17541 14400 FAX0004.ZFX  TRANQUILITY BASE       1  Finished

07-22 21:44 00:32   16941  9600*FAX0005.ZFX  Null ID Received       1  Finished

07-22 22:21 01:22  107392 14400*FAX0006.ZFX  7132424708             1  Finished

The asterick between the fax DCE rate and the filename indicates

the fax was marked as high resolution.  "Not Done" under Notes

means that BGFAX believed more pages were to have followed, but

were not actually received.  "ET" is estimated time.  Everything

else is self explanatory.

DOFAX.BAT

=========

After you successfully receive a fax, BGFAX will exit with an

errorlevel of 4 and create a DOFAX.BAT file.  Example:

C:\BGFAX\EXEFAX.BAT C:\BGFAX\FAX0086.ZFX

You do not have to do anything with this at all, but it is

provided for those of you who wish to print faxes as they are

received.  It will call the EXEFAX.BAT file and pass it the

parameter with the path and name of the fax file that was just

received.  If you are using Smith Micro Software's Quick Link II,

for example, and are one of those people that leave your printer

on 24 hours a day, your EXEFAX.BAT file might contain this:

@echo off

printfax %bgfax%\%1 24pin.qlp

-------------------------------------------------------------

ACKNOWLEDGEMENTS

-------------------------------------------------------------

I would like to thank the following people for their assistance

during the beta test phase.

Bill Huther

Brian Wood

Russell Kroll

Ethan Brofman

Ed Lucas

...and many people from the InterNet.

-------------------------------------------------------------

SHAREWARE

-------------------------------------------------------------

THIS PROGRAM IS SHAREWARE.

If you use this program for more than four weeks and it works to

your satisfaction, it is *HIGHLY* recommended you register the

program at its low cost of only US $25.  (I have to pay off the

new computer I just bought to replace the one that crashed!)

Registrations encourage frequent updates.  If you choose to

register after the trial time period, please fill out the included

REGISTER.FRM file and send it along with your check, money order,

VISA, MASTERCARD, DISCOVER, or AMERICAN EXPRESS card number.

-------------------------------------------------------------

Regards,

B.J. Guillot