Using LENSCAD:

LENSCAD is a stand alone program which will calculate the focal

lengths of a lens system with up to three variable parameters and

three wavelengths of light.  The only limit to the number of elements

is the memory capacity of your computer and the stack size, which has

been set at 3000.

To start LENSCAD type lc <enter>.  You will be asked if you want to

use your old data.  If you enter 'n' you will be asked how many

elements in the system you want to design.  If at any time you need to

change the number of elements you must restart the program.  Once you

tell the program how many elements in your design you will be asked

how many wavelengths of light you want to test the system for and you

will enter those wavelengths.  The red, yellow and blue menu choices

enter hydrogen red, sodium yellow, and hydrogen blue spectral line

wavelengths, respectively.  You will then be in the main menu. Select

the parameters to enter for the system.  You will be prompted for

element and surface numbers, etc.

CONVENTIONS:

The element the ray goes through first is element #1 and the first

surface the ray passes through is surface #1.  The space between

elements is identified by the greatest element number of the elements

adjoining the space. Thus the greatest number of the elements

adjoining the space between elements #1 and #2 is #2.  Rays are

considered to be coming from the left.  The radius of curvature of an

element is negative if the center of the curve is to the left of the

element.  Thus a biconvex lens will have a positive radius of

curvature for surface #1 and a negative radius of curvature for

surface #2.

No space between lenses may be set at zero.  For elements in contact

set the space to a very small number such as .000001. Note that mks

units are used for all entries (distances in meters).  For a plane

surface where the radius of curvature is effectively infinity, use a

large number such as 1000000000 for the radius of curvature.

Any of the three parameters curvature, distance, and element thickness

may be made a variable.  You name which curvature, etc. you want to

vary, and give the range.  Depending on the number of variables, the

program will calculate over either 11 or 101 points over each range.

If you have one variable parameter the program will divide the range

into one hundred regions and will calculate the focal distance over

101 points.  For two or three variable parameters the program divides

the ranges into ten regions and calcualates the focal distance over 11

points.  The focal distance given is the distance from the last

surface.

You may review the parameters you have entered by choosing selection 5

in the menu.  It is recommended that you save the data for the system

you design prior to calculating the focal length(s) for the system.

Use the SAVE DESIGN PARAMETERS option.  Then if you crash the program

you can restart it and use the old data. This is quite handy with a

very large system!

You may choose from a list of standard glass types to establish the

indices of refraction for the elements in your system or you may enter

the indices of refraction for the different wavelengths you are

testing your system for. You must also enter the index of refraction

for the media between elements.

In order to calculate the focal distance of the system you must set

the input to the system.  If you forget to do this the program will

prompt you for the information prior to doing the calculations.  The

input information for the system consists of the angle the input ray

subtends with the optical axis of the system (in radians) and the

height above the axis the ray enters the system.

If you want to change the number of variable paramters or the

parameter(s) to vary select CHANGE OPTIONS from the menu.  You will

then be asked which parameters to vary and the variable ranges.

The MAKE REPORT option will create an ascii text file with a summary

of the design information on your system.  You will be prompted for

the filename under which to save the report.

TIPS FOR USING LENSCAD:

Always review the paramaeters of the system you are designing prior to

calculating the focal distance.  This will avoid a system crash should

you forget to enter a value for a parameter.  Always save your design

parameters before making the focal distance calculation.

The output of LENSCAD consists of the wavelength and variable values

for each variable parameter, and the associated focal distance for

that set of variable values.  This format is repeated for all the

wavelengths the system is being tested for.  LENSCAD makes all

calculations for one wavelength before making calculations for the

next wavelength.  The output for one variable and two wavelengths

would have all the results for the first wavelength in the first half

of the file and all the results for the second wavelength in the last

half of the file,  making it very easy to extract the data.

The output for a three variable paramater system with two or more

wavelengths of light is in excess of 360 kilobytes, therefore the data

must be written to a hard disk or a high capacity floppy disk if you

choose this option. If you have only 360kb drives you might run the

program once for each wavelength.

LENSCAD displays the elements of the system matrix during the

calculation of the focal distances.  You may see the system matrix for

a particular system by changing options to make all the parameters

constant.  The system matrix contains four elements:

SYSTEM(1,1) is the angular magnification of the system.

SYSTEM(1,2) is the equivalent power of the system.  Its

reciprocal is the equivalent focal length.

SYSTEM(2,1) is the negative, reduced thickness of the lens.

SYSTEM(2,2), along with (1,1) and (1,2), give the front vertex

length:

a1=        SYSTEM(1,1)*n

SYSTEM(1,2)

and the back vertex focal length:

a2=       SYSTEM(2,2)*n

SYSTEM(1,2)

The algorithm used by the program was derived from the matrix method

outlined by Jurgen R. Meyer-Arendt, M.D in the 2nd ed. of Introduction

to Classical and Modern Optics (pp.78-89). It is my understanding he

dropped the technique in the 3rd ed. which is a real shame because it

lends itself well to computer approaches to lens design.

****************************************************************************

INTERESTED IN NEURAL NETWORKS? NNET IS A THREE LAYER BACKPROPAGATION NEURAL

NETWORK SHELL DEVELOPED FOR RESEARCHING NEURAL NETWORKS. INCLUDED WITH NNET

IS A VERSION COMPATIBLE WITH THE HP95LX PALMTOP COMPUTER FOR TRULY PORTABLE

NEURAL NETWORKING. SEND $54 FOR EXPRESS SHIPMENT TO J. MICHAEL, BOX 941124,

ATLANTA, GA 3O341.

****************************************************************************