THE WA8BXN MSYS PACKET BULLETIN BOARD SYSTEM

USER'S MANUAL

By Steve Wolf, NO8M @ NO8M.#NEOH.OH.USA.NA

December, 1993

Version 1.16

Please copy it and pass it around!

(Keep the credits and trademark notice!)

Additions, corrections and suggestions:

Steve Wolf, NO8M

27132 Butternut Ridge Road

North OLMSTED, OH  44070-4417

NO8M @ NO8M.#NEOH.OH.USA.NA

MSYS is written by Mike Pechura, WA8BXN, and is copyright 1993 by

HUB COMPUTERS, INC.  It is distributed free for amateur use and

can be obtained by various means including the Cleveland Hamnet

BBS in Cleveland, Ohio

(216-942-7516 - 9600/2400/1200 baud - RBBS).

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 1

THIS MANUAL

This  manual is  intended  to be  a  reference to  the  many

commands available in the MSYS PBBS. It will not teach you how to

operate the board efficiently,  you will have to experiment  with

it. It will give you the tools to experiment.

COMMON ERRORS

The  misspelling  of a  file  name  you  are  attempting  to

download is one of the most often observed errors.   The improper

use of subdirectories is a close second.

The use of the  wrong command is another "most  often made".

If you find yourself in a bind,  check the command description in

this manual.

Another  common  problem  is  attempting to  use  the  nodes

without giving a  port number.   You must give  a port number  to

connect from the nodes.

Any  traffic moving off the board should be signed with your

call and your home PBBS, like this:  "KA8ZVV @ NO8M.OH.USA.NA"

A send to  a station must contain the exact  callsign of the

person  you want the message to go to.   If you send a message to

KA8ZZV  rather  than KA8ZVV,  your  station  will  never get  the

message.

The sysop  is  more than  happy  to answer  your  questions.

Please don't hesitate to leave a message to the call  of the PBBS

asking about your problem!

PRINTING THE MANUAL

The manual can be printed by using the DOS command

TYPE USERMAN.DOC > PRN

KA-Node, KAM and KPC-4 are trademarks of Kantronics, Inc.

NET/ROM is a trademark of Software 2000.

IBM is a trademark of International Business Machines Corp.

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 2

STRUCTURE

MULTI-USER/MULTI-TASKING

The  board is  a  multi-user system.  Many  stations can  be

connected at once.  It will support many connects on many bands.

This is normally transparent  to the user.   However, during

prime  time, you  might observe  some slowing.  Even if  you hear

nothing on your frequency, the  board might be up to its  ears in

tasks  on other frequencies. Disk reads and writes will also slow

a response.

HOUSECLEANING

The board  will automatically kill messages  after a certain

time period. Decisions about what is kept  and what is killed are

made by  the sysops. Should you be out  of town for a long period

of time, you might ask the sysop to hold your mail.

BACKUPS

Every message gets copied to a backup file. Should the board

crash, the messages can be  restored. If a message that you  have

already read  and killed  suddenly  reappears, it  may have  been

restored after a crash.

The  board  is  very  stable  if  run  with  a  conservative

attitude.  Normally  sysops will  tweak and  tune to  get optimum

performance. They will experiment with other ports. They will not

act conservatively.

SECTIONS

There are four basic sections. There is a message section, a

download section,  a TCP/IP section and a  user services section.

The message section is  capable of delivering mail and  bulletins

almost  anywhere  in the  world.  The  download section  contains

information held for reference  and training. The service section

provides a node  and conference  that enhances the  value of  the

software.   Send a message to  the callsign of the  PBBS for more

information on the TCP/IP section.

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 3

THE SERVICES SECTION

The Conference

K-Node

Network Node

THE CONFERENCE

Normally, only two amateurs can connect via packet radio and

hold a conversation. Three or more can talk  in the converse mode

but will be forced to disregard all other traffic on the channel.

MSYS provides a means  by which any number, depending  on channel

congestion, can talk.

To  enter or  start a  conference, you  must connect  to the

PBBS. From there  you issue the conference  command. Every packet

you send will be received and acknowledged by the  board. It will

then  send your  packet  and receive  acknowledgement from  every

other user.

The  first packet received from  a user will  be prefaced by

the user's  callsign. If the  next packet  sent is from  the same

person, the callsign is not included.

Overhead  (channel usage)  is  very high  for a  conference.

However, if the local  area network (LAN) is on  a clear channel,

it will support many users.

You might be asked to join in  a conference.  Do it!  If you

get a  message that says  that someone would  like you to  join a

conference, type the letter C.

THE K-NODE

MSYS  provides the  user  with  two  types  of  nodes.    An

explanation of what a node is and how it operates can be found in

most  download sections. For  now, suffice it to  say that a node

acts somewhat  like a  repeater, extending  the range  of another

packet station.

One MSYS node  is called a K-Node and operates much like the

Kantronics KA-Node. They may also provide digipeating and gateway

service.

To use  the K-node, you  must first connect  to it.  It will

have a different callsign than the PBBS and will normally use  an

alias.   The ID command, described later,   will let you know the

call of the K-Node.

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 4

The following K-Node commands are available:

Bye ------- This is used to disconnect from the K-node.

Connect --- This command allows the user to connect  to

another station.   Two  additional  items, a  port

number and a callsign  are necessary. They MUST be

included.  The ID  command, described  later, will

tell  you what  port  number is  attached to  what

frequency.

C#   call -- Connect on port  # to call, PORT NUMBER IS

REQUIRED!

Help ----- Help.

Just Heard  ----- What  stations the K-node  has heard.

This list is much  more limited than the   command

on  the PBBS.  It  will also take  an operand port

number.

Node -----  This lists K-Nodes  heard and  will take  a

port number operand.

THE NET NODE

The NET  NODE is different from the  K-Node in that the node

listens to  the frequency and  logs what it  hears.  If  it hears

another node, it logs that node as  a "route", a node that it can

directly connect to.  It also logs the nodes that route can hear.

It compares the  other nodes with what is in  its log and figures

out the best path to a destination.

NET  NODES  only talk  to  other NET  NODES,  NET/ROM nodes,

TheNet nodes  and other versions of the network nodes.  They will

typically have  a callsign  with SSID  and an  alias.  Many  have

rather strange numbers  as alias names.   This number  represents

the node's identifier when used with the TCP/IP protocol.

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 5

The following network node commands are available:

BBS --------- Connect to the bulletin board.

B<ye> ------- Disconnect

C#  ---------- Connect on port  # to call  (# is unnecessary

for known nodes)

C<onnect> --- name or call --- for known nodes

H<elp> ------ Help

I<nfo> ------ Basic information about the node

K<Node> ----- Lists the K-Nodes that are known

J<ustheard> - What has been heard (a limited output)

N ----------- Lists the network nodes that are known

N call ------ Information about a specific node

P<orts> ----- The port numbers of the node and the PBBS

R<outes> ---- To nodes that can be directly connected to

R call ------ Routes from a particular node

T<alk> ------ Page the Sysop

U<sers>  ----- Info on the  users and node  connected to the

MSYS node.

THE MESSAGE SECTION

This  is the meat of  the board. Messages  from user to user

and  bulletins  appear here.  The  board  is capable  of  routing

messages to almost  anywhere in  the world.  The bulletins  carry

information that  you may not  otherwise see until  the magazines

come out.

CITY, STATE and ZIP

If  you are  entering a  message for  another party  and the

route has not been proven by a number of other messages that have

made the  trip, please put  the City, State  and Zip code  in the

title line  of the message.   It will really help  the routing of

your message should it get hung up somewhere.

HIERARCHIAL FORWARDING  (Routing Messages)

This is not  near as complicated as  you might think! As  an

example, let's say that your  friend in Tahiti, French Polynesia,

wants to send you a  message. The local PBBS might not  know that

NO8M is in Ohio. It  might not even know how to  get traffic into

Ohio.  However if  the message  is sent  like this:  SP  WB8RNI @

NO8M.OH.USA.NA the message should have no problem.

The local  board will look at  the @PBBS line and  see if it

knows how to  route to NO8M. If it does not,  if will look at the

OH to  see if  it knows  how to  get traffic  into Ohio.  If that

fails,  it will  see if  it  knows about  the  United States.  If

nothing else, it had better know about North America!

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 6

See, nothing to it!  Your PBBS knows all about  the PBBSs in

your  state. In  fact, you  can verify  in-state traffic  will be

routed by  using the PF command. The board will come back and say

to where your  traffic will be sent.  In the United States,  your

PBBS  should recognize a PBBS callsign and respond with a routing

like KY.USA.NA.

If it  does not, and you are SURE that  the @PBBS given is a

forwarding PBBS,  then you  may send  it by  adding a two  letter

state abbreviation after the  @PBBS call. In this way,  MSYS will

send the traffic to a forwarding PBBS in that state  and let that

board figure out what to do with it.

For international traffic that you  are not sure of,  please

send a message to the call of the PBBS and inquiries will be made

in order to obtain the proper routing.

DATABASE

MSYS  has the  facilities  to maintain  a  number of  simple

databases. Adding to a database is done like this:

Line entered                       Explanation

============================       ============================

SP MSYS @ thatbbs                  A message to the PBBS.

DBA 3                              (title  line) Add  to database

#3.

wind damage, Mentor, Ohio \        The "\" character

large tree down at 615 & 20 \           causes one line after

reported by Mentor PD \                 it to be listed

A no injuries                           with the original line.

/EX                                "/ex" to end.

Now if a search (GREP) on database number three were used on

the string "Mentor, Ohio", all four lines of the message would be

sent.   GREP  can be  used  on all  files  ... PBBS  lists,  club

rosters, hazardous materials lists, etc.

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 7

MSYS COMMANDS

Abort ------- This command stops the PBBS  from what it is doing.

For  example, you  have asked  it to list  all the

messages.  After about 300 messages you decide you

have had enough!   Issue  the Abort  command.   It

also works with reading messages, downloads, etc.

Bye --------- This disconnects you from the PBBS.  It resets your

statistics,  too.   If you  just disconnect  or go

away  without disconnecting, the  PBBS will assume

you ran into trouble and log the disconnect.   For

example, on  Monday you check-in and  read all the

new  messages.  You  then disconnect without using

the  Bye.   Then, you  check in  on Tuesday.   You

issue an L (list  since you were last on).   Since

the PBBS ignored your Monday connect, you will get

the  messages since  you last  checked in  and the

messages you read on Monday. Now, had you used Bye

on Monday,  your Tuesday check-in would have given

you just the new messages.

Conference -- The conference  allows as many users as  desired to

connect  with  each  other.     The  best  way  to

understand it is  to try  it with a  friend.   The

first time you  talk, your line that is  sent will

be prefaced with your  callsign, like this:  NO8M:

If the next line does not have a callsign in front

of it, then it came from the same station.  Inside

the conference you can use the following  commands

(send  a control-Z  by  pressing  and holding  the

CONTROL key while tapping the Z.  Release both.

/ex ---- To return to the PBBS (or ^ZQ)

^ZH ---- Help

^ZU ---- List users and their channels

^ZA # -- Ask user on channel # to join conference

Download  ---- To download a file.   You must enter the full name

and  directory  that you  obtained  from  the What

command  or  from  a   list  describing  what   is

available  that  is   sometimes  available.     To

download  a file  in a  subdirectory, use  this: D

subdirectory/filename.   So, to  download the file

APPLICAT.INF  in  the NCARC  directory,  you would

issue the command

"D NCARC/APPLICAT.INF".

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 8

Grep --------  This command searches  a file for  a given set  of

characters.  First just enter a G.  The board will

ask for a filename.  It is asking for something in

the download  section, that you get  with the What

command.   It will then ask you for the string you

want  to  find.   Let's  say  you want  to  search

NCBYLAWS.DOC    for    the   string    "election":

G(ENTER)NCBYLAWS.DOC(ENTER)election(ENTER).    The

board  will  then send  you  all  lines that  have

"election"  in   them.    The   command  does  not

recognize wildcards (*/?).   If you do not specify

a file, the default of DATABASE.0 will be used.

Help -------- A short list of commands.

?x --- A longer explanation of the command, use ?x

where  x is the name of  the command. I would

use the command ?L to get help on LIST.

Info -------- This gives information on the system's hardware.

ID  ----------  This gives  you  the callsigns,  ports  and other

information connected.

Just heard -- This gives information about stations that the PBBS

has  heard.   If the  call is  a funny  alias, the

sponsor's call is given inside the curly brackets.

If you want to connect to someone like that, don't

use the  sponsor's call, use  the alias.   Use the

Path  command  to  determine  how   stations  were

connected  to the  PBBS.  You  can also  limit the

output with  a channel number, like this: JK 0 ...

this lists all KA nodes heard on port 0.

JB -- BBSs

JD -- Digipeaters

JG -- Gateways

JK -- KA nodes

JM -- Other MSYS PBBSs

JN -- NET/ROM nodes

JT -- TCP/IP protocol stations

Kill  -------- This deletes  messages.  You may  kill a number of

messages at the same time:  K 2 34 556.

KM -- Kills messages addressed to you.

KT -- Kills NTS traffic you are going to deliver.

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 9

List --------  List  is  used to  give  you the  headers  of  the

messages.   Used alone, it will  list all messages

since you  last logged  off with the  Bye command.

The  PBBS has  a  function to  force all  personal

messages to end up  as private messages.  If  this

is  on,  any  message  sent except  bulletins  are

changed to private.

L ---------- List since last logged off with B.

L catagory - List for a single catagory.

LC --------- List categories (TO fields).

LM --------- List mine, messages addressed to you.

LN --------- Lists messages not yet read.

LT --------- Lists NTS traffic messages.

LL --------- List the last message entered.

LL # ------- Last # messages.

LO yymmdd -- List OLDER than yymmdd.

LU --------- Lists unread messages to you.

LY --------- Lists messages that have been read

LW --------- Lists weather messages (type W$).

L< call ---- Lists messages from call (or alias).

L> call ---- Lists messages to call (or alias).

L@ call ---- Lists messages with call as an @BBS.

L x -------- Lists  messages since a given number.

To see them all, use L 0.

L x y ------ List numbers between numbers x and y.

L"string" --  Lists  messages with  the  character

string  you  specify in  the title.

The quote marks are required.  This

is  case insensitive,  "Yaesu" will

match "YAESU", "yaesu", etc.

L'string'  -- Lists  messages  with the  character

string  you  specify in  the title.

The   single    quote   marks   are

required.  This is  case sensitive,

"Yaesu" will not match "yaesu".

L$ --------- Lists type $, list bulletins.

Message ----- This will send the message of the day. This message

contains  information on  happenings.  It  will be

sent  on  the initial  check  in  UNLESS you  have

selected the eXpert  mode.  If you  have, you must

use  M to read  it.  Expert  users DO  NOT get the

message of the day!

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 10

Name -------- This command is used to register with the PBBS.

N --- Your first name  with only the first  letter

capitalized.

NQ -- Your QTH, for example:  North Olmsted, OH.

NZ -- Your zip code (only the five digit code).

NH  -- Your home PBBS,  where you want  to pick up

mail  addressed  to  you.   Traffic  may

arrive to  you on  other boards  in your

area, you may want to declare your  home

PBBS on those.  If you change this, make

sure you  do it  on all boards  you have

checked into  And only  declare one PBBS

as  your  home  PBBS.   Otherwise,  your

message may  end up  as the potato  in a

game   of   hot-potato.      One   board

forwarding  to  another and  back again.

Declare only  one home PBBS.   NH should

be a full service PBBS, not a TNC PBBS.

Path -------- This command will tell you if the PBBS knows how to

route traffic to a  PBBS or if the PBBS  knows the

amateur.

P call -- If the person is known, information

that  was   entered  is  displayed.

This  is  a good  way  to  find out

someone's name,  their last connect

and  where  their  traffic will  be

sent.    A response  that indicates

that  the  person did  not  enter a

home board and  has not  checked-in

for  a long  time would  indicate a

message  left for them might not be

received.

PF call - This is used to  indicate if the PBBS knows how to

route traffic to another PBBS.   If this PBBS does

not know how to route to the PBBS of interest, you

can insure  routing by  adding a two  letter state

abbreviation  to  the   end  of  the  PBBS   call:

KA8ZVV.OH.  If  you are routing traffic  to a PBBS

in  Ohio and this PBBS  does not know  it, leave a

message for the sysop.

PC call -  If there is a callbook (REQQTH) server enabled at

the  PBBS,  this command  can  be  used to  obtain

information about a callsign from the callbook.

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 11

Read  -------- This is used  to read messages  on the board. Many

messages  may be  strung  together:   R  1 45  68.

Wildcards  are  supported:    R>  DX*  will   read

anything starting with "DX".

R@ call - Read messages having a certain @BBS.

R> xxxx - Read messages to xxxx TO field.

R< xxxx - Read messages from xxxx in FROM field.

R cat --- Reads all messages in catagory.

RE ------  The read  for export.   This  reads the

message in a format where it can then be

easily  uploaded into  a  PBBS  or  mail

drop.

RH ------ The read  with headers.  This  shows the

forwarding  headers  that  are added  at

each PBBS  as the message is  sent.  The

routes are not the same at all times and

it can be  fun to see how  a message got

from tither to yon.  If a message author

did not give the  home PBBS for a return

reply, use  this to determine  where the

message originated.  Normally, that PBBS

can be tried for a response.

RN  ------ This reads only the text of the message

and  is a  good way  to save  time  on a

marginal path.

RM ------ Reads unread messages addressed to you.

RP ------  Reads a  message without marking  it as

having been read  so it  will be  listed

the next time you check in.

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 12

Send  -------- This is used to send  a message.  The basic format

is Sx call  @PBBS.  Sx is a send  with the type of

message  it is.   The @PBBS is  the location where

the addressee picks  up their mail.   Call is  the

exact callsign  of the recipient.   If you  make a

mistake on the addressee's call,  the message will

probably end up in limbo, never to be received.

(control-A) -- Aborts the message; to start over.

CC  callsign -- This causes a copy of a message to

be  sent  to  the  calls  following  the

carbon copy command.   The CALLSIGN  can

t  a  k  e       t  h  e      f  o  r  m

CALLSIGN1@PBBS;CALLSIGN2@PBBS   etc.  to

send the same message to many users.

REPLY  #  --  Allows  the  PBBS  to  automatically

address  a  message replying  to another

message.  If you  add a period after the

message number, like this:   "REPLY #.",

the PBBS will also automatically enter a

title for you.

SP -------  A personal  message listed or  read by

addressee and author.

SR # -- Allows the PBBS to automatically address a

message replying to another message.  If

you  add  a  period  after  the  message

number, like this:  "REPLY #.", the PBBS

will  also  automatically enter  a title

for you.

ST  ------- NTS Traffic.  Most  boards have a help

file -- READ IT!

SB  ------- Send  a bulletin.   Without  an @PBBS,

these will stay on  your PBBS and may be

addressed in whatever way you wish.  For

example, in northern Ohio,  when sending

a bulletin, the @PBBS  field may be  set

to the following:

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 13

WW -- To quote Jim, WK8G,  messages addressed

to    ALLBBS     are,    "intergalactic,

omnipresent,  transuniversal All  Points

Bulletins"!   This  message would  go to

every PBBS  in the  world.   PLEASE  USE

THIS  WITH CAUTION.   The packet network

in Guam does not want to know about your

wattmeter for sale!.  Some areas use WW.

ALLUS  -- This would go  to every PBBS in the

U.S.  USE WITH  CAUTION as your for sale

would  normally  NOT  be  in  the proper

forum if sent in this manner!

OKIPN --  (Or  your wide  area  flood  route)

Ohio,  Kentucky and  Indiana.   Again, a

message to NEOH or ALLOH is better!

ALLOH -- All Ohio (or ALLMI, ALLHI, etc.).

NEOH  --  (Or  your local  area  flood route)

Northeast   Ohio.    This  is  the  best

routing for the "for sale" messages.

PBBSCALL -- For all at a PBBS.

Talk -------- Page the sysop.

Users  -------  Gives the  current users  of  the system  and the

number  of  messages.    If you  want  to  talk to

someone using  the board, you may want  to start a

Conference and invite them in.

Version -----  The version of  the software and  the date it  was

last started.

What  --------  This gives  a list  of what  is available  in the

download section.

W   subdirectoryname -- Gives a list of what is in

the subdirectory.   W NCARC or W  AMSAT/BULLS

are examples.

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 14

Xpert -------  This sets and  resets your expert  status.  As  an

expert, you get a short  command prompt and do not

receive the message of the day.

X   x -- Where x  is a number, sets  the number of

lines you receive  before the PBBS stops  and

asks you  if you want "More?".   Setting this

to  zero defeats the  "More?".  Answering the

"More?" with a C defeats it for that message.

XC  --  Toggles  the listing  of  catagories  when

connecting to  the  BBS while  in  non-eXpert

mode.    In eXpert  mode  you  never get  the

automatic catagory listing.

XF -- Sets  the PBBS to  send you multiple  lines.

Use this with a good route.

XR -- Toggles the automatic question asking if you

want a  reply to  the message you  just read.

This  can  be  useful  if you  get  a  lot of

personal mail.

XS  -- Sets  the PBBS  to send  you one line  at a

time.  Use this with a poor route.

Yapp  -------- This command allow  you to download  a binary file

from  the YAPP directory.   You may also upload if

you have been authorized.

YW -- What is in the YAPP directory.

YU -- Upload a file to the YAPP directory.

YD -- Download a file from the YAPP directory.

*  ------------ This makes the line after the asterisk a comment.

It can be used to answer the sysop after receiving

a "Message from sysop" line.

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 15

DEALING WITH TOO MANY BULLETINS

If you live in an  area that is served by a good network and

attentive sysops, you may experience bulletin overload.  If there

are just too many bulletins coming  in, keeping up can be quite a

chore.

For just this reason,  a number of commands are  designed to

help you.  The first command  is "LC".  This gives you a  list of

the different TO fields in the bulletins.  You can examine the TO

fields and determine those that interest you.

You can  then go to a specialized  READ commands to read the

bulletin categories  you  have selected.    "R> xxxx"  reads  all

bulletins which have the xxxx you entered in the TO field.

Let's say  you wanted to check  all the DX bulletins  on the

bulletin board.  You would give the  command "LC" and perhaps see

DX,  DXING,  DXER  as some  of  the  categories  that cover  your

subject.   You may then give  the command "R DX*".   This command

means  READ ALL BULLETINS WHICH START WITH THE CHARACTERS "DX" IN

THE TO FIELD.  The asterisk means "everything" or "all".

OTHER OVERLOAD HELPERS

You can also read everything from a certain author.  You can

use the command "R< KA8ZVV" to read everything from KA8ZVV.

You can read all the bulletins sent to a certain @PBBS.  You

can use  "R@ ARRL" to  read all  the bulletins sent  to the  ARRL

flood route.  "R@  ALLBBS" will read  all the bulletins going  to

the ALLBBS route.

You can also get  specific in your LIST  commands.  You  can

"L> YAESU"  to see all bulletins  sent to YAESU.   Of course, you

will miss those bulletin entered by those who own what they think

are "YEASU"  owners.  The "L> xxxx"  command will not correct for

poor spellers!

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 16

KEEPING LINE LENGTHS LESS THAN 80 CHARACTERS

There are a  number of reasons you want to  keep the lengths

of your lines less than 80 characters.  This will explain why you

need to do this.

Depending on your computer, you may see 80, 60 or even  less

characters on your screen.  This is not the line length.   A line

ends in a carriage  return (where you hit the ENTER key).  If you

only  have carriage returns at  the end of  your paragraphs, then

your line length is the length of the paragraph.

PBBS EDITORS

Let's say  you enter  a message into  the PBBS but  you have

made a  mistake.  You signed  your message in the  proper manner,

with  the full  hierarchial return  address, like  this:   NO8M @

NO8M.OH.USA.NA.  But this time, you made a mistake.   You entered

your  home call wrong, like this:  NO8M@NM8O.OH.USA.NA.  In these

cases  your sysop can edit  your message and  correct your error.

It is easier than sending the message back to you with a note.

However, the  editor on the MSYS  PBBS is limited.   It will

work  with only 80  character lines.   This is to  limit the code

that  is required  for  the  editor.   A  better  editor  can  be

installed  but   it  would  take  space  better  suited  to  PBBS

functions.

If your line  lengths are over  80 characters, your  message

would have  to be taken to  another program and edited.   You are

more likely to get it sent back.

SOLUTIONS

USING A TERMINAL PROGRAM

If you are entering a message from a terminal program, hit a

carriage return (the ENTER key) before you send 80 characters.

USING A WORD PROCESSOR

Many word processors  will allow you  to enter your  message

without  regard  to  line-length.    This  makes  editing,  spell

checking  and  other functions  easier.   They  will then  have a

special function to allow the file to be saved in an 80 character

per line format.

This  file was done in  the WordPerfect word  processor.  It

was saved with carriage returns using the "DOS SAVE" function.

It might help if you can set your word processor to show carriage

returns on the screen.  Consult your manual.

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 17

DON'T GET FANCY

Many newsletter editors watch  packet messages for ones they

would like  to print.  Do  not add fancy stuff  the messages that

you might want  to see  printed.   It causes  additional work  to

reformat  them into  usable  text.   Do  not justify  your  text.

Justification adds spaces between words to make both margins come

out straight.   It is very  difficult to edit  these extra spaces

out.

Do  not indent your whole message.  These indentations are a

pain to take out.  Add two spaces between sentences.   After each

period, question mark, etc., there should be two spaces.

Single space your messages.  This saves network time as the extra

carriage return does  not need to  be sent over  and over as  the

message gets sent to other boards.  Again, the newsletter editors

will not have to remove the extra carriage returns.

The use of capital  letters is a questionable practice.   Studies

have been done showing  that messages in all capital  letters are

more difficult to read.

APLINK

Many  times, the AMTOR PBBSs, called APLINK, will be used to

send traffic.  AMTOR has a very restricted character set.   There

are no lower case letters.  Many characters, such as "~", "^" and

"_" will be ignored.

PACTOR

MSYS will  handle the link  direction.  There is  no need to

worry  about the direction.   Otherwise, the connect appears much

as it does on a normal packet port.

PROOFREAD YOUR MESSAGES!

After  sending a bulletin,  read it and see  if that is what

you meant it to say and how you meant it to  look.  After sending

it,  use the command "LL" (list last)  to get the number and read

it.

EXPERIMENT

You  can send messages to yourself.   You can then read them

back to see if what you have done is what you wanted to do.

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 18

NTS BASICS

WHY THE GOOFY FORMAT?

Your message may not stay on  the packet network.  It may be

forwarded by nets using RTTY, CW, phone or whatever.  These other

modes are  NOT error  free.   The  preamble  and format  of  your

message  will help standardize and check  your message for better

handling.

NTS IS PICKY

Danged right, they are!   NTS operators want to  insure that

the  system works now, before the disaster.   If they can get you

using the standard now, you will not be a problem later.

Right now you are handling messages that might not mean much

if  they don't get to their  destination.  After a disaster, your

message  content  may  have  much  more  meaning.    You  may  be

transferring messages about critical  supply needs, about medical

supplies  that will save someone's life.  Information to a family

who's relatives assume they are dead.

If you do something wrong while handling NTS traffic, expect

to get corrected.  You will be one heck of an asset in a disaster

if  you know  what you  are doing.   You  will be  one heck  of a

liability if you don't.

A TYPICAL MESSAGE

|    ST 44070 @ NTSOH

|    TITLE:  North Olmsted, OH  (216)777

|    Enter message--End with /EX or CTRL-Z

|    NR 15 R HXG N8GNJ 4 Seattle, WA 9-18-89 0045Z

|    To:  Steve Wolf

|         North Olmsted, OH

|         216-777-1177

|    -bt-

|    Request home PBBS call

|    -bt-

|    Steve N8GNJ, Seattle, WA

|    Operator's note:  Reply to

|    N8GNJ@N8GNJ.WA.USA.NA  Seattle, WA

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 19

Let's examine the above message line by line:

|    ST 44070 @ NTSOH

When you send a piece of NTS traffic, it has  a special send

command.  The traffic indicator will help it on its way.  It will

also (sometimes) cause the  last PBBS in  the line to generate  a

message back to you telling you who took it off the packet system

and where.  In many cases,  this might not be at its destination.

An operator in  Iowa might pull an Ohio message  because he knows

he can get it there quickly.   He might be getting on a net  that

has an operator  in the town  that the message  is destined  for.

ALWAYS  use a ST zip  @NTSxx command as  all the forwarding PBBSs

will recognize that without interpretation.

|    North Olmsted, OH  216-777-1177

This  title  line tells  that there  is  one message  in the

packet message and that it is destined for Ohio.  If NOT disaster

related,  there may be many  messages in the  packet messages, as

long  as they are destined for the  same location.  The inclusion

of an  area code  (AC 216)  is sometimes used  but does  not mean

diddley  in many areas.   If the precedence  (explained below) is

EMERGENCY or P, then that could be added to  the title line.  The

title line is limited to 37 characters!

|    NR 15 R HXG N8GNJ 4 Seattle, WA 9-18-89 0045Z

This is the preamble.  NR 15 is the number of the message at

the sender's station.  On January 1, you start  with message one.

You assign each message going out of your station a number.

"R" is ROUTINE.   Other precedents are  "W"elfare, "P"riority and

"EMERGENCY".   EMERGENCY is  always spelled out!   Any precedents

other than routine should appear in the title line.

"EMERGENCY" deals  with information related to the safety of life

and  property.    "Priority"  is everything  that  is  almost  an

emergency.  It includes  important messages having a  time limit,

notices  of death  and  injury  in  a  disaster  area  and  press

dispatches.   "Welfare" is  for health and  welfare inquiries and

responses that indicate  all is well.   If all  is not well,  the

message  should be  sent as  Priority.   "Routine" is  everything

else.

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 20

The HX-  is the  handling instructions  (if any).   Handling

instructions are rarely used.

HXA# -- Collect call within # miles (or unlimited if no #).

HXB# -- Cancel in # hours/notify originator.

HXC  -- Report time and date of delivery.

HXD  -- Report time, date, method and call delivering.

HXE  -- Obtain and send reply.

HXF# -- Hold message until # (date).

HXG  -- Mail or toll call delivery not required.

The number five is the check of the message.  It is how many

words you are sending.  The stop character, the "x", is counted.

The rest  of the line is  self-explanatory.  For gosh  sakes, use

UTC!  There are other people in the world and they don't all  use

EDST!

|    To:  Steve Wolf

|         North Olmsted, OH

|         216-777-1177

This is the address that the message is to go to.  The phone

number  is almost  always mandatory.   It  will probably  just be

serviced back without it.

|    -

|    -bt-

|    Request home PBBS call

|    -bt-

The meat, the message.   Preceded and followed by a  -bt- to

set  it  off, the  message should  be  no longer  than  19 words.

Rambling messages do not make it!

|    Steve N8GNJ, Seattle, WA

|    Operator's note:  Reply to

|    N8GNJ@N8GNJ.WA.USA.NA  Seattle, WA

This  is  the  signature,  to  whom  a  response  is  to  be

forwarded.  It is a real good  idea to add your packet address to

the end.

CHECKS--COUNTING THE WORDS

If you are going to run  into trouble, this is where it will

be.  Probably the best  available advise is to not banter  on the

air about disagreements over the  check.  If they know more  than

you, they are right.  If they know less, they are wrong.

WA8BXN MSYS PBBS User's Manual

Version 1.16     December, 1993

Page 21

Here is a quick list of some sticky check situations:

Alfred E. Neumann   3 words

Grand Cayman        2 words

NY                  1 word

New York City       3 words

ARL SIXTY THREE     3 words

Fifty Six           2 words

6146B               1 word

You can change an improper check.  DON'T CHANGE THE WORDING!

If you  do change the check, use a slant bar between the old

and new values.  For example, if you received a check of five and

the check should be seven, note it 5/7.

ARRL NUMBERED RADIOGRAMS

These are "standard" messages.   The text of the  message is

simply ARL  followed by the number spelled out:  ARL SIXTY EIGHT.

It  allows the operator to quickly make  up standard "are you OK"

and the like type messages.  There should be a file on your board

describing all the different ARL numbers.

The one seen in a disaster situation is ARL NINETEEN.   Here

is an example:

|    NR 132 W NO8M ARL 2 N. OLMSTED, OH AUG 5

|    JANE DOE

|    1234 WEST 5TH

|    GARY, IN

|    PHONE 123-456-7890

|    ARL NINETEEN

|    JOHN SMITH 987-654-3210

|    OR REPLY NO8M@NO8M.OH.USA.NA

(Note  that this message could be sent either to Jane Doe, a

person in the disaster area, or  to an agency who could follow it

up.  For example, it could be addressed to the American Red Cross

agency  in  the area  with  the text  requesting a  check  on the

person.)

DO NOT BOOK DISASTER TRAFFIC!

Booking  traffic is  sending a  number of messages  with the

same information in one message.  The stations in a disaster area

do not have the time to separate them.

WA8BXN MSYS PACKET BBS COMMAND SUMMARY ... Version 1.16

=======================================================

Abort..........Stop current action           PC........Request a callsign

Bye............Disconnect               Read...........Read a message

Conference.....Multi-user\DX Node            R x ......Read all catagory x

^ZH.......Conference help               R@ xxxx...Read all at xxxx BBS

^ZQ.......Return to PBBS                R> xxxx...Read all to xxxx

^ZU.......List users                    R< xxxx...Read all from xxxx

^ZA #.....Invite user on #              RM........Read Mine

Download.......Download from FILES           RE........Read for export

Grep...........String search file            RH........Read with headers

Help...........Help                          RN........Read only text

Information....PBBS hardware                 RP........W/O mark as read

ID.............Port definitions         Send...........Send a message

Just Heard.....Log                          ^A.........Abort message

JB........BBS Stations                 CC.........Carbon copy

J.........Digipeaters                  REPly #....Reply auto title

J........Gateways                      SP.........Send personal

JK........K and KA nodes               ST.........Send traffic

JM........MSYS PBBSs                   SB.........Send bulletin

JN........Net/Rom                  Talk...........Page sysop

JT........TCP/IP protocol          Users..........Users

Kill...........Kill a message           Version........Version

KM........Kill mine                What...........Download list

KT........Kill traffic                  W x.......What in x=subdir

List...........Message headers          eXpert.........Toggle expert

L x.......List catagory x               X #.......Lines per page

LC........List catagories               XC........Toggle catagory

LM........List mine                     XF........Multiple lines

LN........List not read                 XR........Toggle reply

LT........List traffic                  XS........1 line per reply

LL........List last message        Yapp...........Down and uploads

LL #......List last #                   YW........YAPP Directory

LO #......List older #=yymmdd           YD........Download a file

LU........List unread to you            YU........Upload a file

LY........List read to you         * .............Comment to sysop

LW........List weather

L?........List ? forward           NETWORK NODE COMMANDS

L<< x.....List fm x=callsign       BBS............Connect to the PBBS

L>> x.....List to x=callsign       Bye............Disconnect

L@ PBBS...Messages to PBBS         Connect CALL...Connect to call

L #.......List since #                  C# CALL...C on port # to call

LL #......List last #              Help...........Help

L # #.....List from # to #         Info...........Basic Information

L"x"......List with x=string       Justheard......Limited output JH

L'x'......List with x=sTrInG       K-Nodes........K-Node output.....

L$........List bulletins           Nodes..........Nodes heard

Message........Message of the day          Nodes CALL..Info on node

Name...........First name               Ports..........Port information

NH........Home board               Routes.........Direct connect nodes

NQ........QTH=City, State             Route CALL..Other node

NZ........Zip code                 Talk...........Page Sysop

Users..........Users/nodes

Path...........Path to a user

PF........Path to a PBBS           Version 1.16 December 1993