XIRCOM CREDITCARD ADAPTER FLASH UPDATE UTILITY

README DOC

Release 2.13 Revision E

December 16, 1993

========================================================================

Thank you for purchasing the Xircom CreditCard PCMCIA Ethernet Adapter.

Xircom has done everything to ensure that our PCMCIA Ethernet Adapter is

compatible with the Personal Computer Memory Card International

Association's (PCMCIA) 2.0 to 2.1 I/O slot configuration.

The CEUPDATE.EXE utility enables end users to upgrade to the latest

revision of firmware available for the Xircom CreditCard Ethernet Adapter.

If you have compatibility issues related to your specific environment,

or you have general questions, please contact Xircom Technical Support.

Information on Xircom's Technical Support is available at the end of this

file.

Refer to the INSTALL.DOC for instructions on the usage of CEUPDATE.EXE.

FEATURES OF NEW RELEASE

=======================

Once the Card has been flashed, through the use of CEUPDATE.EXE, the

revision of the firmware will be 'REV E'.  This revision provides the

following benefits.

-  Increased performance from Memory Mapped I/O mode.  Be certain

to use the latest network drivers to obtain maximum performance

from the upgraded flash.  Use network drivers version 2.0 or

greater.  The increased performing 'Memory Mapped I/O mode' is

automatically enabled by loading the file CECFG.EXE prior to

loading a network driver.

If you are using a memory manager such as EMM386 or QEMM, you will

need to modify your exclude statement to increase the window size

from 4k to 8k to support the new Memory Mapped I/O mode.

Device=C:\DOS\EMM386 NOEMS X=D000-D0FF

Change to:

Device=C:\DOS\EMM386 NOEMS X=D000-D1FF

-  Support of the Cirrus Logic PCMCIA controller for operating the

PCMCIA adapter in 16 bit mode, previous firmware would only

function in 8 bit mode properly.

The utility does not function on the following computers:

=========================================================

Gateway 2000 Colerbook:  PCMCIA slot does not go to 12v.

Grid Convertible Pentop: PCMCIA slot does not go to 12v.

IBM 720 Thinkpad

XIRCOM TECHNICAL SUPPORT

========================

To obtain technical support for your Xircom product, please call or send

a facsimile to the appropriate number listed below.  Access the Xircom

INFOSYS BBS at up to 14400 bps, No parity, 8 data bits, 1 stop bit

(N-8-1).

Corporate Offices        Technical Support and Information

-----------------------  ---------------------------------------------

Xircom, Inc.             North and Latin America... TEL:  800-874-4428

Corporate Headquarters   International ............ TEL:  818-878-6280

26025 Mureau Road        24-Hour Facsimile......... FAX:  818-878-7175

Calabasas, CA 91302      24-Hour Data Lines ....... BBS:  818-878-7618

TEL: (818) 878-7600   24-Hour Internet.......... INT: cs@xircom.com

FAX: (818) 878-6630   CompuServe                 CIS:  GO XIRCOM

HOURS:  6:00 a.m. to 4:30 p.m., Monday through Friday (Pacific)

Xircom Europe NV         Europe, Mid-East, Africa.. TEL:  32 3 326 3494

Bisschoppenhoflaan 82-84 24-Hour Facsimile......... FAX:  32 3 326 3150

2100 Antwerp, Belgium    24-Hour Data Lines........ BBS:  32 3 326 2368

TEL: 32 3 326 34 94

FAX: 32 3 326 31 50

HOURS:  8:30 a.m. to 5:30 p.m., Monday through Friday

8:30 a.m. to 4:30 p.m., Friday

Xircom Asia LTD.         Asia Pacific.............. TEL:  852-525-2078

1604 Centry Square       24-Hour Facsimile......... FAX:  852-525-8955

1-13 D'Aguilar Street    24-Hour Data Lines........ BBS:  852-537-6048

Hong Kong

TEL: 852-525-2078

FAX: 852-525-8955

HOURS: 9:00 A.M. TO 5:00 P.M., Monday through Friday

---------------------------------------------------------------

Thank you for making Xircom a part of your network.

------------------------------------------------------------------------

Xircom acknowledges all tradenames and trademarks used in this document

as the property of their respective owners.

===============================[  END  ]================================