July 7, 1994

_ _ _

/ / (_)

_\ \ \

(_)_/_/  ASPI-ID v1.0 -

Displays SCSI Mode Sense and Inquiry information via the ASPI device

driver.

Copyright 1994, Seagate Technology, Inc. All rights reserved.

Seagate Technology, Inc.

B-5 Technical Support

920 Disc Drive

Scotts Valley, CA 95066

===============================================================

Usage: ASPI-ID [SCSI ID] [optional: R or D]  or....

ASPI-ID /filename.ASP

switches:

SCSI ID        - A valid SCSI ID in the range 0 to 6

The optional R and D switches will only work if the SCSI ID

switch is also set on the command line.  ex: ASPI-ID 0 D

R            - allow command line Redirection; this feature is

useful for redirecting the output to a printer or a

file.

D            - Dump the SCSI Mode Sense and Inquiry binary data to

<filename>.ASP; this allows a copy to be saved that

can be studied at a later time. A "collection" of

various drive files might be useful for reference

purposes.

/filename.ASP - load a previously saved ASPI data dump.  The ASPI

device driver is not required in memory for this

feature. Therefore, non-SCSI systems can be used to

load and display a saved file.

(no switches)  - prompt for SCSI ID info

ASPI-ID is designed for the support of users with Seagate disc drives and

has been tested with Adaptec, Acculogic, Always, DPT and Ultrastor

SCSI host adapters with their associated ASPI device drivers.

(These represent the HA's we had laying around in Tech Support, other

HA's with ASPI drivers should also work.)

===============================================================

A little background:

"ASPI" stands for Advanced SCSI Programming Interface.  It is an

Adaptec-developed interface specification for sending commands to SCSI

host adapters.  The interface provides an abstraction layer that

insulates the programmer from considerations of the particular host

adapter used.  The ASPI manager accepts ASPI commands and performs the

steps necessary to send the SCSI command to the target.  For example,

although the Adaptec AHA-152x and AHA-274x host adapters have very

different hardware, the ASPI interface to these boards is the same.

Unlike AT bus based disc drive interfaces, which tend to have common

well definded register level instructions, SCSI controllers are not

nearly so constrained.  In other words, it is up to the host adapter

manufacturer to supply the link between the actual hardware

controlling the SCSI bus, and the service calling software, usually

the operating system.  Although this increases flexibility for changes

to the host adapter, it precludes direct universal hardware port

calls.

The "linking" software has to be supplied by the host adapter

manufacturer either in the form of a BIOS, or equivalent software

module in the form of a device driver.  Because each host adapter

manufacturer uses different SCSI support hardware, it is impossible

for a drive manufacturer to supply software support for all SCSI host

adapters.

Fortunately, thanks to the ASPI, a common ground is established that

allows a software program to communicate directly with the SCSI

device.  The only requirement is that the user must install an ASPI

device driver (usually in CONFIG.SYS) that was written by the host

adapter OEM. DOS ASPI device drivers are usually supplied with the

host adapter or are freely available via support BBS's because of

their common usage with SCSI CD-ROMs and tape backup systems.

The ASPI command codes utilized in ASPI-ID are:

Host Adapter Inquiry -

ID ofHost Adapter

SCSI Manager ID (the name given to the ASPI driver by the host

adapter manufacturer)

Host Adapter ID (the name of the host adapter)

Get Device Type -

Peripheral Device Type of target

Execute SCSI I/O Command -

SCSI Inquiry (12h) (returns information about the drive. see

Appendix D for detailed information)

SCSI Mode Sense (1Ah) (returns many pages of detailed

information about the drive. see

Appendix C for detailed information)

SCSI Read Capacity (25h) (returns the number of the last

logical block and the block size)

Get Disk Drive Information -

INT13 drive numnber

Preferred Head Translation

Preferred Sector Translation

===============================================================

The follow SCSI command and mode page definitions are taken from

a Seagate SCSI drive product manual.  The purpose for including

it in this doc file is to help clarify the information returned

by the ASPI-ID program.  It is not intended to be a complete

SCSI reference.

3.4.13    Mode Sense command (1AH)

===============================================================

When the initiator sends this command to the drive, it returns

mode-page parameters to the initiator. This command is used in

conjunction with the Mode Select command.

Bits    7       6       5       4       3       2       1       0

Bytes

0       0       0       0       1       1       0       1       0

1       LUN                     0       0       0       0       0

2       PCF             Page code

3       0       0       0       0       0       0       0       0

4       Allocation length

5       0       0       0       0       0       0       Flag    Link

Byte 2    The page control field (PCF) determines the content of Mode

Parameter bytes. Regardless of the value of the PCF, the

block descriptor always contains the current values.

PCF bit 7       PCF bit 6       Effect

0               0               Return current values.

0               1               Return changeable values.

1               0               Return default values.

1               1               Return saved values.

The page code is the designator that is unique to each page.

The page codes are listed in Section 3.4.13.1.

Byte 4    The allocation length specifies the number of bytes that the

initiator has allocated for returned Mode Sense data. An

allocation length of 0 means that no Mode Sense data is to

be transferred. This condition is not considered an error.

Any other value represents the number of bytes to be

transferred. For a description of the allocation length, see

Section 3.4.13.1.

3.4.13.1  Page code and allocation length

The Mode Sense command descriptor block contains a page code (byte 2,

bits 5--0) and an allocation length (byte 4). These parameters are

described in the following table. You can transfer mode pages to the

initiator either of two ways:

- Transfer all mode pages at once by using page code 3FH, as de-

scribed in the last row of this table, or

- Transfer one mode page at a time by using the page code and

allocation length of the mode page.

Page    Allocation      Mode Sense data returned

code    length

01H     18H             4   bytes of Mode Sense header

8   bytes of block descriptor

2   bytes of mode-page header

10 bytes of Error Recovery parameters

02H     18H             4   bytes of Mode Sense header

8   bytes of block descriptor

2   bytes of mode-page header

12 bytes of Disconnect/Reconnect parameters

03H     24H             4   bytes of Mode Sense header

8   bytes of block descriptor

2   bytes of mode-page header

24 bytes of Format Device parameters

04H     20H             4   bytes of Mode Sense header

8   bytes of block descriptor

2   bytes of mode-page header

20 bytes of Rigid Disc Geometry parameters

08H     20H             4   bytes of Mode Sense header

8   bytes of block descriptor

2   bytes of mode-page header

20 bytes of Caching parameters

0CH     24H             4   bytes of Mode Sense header

8   bytes of block descriptor

2   bytes of mode-page header

24 bytes of Notch and Partition parameters

0DH     18H             4    bytes of Mode Sense header

8    bytes of block descriptor

2    bytes of mode-page header

12  bytes of Power Condition parameters

38H     1CH             4    bytes of Mode Sense header

8    bytes of block descriptor

2    bytes of mode-page header

16  bytes of Cache Control parameters

3CH     0FH             4    bytes of Mode Sense header

8    bytes of block descriptor

2    bytes of mode-page header

3    bytes of Soft ID parameters

00H     10H             4    bytes of Mode Sense header

8    bytes of block descriptor

2    bytes of mode-page header

4    bytes of Operating parameters

3FH     143 or 144      4    bytes of Mode Sense header

8    bytes of block descriptor

2    bytes of mode-page header

143 or 144 bytes of mode parameters,

including all mode pages

* The allocation length depends on whether the Operating page has 2 or

3 bytes. The Operating page is described in Appendix C.10.

3.4.13.2  Mode Sense data

The Mode Sense parameter list contains a 4-byte header followed by an

8-byte block descriptor (if any), followed by the mode pages. The

header and block descriptor are shown below. The mode pages are

described in Appendix C.

Bits    7       6       5       4       3       2       1       0

Bytes

0                  Mode Sense data length

1 (default)        Medium type (00H)

2       WP=0       Reserved

3 (default)        Block descriptor length (08H)

Block descriptor

4 (default)        Density code (00H)

5--7               Number of blocks

8 (default)        Reserved (00H)

9--11              Block length

Mode pages

12--n              Mode pages

Byte 0  The Mode Sense data length specifies the number of

bytes minus 1 of the Mode Sense data to be transferred

to the initiator.

Byte 1  The medium type is always 0.

Byte 2  The WP (write protect) bit is always 0, which means the media

is write-enabled.

Byte 3  The block descriptor length is the number of bytes in the

block descriptor. This value does not include the page headers

and mode pages that follow the block descriptor, if any.

Byte 4  The density code is not supported.

Bytes 5--7      The number of blocks field contains the total number

of blocks available to the user, which is specified on page 1.

Byte 8  Reserved

Bytes 9--11     The block length specifies the number of bytes

contained in each logical block described by the block

descriptor.

Appendix B. Sense data

===============================================================

The appendix contains the descriptions for sense data returned by the

Request Sense command. For more information on the Request Sense

command, see the Seagate SCSI-2 Interface Manual, publication number

77738479.

B.1  Additional sense data

===============================================================

When the initiator issues a Request Sense command, the drive returns

the following additional sense data.

Bit            7       6       5       4       3       2       1       0

Bytes

0              Valid   Error code

1       1       1       0       0       0       X

1              Segment number (00H)

2              0       0       ILI     0       Sense key

3--6           Information bytes

7              Additional sense length

8--11          Command specific data

12             Additional sense code

13             Additional sense code qualifier

14             FRU code

15             SKSV

16--17         Sense key specific

18--22         Product-unique sense data (00H)

Byte 0    If the valid bit is one, the information bytes (bytes 3

through 6) are valid. If the valid bit is zero, the

information bytes are not valid.

If the error code contains a value of 70H, the error

occurred on the command that is currently pending. If the

error code contains a value of 71H, the error occurred

during the execution of a previous command for which a good

status has already been returned.

Byte 1    The segment number is always zero.

Byte 2    If the incorrect length indicator (ILI) bit is zero, the re-

quested block of data from the previous command did not

match the logical block length of the data on the disc. If

the ILI bit is one, the requested block of data from the

previous command matched the logical block length of the

data on the disc.

The sense key indicates one of nine general error cate-

gories. These error categories are listed in Appendix B.2.

Bytes 3--6     When the valid bit is 1, the information bytes contain

the logical block address of the current logical block

associated with the sense key. For example, if the sense

key is media error, the information bytes contain the

logical block address of the offending block.

Byte 7    The additional sense length is limited to a maximum of 0EH

additional bytes. If the allocation length of the command

descriptor block is too small to accommodate all of the

additional sense bytes, the additional sense length is not

adjusted to reflect the truncation.

Bytes 8--11    These bytes contain command-specific data.

Bytes 12--13   The additional sense code and additional sense code

qualifier provide additional details about errors. See Ap-

pendix B.3.

Byte 14   The field replaceable unit (FRU) code is used by field

service personnel only.

Bytes 15--22   These bytes are not used and are always zero.

B.2  Sense key

===============================================================

The sense keys in the lower-order bits of byte 2 of the sense data

returned by the Request Sense command are described in the following

table. You can find a more detailed description of the error by

checking the additional sense code and the additional sense code

qualifier in Section B.3.

Sense key Description

0H        No Sense. In the case of a successful command, no specific

sense key information needs to be reported for the drive.

1H        Recovered error. The drive completed the last command

successfully with some recovery action. When many recovered

errors occur during one command, the drive determines which

error it will report.

2H        Not ready. The addressed logical unit cannot be accessed.

Operator intervention may be required to correct this

condition.

3H        Medium error. The command was terminated with a

nonrecoverable error condition, probably caused by a flaw in

the media or an error in the recorded data.

4H        Hardware error. The drive detected a nonrecoverable hardware

failure while performing the command or during a self-test.

This includes, for example, SCSI interface parity errors,

controller failures and device failures.

5H        Illegal request. An illegal parameter in the command

descriptor block or in the additional parameters supplied as

data for some commands (for example, the Format Unit

command, the Mode Select command and others). If the drive

detects an invalid parameter in the CDB, it terminates the

command without altering the media. If the drive detects an

invalid parameter in the additional parameters supplied as

data, the drive may have already altered the media.

6H        Unit attention. The drive may have been reset. See the

Seagate SCSI-2 Interface Manual for more details about the

Unit Attention condition.

BH        Aborted command. The drive aborted the command. The

initiator may be able to recover by retrying.

EH        Miscompare. The source data did not match the data read from

the media.

B.3  Additional sense code and additional sense code qualifier

===============================================================

The additional sense code and additional sense code qualifiers

returned in byte 12 and byte 13, respectively, of the Sense Data

Format of the Request Sense command are listed in the following table.

Error code (hex)  Description

Byte 12 Byte 13

00      00      No additional information is supplied.

01      00      There is no index/sector signal.

02      00      There is no seek complete signal.

03      00      A write fault occurred.

04      00      The drive is not ready and the cause is not

reportable.

04      01      The drive is not ready, but it is in the process of

becoming ready.

04      02      The drive is not ready; it is waiting for the

initializing command.

04      03      The drive is not ready; human intervention is

required.

04      04      The drive is not ready; the format routine is in

process.

05      00      The drive does not respond when it is selected.

06      00      Track 0 was not found.

07      00      More than one drive is selected at a time.

08      00      There was a drive communication failure.

08      01      A drive communication time-out occurred.

08      02      A drive communication parity error occurred.

09      00      A track following error occurred.

0A      00      An error log overflow occurred.

0C      01      A write error occurred, but the error was recovered

using auto-reallocation.

0C      02      A write error occurred. Auto-reallocation was

attempted, but it failed.

10      00      An ID CRC or ECC error occurred.

11      00      An unrecovered read error occurred.

11      01      The read retries were exhausted.

11      02      The error was too long to correct.

11      03      There were multiple read errors.

11      04      A read error occurred. Auto-reallocation was

attempted, but it failed.

12      00      The address mark was not found in the ID field.

13      00      The address mark was not found in the data field.

14      00      No record was found.

14      01      No record was found.

15      00      A seek positioning error occurred.

15      01      A mechanical positioning error occurred.

15      02      A positioning error was detected by reading the media.

16      00      A data synchronization mark error occurred.

17      00      The data was recovered without applying error

correction or retrying.

17      01      The data was recovered with retries.

17      02      The data was recovered with positive head offset.

17      03      The data was recovered with negative head offset.

17      05      The data was recovered using the previous sector ID.

17      06      The data was recovered without ECC. The drive uses

data auto-reallocation.

18      00      The data was recovered with ECC.

18      01      The data was recovered with ECC and retries.

18      02      The data was recovered with ECC, retries, and

auto-reallocation.

19      00      There is an error in the defect list.

19      01      The defect list is not available.

19      02      There is an error in the primary defect list.

19      03      There is an error in the grown defect list.

1A      00      A parameter overrun occurred.

1B      00      A synchronous transfer error occurred.

1C      00      The defect list could not be found.

1C      01      The primary defect list could not be found.

1C      02      The grown defect list could not be found.

1D      00      During a verify operation, a compare error occurred:

the source data did not match the data read from the

media.

1E      00      An ID error was recovered.

20      00      The drive received an invalid command operation code.

21      00      The logical block address was not within the

acceptable range.

22      00      The drive received a CDB that contains an invalid bit.

(This error code applies to direct-access devices.)

24      00      The drive received a CDB that contains an invalid bit.

(This error code applies to all SCSI devices.)

25      00      The drive received a CDB that contains an invalid LUN.

26      00      The drive received a CDB that contains an invalid

field.

26      01      The drive received a CDB containing a parameter that

is not supported.

26      02      The drive received a CDB containing an invalid

parameter.

26      03      The drive received a CDB containing a threshold

parameter that is not supported.

29      00      A power-on reset or a bus device reset occurred.

2A      00      Some parameters were changed by another initiator.

2A      01      The Mode Select parameters were changed by another

initiator.

2B      00      The microcode was downloaded.

2F      00      The tagged commands were cleared by another initiator.

30      01      The media cannot be read because the format is not

recognized.

30      02      The media cannot be read because the format is

incompatible with certain parameters.

31      00      The media format is corrupted.

31      01      The format command failed.

32      00      There are no spare defect locations available.

32      01      An error occurred when the defect list was being

updated.

37      00      A rounded parameter caused an error.

3D      00      The identify message contains invalid bits.

3F      00      The target operation command was changed.

3F      01      The microcode was changed.

3F      02      The drive was operating as a SCSI drive and is now

operating as a SCSI-2 drive, or vice versa.

3F      03      The inquiry data was changed.

40      00      The RAM failed.

40      8x      A correctable ECC error occurred; x equals the length

of the error.

40      90      A configuration error occurred.

40      A0      The self-test routine discovered an error in a ROM.

40      A1      The self-test routine discovered an error in the

processor RAM.

40      A2      The self-test routine discovered an error in the

buffer RAM.

40      A3      The self-test routine discovered a SCSI protocol

error.

40      A4      The self-test routine discovered a DMA error.

40      A5      The self-test routine discovered an error in the disc

sequencer.

40      A6      The self-test routine discovered an error in the disc

sequencer RAM.

40      A7      A self-test error occurred.

40      A8      The flash memory cannot be read or written.

40      A9      The flash memory directory cannot be read, or it is

corrupted.

40      AA      The flash memory contains an incompatible version

number.

40      AB      The flash memory contains an incompatible revision

number.

40      AC      A flash memory checksum error occurred.

40      AD      The flash memory contains invalid parameters.

40      AE      The flash memory is incompatible with the HDA and the

circuit board. The flash memory must be reconfigured.

40      B0      The servo command timed out.

40      B1      The servo command failed.

40      B2      The servo command was rejected.

40      B3      The servo interface does not work.

40      B4      The servo either failed to lock on track during spinup

or has wandered off track.

40      B5      An internal servo error occurred.

40      B6      During spinup, a servo error occurred.

40      B7      The servo pattern is inconsistent.

40      B8      A seek recovery error occurred.

40      B9      The actuator did not achieve high-speed calibration.

40      C0      The defect list is full.

40      C1      A failure occurred while the grown defect list was

being written.

40      C2      The write life-cycle of the flash memory has been

exceeded.

40      C3      There was an attempt to add an illegal entry to the

grown defect list.

40      C4      There was an attempt to add a duplicate entry to the

grown defect list.

41      00      A data path diagnostic failed.

42      00      A power-on or self-test failure occurred.

43      00      A message reject error occurred.

44      00      An internal controller error occurred.

45      00      An error occurred during a selection or a reselection.

47      00      A SCSI interface bus parity error occurred.

48      00      The initiator has detected an error.

49      00      The initiator received an invalid message from the

drive.

4C      00      The drive failed to self-configure.

4E      00      The drive attempted to perform overlapped commands.

5B      00      There was a log exception.

5B      01      A threshold condition was met.

5B      02      The log counter has reached its maximum value.

5B      03      All the log list codes have been used.

5C      00      There was a change in the RPL status. The drive lost

synchronization.

Appendix C. Mode pages

===============================================================

Mode pages are groups of parameters stored by the drive. These

parameters can be read using the Mode Sense command and changed using

the Mode Select command. These commands are described in Sections

3.4.10 and 3.4.13.

This appendix contains the default parameters and the changeable

parameters for the mode pages. The current parameters and the saved

parameters are not shown.

Note. The default values contained in this appendix may differ from

the default values actually contained in your drive. To determine the

default values, use the Mode Sense command.

Contains

changeable

Mode page                       Page code       Bytes   parameters

Error Recovery page             01H               10      Yes

Disconnect/Reconnect page       02H               14      Yes

Format Device page              03H               22      Yes

Rigid Disc Geometry page        04H               22      Yes

Caching page (SCSI-3)           08H               18      Yes

Control Mode page               0AH               10      Yes

Notch page                      0CH               22      No

Cache Control page              38H               14      No

Soft ID page                    3CH               1       Yes

Operating page                  00H               2 or 3  Yes

For all mode pages:

- If the changeable value is  0, the initiator cannot change the bit

directly. If the changeable value is 1, the initiator can change the

bit directly.

For example, in the header below, the changeable value for the page

code bits is 0, which means that the page code cannot be changed; the

changeable value of the PS bit is one, which means that the PS bit can

be changed.

- During the Mode Sense command, the PS (parameter savable) bit is 1,

which means the mode page is saved on the disc. During the Mode

Select command, you must set the PS bit to 0.

- An X means that the value of the bit cannot be specified. For

example, the default value of bit 0 of byte 1 of page 00H (the

Operating Page) cannot be specified because the bit can be either 1

or 0.

All mode pages contain a 2-byte header that contains the page code and

the page length for that particular page. The header is shown below.

Bits           7       6       5       4       3       2       1       0

Bytes

0              PS      Page code

changeable     1       0       0       0       0       0       0       0

1              Page length

changeable     00H

Byte 0    During the Mode Sense command, the PS (parameter savable)

bit is 1, which means the mode page is saved on the disc.

During the Mode Select command, you must set the PS bit to

0. The page code is the unique code that identifies the

page.

Byte 1    The page length is the length, in bytes, of the page.

C.1  Error Recovery page (01H)

===============================================================

The Error Recovery page is shown below. This table summarizes the

function, the default value and the changeability of each bit.

Bits           7       6       5       4       3       2       1       0

Bytes

0              PS (1)  Page code (01H)

1              Page length (0AH)

2              AWRE    ARRE    TB      RC      EER     PER     DTE    DCR

default        0       0       0       0       0       0       0       0

changeable     1       1       1       1       1       1       1       1

3 (default)    Read retry count (20H)

changeable     FFH

4 (default)    Correction span (16H)

changeable     00H

5 (default)    Head offset count (00H)

changeable     00H

6 (default)    Data strobe offset count (00H)

changeable     00H

7 (default)    Reserved (00H)

changeable     00H

8 (default)    Write retry count (20H)

changeable     00H

9 (default)    Reserved (00H)

changeable     00H

10--11 (default) Recovery time limit (FFFFH)

changeable     0000H

Byte 2    When the automatic write reallocation enabled (AWRE) bit is

1, the drive automatically reallocates bad blocks detected

while writing to the disc. When the AWRE bit is 0, the drive

does not perform automatic reallocation; instead, the drive

reports a check condition status with a sense key of media

error.

Note. The AWRE bit does not apply during the Format com-

mand.

When the automatic read reallocation enabled (ARRE) bit is

1, the drive automatically reallocates bad blocks detected

while reading from the disc. When the ARRE bit is 0, the

drive does not automatically reallocate bad blocks. Instead,

a check condition status is reported with a sense key of

media error.

The transfer block (TB) bit is not supported.

When the read continuous (RC) bit is 1, the drive sends all

data without doing any corrections. This function supersedes

other bits in this byte. When the RC bit is 0, the

correction is performed according to the other bits in this

byte.

The enable early recovery (EER) bit is not supported.

The post error (PER) bit is not supported.

The disable transfer on error (DTE) bit is not supported.

When the disable correction (DCR) bit is 1, the drive does

not apply offline ECC to the data even if it can correct the

data.

Byte 3    The read retry count field is the maximum number of times

the drive attempts its recovery algorithms. If the EER bit

of byte 2 is 1, the number of retries specified by read

retry count (up to a maximum of nine retries) is performed

before ECC is applied. The read retry count field has a

range of 0 through 20H. A read retry count of 0 means that

no retries are performed.

Byte 4    The correction span is the size of the largest read data

error, in bits, on which ECC correction is attempted. Longer

errors are reported as nonrecoverable.

Byte 5    The head offset count is not implemented. Head offsets are

performed as part of the drives retry algorithms.

Byte 6    The data strobe offset count is not implemented.

Byte 7    Reserved

Byte 8    The write retry count field contains the maximum number of

times the drive attempts its recovery algorithms. If the EER

bit is set, the number of retries specified by the retry

count, up to a maximum of nine retries, is performed before

ECC is applied. The write retry count field has a range of 0

through 20H. A write retry count of 0 means that no retries

are performed.

Byte 9    Reserved

Bytes 10--11   The recovery time limit field always has a value of

FFFFH, which means that the recovery time is unlimited.

C.2  Disconnect/Reconnect page (02H)

===============================================================

The Disconnect/Reconnect page is shown below. This table summarizes

the function, the default value and the changeability of each bit.

Bits           7       6       5       4       3       2       1       0

Bytes

0              PS (1)  Page code (02H)

1              Page length (0EH)

2 (default)    Buffer full ratio (F0H)

changeable     FFH

3 (default)    Buffer empty ratio (10H)

changeable     FFH

4--5 (default) Bus inactivity limit (0000H)

changeable     0000H

6--7 (default) Disconnect time limit (0000H)

changeable     0000H

8--9 (default) Connect time limit (0000H)

changeable     0000H

10--11 (default) Maximum burst size (0000H)

changeable     0000H

12--15 (default) Reserved (00000000H)

changeable     00000000H

Byte 2    The buffer full ratio field indicates, on Read commands, how

full the drives buffer is before reconnecting. The drive

rounds up to the nearest whole logical block. This parame-

ter is the numerator of a fraction that has 256 as its

denominator.

Byte 3    The buffer empty ratio field indicates, on Write commands,

how empty the drives buffer is before reconnecting to

fetch more data. The drive rounds up to the nearest whole

logical block. This parameter is the numerator of a fraction

that has 256 as its denominator.

Bytes 4--5     The bus inactivity limit field indicates the time, in

100-msec increments, that the drive can assert the Busy

signal without handshakes until it disconnects. The drive

can round down to its nearest capable value. If the bus

inactivity limit is 0000H, the drive maintains the BSY--

signal for 1 msec without handshakes.

Bytes 6--7      The disconnect time limit field indicates the minimum

time, in 100-msec increments, the drive remains discon-

nected until it attempts to reconnect. A value of 0

indicates that the drive is allowed to reconnect

immediately.

Bytes 8--9      The connect time limit field indicates the maximum

time, in 100-msec increments, that the drive should remain

connected until it attempts to disconnect. The drive may

round to its nearest capable value. A value of 0 means that

the drive can remain connected indefinitely until it tries

to disconnect.

Bytes 10--11   The maximum burst size field limits the amount of data

that can be transferred during the data phase before the

drive disconnects from the host. The value, multiplied by

512, indicates the maximum number of bytes that can be

contained in a single burst. A value of 0 means that there

is no limit to how many bytes can be transferred during a

single burst.

Bytes 12--15   Reserved

C.3  Format Device page (03H)

===============================================================

The Format Device page is shown below. This table summarizes the

function, the default value and the changeability of each bit. This

page is sent only before the Format Unit command is sent. The drive

parameters are updated immediately, but any changes between these

current parameters and the existing media format do not take effect

until after the Format Unit command is completed.

Bits           7       6       5       4       3       2       1       0

Bytes

0              PS (1)  Page code (03H)

1              Page length (16H)

2--3 (default) Tracks per zone (0001H)

changeable     0000H

4--5 (default) Alternate sectors per zone (0001H)

changeable     0000H

6--7 (default) Alternate tracks per zone (0000H)

changeable     0000H

8--9 (default) Alternate tracks per volume (0008H)

changeable     0000H

10--11 (default) Sectors per track (0058H)

changeable     0000H

12--13 (default) Data bytes per physical sector (0200H)

changeable     0000H

14--15 (default) Interleave (0001H)

changeable     0000H

16--17 (default) Track skew factor (0002H)

changeable     0000H

18--19 (default) Cylinder skew factor (0009H)

changeable     0000H

20             SSEC    HSEC    RMB     SURF    Reserved

default        1       0       0       0

changeable     0       0       0       0       0       0       0       0

21--23 (default) Reserved (000000H)

changeable     000000H

Bytes 2--3     The tracks per zone field indicates the number of

tracks the drive allocates to each defect-management zone.

Spare sectors or tracks are placed at the end of each

defect- management zone. If each zone is treated as

containing one track, the valid value for tracks per zone is

1. If each zone is treated as containing one cylinder, the

valid value is equal to the number of read/write heads.

Bytes 4--5      The alternate sectors per zone field indicates the

number of spare sectors to be reserved at the end of each

defect- management zone. The drive defaults to one spare

sector per zone. If each zone is treated as containing one

track, the valid value for alternate sectors per zone is 1.

If each zone is treated as containing one cylinder, the

valid values are 1 through 3.

Bytes 6--7      The alternate tracks per zone field indicates the

number of spare tracks the drive reserves at the end of each

defect-management zone. A value of 0 indicates that no spare

tracks are reserved at the end of each zone for defect

management.

Bytes 8--9      The alternate tracks per volume field indicates the

number of spare tracks to be reserved at the end of the

drive for defect management. The default is equal to twice

the number of read/write heads.

Bytes 10--11   The sectors per track field indicates the number of

physical sectors the drive allocates per track. The drive

reports the average number of physical sectors per track

because the number of sectors per track varies between the

outer and inner tracks.

Bytes 12--13   The data bytes per physical sector field indicates the

number of data bytes allocated per physical sector.

Bytes 14--15   The interleave field is the interleave value sent to

the drive during the last Format Unit command. This field is

valid only for Mode Sense commands. The drive ignores this

field during Mode Select commands. The interleave is always

1:1.

Bytes 16--17   The track skew factor field indicates the number of

physical sectors on the media between the last logical

block of one track and the first logical block of the next

sequential track of the same cylinder. The actual track skew

factor that the drive uses is different for every zone. The

default value is 0002H, which is the track skew factor for

the first zone. This default value is only used when tracks

per zone and alternate sectors per zone are set to 1.

Bytes 18--19   The cylinder skew factor field indicates the number of

physical sectors between the last logical block of one

cylinder and the first logical block of the next cylinder.

The actual cylinder skew factor that the drive uses depends

on the zone. The default value is 0009H, which is the

cylinder skew factor for the first zone.

Byte 20   The drive type field bits are defined as follows:

The soft sectoring (SSEC) bit is set to 1. This bit is

reported as not changeable. Although it can be set to

satisfy system requirements, it does not affect drive per-

formance.

The hard sectoring (HSEC) bit is set to 0. This bit is

reported as not changeable. Although it can be set to

satisfy system requirements, it does not affect drive per-

formance.

The removable media (RMB) bit is always set to 0, indi-

cating that the drive does not support removable media. This

same bit is also returned in the Inquiry parameters.

The surface map (SURF) bit is set to 0, indicating that the

drive allocates successive logical blocks to all sectors

within a cylinder before allocating logical blocks to the

next cylinder.

Bytes 21--23   Reserved 92

C.4  Rigid Disc Geometry page (04H)

===============================================================

The Rigid Disc Geometry page is shown below. This table summarizes the

function, the default value and the changeability of each bit.

Bits           7       6       5       4       3       2       1       0

Bytes

0              PS (1)  Page code (04H)

1              Page length (16H)

2--4           Number of cylinders

changeable     000000H

5              Number of heads

changeable     00H

6--8           Starting cylinder for write precompensation

default        000000H

changeable     000000H

9--11          Starting cylinder for reduced write current

default        000000H

changeable     000000H

12--13 (default) Drive step rate (0000H)

changeable     0000H

14--16 (default) Loading zone cylinder (000000H)

changeable     000000H

17             Reserved                                        RPL

default                                                        0       0

changeable     0       0       0       0       0       0       1       1

18 (default)   Rotational offset (00H)

changeable     FFH

19 (default)   Reserved (00H)

changeable     00H

20--21         Media rotation rate

default        1194H

changeable     0000H

22--23 (default) Reserved (0000H)

changeable     0000H

Bytes 2--4     The number of cylinders field specifies the number of

user-accessible cylinders, including two spare cylinders set

aside for defects.  The drive uses the additional cylin-

ders for storing parameters and defect lists or for diagnos-

tic purposes.  The number of cylinders is specified on page

1.

Byte 5    The number of heads field specifies the number of read/write

heads on the drive. The number of heads is specified on page

1.

Bytes 6--16    The starting cylinder for reduced write current,

starting cylinder for reduced read current, drive step rate

and loading zone cylinder bytes are not used by the drive.

Byte 17   When the rotational position locking (RPL) bits are 00Bi-

nary, the rotational position locking is changeable. When

the RPL bits are 01Binary, the drive automatically synchro-

nizes its spindle with the synchronized master. When the RPL

bits are 10Binary or 11Binary, the drive is the synchro-

nized-spindle master.

For more information about external spindle-clock synchronization,

see Section 2.11.

Byte 18   The rotational offset is the rotational skew the drive uses

when synchronized. The rotational skew is applied in the

retarded direction (lagging the sync spindle master). A

value of zero means no rotational offset is used.

Byte 19   Reserved.

Bytes 20--21   The medium rotation rate is the spindle speed, which is

specified on page 1.

Bytes 22--23   Reserved.

C.5  Caching page (08H)

===============================================================

The drive uses read look-ahead, read caching and write caching to

improve seek times and performance.

C.5.1     Read look-ahead and read caching

The drive uses an algorithm that improves seek performance by reading

the next logical sectors after the last requested sector. These unre-

quested sectors are read into a buffer and are ready to be transmitted

to the host before they are requested. Beca use these sectors are read

before they are requested, access read time for the sectors is

virtually eliminated. This process is called either read look-ahead or

read caching.

Read look-ahead and read caching are similar algorithms. Read look-

ahead occurs when a Read command requests more data than can be

contained in one buffer segment. Read caching occurs when a Read

command requests less data than can be contained in one b uffer

segment.

The buffer used for read look-ahead and caching can be divided into

segments as shown in the following table. To change the number of

segments, use byte 13 of the Caching page, which is described in

Appendix C.5.3. The default is one, 256-Kbyte segment.

Number of segments      Size of segment (in Kbytes)

1                        256

2                        128

4                        64

8                        32

16                       16

When the buffer is divided into multiple segments, each segment func-

tions as an independent buffer, causing dramatically increased

performance in multitasking and multiuser environments.

C.5.2     Write caching and write merging

===============================================================

Write caching. The drive uses the write segment to store write com-

mands and data. After the drive caches the commands and data, it is

immediately ready to process new commands. The drive writes the data

to the disc at its next convenient opportunity.

Write merging. The drive accepts contiguous write commands and

executes them sequentially as one command.

C.5.3     Caching page description

The Caching page is shown below. This table summarizes the function,

the default value and the changeability of each bit.

Bits           7       6       5       4       3       2       1       0

Bytes

0              PS (1)  Page code (08H)

1              Page length (12H)

2              IC      ABPF    CAP     DISC    SIZE    WCE     MF     RCD

default        1       0       0       1       0       1       0       0

changeable     1       0       0       0       0       1       1       1

3              Demand read                     Write retention priority

retention priority

default        0       0       0       0       0       0       0       0

changeable     0       0       0       0       0       0       0       0

4--5 (default) Disable prefetch transfer length (FFFFH)

changeable     0000H

6--7 (default) Minimum prefetch (0000H)

changeable     0000H

8--9 (default) Maximum prefetch (FFFFH)

changeable     FFFFH

10--11 (default) Maximum prefetch ceiling (FFFFH)

changeable     FFFFH

12             FSW     Rsrvd   DRA     Reserved

default        0       0       0       0       0       0       0       0

changeable     0       0       1       0       0       0       0       0

13             Number of cache segments

default        0       0       0       0       0       0       0       1

changeable     1       1       1       1       1       1       1       1

14--15 (default) Cache segment size (0000H)

changeable     0000H

16 (default)   Reserved (00H)

changeable     00H

17--19 (default) Noncache segment size (000000H)

changeable     000000H

Byte 2    The initiator control (IC) bit is not supported.

When the abort prefetch (ABPF) bit is 0, the drive controls

completion of prefetch. See the description for the DISC

bit, below. This is the default value and it is not

changeable.

The caching analysis permitted (CAP) bit is not supported.

When the discontinuity (DISC) bit is 1, the drive may

prefetch across cylinder boundaries, where head seeks

consume additional processing time. This is the default

value and it is not changeable.

The size enable (SIZE) bit is not supported.

When the write cache enable (WCE) bit is 0, the drive

returns a good status for a Write command after successfully

writing all the data to the media. When the WCE bit is 1,

the drive returns a good status for a Write command after

successfully receiving the data and before writing it to the

media.

When the multiplication factor (MF) bit is 0, the drive

interprets the minimum prefetch and maximum prefetch fields

as the number of logical blocks to be prefetched. When the

MF bit is 1, the drive interprets the minimum prefetch and

maximum prefetch fields in terms of a number which, when

multiplied by the transfer length of the current command,

yields the number of logical blocks to be prefetched.

When the read cache disable (RCD) bit is 0, the drive may

return data requested by a Read command by accessing either

the cache or the media. If the RCD bit is 1, the cache is

not used.

Byte 3    The demand read retention priority field is not used. The

initiator cannot assign any special retention priority to

the drive.

The write retention priority field is not used. The

initiator cannot assign any special retention priority to

the drive.

Bytes 4--5     The disable prefetch transfer length always has a value

of FFFFH, which means that the drive attempts an antici-

patory prefetch for all Read commands.

Bytes 6--7     The minimum prefetch field specifies the minimum num-

ber of blocks the drive prefetches, regardless of the delays

it may cause in executing subsequent pending commands.

When the minimum prefetch field contains 0, the drive

terminates prefetching whenever another command is ready to

be executed. If the minimum prefetch equals the maximum

prefetch, the drive prefetches the same number of blocks

regardless of whether there are commands pending.

Bytes 8--9     The maximum prefetch field specifies the maximum num-

ber of blocks the drive prefetches during a Read command

if there are no other commands pending. The maximum prefetch

field represents the maximum amount of data to prefetch into

the cache for any single Read command.

Bytes 10--11   The maximum prefetch ceiling field should be equal to

the maximum prefetch field. The maximum prefetch ceiling and

maximum prefetch fields are the same if the MF bit is 0.

Byte 12   The force sequential write (FSW) bit is not supported.

When the disable read-ahead (DRA) bit is 1, the drive does

not read into the buffer any logical blocks beyond the

addressed logical blocks. When the DRA bit equals 0, the

drive can continue reading logical blocks into the buffer

beyond the addressed logical blocks.

Byte 13   The number of cache segments field determines the number of

segments into which the cache should be divided. Valid

values are 1, 2, 4, 8, 16 and 32.

Bytes 14--15   The cache segment size field indicates the segment size

in bytes. The cache segment size field is valid only when

the SIZE bit is 1.

Byte 16   Reserved.

Bytes 17--19   The noncache segment size field always contains zeros.

This means that the entire buffer is available for caching.

C.6  Control Mode page (0AH)

===============================================================

The Control Mode page is shown below. This table summarizes the

function, the default value and the changeability of each bit.

Bits           7       6       5       4       3       2       1       0

Bytes

0              PS (1)  Page code (0AH)

1              Page length (0AH)

2              Reserved                                               RLEC

default        0       0       0       0       0       0       0       0

changeable     0       0       0       0       0       0       0       1

3              Queue algorithm modifier        Reserved        QErr    DQue

default        0       0       0       0       0       0       0       0

changeable     1       1       1       1       0       0       0       1

4              EECA    Reserved                        RAENP   UAAENP  EAENP

default        0       0       0       0       0       0       0       0

changeable     00H

5 (default)    Reserved (00H)

changeable     00H

6--7 (default) Ready AEN hold-off period (0000H)

changeable     0000H

8--9 (default) Busy timeout period (FFFFH)

changeable     0000H

10--11         Reserved (0000H)

changeable     0000H

Byte 2    The RLEC bit is not implemented.

Byte 3    The queue algorithm modifier field is only effective if the

disable queuing bit is zero. When bit 4 in the queue

algorithm modifier field contains one, the drive may use

tagged command queuing to change the order in which it

executes commands. When bit 4 in the queue algorithm

modifier field contains zero, the drive always executes

commands according to the order indicated by the simple

queue tag.

When the disable queuing (DQue) bit is zero, tagged command

queuing is enabled. When the DQue bit is one, tagged command

queuing is disabled.

Byte 4    Not implemented

Byte 5    Reserved

Bytes 6--7     Not implemented

Bytes 8--9     The busy timeout period field contains the maximum

possible value, which means that the drive can remain busy

an unlimited amount of time.

Bytes 10--11   Reserved

C.7  Notch page (0CH)

===============================================================

The Notch page contains parameters that describe the notches. The

table below summarizes the function, default value and the

changeability of each bit.

The drive uses Zone Bit Recording, which means that the outer

cylinders of the disc contain more logical blocks than the inner

cylinders. The cylinders are organized into groups, called zones or

notches. Every logical block is part of a notch. Notches do not

overlap.

Bits           7       6       5       4       3       2       1       0

Bytes

0              PS (1)  Page code (0CH)

1              Page length (16H)

2              ND      LPN     Reserved

default        1       0       0       0       0       0       0       0

changeable     0       0       0       0       0       0       0       0

3 (default)    Reserved (00H)

changeable     00H

4--5 (default) Maximum number of notches (0013H)

changeable     0000H

6--7 (default) Active notch (0000H)

changeable     0       0       0       1       1       1       1       1

8--11 (default) Starting boundary (00000000H)

changeable     00000000H

12--15 (default) Ending boundary (000BB903H)

changeable     00000000H

16--23 (default) Pages notched (0000000000000008H)

changeable     0000000000000000H

Byte 2    The notched drive (ND) bit is always 1, which means the disc

contains notches of different recording densities. For each

supported active notch value, this page defines the starting

and ending boundaries of the notch.

The logical or physical notch (LPN) bit is 0, which means

the notch boundaries are based on the physical parameters

of the logical unit. The cylinder is most significant; the

head is least significant.

Byte 3    Reserved.

Bytes 4--5     The maximum number of notches field indicates the maxi-

mum number of notches supported by the drive.

Bytes 6--7     The active notch field identifies the notch to which

this, and all future Mode Select and Mode Sense commands

refer, until the active notch is changed by a later Mode

Select command. The value of the active notch field must be

greater than or equal to 0 and less than or equal to the

maximum number of notches. An active notch value of 0 means

that current and future Mode Select and Mode Sense commands

refer to the parameters that apply for all notches.

Bytes 8--11    The starting boundary field indicates the beginning of

the active notch if the active notch is not 0, or the

starting boundary of the logical unit if the active notch is

0. This field is ignored by the Mode Select command. When

the LPN bit is 0, the three most significant bytes represent

the cylinder number and the least significant byte

represents the head number.

Bytes 12--15   The ending boundary field indicates the end of the

active notch if the active notch is not 0, or the end of the

logical unit if the active notch is 0. The default is equal

to the end of zone 1.

When the LPN bit is 0, the three most significant bytes

represent the cylinder number and the least significant byte

represents the head number.

Bytes 16--23   The pages notched field contains a bit map of the mode

page codes that indicates which pages may contain dif-

ferent parameters for each notch. When a bit is 1, the

corresponding mode page can contain different parameters

for each notch. When a bit is 0, the corresponding mode page

contains the same parameters for all the notches. The most

significant bit of this field corresponds to page code 3FH

and the least significant bit corresponds to page code 00H.

C.8  Cache Control page (38H)

===============================================================

The Cache Control page is shown below. This table summarizes the

function, the default value and the changeability of each bit.

Bits           7       6       5       4       3       2       1       0

Bytes

0              PS (1)  Page code (38H)

1              Page length (0EH)

2              Rsrvd   WIE     Rsrvd   CE      Cache table size

default        0       X       0       X       X       X       X       X

changeable     (00H)

3 (default)    Prefetch threshold (00H)

changeable     00H

4 (default)    Maximum prefetch (FFH)

changeable     00H

5 (default)    Maximum prefetch multiplier (00H)

changeable     00H

6 (default)    Minimum prefetch (00H)

changeable     00H

7 (default)    Minimum prefetch multiplier (00H)

changeable     00H

8--15 (default Reserved (0000000000000000H)

changeable     0000000000000000H

Byte 2    The cache enable (CE) bit is always the inverse of the RCD

bit in Mode Page 08H.

The write index enable (WIE) bit controls the creation of

cache data on Write commands. If bit 6 is 0, the next

command treats the cache area as empty.

The cache table size field contains the same values as Mode

Page 08H, byte 13, bits 3 through 0.

Byte 3    The prefetch threshold  is not implemented. The drive reads

until the buffer is full upon receipt of a Read command.

Byte 4    The maximum prefetch field always contains the same value as

byte 9 of the Caching page. The initiator cannot change this

byte directly.

Byte 5    The maximum prefetch multiplier field always contains the

same value as byte 9 of the Caching page, which is de-

scribed in Appendix C.5.3. The initiator cannot change this

byte directly.

Byte 6    The minimum prefetch field always contains the same value as

byte 7 of the Caching page. The initiator cannot change this

byte directly.

Byte 7    The minimum prefetch multiplier field always contains the

same value as byte 7 of the Caching page. The initiator

cannot change this byte directly.

Byte 8--15     Reserved.

C.9  Soft ID page (Flash memory) (3CH)

===============================================================

The Soft ID page is shown below. This table summarizes the function,

the default value and the changeability of each bit. This page is

saved in flash memory that has a life span of 10,000 writes.

Bits           7       6       5       4       3       2       1       0

Bytes

0              PS (1)  Page code (3CH)

1              Page length (01H)

Soft    Soft   Param   Soft     Remote ID 2    ID 1    ID 0

2              ID      Parity enable  remote   S/S

default        0       0       0       0       0       0       0       0

changeable     1       1       1       1       1       1       1       1

Byte 2    When the soft ID bit is 0, the drive ignores ID0, ID1 and

ID2 and uses the SCSI ID jumpers to determine the SCSI ID.

When the soft ID bit is 1, the drive ignores the SCSI ID

jumpers and uses ID0, ID1 and ID2 to determine the SCSI ID.

See Figure 5 on page 21 for jumper settings.

When the soft parity bit is 0, the drive uses the parity

jumper settings to determine whether the drive uses parity.

When the soft parity bit is 1, the drive ignores the parity

jumper settings.

When the soft remote bit is 0, the drive uses the remote

start jumper setting to determine whether remote start is

implemented. When the soft remote bit is 1, the drive

ignores the jumpers and uses the remote S/S bit to determine

whether remote start is implemented.

Byte 2    When the remote S/S bit is 0, the drive spins up after a

delay specified by the spinup delay field (byte 4 of the

Operating page, 00H). When the remote S/S bit is 1, the

drive spins up when it receives the Start Unit command. This

bit is only valid if the soft remote bit is 1.

The ID0, ID1 and ID2 bits are the SCSI ID bits. These bits

are only valid when the soft ID bit is 1.

When the param enable bit is 0, the drive does not check

parity. When the param enable bit is 1, the drive checks

parity. This bit is only valid if the soft parity bit is 1.

C.10 Operating page (Flash memory) (00H)

===============================================================

The Operating page is shown in the table below. This table shows the

function, the default value and the changeability of each bit.

The drive accepts an Operating page of two lengths: two bytes or three

bytes. If the length is two bytes, then byte 4, the spinup delay

field, is not written and is assumed to be unchanged.

In addition to being saved on the media, this vendor-unique page is

saved in flash memory that has a life span of 10,000 writes.

Bits           7       6       5       4       3       2       1       0

Bytes

0              PS (1)  Page code (00H)

1 (default)    Page length (02H or 03H)

changeable     0       0       0       0       0       0       1       X

2              Usage   SSM     RSVD    ATOFF   Reserved

default        1       0       0       0       0       0       0       0

changeable     1       1       0       1       0       0       0       0

3 (default)    Rsrvd   Device type qualifier (00H)

changeable     00H

4 (default)    Spinup delay (00H)

changeable     00H

Byte 2    When the usage bit is 1, a warning message is enabled. When

the write life span of the flash memory is exceeded, a

warning message is generated. See additional sense error

code C2 in Appendix B.3. When the usage bit is 0, the

warning message is disabled. If requested, the flash memory

data and the write counter is updated even after the write

life span is exceeded, but the integrity of the data cannot

be assured.

Byte 2    When the synchronous select mode (SSM) bit is 0, the drive

does not send a synchronous data transfer message unless the

initiator has already issued a synchronous data transfer

message. When the SSM bit is 1, the drive can send a

synchronous data transfer message, even when the initiator

has not sent a synchronous data transfer message.

When the disable unit attention (ATOFF) bit is 0, the drive

generates a unit attention condition during power up. When

the disable unit attention (ATOFF) bit is 1, the drive does

not generate a unit attention condition during power up.

Byte 3    The device type qualifier field is not supported.

Byte 4    The spinup delay field controls the drive when it is not in

the remote mode. When the value is 00H, the drive spins up

without delay. When the value is FFH, the drive delays

spinup to a duration whose value in seconds equals five

times the drives SCSI bus ID number. When the value is

between 01H and FEH, the drive delays spinup for the

corresponding decimal duration, in seconds.

Appendix D. Inquiry data

===============================================================

When the initiator issues an Inquiry command, the drive returns either

of the following two types of data, depending on the value in the EVPD

bit in byte 1 of the Inquiry command descriptor block:

- Inquiry data

- Vital product data

Both types of data are discussed in this appendix. The Inquiry command

is described in Section 3.4.9.

D.1     Inquiry data

===============================================================

When the initiator issues an Inquiry command, and the EVPD bit in byte

1 of the Inquiry command descriptor block is 0, the drive returns the

following data. If the EVPD bit in byte 1 of the Inquiry command

descriptor block is 1, see Appendix D.2.

Bit      7       6       5       4       3       2       1       0

Byte

0        Peripheral              Peripheral

qualifier               device type

0       0       0       0       0       0       0       0

1        RMB     Device type modifier

0       0       0       0       0       0       0       0

2        ISO             ECMA                    ANSI version

0       0       0       0       0       0       1       0

3        AENC  TrmIOP            Response data format

0       0       Resrvd          0       0       1       0

4        Additional length (8FH)

5--6     Reserved (00H)

7        Rel Adr Wbus32  Wbus16  Sync    Linked  0       CmdQue  Sft Re

8--15    Vendor identification

16--31   Product identification

32--35   Product revision level

36--43   Drive serial number

44--95   Reserved

96--143  Copyright notice

144--147 Servo PROM part number

Byte 0  The peripheral qualifier field contains zero, which means that

the drive is currently connected to the logical unit that is

issuing the Inquiry command. The peripheral device type field

contains zero, which means that the drive is a direct access

device.

Byte 1  The RMB bit is 0, which means the discs are not removable. The

device type modifier is not used.

Byte 2  The ISO version field contains zero, which means that we do

not claim compliance with ISO 9316.

The EMCA version field contains zero, which means that we do

not claim compliance with EMCA-111.

The ANSI version field contains two, which means that the

drive complies with ANSI SCSI-2 standard X3.131-199x.

Byte 3  The asynchronous event notification (AENC) bit is zero, which

means that the drive does not support asynchronous event

notification.

The terminate I/O process (TrmIOP) bit is zero, which means

that the drive does not support the terminate I/O process

message.

The response data format field contains two, which means that

the inquiry data is in standard SCSI-2 format.

Byte 4  The additional length field contains 143, which is the num-

ber of bytes contained in the inquiry data beyond byte 4. This

value represents a total inquiry data length of 148 bytes. If

the allocation length in the CDB of the Inquiry command is

less than 148, the inquiry data is truncated, but the

additional length does not change.

Bytes 5--6      Reserved

Byte 7  The RelAdr bit is one, which means that the drive supports the

relative addressing mode.

The WBUS32 bit is zero, which means that the drive does not

support 32-bit data transfers.

The WBUS16 bit is zero, which means that the drive does not

support 16-bit data transfers.

The SYNC bit is one, which means that the drive supports

synchronous data transfer.

The Linked bit is one, which means that the drive supports

linked commands.

The CmdQue bit is one, which means that the drive supports

tagged command queuing.

The Soft Re bit is zero, which means that the drive responds

to a reset with a hard reset.

Bytes 8--15     The vendor identification field contains "SEAGATE"

in ASCII text.

Bytes 16--31    The product identification field contains the model

num- ber of the drive in ASCII text.

Bytes 32--35    The product revision level field contains the last

four digits of the firmware release number in ASCII.

Bytes 36--43    The drive serial number field contains the serial

number of the drive in ASCII.

Bytes 44--95    These bytes are reserved; they contain only zeros.

Bytes 96--143The copyright notice field contains the following ASCII

string: "Copyright (c) 1993 Seagate. All rights reserved."

Bytes 144--147  The servo PROM part number field is reserved.

D.2     Vital product data pages

===============================================================

When the initiator issues an Inquiry command, and the EVPD bit in byte

1 of the Inquiry command descriptor block is 1, the drive returns

vital product data pages. If the EVPD bit in byte 1 of the Inquiry

command descriptor block is 0, see Appendix D.1.

All vital product data pages contain a 4-byte header, shown below.

Bits     7       6       5       4       3       2       1       0

Bytes

0       Peripheral qualifier     Peripheral device type

1       Page code

2       Reserved (00H)

3       Page length

Byte 0  The peripheral qualifier field contains zero, which means that

the drive is currently connected to the logical unit issuing

the Inquiry command.

The peripheral device type field contains zero, which means

that the drive is a direct-access device.

Byte 1  The page code field contains the same value contained in the

page code field in byte 2 of the Inquiry command descriptor

block.

If the page code field contains any of the page codes shown in

the table below, the drive returns the corresponding page. The

available page codes are:

Page code       Description

00H             Supported vital product data pages

80H             Unit serial number page

81H             Implemented operating definitions page

C0H             Firmware numbers page (vendor-unique)

C1H             Date code page (vendor-unique)

C2H             Jumper settings page (vendor-unique)

Byte 2  Reserved

Byte 3  The page length field contains the length of the supported

page list.

D.2.1   Unit Serial Number page (80H)

The Unit Serial Number page is shown below. The table summarizes the

function and the default value of each bit.

Bits     7       6       5       4       3       2       1       0

Bytes

0       Peripheral qualifier     Peripheral device type

1       Page code (80H)

2       Reserved (00H)

3       Page length (0EH)

4--17   Product serial number

Bytes 4--17     The product serial number field contains the serial

number for the drive in ASCII. If the drive does not return

the serial number, it returns spaces (20H).

D.2.2   Implemented Operating Definition page (81H)

The Implemented Operating Definition page is shown below. The table

summarizes the function and the default value of each bit.

Bits     7       6       5       4       3       2       1       0

Bytes

0       Peripheral qualifier     Peripheral device type

1       Page code (81H)

2       Reserved (00H)

3       Page length (05H)

4       SAVIMP   Current operating definition

0

5       SAVIMP   Default operating definition

0

6--8    SAVIMP   Supported operating definition

0

Byte 4  The current operating definition field contains the value of

the current operating definition.

Byte 5  The SAVIMP bit is always zero; therefore, the current

operating definition parameter cannot be saved. If the SAVIMP

bit is one, the current operating parameter can be saved.

The default operating definition field contains the value of

the default operating definition. If no operating definition

is saved, the drive uses the default operating definition.

Bytes 6--8      If the SAVIMP bit is zero, the default definition

parameter cannot be saved. If the SAVIMP bit is one, the

default definition parameter can be saved.

The supported operating definition field contains the value of

the supported operating definition. If no supported oper-

ating definition is saved, the drive uses the default

operating definition.

D.2.3   Firmware Numbers page (C0H)

The Firmware Numbers page is shown below. The table summarizes the

function, and default value of each bit.

Bits     7       6       5       4       3       2       1       0

Bytes

0       Peripheral qualifier     Peripheral device type

1       Page code (C0H)

2       Reserved (00H)

3       Page length (10H)

4--7    Download firmware number

8--11   Controller PROM number

12--15  Servo PROM number

16--19  EEPROM image number

Bytes 4--8  The download firmware number field contains the firm- ware

number in ASCII.

Bytes 9--11The controller PROM number field contains the controller

PROM number in ASCII.

Bytes 12--15The servo PROM number field contains the servo PROM in

ASCII.

Bytes 16--19The EEPROM image number field contains the EEPROM image

number in ASCII.

D.2.4   Date Code page (C1H)

The Date Code page is shown below. The table summarizes the function

and the default value of each bit.

Bits     7       6       5       4       3       2       1       0

Bytes

0       Peripheral qualifier     Peripheral device type

1       Page code (C1H)

2       Reserved (00H)

3       Page length (03H)

4       Year

5--6    Week

Bytes 4 The year field contains the year, in ASCII, that the firmware

was released.

Bytes 5--6      The week field contains the week, in ASCII, that the

firm- ware was released.

D.2.5   Jumper Settings page (C2H)

The Jumper Settings page is shown below. The table summarizes the

function and the default value of each bit.

Bits     7       6       5       4       3       2       1       0

Bytes

0       Peripheral qualifier     Peripheral device type

1       Page code (C2H)

2       Reserved (00H)

3       Page length (01H)

4       Rsrvd                     MS     PE      SCSI ID

Byte 4  If the motor start (MS) bit is 1, the remote start enable

jumper is installed on pins 3 and 4 of  the options jumper

block. If the MS bit is 0, the remote start enable jumper is

not installed.

If the parity enable (PE) bit is 1, the parity enable jumper

is installed on pins 1 and 2 of  the options jumper block. If

the PE bit is 0, the parity enable jumper is not installed.

SCSI ID is the SCSI ID of the drive.

=====Rev=Hist===============

Written with Borland Pascal v6.0 by B.Rudock, R.Dimick and R.Stacy.

Revision History:

7/94   v1.0    Public release.

=====Legal=Disclaimer=======

LICENSE AGREEMENT

Seagate provides the accompanying object code software ("Software")

and nonexclusively licenses its use on the following terms and

conditions. The Software is copyrighted by Seagate.  YOU ASSUME FULL

RESPONSIBILITY FOR THE SELECTION OF THE SOFTWARE TO ACHIEVE YOUR

INTENDED PURPOSES, FOR THE PROPER INSTALLATION AND USE.  SEAGATE DOES

NOT WARRANT THAT THE SOFTWARE WILL MEET YOUR REQUIREMENTS, THAT THE

SOFTWARE IS FIT FOR ANY PARTICULAR PURPOSE OR THAT THE USE OF THE

SOFTWARE WILL BE ERROR FREE.  SEAGATE EXPRESSLY DISCLAIMS ALL

WARRANTIES, WHETHER ORAL OR WRITTEN, EXPRESSED OR IMPLIED, INCLUDING

WITHOUT LIMITATION WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A

PARTICULAR PURPOSE.  IN NO EVENT WILL SEAGATE BE LIABLE TO YOU, YOUR

CUSTOMERS OR OTHER USERS FOR ANY INDIRECT, INCIDENTAL, CONSEQUENTIAL,

SPECIAL OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE

USE OR INABILITY TO USE THE SOFTWARE.

End of License agreement.

=====Other==================

Available on the Seagate Tech Support BBS (408)438-8771 and many fine

and helpful BBS's around the world:

Specifications and jumper drawings for all Seagate Disc Drives and

Controllers.

Reprints of Installation Guides.

Troubleshooting essays.

FINDTYPE - Utility which displays bios drive type table and matches a

Seagate model to the best drive type.  Also prints complete

specifications lists and much more!

FINDINIT - Utility for Seagate controllers and host adapters that have

onboard bios. Queries the system to determine bios memory address and

initiates low-level format.

DESK REFERENCE - Hypertext data system for all Seagate products,

troubleshooting, other OEM phone numbers and much, much more. A must

for dealers who do a fair amount of support for Seagate products.

SEAGATE FORMAT - A lo-level and mid-level formatting utility for AT

systems (286+) which supports MFM, RLL, ESDI and ATA interface drives.

FIND-ATA - An ATA interface identify drive utility. Ask the drive directly

to determine cylinder, head and sectors, Fast-ATA features like Read/Write

Multiple mode, PIO modes, DMA modes and more.  Save the data to a file.

Supports Primary and Secondary port addresses.

-=EOF: ASPI-ID.DOC=-