(C) '94 CARD GUIDE     GLOSSARY OF TERMS/KEY TO ABREVIATIONS

COMMON SERIAL PORT ADDRESSES                  PRINTER PORT ADDRESSES

----------------------------                  ----------------------

COM1 - 3F8  NORMALLY IRQ4                     * LPT1: 378  IRQ7

COM2 - 2F8  NORMALLY IRQ3                       LPT2: 278  IRQ5

COM3 - 3E8  NORMALLY IRQ4 (SHARED WITH COM1)

COM4 - 2E8  NORMALLY IRQ3 (SHARED WITH COM2)  * ON SOME BRAND NAME PCS

LPT1 WILL BE AT 3BC & LPT2

& LPT3 WILL BE MOVED TO

OTHER ADDRESSES AS A RESULT

MULTI I/O - SPECIFIES A CARD THAT HAS TWO OR MORE I/O FUNTIONS BUILT

ONTO IT.  USUALLY THE FUNCTIONS WILL BE A COMBINATION OF SERIAL,

PARALLEL, AND GAME/JOYSTICK PORT.

IDE MULTI I/O - SPECIFIES A CARD THAT INCLUDES EVERYTHING THE ABOVE

MULTI I/O HAS PLUS AN IDE HARD DRIVE INTERFACE AND USUALLY

A FLOPPY DRIVE INTERFACE AS WELL.

SERIAL I/O - SPECIFIES A CARD THAT HAS ONLY SERIAL PORTS

IDE AND FDC I/O ONLY - SPECIFIES A CARD THAT HAS ONLY HAS IDE HARD DRIVE

AND FLOPPY DRIVE I/O PORTS

ENA - SPECIFIES THE SETTING FOR ENABLING THE PORT

DIS - SPECIFIES THE SETTING FOR DISABLING THE PORT

SEL - ABBREVIATION FOR SELECT

JPX - ABBREVIATION FOR JUMPER (X IS NO. THAT IS LABELLED ON CARD)

CN - ABBREVIATION FOR CONNECTOR

PRI - ABBREVIATION FOR PRIMARY (OR MAIN) SETTING OF PORT

SEC - ABBREVIATION FOR SECONDARY SETTING OF PORT

IDE - SPECIFIES THE IDE HARD DRIVE PORT

FDC/FDD - SPECIFIES THE FLOPPY DISK CONTROLLER OR DRIVE PORT

COM1/3 - SPECIFIES THE SERIAL PORT WHICH IS SWITCHABLE BETWEEN COM1 AND

COM3

COM2/4 - SPECIFIES THE SERIAL PORT WHICH IS SWITCHABLE BETWEEN COM2 AND

COM4

UART - ANOTHER TERM USED TO SPECIFY THE SERIAL PORT USUALLY WHEN IT IS NOT

CONFINED TO BEING ANY SPECIFIC COM NUMBER.

LPT - SPECIFIES THE PARALLEL PORT

GAME - SPECIFIES THE JOYSTICK PORT

LED - SPECIFIES THE CONNECTOR TO WHICH THE HARD DRIVE LIGHT (LED) IS

CONNECTED

IRQ - SPECIFIES AN INTERRUPT REQUEST LINE SETTING

RS232/ASYNC - OTHER TERMS SOMETIMES USED TO DESCRIBE SERIAL CARDS AND

PORTS