ALL ABOUT IRQ LINES

by Ralph Griffin  - (typo corrections 1995 & reformatted DOS)

What is an IRQ line:

Some adapter cards designed to be installed in IBM PC, PC/XT,

PC/AT, or compatible computer require Interrupt Request (IRQ)

line. These cards usually give the user a narrow selection of

IRQ lines and accompanying documentation gives little advice

on which IRQ line to use.  If you choose the wrong line, it

will interfere with operation of some other hardware on your

system such as the COM ports or printer ports.  This document

explains how to select IRQ lines.

It will be helpful to review the function of the IRQ lines.

The IBM PC, IBM PC/XT, and PC/XT compatibles have 8 IRQ lines

(numbered IRQ0 through IRQ7).

The IBM PC/AT, and PC/AT compatibles including computers with

386 or higher processors which still use the PC/AT interrupt

controller) have 16 IRQ lines (numbered IRQ0 through IRQ15).

IRQ10 through IRQ15 may also be called IRQA through IRQF.

The IRQ lines are physical connections in the computer. When

a hardware device such as an adapter card activates the IRQ

line it is detected by the programmable interrupt controller

(PIC).

The PIC then activates the interrupt pin on the processor.

The processor responds by stopping whatever it is doing and

then asks the PIC which interrupt should by executed.

PIC tells the processor which interrupt to execute depending

on which IRQ line was activated. The processor then gets the

vector for that interrupt by looking in the interrupt vector

table in memory.  The processor then starts executing the

procedure at the address that is pointed to by the vector.

When the procedure is finished, the processor resumes doing

whatever it was doing before it was interrupted.

Each of the IRQ lines can be enabled or disabled by clearing

or setting a bit in the PIC mask. If a device activates a IRQ

line whose PIC mask bit is set, then the PIC will ignore the

signal and no interrupt will be generated.

The PIC mask for IRQ0 through IRQ7 is a byte that can by read

or written at I/O port address 21(hex). The PIC mask for IRQ8

through IRQ15 is a byte that can by read and written at I/O

port address A1(hex).

Every IRQ line has a designated interrupt, and many IRQ lines

have designated functions.  Some such functional designations

are a little flexible. The following list shows the interrupt

number and function of all the IRQ lines:

LINE      INT(x)    FUNCTION

IRQ0      08        system timer

IRQ1      09        keyboard

IRQ2      0A        PC/XT: EGA vertical retrace or (used)

PC/AT: see explanation below

IRQ3      0B        COM2 or COM4 or maybe available

IRQ4      0C        COM1 or COM3 or maybe available

IRQ5      0D        PC/XT: hard disk drive

PC/AT: LPT2 or maybe available

IRQ6      0E        floppy disk drive

IRQ7      0F        LPT1 or maybe available

IRQ8      70        PC/AT: CMOS Real Time Clock

IRQ9      71        PC/AT: see explanation below

IRQ10     72        PC/AT: probably available

IRQ11     73        PC/AT: probably available

IRQ12     74        PC/AT: probably available

PS/2: mouse

IRQ13     75        PC/AT: numeric coprocessor

IRQ14     76        PC/AT: hard disk drive

IRQ15     77        PC/AT: probably available

As you can see from the above table, it appears that all of

the lower 8 IRQ lines are all reserved for something, and

that makes it hard to add anything. But you may not actually

have all the equipment that uses all those lines which means

that the lines are free to be used for something else.

IRQ3:

If a computer has a COM2 or COM4 serial port then IRQ3 must

be used for this purpose. The IRQ3 line indicates when

incoming data has been received on COM2 or COM4. The computer

can enable reception of data at either COM2 or COM4 but not

both at the same time. If a computer has neither COM2 nor

COM4 (or while the computer has enabled reception of data at

neither COM2 nor COM4), then IRQ3 is available.

IRQ4:

When computers have COM1 or COM3 serial port, IRQ4 must be

used for this purpose. The IRQ4 line indicates when incoming

data has been received on COM1 or COM3.

The computer can enable reception of data at either COM1 or

COM3 but not both at the same time. If a computer has neither

COM1 nor COM3 (or while the computer has enabled reception of

data at neither COM1 nor COM3), then IRQ4 is available.

IRQ5:

On PC/XT's, the IRQ5 line is reserved for the hard disk

drive. On PC/AT's, it is reserved for the second parallel

printer port (LPT2), but most PC/AT's have only one parallel

printer port (LPT1), so the IRQ5 line is in fact available

for other uses.

If you do have a second parallel port, and if you do not need

it, you may be able to use jumper/switch settings to disable

it or its access to the IRQ5 line.

Disabling LPT2 or its assess to IRQ5 would make the IRQ5 line

available for other uses. Even if you can not disable IRQ5

line access of LPT2, you can still probably treat the IRQ5

line as if it were available because it is very common for

the IRQ line of the printer port to be disabled by software.

(see "Printing Without an IRQ Line" below).

IRQ7:

The IRQ7 line is reserved for the first parallel printer port

(LPT1), and most computer have at least one printer port. If

you don't need your LPT1 printer port (because you don't have

a printer connected directly to your computer, or because you

are using a serial printer, or because you are using LPT2

instead of LPT1), then you may be able to use jumper/switch

settings to disable it or its access to the IRQ7 line.

Disabling LPT1 or its access to IRQ7 would make the IRQ7 line

available for other uses. Even if you can not disable the

IRQ5 line access of LPT2, you can still probably treat the

IRQ5 line as if it were available because it is very common

for the IRQ line of the printer port to be disabled by the

software (see "Printing Without an IRQ Line" below).

IRQ2:

The IRQ2 line has no predetermined function, but it is

commonly used by EGA and VGA adapters, network adapters, and

bus mouse adapters. The IRQ2 line can be used for any one of

these purposes or for any other purpose, as long as there is

no conflict with other adapters.

If the IRQ2 line is already being used by an EGA or VGA

adaptor, you may be able to disable it for reasons explained

below, and that would free it up for some other use.

On a PC/AT the IRQ9 line may be considered the same as the

IRQ2 line (more on that later). So if one adaptor uses the

IRQ2 line and another uses the IRQ9 line, then there is a

potential conflict.

Most EGA adaptor cards have a problem that snow may appear on

the display if a program does a lot of writing directly to

the video memory in graphics mode. This problem is caused by

the adaptor bus and video output circuits trying to access

the same video memory at the same time.

To get around the snow problem, programs can set up the EGA

adaptor to generate an interrupt every time the display goes

into a vertical retrace, and set up an interrupt handler to

write to the display memory only while the display is doing a

vertical retrace (when the video output circuits are not

accessing the video memory).  The EGA adaptor uses IRQ2 for

this purpose.

The IRQ2 line is also useful for scrolling graphics and large

text on the screen. By updating the screen only during a

vertical retrace, a program can avoid the appearance of

jagged edges within the scrolling image.

If your EGA adaptor has a jumper that allows you to enable or

disable the IRQ2 line, then you could disable it, and you may

not notice any change in performance, especially if you do

not have any software that utilizes it.

There is also a chance that your EGA adaptor does not have

the snow problem anyway, and the IRQ2 jumper is there only

for the remote possibility that you may be running some

software that won't work properly without the IRQ2 feature.

If your EGA adaptor has no such jumper, then chances are that

the IRQ2 line is enabled.

Some VGA adaptor cards also have a jumper that allows you to

enable or disable the IRQ2 line. VGA adapters are not

supposed to have the snow problem even when the IRQ2 line is

disabled, so it is very unlikely that you need to enable it.

The IRQ2 jumper is there only for the remote possibility that

you may be running some software that won't work properly

without the IRQ2 feature.  Most software that uses the IRQ2

can automatically detect it and adjust its operations to work

with or without it.

Some software that uses the IRQ2 feature selects the mode of

operation during installation, so you may need to reinstall

or re-setup after disabling the IRQ2 line of the EGA or VGA

adaptor.

If your VGA adaptor has a 16 bit interface instead of an 8

bit interface, then the jumper will be referred to as IRQ9

enable instead of IRQ2 enable. It is really the same line as

explained below.

IRQ2 and IRQ9 Working Together:

On PC/AT's and those computers with 386 or higher processors

which still use the PC/AT interrupt controller, the IRQ2 and

IRQ9 lines are used in mysterious ways which require lengthy

explanation.

Most technical references provide confusing, scant and often

contradictory information about this line and its interrupt.

The following is an attempt to clear up the confusion.

Comparison of the pin-out diagrams for the adapter interface

of the PC/XT and PC/AT shows that the IRQ9 terminal on the

PC/AT is in the same location as the IRQ2 terminal on the

PC/XT.

Furthermore, the PC/AT does not even have an IRQ2 terminal.

If a card is designed to work in the PC/XT, it has an 8 bit

interface and it may use the IRQ2 line.

If the card is plugged into a PC/XT system if it activates

its IRQ2 line, then it will also activate the IRQ2 line on

the computer motherboard and that will cause the computer to

execute interrupt 0A.

If the same card is plugged into a PC/AT and if it activates

(what it thinks is) the IRQ2 line, then it will actually be

activating the IRQ9 line on the computer motherboard (the

IRQ2 terminal of the card contacts the IRQ9 terminal of the

interface connector) and that will cause the computer to

execute interrupt 71.

The procedure for interrupt 71 is set up by the BOIS and it

includes a software call to interrupt 0A (the interrupt for

IRQ2).

In other words, the PC/AT computer uses hardware to redirect

the IRQ2 line of an adaptor card onto the IRQ9 line of the

motherboard, and it uses software to redirect interrupt 71

(the procedure for IRQ9) back to interrupt 0A (the procedure

for IRQ2). Thus the real IRQ2 line on the motherboard is

uninvolved, except for an indirect way (cascading) which is

discussed below.

You can install an 8 bit card in a PC/AT computer and enable

IRQ2. It is possible to run software which is specifically

designed to work with that card (it expects to see interrupt

0A executed every time the card activates the IRQ2 line) and

it should work, provided that there are no conflicts with

other adaptor cards, and provided that the software does not

mishandle the PIC mask (see "About the PIC Mask" below).

There are a number of things that could possibly go wrong

with using IRQ2 of an 8 bit adaptor card plugged into PC/AT

computer.

None of these problems will occur will if the software is

written properly, but there is some bad software out there

because this subject is not well documented. The software (or

device driver) which is designed to work with that 8 bit

adaptor card needs to be aware of weather it is running on an

XT or an AT.

When accessing the PIC mask (see "About the PIC Mask" below),

the software must enable/disable IRQ2 when it is running on

an XT, and it must enable/disable IRQ9 when it is running on

an AT. If the software disables IRQ2 (when not in use) on an

AT, it will be disabling IRQ8 -IRQ15, and the result is a

disabled hard disk among other things.

If the software see's the IRQ2 of an AT is already enabled

(IRQ2 must always be enabled on an AT), then the software may

refuse to initialize because it incorrectly concludes that

some other software is already using IRQ2.

The software can get by without knowing whether it is on an

XT or AT if it makes sure that the IRQ2 line is enabled (not

caring if it was already enabled), and if it never disables

the IRQ2 line, and if the BIOS boots up with IRQ9 enabled

(which is common practice).

Many technical references state that the PC/AT computer uses

the IRQ2 line as a cascade for IRQ8 through IRQ15.  That is

true.  It means that whenever any one of the IRQ8 through

IRQ15 lines is activated, it triggers a slave PIC which then

activates the IRQ2 line going into the master PIC.

The master PIC then activates interrupt pin of the processor.

However, that IRQ2 line is not accessible by the adaptor

interface, so it is not interfered with by using the IRQ2

line of an 8 bit adaptor card. Furthermore, the cascading

process is based on hardware protocol and it does not involve

the use of interrupt 0A.

Many technical references state that the IRQ2 line or int 0A

is reserved.  That is not true. It may be that IBM wanted to

reserve IRQ2 for its own use, but the only things that IBM

ever commonly used it for are listed above.

IBM lost authority to dictate details of the AT architecture

when it became a minority player in the PC market. It may be

that the technical references are referring to the IRQ2 line

which is used for cascade, but is not even accessible to the

adapter cards of the PC/AT. But that is misleading because it

leaves the reader with the untrue impression that it is not

permissible to enable the IRQ2 line on an 8 bit adaptor card

which is plugged into a PC/AT.

Printing Without an IRQ line:

It is possible to operate a printer port without an IRQ line.

By disabling the IRQ7 line of LPT1, or the IRQ5 line of LPT2,

you can make that IRQ line available for other uses.

Parallel printer ports can be set up to generate an interrupt

whenever acknowledge line of the port is pulsed, but normally

this feature is not used. The BIOS printer support routine

simply sends out a byte and then goes into a loop that looks

for a pulse on the acknowledge line.

The BIOS routine does not return successfully until a pulse

is detected. If too much time passes without detecting a

pulse on the acknowledge line, then the BIOS routine returns

with a "Timeout" error.

Multitasking programs and operating systems can bypass the

BIOS and go directly to the printer port hardware and PIC

mask to enable the IRQ feature.

This allows them to do something else while waiting for the

pulse on the acknowledge line, but such programs are the

exception instead of the rule.

The author of this document does not know about Windows' and

OS/2's usage of the printer port IRQ lines, but it would make

sense if those operating systems did use the printer port IRQ

lines.  If you disable the IRQ line of the parallel printer

port, your DOS programs can still send data to the printer

and it will be printed.

The parallel interface still has status lines which enable

your programs to avoid overflowing the printers input buffer

and detect basic errors such as "Out of Paper" and "Device

not Ready".

Sharing an IRQ line:

It is also possible for more than one device to share an IRQ

line. For example, you could have a scanner and a modem both

using the IRQ3 line. As long as you don't try to run the

scanner program and the modem program at the same time, you

may be able to make it work. The most likely problem that may

arise here is a software incompatibility. For example, you

may need to load a driver for the scanner in the CONFIG.SYS

file, and that may conflict with the modem program even when

you are not running the scanner program. It may be that you

will have to change the configuration and reboot in order to

switch between the scanner and the modem. You may find that

you can run the scanner just fine, and then run the modem

just fine, but you can't run the scanner after running the

modem because the modem program does not properly restore the

state of the interrupt vectors and the PIC mask (see "About

the PIC Mask" below). Such an incompatibility can be overcome

without rebooting, but it requires that the modem program be

run from a batch file. The batch file would contain commands

to run the modem program and then a custom program that

restores the interrupt vectors and PIC mask to the proper

values for compatibility with the scanner (this solution is

most elegant once you get it working but it is not for

novices).

As another example, you could have a scanner and an EGA card

both using the IRQ2 line. As long as the scanner program does

not use the EGA IRQ feature, it may work. A program or driver

must directly access the I/O ports of the EGA adaptor in

order to activate the EGA IRQ feature. The feature is not

supported by the BIOS or DOS. Of course this example is also

subject to the same kind of potential software

incompatibility problems that were described in the example

of sharing the IRQ3 line.

Still another example, you could have a scanner and a printer

port both using the IRQ7 line. As long as the scanner program

does not use the printer port IRQ feature, it may work.

A program or driver must directly access the I/O ports of the

printer port in order to activate printer port IRQ features,

but fortunately, very few programs actually do that so there

is little chance of a conflict.  Likewise, DOS and the BIOS

do not use the IRQ feature of the I/O port, so we will have

no conflict there either.  Of course this example is also

subject to same kind of potential software incompatibility

problems that were described in the example of sharing the

IRQ3 line.

Yet another example, you could have a serial printer on COM1

and a modem on COM3. Both devices use IRQ4, but if there is

no need or expectation for the printer to send data back to

the computer (as is usually the case), then COM1 will never

recieve any data (it only sends data), and therefore COM1

will never actually use the IRQ4 line. The printer can signal

its readiness status to the computer using the serial data

status lines, which do not involve the transmition of actuall

serial data to the computer and thus it does not invlove the

use of the IRQ4 line. Thus there will not be a conflict with

the modem on COM3.

About the PIC Mask:

This section is for programmers only. If you are not a

programmer, then you are finished reading. There are many

ways to disable an IRQ line. The first way is to not install

the device that would attempt to use the IRQ line. The second

way would be to completely disable the device using switch or

jumper settings. The third way would be to allow the device

to work but disable its access to the IRQ line using switch

or jumper settings. The fourth way would be to use software

that simply does not activate the IRQ line feature of the

device in question. The computer boots up with the IRQ line

feature of most such devices deactivated, so all you have to

do is make sure the feature does not get activated, or that

it is deactivated when it is no longer needed. Now that we

have got that out of the way, we are ready to talk about the

PIC mask.

The fifth way to disable an interrupt is to set its bit in

the PIC mask. The PIC (Programmable Interrupt Controller) is

the device which reads the IRQ lines and sends a signal to

the interrupt pin of the processor when it receives a signal

on one of the IRQ lines.

If the mask bit for that IRQ line is set, then the PIC will

ignore that IRQ line and the processor will not be notified.

Bits 0 through 7 of the PIC mask control IRQ0 through IRQ7,

and they can be read and written at I/O port address 21h.

Bits 8 through 15 of the PIC mask control IRQ8 through IRQ15,

and they can be read and written at I/O port address A1h. On

an XT, bits 0, 1, 5 and 6 should always be clear.  On an AT

or higher, bits 0, 1, 2, 6, 14 should always be clear.

On an AT or more advanced computer, setting bit 2 has same

effect as setting bits 8 though 15. Normally, any software

that activates the IRQ line feature of a device will also

clear the PIC mask bit for that IRQ line.

The following procedures show how to set and clear various

bits of the PIC Mask using C.

Clear bit 3, thus enabling IRQ3.

outportb(0x0021,inportb(0x0021)&0xF7));

Set bit 7, thus disabling IRQ7.

outportb(0x0021,inportb(0x0021)|0x80));

Clear bit 12, thus enabling IRQ12.

outportb(0x00A1,inportb(0x00A1)&0xEF);

The following instruction sets bit 2, thus disabling IRQ2 of

an XT. Never do this on an AT, because it will disable IRQ8

through IRQ15, which disables the hard disk among other

things.

outportb(0x0021,inportb(0x0021)|0x04));

The following instruction clears bit 2, thus enabling IRQ2 of

an XT. If a program does this on an AT, then it probably does

not hurt anything, but it does not help either because what

really needs to be enabled is IRQ9.  Fortunately, the AT

probably booted up with IRQ9 enabled.

If a TSR program or device driver enables IRQ2/9 once during

initialization, and if it never disables IRQ2/9, then it can

use this one instruction and get away with not knowing if the

machine was an XT or an AT.

Also, if some program has wrongfully set bit 2 by attempting

to disable IRQ2 of an XT when in fact it was an AX, then the

following instruction can be used to correct the problem.

outportb(0x0021,inportb(0x0021)&0xFB));

The following instruction clears bit 9, thus enabling IRQ9 of

an AT, and records the original state for later reference.

outportb(0x00A1,(original_A1=inportb(0x00A1))&0xFD));

The following instruction sets bit 9, thus disabling IRQ9 of

an AT, but only if it was set when the program first looked

at it. If a program sets bit 9 where it had previously been

clear, then it runs the risk of creating incompatibilities

with the kind of programs that enable IRQ2/9 without knowing

whether the system is an XT or an AT.

if(original_A1&0x02)outportb(0x00A1,inportb(0x00A1)|0x02);

The sixth way to disable an interrupt is to execute a

processor instruction which clears the interrupt bit of the

flags register. The processor will ignore all hardware

interrupt as long as the interrupt bit of the flags register

is clear.

This is good in critical tasks that must not be interrupted,

but it also disables the timer and keyboard, so you can not

leave the processor in this state for a long time.

Also, when your program call other system functions, it is

safe to assume that those functions will set the interrupt

bit or fail to work properly if it is clear.#############################################################################################################################################################################################################################################################################################################################################################################################################