Earth Monitor

Earthquake Monitoring and Display Software

for the IBM Personal Computer

Document created with version 5.17

Current version is 6.1 (as of 1 May 1995)

June 26, 1995

Ted Blank

Address:

954 Foxswallow Ct

San Jose, California USA 95120

(408) 323-8514

Compuserve: 71651,1416

Internet:   tblank@netcom.com

FTP: /pub/tb/tblank at ftp.netcom.com

CONTENTS

Introduction   . . . . . . . . . . . . . . . . . . . . . . . . . . 1

Where to find a seismometer  . . . . . . . . . . . . . . . . . . 1

The EMON Software Package  . . . . . . . . . . . . . . . . . . . . 1

What kind of PC should I use?    . . . . . . . . . . . . . . . . 1

Package Contents   . . . . . . . . . . . . . . . . . . . . . . . . 2

Monitoring Seismic Activity with EMON  . . . . . . . . . . . . . . 3

Customizing EMON: the OPT file   . . . . . . . . . . . . . . . . 4

Displaying earthquake recordings with QUAKEVU  . . . . . . . . . . 4

QUAKEVU V6 special note  . . . . . . . . . . . . . . . . . . . . 7

Customizing QUAKEVU startup: the INI file  . . . . . . . . . . . 7

Utility programs   . . . . . . . . . . . . . . . . . . . . . . . . 8

EL (EMONList)  . . . . . . . . . . . . . . . . . . . . . . . . . 8

HOWFAR   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 8

MSPLICE  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 9

Hardware  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 11

PC Clock  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 11

Analog-to-Digital Converter (ADC) Card  . . . . . . . . . . . . . 11

Addressing  . . . . . . . . . . . . . . . . . . . . . . . . . . 12

Signal Amplifier and Filter   . . . . . . . . . . . . . . . . . . 14

Amplification level ("gain")  . . . . . . . . . . . . . . . . . 14

Software description  . . . . . . . . . . . . . . . . . . . . . . 15

Data collection   . . . . . . . . . . . . . . . . . . . . . . . . 15

Trigger Algorithm   . . . . . . . . . . . . . . . . . . . . . . 15

EMON Options File   . . . . . . . . . . . . . . . . . . . . . . 16

Parameters which are new in Version 6.1   . . . . . . . . . . 21

Parameters which are new in Version 6.0a-k  . . . . . . . . . 21

Parameters which existed in Version 5.xx  . . . . . . . . . . 24

EMON Main Screen  . . . . . . . . . . . . . . . . . . . . . . . 29

Header area   . . . . . . . . . . . . . . . . . . . . . . . . 30

Seismometer output display area   . . . . . . . . . . . . . . 30

Filename display area   . . . . . . . . . . . . . . . . . . . 30

EMON Keyboard Actions   . . . . . . . . . . . . . . . . . . . . 31

ADC boards supported  . . . . . . . . . . . . . . . . . . . . . 32

Displaying quake files with QUAKEVU   . . . . . . . . . . . . . . 32

QUAKEVU features  . . . . . . . . . . . . . . . . . . . . . . . 33

Mouse Interface to QUAKEVU  . . . . . . . . . . . . . . . . . . 33

Arrow keys while Zoom'ed  . . . . . . . . . . . . . . . . . . . 33

Printing the screen from QUAKEVU  . . . . . . . . . . . . . . . 34

QUAKEVU example   . . . . . . . . . . . . . . . . . . . . . . . 34

Saving a subset of the original file  . . . . . . . . . . . . . 39

Printing the screen contents  . . . . . . . . . . . . . . . . . 39

Data files  . . . . . . . . . . . . . . . . . . . . . . . . . . . 39

Data File Names   . . . . . . . . . . . . . . . . . . . . . . . 40

Data file names for files created by EMON V5  . . . . . . . . 40

Data file names for files created by EMON V6  . . . . . . . . 40

Bibliography  . . . . . . . . . . . . . . . . . . . . . . . . . . 41

General Background  . . . . . . . . . . . . . . . . . . . . . . . 41

Seismic Monitoring Station Practices  . . . . . . . . . . . . . . 41

Home Seismometer Construction and Design  . . . . . . . . . . . . 41

Contents  ii

Earthquake Interpretation   . . . . . . . . . . . . . . . . . . . 41

Personal Computer Data Acquisition  . . . . . . . . . . . . . . . 42

Seismic Software Design   . . . . . . . . . . . . . . . . . . . . 42

Seismometers in the Classroom   . . . . . . . . . . . . . . . . . 42

Contents  iii

FIGURES

Figure 1. EMON display   . . . . . . . . . . . . . . . . . . . . . 4

Figure 2. Sample QUAKEVU screen  . . . . . . . . . . . . . . . . . 4

Figure 3. NEIC earthquake location information.    . . . . . . . . 7

Figure 4. Contents of HOWFAR.LOC file  . . . . . . . . . . . . . . 9

Figure 5. Combining parts of two files into one file with MSPLICE  10

Figure 6. OPT file for single-address, single-channel ADC   . . . 13

Figure 7. OPT file for single-address, multi-channel ADC  . . . . 13

Figure 8. Noise detection in trigger algorithm  . . . . . . . . . 15

Figure 9. EMON typical screen display   . . . . . . . . . . . . . 30

Figure 10. Initial QUAKEVU display using QuickDisplay (/Q) option  36

Figure 11. Zoom display of the area of interest.    . . . . . . . 37

Figure 12. QUAKEVU screen after turning QuickDisplay off  . . . . 38

Figure 13. Origin time & distance calc. using P/S pointers  . . . 39

Figures  iv

INTRODUCTION

The personal computer  has been used productively in laboratories as a

data collection device for many  years.  Now,  amateur scientists  can

also  take  advantage of the power of the  personal computer.  If  you

have  a  PC and an interest  in the Earth sciences, you might  want to

consider  setting  up a seismic monitoring station at  home  or  in  a

classroom.  Excellent  results  can  be   obtained  with  a  home-made

seismometer,  constructed  from readily available  materials.  Several

Scientific  American  articles  over  the  past  15   years  (see  the

bibliography)   give  instructions  for   building  various  types  of

seismometers.   Mercury  tilt-meters,  pendulum-based   designs,   and

capacitance-based designs are  just some of  the approaches that  have

been tried with excellent results.

All of the early amateur seismograph designs  used  some sort of chart

recorder to make a permanent record of an amplified earthquake signal.

With the EMON software package, your  PC can do  the  job of the chart

recorder- and much more!  Saving earthquake records to disk allows the

power of the  PC to be applied to the  data analysis - a task at which

the computer excels.

WHERE TO FIND A SEISMOMETER

While you might salvage a seismometer from a local college or research

laboratory, don't dismiss the homemade idea prematurely.  Building one

can be lots of fun and makes an excellent  home or  classroom project.

It  is decidedly low-tech in  nature,  and inspires rich possibilities

for further exploration.

THE EMON SOFTWARE PACKAGE

The collection of programs in this package will let you

�  view the signal from your seismometer in real time,

�  save earthquake recordings to disk,

�  display and analyze the earthquake records, and

�  zoom in on interesting parts of the earthquake records.

It  is  the  PC  of  course  that  makes the last  three  capabilities

possible.

WHAT KIND OF PC SHOULD I USE?

The same PC can be used for both data collection  and analysis, or you

can use two different  PCs.  The PC  you use for data collection  does

not need to be particularly powerful.  In fact, an old  IBM PC/XT (the

first PC to have a hard  drive) with an  internal  or  external 3 1/2"

floppy drive  installed  is  just  about the  perfect data  collection

Introduction  1

machine.  The  hard drive allows lots of data to be collected, and the

floppy drive  allows you to archive  the important files or share them

with friends.

When it is  time  to display  and  analyze the  data  though, a faster

machine (minimum of 286 class,  but the faster  the better) makes this

part much  more  enjoyable.  If you  want to display your  data  on  a

different PC than the one on which it was  collected, just  make  sure

the  two machines have at least one  size of floppy disk in common, or

some sort  of  serial or parallel  port  connection  which allows file

transfer.

Also,  remember to keep good  written records.  From the day you start

monitoring for earthquakes, keep a log  or  journal.  Anytime you make

any change to your  system, write down  what you've done in  your log.

This way others will know what has changed.

You might want to start a folder for  each significant  earthquake you

record.  Keep  a printout of the record in the folder,  along  with  a

disk containing a copy of the quake file.  You might  also want to put

a  map  on  the wall  of your station  with a pin  in  the location of

earthquake you've recorded.

PACKAGE CONTENTS

��������������������������������������������������������������������Ŀ

Table 1. EMON package contents.  This table describes the contents�                                                                    �

of the EMON package.�                                                                    �

��������������������������������������������������������������������Ĵ

Filename       Description                                        �              �                                                     �

��������������������������������������������������������������������Ĵ

EMON.EXE       Monitors, displays and records output of the       �              �                                                     �

seismometer                                        �              �                                                     �

��������������������������������������������������������������������Ĵ

EMON6.OPT      Options file you use to customize EMON for your    �              �                                                     �

location.  First time users of EMON should rename  �              �                                                     �

this file as EMON.OPT and then customize.          �              �                                                     �

��������������������������������������������������������������������Ĵ

EMON6NEW.OPT   File containing only options new to Version 6 of   �              �                                                     �

EMON.  Current users of EMON should add the        �              �                                                     �

contents of this file to their existing EMON.OPT   �              �                                                     �

file, and then customize the new options.          �              �                                                     �

��������������������������������������������������������������������Ĵ

EMON61NW.OPT   File containing only options new to Version 6.1 of �              �                                                     �

EMON.  Current users of Version 6 of EMON should   �              �                                                     �

add the contents of this file to their existing    �              �                                                     �

EMON.OPT file, and then customize the new options. �              �                                                     �

��������������������������������������������������������������������Ĵ

QUAKEVU.EXE    Displays and lets you analyze quake files          �              �                                                     �

����������������������������������������������������������������������

Introduction  2

��������������������������������������������������������������������Ŀ

� Table 1 (Page 2). EMON package contents.                           �

��������������������������������������������������������������������Ĵ

Filename       Description                                        �              �                                                     �

��������������������������������������������������������������������Ĵ

QUAKEVU.INI    File which tells QUAKEVU in which mode to start.   �              �                                                     �

��������������������������������������������������������������������Ĵ

EL.EXE         Displays full screen list of quake files on disk.  �              �                                                     �

From this screen you can display files with        �              �                                                     �

QUAKEVU.                                           �              �                                                     �

��������������������������������������������������������������������Ĵ

MSPLICE.EXE    Splices together two or more consecutive quake     �              �                                                     �

files into one file.  A full screen list of files  �              �                                                     �

is presented.  You may select any number for       �              �                                                     �

splicing.                                          �              �                                                     �

��������������������������������������������������������������������Ĵ

HOWFAR.EXE     Calculates the great circle distance between two   �              �                                                     �

points on the earth's surface given their latitude �              �                                                     �

and longitude.                                     �              �                                                     �

��������������������������������������������������������������������Ĵ

EMON.DOC       This file.  Print via COPY EMON.DOC LPT1: command. �              �                                                     �

��������������������������������������������������������������������Ĵ

READ.ME        Instructions for printing documentation, etc.      �              �                                                     �

��������������������������������������������������������������������Ĵ

HOWFAR.LOC     Sample of a file containing the name, latitude and �              �                                                     �

longitude of your station, for use by the HOWFAR   �              �                                                     �

program.  Optional.                                �              �                                                     �

��������������������������������������������������������������������Ĵ

93019OIN.TBJ   Quake file from a magnitude 6.0 earthquake of Jan  �              �                                                     �

19, 1993 at 14:39:27.4 UTC off the island of       �              �                                                     �

Hokkaido in the Sea of Japan.  This was an         �              �                                                     �

aftershock of the 6.9 quake of January 15 which was�              �                                                     �

the largest the area had experienced in 11 years.  �              �                                                     �

The recording was made in downtown Tokyo.          �              �                                                     �

��������������������������������������������������������������������Ĵ

ALASKA.DAT     Quake file from a magnitude 6.6 earthquake in      �              �                                                     �

Alaska. The recording was made in New York State.  �              �                                                     �

��������������������������������������������������������������������Ĵ

NEWHAMP.DAT    Quake file from a smaller earthquake in Berlin, New�              �                                                     �

Hampshire.  This recording was also made in New    �              �                                                     �

York State.                                        �              �                                                     �

����������������������������������������������������������������������

MONITORING SEISMIC ACTIVITY WITH EMON

The Earth Monitor (EMON) program is designed to run continuously in

the PC.  It  periodically checks the input from the seismometer and

amplifier for earthquake-like activity.  An earthquake data file is

saved  to disk only  when an  earthquake  signal  is  detected.  An

options file called  EMON.OPT is  read at startup.  You  can change

the parameters that control saving data  by changing entries in the

OPT  file.  You  can collect data  from  up  to  three seismometers

simultaneously.  A separate  quake file  will  be  written for each

Introduction  3

device.

Figure 1 shows a sample of  how the EMON  screen appears when it is

monitoring for  earthquakes.  Notice  that  three  quake files have

already been  saved to disk, and their names appear in the space at

the  bottom of  the screen.  If your PC has graphics capability (as

this one  did), the box  at  the  upper right displays  a real-time

trace of the output of the seismometer.  However,  EMON can collect

data on  a  PC which does not  have graphics capability.  You  will

just  need to use a different PC to  display the data later.  "EMON

Main Screen" on page 29  goes into more detail on using EMON.

�����������������������������������������������������Ŀ

�������������������������������������������������������

Figure 1. EMON display

CUSTOMIZING EMON: THE OPT FILE

Earthquake recording stations differ in more than just their location.

For instance,  a  quiet location in  the  country  could record small,

distant  events without worrying about interference from local  noise,

whereas a location in  a  large  city  might need  to  more  carefully

discriminate earthquake activity from  earth movements due to  trains,

subways or the like.

You customize EMON for the characteristics of your location through an

options  file called  EMON.OPT.  You can edit this file with any ascii

text editor.  Do not use word processors!  The  EMON.OPT file  is read

from disk when the program starts.  It should be in the same directory

as the EMON program.  In  the  OPT file you give EMON information like

the local noise level (used to discriminate true seismic activity from

local  noise).  Other  information  you  supply  includes  the   three

character ID of your station (which becomes  the file extension on all

saved files) and the disk (or  directory)  to which you want the quake

files to be saved.  You can also change some  of these same parameters

on  the screen after EMON has  started.  However, changing a parameter

on the  screen does notmake the  change  permanent.  You must edit the

EMON  file  and make the same change if  you  wish it  to be effective

automatically every time the EMON program starts.  "EMON Options File"

on page 16 explains each parameter in the options file.

DISPLAYING EARTHQUAKE RECORDINGS WITH QUAKEVU

The QUAKEVU program displays the data files created by EMON.  Figure 2

shows a sample of the QUAKEVU screen.

��������������������������������������������������������������������Ŀ

����������������������������������������������������������������������

Figure 2. Sample QUAKEVU screen

QUAKEVU allows you to identify the P and S wave arrivals on the screen

by moving small pointers via either  the mouse  or  the keyboard arrow

Introduction  4

keys.  Once  you  have  picked out  the P  and S  wave arrival points,

QUAKEVU then automatically calculates for you

�  the time the P wave arrived at the recording station

�  the time the S wave arrived at the recording station

�  the time the quake occurred (the Origin time)

�  the  distance from the recording  station  to the epicenter of  the

quake along a great-circle arc (in kilometers, miles and degrees)

This information is updated  every time  you move  either the P  or  S

pointers.

Notice that QUAKEVU  does not calculate the locationof the earthquake,

just the distance from the recording station.  You will need to obtain

distance calculations from at least three stations  in different parts

of  the  globe, and use triangulation on a map  or globe to figure out

the actual  location.  This  makes  it fun  to trade  quake files with

other groups who also monitor earthquakes.

QUAKEVU will also allow you to save  a subset of the data to a smaller

file  for  easier  transmission  across telephone  lines.  During  the

process  of  saving the subset file you  can also add a comment to the

file being saved (up to 60  characters)  as  well  as the true  origin

time, depth, location etc. if you know them.

QUAKEVU will work only on a PC with a Color Graphics Adapter (CGA) and

graphics  display,  or  its   equivalent.  QUAKEVU  uses  the  CGA  in

"hi-resolution" (640x200) mode,  which is equivalent  to "SCREEN 2" in

BASIC.  (In  this  mode  only  black  and white are  available.)  Most

computers with any type of  graphics  capability support this graphics

mode, since it has been around almost since the PC was invented.

You can display from one to three recordings of the same earthquake on

the screen at the same time.

Note: An OS/2  and  Windows  version of  QUAKEVU  called  WinQuake  is

available  from  Larry  Cochrane,  24   Garden   St.,  Redwood   City,

California.  This package  is a major  improvement on  QUAKEVU  in all

respects but  does require  Windows  (either DOS/Windows  or  the OS/2

Windows) and a 386 or 486 CPU.  It is available via anonymous FTP from

the /pub/cochrane directory of the ftp.netcom.com Internet system.

Each earthquake data file  contains  reserved space for a 60 character

comment describing the contents  of  the file.  QUAKEVU will allow you

to  to insert text into this reserved area when you save a file.  Once

a comment area is  filled  in, QUAKEVU displays it on the screen  when

the   file   is   displayed.   This   allows   a   limited   form   of

self-documentation for  the data files.  For  instance, you might want

to add a comment like "ML6.6 Solomon Islands  11S 163.5E".  This would

remind when you  displayed  the  quake file of the  magnitude  (on the

Richter scale), the  location, and the latitude and longitude  of  the

quake being displayed.  Note that the recording station name (up to 15

Introduction  5

characters) is already  saved in the file  in  a  different  place, so

there is no need to add that to the comment.

You can get information on the true  location of  the  earthquake from

news  reports  or, if  your PC has  a modem,  via  a call to the  U.S.

National Earthquake Information Center (NEIC) computer bulletin board.

Within a few  hours of a  major  quake, the NEIC  in Golden, Colorado,

makes a preliminary determination of its location using data just like

you  will  be  collecting  from  stations  all  around the world  that

recorded the event.  An  example of one of  their messages is given in

Figure 3  on  page 7.  It  is  sent out as  soon as possible to assist

disaster relief  agencies around the world  in  responding quickly  to

major earthquakes.  Location of smaller quakes are  usually calculated

and available within 1-2 weeks after the event.

Introduction  6

����������������������������������������������������������������������

U.S. GEOLOGICAL SURVEY  NATIONAL EARTHQUAKE INFORMATION CENTER

World Data Center A for Seismology

the following is from the united states geological survey,  national

earthquake   information   center:    preliminary   hypocenter   for

earthquake of 1993 jan 19, SEA  OF  JAPAN:   latitude  38.7  degrees

north  longitude 133.5 degrees east origin time 14 39 27.4 utc depth

460, magnitude 6.0 mb.

stations used:  mat p 144044.0 smy p 144507.9 adk p 144552.9

sdn p 144707.8 brw p 144719.7 tta p 144721.5 svw p 144724.9

ima p 144727.3 ap 144845.3 bgl p 144736.2 crp p 144736.5

ap 144907.5 cp2 p 144736.8 kdc p 144738.8 ap 144850.9

.

.

.

tuc p 145126.6 alq p 145130.1 ap 145312.8 wmok p 145148.8

tbr p 145208.0 nav p 145214.4 bla p 145216.3 cvl p 145216.7

ceh p 145223.6 prm p 145225.3 jsc p 145226.7 lhs p 145226.9

����������������������������������������������������������������������

Figure 3. NEIC earthquake location information.

QUAKEVU V6 SPECIAL NOTE

In version 6  and higher  versions of EMON, additional space is placed

in the data file describing  the sensor  that was used to  record  the

quake.  Additional  space is reserved in the file  into  which you can

add

�  Magnitude

�  Method by which magnitude was computed (4 chars)

�  Depth (in km)

�  Latitude and longitude

CUSTOMIZING QUAKEVU STARTUP: THE INI FILE

Different users found they used QUAKEVU in different ways.  Some  with

slower  PCs  start in QuickDisplay mode, zoom  in on a portion of  the

file,   then  display  at  full   resolution.  Others  like  to  start

immediately identifying  the P  and  S  waves  to get an  estimate  of

distance.

Since there was no way to  satisfy  all these  different requirements,

QUAKEVU V6 now includes a  file called QUAKEVU.INI which tells QUAKEVU

how to start  up.  After  starting the program you can continue to use

the keyboard or mouse as  before.  You  can edit the INI file with any

ASCII text editor.  Don't use word processors!

Introduction  7

UTILITY PROGRAMS

Several utility programs are included to  assist in handling the files

after they are collected.

EL (EMONLIST)

EL.EXE is the EMONList program.  EL  ?  gets you help.  Just  supply a

filemask  in standard form,  like *.*  or  93*.*  and EL will find all

matching  files  in  the  current  directory  (or  the  directory  you

specify).  It  lists the peak sample, date and  time sampling started,

and the first  40  characters or so of the  comment.  Hit enter on any

line to view that file with QUAKEVU.  You  can  also  use the UpArrow,

DownArrow,  PgUp, PgDn,  Home  and End  keys to  move around the list.

Pressing the Del key will erase the file from disk (after giving you a

chance to confirm).  Deleting a file will remove it from the on-screen

list as well.

If you have several files for the same quake from different locations,

you can view all of them on the same QUAKEVU screen.  To do this, mark

up to three files with the spacebar.  An  "X"  will appear in front of

the name of the file.  When all files have been marked,  press enter -

QUAKEVU  will  then  load all three  and  display  them  on  the  same

timescale.

HOWFAR

The HOWFAR program computes the great circle  distance between any two

points  on  the  earth given  the latitude and longitude of each.  You

will be prompted for both sets of coordinates.  However, if you create

a file called HOWFAR.LOC containing the latitude and longitude of your

station,  the HOWFAR program will use that information as  one of  the

locations and you only need to enter the second  one.  The  HOWFAR.LOC

file must be in the same directory as HOWFAR.EXE.  The  format of  the

HOWFAR.LOC file is:

Introduction  8

����������������������������������������������������������������������

LINE 1 - Text describing the place that this set of coordinates

refers to.  For instance, you might put something like:

XYZ School, Woonsocket, RI

LINE 2 - latitude in degrees, e.g. 43.52.  Use negative numbers

if you are SOUTH of the equator.

LINE 3 - longitude in degrees, e.g. 108.16.  Use negative numbers

if you are in the half of the world WEST of Greenwich.

Example for Tokyo, Japan station at 35.65 N latitude and 139.72 E longitude:

Tokyo, Japan

35.65

139.72

����������������������������������������������������������������������

Figure 4. Contents of HOWFAR.LOC file

MSPLICE

The recent 7.8 quake in Bolivia released an enormous amount of energy.

At one user's  station,  6  EMON files were saved from this one  great

quake.  Rather  than dealing with  these  files one  at  a  time,  the

MSPLICE program allows you to splice together two  or more  data files

which, back-to-back, span a long event.

Figure 5 on page 10 shows  this procedure.  The earthquake starts late

in the first file and continues into the second file.  MSPLICE subsets

the samples in both files until the total  is  less  than  or equal to

25000.  (EMON  data files can only contain up to 25000 samples.)  Then

both sets of data are are combined into a new file spanning the entire

event, although at a lower resolution.

Introduction  9

����������������������������������������������������������������������

Original files

(25000 samples  ��������������******###       #########*******��������

each)

�                  �

�                  �

v                  v

Subset files

(Every other     � � � � � � � * * * # #       # # # # # * * * � � � �

sample

discarded)                       �                  �

��������������������

�

�

v

Combined file

(25000 samples)               �******############*******�

����������������������������������������������������������������������

Figure 5. Combining parts of two files into one file with MSPLICE

Introduction  10

HARDWARE

Up to now  we've  talked  only  about the  software side of running  a

seismic monitoring station.  However  it  is  important  to  have some

familiarity  with  the hardware  required to interface  a  PC  to  the

"outside world".  The three pieces of hardware we will discuss are the

PC clock, the  analog-to-digital  converter (ADC) card, and the signal

amplifier.

PC CLOCK

An  accurate  PC  clock  is  a  must  for  seismic   data  collection.

Scientists  have  agreed   that   all  monitoring  stations  will  use

Coordinated Universal  Time (UTC,  formerly  known as  Greenwich  Mean

Time).  You should therefore keep your PC set  to UTC time rather than

local time.  In the U.S.A., a call to 1-900-410-TIME  will cost you 50

cents and  will allow you  to  set your  PC  clock to  the  U.S. Naval

Observatory Master Clock.  If you have a short-wave radio, the British

Broadcasting Company  (BBC) accurately  marks the  time  on  the hour.

Also, several packages are on the market  to  continuously synchronize

your PC clock to the WWV time  signal which can be received in most of

the  continental US.  Beware of daylight savings time - it may  affect

the number of hours between your time and UTC.

If you find that  your  PC clock does not maintain  the  correct time,

there may be  add-in clock boards available which keep excellent time.

The factory clocks  in some older PCs may drift as  much as a minute a

week.  You should try to keep  your PC within one second  of true UTC.

Shareware programs like RighTime and AccuSet, available on  Compuserve

and  other  bulletin board systems,  can  also  help  your PC stay "on

time".

ANALOG-TO-DIGITAL CONVERTER (ADC) CARD

The job of an ADC card is to convert an analog signal from the outside

world,  usually a voltage, into a digital  signal - a range of numbers

that  a  computer  can  manipulate.  (This  process  is also  known as

"digitizing").  Low voltage = small number, high voltage = big number.

(Sometimes the range is from a negative number to  the  same  positive

number.)  Each card has  a different  "smallest" and "biggest" number.

Here, as in most places, you get what you pay for.  An inexpensive ADC

card might  convert a range of input voltages (say, 0 to 5 volts) into

numbers from zero to 255.  Zero would represent the lowest voltage and

255 would represent the highest voltage.

A more sensitive  (and expensive) ADC card might be able to divide the

same  5-volt range into many  more than  just 255 separate steps.  For

seismographic work  you  should  choose  only  an  ADC card which  can

convert a  voltage range  into a  range of  at  least  4096  different

numbers.  A card with this capability is called a "12-bit" card, since

it's  output has 12 "bits" (binary digits)  of data, which  gives 4096

possible combinations  of ones and zeros.  "13-bit" and "14-bit" cards

are also available, but will probably only be worth the extra money in

Hardware  11

very quiet locations.

The  ADC card  usually fits into  an  expansion  slot  on the PC  main

circuit board.  Since there  may be several cards installed, all cards

are  designed  to  have  different  addresses and  to respond  only to

commands  directed to that address.  The EMON program uses this scheme

to send commands to the ADC card.  So how do you tell EMON the address

of your ADC card?

ADDRESSING

Each add-on card in  a  PC  is  designed to  respond to  commands to a

specific address.  However, different  manufacturers don't always know

which addresses other manufacturers are using.  Most companies now put

switches  on their cards so that  the  user (that's you!)  can  choose

another address for  the  card  if the  address set  at the factory is

already in use by another card in your  PC.  You  will need the manual

supplied with  the  ADC card to change the switches from their factory

setting.  However, most of the time you can use the address set at the

factory.  You will find this in  the documentation that  came with the

card.

ADC card designers are very creative.  There  are many different types

of  ADC  cards  on  the market  with  many  different  features.  EMON

currently supports 9 different types of cards which are  listed in the

EMON.OPT file.  For  amateur purposes I group them  into the following

categories:

Single-address, single-channel

Single-address, multi-channel

Multi-address, multi-channel

Each section  below includes  an  example of how  you might set up the

EMON.OPT file for that type of card.  Each example assumes you want to

monitor more than one seismometer.  If you have only one, just set the

second and third seismometer addresses to zero  and EMON  will  ignore

them.  In EMON version 6.1 and higher, use  the FirstSeismometerActive

(etc.) options in the EMON.OPT file to control  how many channels EMON

will sample from.  Starting in v6.1, only these parameters and not the

zero  address  controls the  devices  which  are  monitored.  This  is

because  some  cards can use three zero addresses and only change  the

channel being sampled.

�  Single-address, single-channel

Only the very  cheapest and  simplest cards are of this  type.  One

card can monitor only one  device.  Each  seismometer you wished to

monitor would therefore require a separate card  and thus an entire

expansion slot.

Here is an  example  of  part of the OPT file for a single-address,

single-channel card.

Hardware  12

����������������������������������������������������������������������

FirstSeismometerActive =     Yes      'Active and being sampled (EMON v6.1+)

FirstSeismometerOrientation =  N      'Orientation (N = North/South)

FirstSeismometerAddress =    340      'Decimal Address of 1st seismometer

FirstSeismometerChannel =      0      'Channel for 1st seismometer

SecondSeismometerActive =    Yes      'Active and being sampled

SecondSeismometerOrientation = E      'Orientation (E = East/West)

SecondSeismometerAddress =   341      'Decimal Address (0 if not installed)

SecondSeismometerChannel =     0      'Channel for 2nd seismometer

ThirdSeismometerActive =     Yes      'Active and being sampled

ThirdSeismometerOrientation =  Z      'Orientation (Z = Vertical)

ThirdSeismometerAddress =    342      'Decimal Address (0 if not installed)

ThirdSeismometerChannel =      0      'Channel for 3rd seismometer

����������������������������������������������������������������������

Figure 6. OPT file for single-address, single-channel ADC

�  Single-address, multi-channel

This is  the most common type  of  card.  Several  different  input

channels (signals)  can be  monitored by  this  type  of card while

taking up only one expansion slot.

The OPT file for this type of card might look like this:

����������������������������������������������������������������������

FirstSeismometerActive =     Yes      'Active and being sampled

FirstSeismometerOrientation =  N      'Orientation (N = North/South)

FirstSeismometerAddress =    340      'Decimal Address of 1st seismometer

FirstSeismometerChannel =      0      'Channel for 1st seismometer

SecondSeismometerActive =    Yes      'Active and being sampled

SecondSeismometerOrientation = E      'Orientation (E = East/West)

SecondSeismometerAddress =   340      'Decimal Address of 2nd seismometer

SecondSeismometerChannel =     1      'Channel for 2nd seismometer

ThirdSeismometerActive =     Yes      'Active and being sampled

ThirdSeismometerOrientation =  Z      'Orientation (Z = Vertical)

ThirdSeismometerAddress =    340      'Decimal Address of third seismometer

ThirdSeismometerChannel =      2      'Channel for 3rd seismometer

����������������������������������������������������������������������

Figure 7. OPT file for single-address, multi-channel ADC

�  Multi-address, multi-channel

In this type of ADC, the same card responds to commands directed to

any of several  different  addresses.  One card can monitor several

different channels, with each address corresponding  to a different

Hardware  13

channel.  The  "base  address"  of  the  card  monitors  the  first

channel,  the  "base address plus 1" reads the second channel, etc.

The  OPT  file  entries  would  be  identical  to   those  for  the

"single-address,  single  channel" card  described above.  However,

commands to any of the three addresses would  actually be processed

by  the same card  instead  of being  processed by  three different

cards.

SIGNAL AMPLIFIER AND FILTER

ADC cards usually are able to  senser  voltages as  low as 1 millivolt

(1/1000  volt).  However,   the  voltage  output  from  a  seismometer

responding to the passage  of  an earthquake wave from a distant event

might be  only 1 microvolt (1/1,000,000 V).  In  this  case  no signal

would be recorded.

For this  reason  the voltage output of  the  seismometer  is  usually

amplified  by  a factor  of from several  hundred to several  thousand

before it is sent to the ADC card.  In addition, noise from electrical

equipment is usually filtered out before the signal is amplified.

AMPLIFICATION LEVEL ("GAIN")

If you use a  higher amplification level (also called "gain") you will

be able to record more distant large events, and smaller local events.

However if  your local environment  is noisy, turning up the gain will

amplify local noise as well.  This noise will usually be of two types:

60 cycle noise  from home electrical devices, and  low-frequency noise

from  traffic,   wind,  home  appliances  and  (of  course)  children.

Electronic  filters can  remove  most 60 cycle noise.  However,  lower

frequency  noise can  be difficult  to  distinguish  from the  seismic

signals themselves.  A general rule  then is to turn the amplification

level on your amplifier up until  you begin to see  too much noise  in

the  signal (too  much vertical jitter)  or begin  recording  too many

"false alarms", and then turn it back down a bit.  Now experiment with

the      EMON.OPT       parameters      "NoiseRangeToIgnore"       and

"SampleCountToSaveFile" to minimize the number of false alarms.

Now let's take a more detailed look at the programs.

Hardware  14

SOFTWARE DESCRIPTION

DATA COLLECTION

The EMON program performs the monitoring and collection of the seismic

data.  The  program  saves  data to disk only when it  appears that  a

quake  has been  detected.  For this  purpose it  includes  a  trigger

algorithm - a kind of decision-making process which you can control.

TRIGGER ALGORITHM

The  purpose  of a  trigger algorithm is to detect earthquake activity

while screening  out false  alarms.  The  algorithm  I chose uses  the

concept of a "noise  zone"  around the normal output value  of the A/D

converter.  You specify the  width of this "noise zone" (in percentage

of full range  of  the ADC card) in the "NoiseRangeToIgnore" parameter

in   EMON.OPT.  See  "hdref  refid=opt..  for   descriptions  of   the

parameters in the EMON.OPT  file.  Signals  outside  (above  or below)

this noise  zone  are the important ones.  The  number of samples that

fall above or  below the noise  zone are counted as the program  runs.

Periodically  EMON  checks  to  see  if  at least a certain  number of

samples   (a   number    you    choose    via   the    OPT   parameter

"SampleCountToSaveFile")  has  been observed both above and below this

zone.

����������������������������������������������������������������������

o

Upper "Quiet" zone     o o      <- 5 samples above upper boundary

o   o

����������������������������������-   upper noise zone boundary

N  Z       SIGNAL    o    o

O  O    o   o o   o o                  o o

I  N    �o�o���o�o����������o���������o���o�

S  E      o     o            o       o

E                                   o

����������������������������������-   lower noise zone boundary

o    o

Lower "Quiet" zone             o  o

oo     <- 6 samples below lower boundary

����������������������������������������������������������������������

Figure 8. Noise detection in trigger algorithm

If this has  indeed  happened,  the program  assumes a quake event has

occured and begins accumulating data to save to disk.  (A small amount

of  data from before the event started is  also included.)  Otherwise,

the counters are reset to zero and monitoring continues.

If you set  the width of this "noise zone" too narrow, many files with

nothing but noise will be saved to disk  and you will have to  spend a

lot of  time going through them to determine if  they contain anything

of interest.

Software description  15

On the other hand, if you set the "noise zone" too wide, only the very

largest earthquakes will be recorded, and you may only  see one or two

files  per  year!  The  proper  settings  for  the  trigger  algorithm

parameters  are  different  for  every  location,   and  can  only  be

determined by trial and  error.  They may change with the seasons too.

If you  live  near the ocean,  for example, winter storm waves hitting

the shore might generate enough  local seismic noise to force  you  to

temporarily raise the value of "NoiseRangeToIgnore".

EMON OPTIONS FILE

The EMON options file (default name: EMON.OPT) allows you to customize

the  EMON program for your setup  and local seismic  environment.  You

can use any text editor to modify  the file.  Following  is an example

of   an  OPT   file.  The   station  it  is  from  has  the  following

characteristics:

�  A single IBM  Data  Acquisition  and Control  Adapter card is being

used to monitor the amplified signals.

�  Two  seismometers  are being monitored.  The  first (at address 544

channel  0)  is  in a N-S orientation, the second (at  address  544

channel 1) is in an E-W orientation.

�  the local environment is a fairly noisy urban  environment,  making

it necessary to set  NoiseRangeToIgnore  (or "noise zone")  to 15%.

In quieter environments, a value as  low as 1 or 2 may be possible.

Values below 1 are accepted but would  only make sense in extremely

quiet environments.

Software description  16

*

* Filename: EMON6.OPT

*

* Sample options file for Earth Monitor (EMON) Seismic Data Acquisition System

*

* If you are a first-time user of EMON, rename this file to EMON.OPT

*                                       ----------------------------

* If you are an existing EMON user you can either:

*

*    1) copy the "New Options" section of this file into your existing

*       EMON.OPT file, or

*

*    2) insert the file EMON6NEW.OPT, which contains only the new options

*       for V6, into your existing EMON.OPT file.

*

* Compatible with version 6.00 and up

* Shipped with version 6.01

* Blank lines or lines beginning with * are comments

* See EMON User's Guide for complete explanations of each parameter.

* Ted Blank, tblank@netcom.com or 71651,1416 on Compuserve

*****************************************************************************

*        New options for EMON Version 6.01                                  *

*****************************************************************************

DisplayTimeCompression =       1      'Compresses display horizontally.

*                                     'Higher numbers display every nth

*                                     'sample, allowing display to run slower

*                                     'than sampling rate.  This leaves data

*                                     'on the screen for a longer time.

FirstSeismometerActive =     Yes      'Specify Yes or No to turn on/off

SecondSeismometerActive =     No      'sampling for each of the three

ThirdSeismometerActive =      No      'possible devices.

*****************************************************************************

*        New options for EMON Version 6.00                                  *

*****************************************************************************

FileNameDisplay =          SHORT      'SHORT: display name only (as in EMON V5)

*                                     'LONG:  display both name and extension

PromptOnQuit =                NO      'YES or NO: safety prompt when quitting?

BeepOnFileSave =              No      'NO     Silent operation.

*                                     'YES    One beep when quake file is saved.

*                                     'ALARM  Beep every 30s until key pressed,

*                                     '       but start when file is saved.

*                                     'IMMEDIATE - Same as ALARM but start

*                                     '       beeping when quake is detected.

AlarmLevel =                  50      'Value from 0 to 99.  Sound alarm only

*                                     'when quake has relative strength at least

*                                     'this large.  NOTE: does not apply if

*                                     'the IMMEDIATE option was selected above.

Software description  17

FileFormat =                 New      'OLD or NEW.  NEW requires QUAKEVU V6.00+

*                                      or WinQuake V1.6+ to view - but these

*                                      new versions of the display programs

*                                      will still work the same with all your

*                                      old data files.  Use OLD only if you

*                                      do not have access to new display pgms.

ActivityLog =                YES      'Mini-trace of EMON activity - start and

*                                      stop times, files saved, etc.

ActivityLogFile =       EMON.LOG      'Log file path, or LPT1: to log to printer

*                                     'NOTE: logging to printer is NOT recom-

*                                     '      mended, since a printer error like

*                                     '      paper-out will stop sampling!

FirstSeismometerMagCorr =       0     'Magnitude correlation factor (WinQuake)

SecondSeismometerMagCorr =      0     'Magnitude correlation - 2nd seismometer

ThirdSeismometerMagCorr =       0     'Magnitude correlation - 3rd seismometer

* FileNamePattern allows you to choose how your data files are named.  Assemble

* the pattern from the building blocks below.  Separate the building blocks with

* either slash /, plus + or dash - characters.  Use the dot . to identify the

* start of the extension.  Note: do not put separator characters around the dot.

FileNamePattern = YY-MM-DD-S-O.III                  'See building blocks below

* ���������������������������������������������������������������������������Ŀ

* �            File Name Pattern building blocks to choose from:              �

* �                                                                           �

* �  YY   = year                            Example:                          �

* �  MM   = month                           Quake at 3:03 am on 2 May 94      �

* �  DD   = day                             Vertical sensor, 1st quake of day �

* �  JJJ  = Julian date (001-366)                                             �

* �  HH   = hour (00-23)                    Pattern ----------> File name     �

* �  H    = hour (A-X)                      ����������������    �����������-  �

* �  TT   = minute (00-59)                  MM-DD-YY-O-S.III    050294Z1.TBJ  �

* �  T    = two minute interval (A-Z,0-3)   (QK)+YY+MM+DD.II+S  QK940502.GC1  �

* �  O    = Orientation (N, E or Z)         YY/MM/DD/O/S.III    940502Z1.RWC  �

* �  S    = seq no. (A-Z,0-9)               YY-MM-DD-II.O-SS    940502RC.N01  �

* �  SS   = seq no. (01-99)    "            YY-JJJ-H-M-O.III    9412232Z.MSS  �

* �  SSS  = seq no. (001-999)  "                                              �

* �  II   = station ID (1st two chars)   * Filename is maximum 8 characters   �

* �  III  = station ID (three chars)     * Extension is maximum 3 characters  �

* �  (cc) = any character string in ()   * You must use "O" somewhere in the  �

* �  .    = . (separates name from ext)     pattern if you have >1 seismometer�

* �                                                                           �

* �����������������������������������������������������������������������������

*****************************************************************************

*        Original options from Version 5                                    *

*****************************************************************************

*****************************************************************************

*        Source of input data                                               *

*****************************************************************************

Software description  18

OperatingMode =           Random      ' Normal = data from sensor

*                                     ' Random = data generated randomly

*                                     ' File   = data from pre-existing file

*                                                (name will be prompted for)

*****************************************************************************

*        Describe your hardware (type of A/D card, addresses, etc.)         *

*****************************************************************************

AnalogToDigitalConverterType = 9      'See list below for details

FirstSeismometerOrientation =  E      'Orientation (N = North/South)

FirstSeismometerAddress =    544      'I/O Address of 1st seismometer

FirstSeismometerChannel =      0      'Channel of 1st seismometer

SecondSeismometerOrientation = N      'Orientation (E = East/West)

SecondSeismometerAddress =     0      'I/O Address (0 if not installed)

SecondSeismometerChannel =     1      'Channel (specify 0 if not installed)

ThirdSeismometerOrientation =  Z      'Orientation (Z = Vertical)

ThirdSeismometerAddress =      0      'I/O Address (0 if not installed)

ThirdSeismometerChannel =      2      'Channel (specify 0 if not installed)

SeismometerToMonitor =         E      'Choose one device to monitor (N, E or Z)

AdcValueAtMaxVolts =        4095      'Digital value when signal >= max Volts

AdcValueAtZeroVolts =       2048      'Digital value when signal = 0 Volts

AdcValueAtMinVolts =           0      'Digital value when signal <= min Volts

*****************************************************************************

*        "Trigger" - screens out as many "false alarms" as possible         *

*****************************************************************************

NoiseRangeToIgnore =           5      'Range (%) around ADC zero value to ignore

SampleCountToSaveFile =       25      '# samples outside null zone required

SamplesPerDataPoint =          5      '# samples to average for one data point

*****************************************************************************

*        Describe your station                                              *

*****************************************************************************

*                1---5---10---15

StationName =       San Jose, CA      'Max of 15 characters, single blanks only

StationID =                  FOX      '3 character (A-Z, 0-9) ID for your site

RecordingLatitude =       111.22      'Lat. of station (neg. if S of Equator)

RecordingLongitude =     -333.44      'Long. of station (neg. if W of Greenwich)

*****************************************************************************

*        Customize the look and operation of your station                   *

*****************************************************************************

DisplayLines =                 5      '# lines on screen (0 for no display)

DisplayCompression =          20      'Vertical compression of displayed signal

*                                     ����������������������������������������Ŀ

Software description  19

SavePath =               C:\QB45      �Path to use when saving quake files.    �

*                                     �Example   Path used                     �

*                                     �-------   ------------------------------�

*                                     �C:        Current directory on C drive  �

*                                     �C:\       Root directory on C drive     �

*                                     �C:\QUAKES QUAKES directory on C drive   �

*                                     �          (first level directory)       �

*                                     �QUAKES    QUAKES dir in current PATH    �

*                                     �          (might be 1st lvl or sub-dir) �

*                                     �\QUAKES   QUAKES dir on current drive   �

*                                     �          (first-level directory)       �

*                                     �A:        Current directory on A drive  �

*                                     �A:\       Root dir on A drive           �

*                                     ������������������������������������������

*

TemporaryBufferSize =       4000      'Size of each temp buffer (max = 8000)

UserCommand1 =         QUAKEVU /      'Commands to execute on marked file ("/"

UserCommand2 =         COPY / A:      'will be replaced by the name of the

*                                     'currently marked file.  Single blanks

*                                     'only please.

ADCOffset =                    0      'Offset to add to each sample if your

*                                      amlifier does not have an "offset" or

*                                      "zero" adjustment.  This value should

*                                      normally be set to zero.

ADCProgrammableGain =         10      'Gain to set when setting the on-board

*                                     'amplifier on some ADC boards like DR110

*                                      (ignored if board in use doesn't support)

*                                      DR110 supports values of: 1, 10 and 100

*****************************************************************************

*        These parameters are no longer used, but are included for          *

*        compatibility with earlier versions of the program, which          *

*        required that they be present.  The are not listed on the          *

*        startup screen.

*****************************************************************************

*

DirectoryProgram =           DIR      'Program to use for listing directories

CopyPath =                    A:      'Path to use when copying files

*

*****************************************************************************

*        The list below shows the A/D devices which are supported by        *

*        EMON.                                                              *

*****************************************************************************

*                                      0 = Burr Brown ADC674A

*                                      1 = IBM Data Acquisition and Control

*                                          Adapter.  Channels 0-3 are

*                                          available for analog input.

*                                          Use Address=0 for the first card

*                                          when using DAC.COM or DACAT.COM.

*                                          (Address 544 is used by special

*                                           versions of the code that write

*                                           directly to the card, bypassing

*                                           DAC.COM).

*                                      2 = Metrabyte DAS-4

*                                      3 = Modified IBM Parallel Printer

*                                          via Jan Froom's COLLECT routine

Software description  20

*                                      4 = A-BUS AN/146

*                                      5 = PC-LAB 714 Super Card.  544 is the

*                                          the default address (hex 220) with

*                                          switches set to 0-0-0-1-0.  For

*                                          this device, the sampled value is

*                                          echoed to D/A channel 1 to drive a

*                                          plotter.  Value of 0 gives -5V,

*                                          value of 16383 gives +5V on D/A.

*                                      6 = MetraByte DAS-16

*                                      7 = PC-LAB 711S

*                                      8 = ACQUTEK PA-CP12

*                                      9 = Real Time Devices DR110 13-bit A/D

The parameters in the OPT file are described in more detail below.

Parameters which are new in Version 6.1

�  DisplayTimeCompression

This is a number which controls what portion  of the data collected

is displayed on the  screen.  A  value  of  1  will  display  every

sample.  A value of N (where N > 1)  will  display  only  every Nth

sample.  This will  allow data to remain on  the screen longer, but

will not affect the sampling rate.

�  FirstSeismometerActive

�  SecondSeismometerActive

�  ThirdSeismometerActive

-  YES - device will be sampled and data saved.

-  NO  - device will not be sampled.

Note: devices must be activated in order, from 1st to 3rd.  Do  not

skip a device.

Parameters which are new in Version 6.0a-k

�  FileNameDisplay

-  SHORT - Display only name of file being saved (not extension).

-  LONG  - Display  entire name  plus extension of  the file  being

saved.  To make  room for the  additional  characters,  only the

hour of the time the file was saved is displayed.

In the previous version of EMON,  only  the  name of the quake data

file was displayed on the screen  but not its  extension.  This was

because the extension was a constant, the location ID.

Software description  21

In EMON V6 you can now put  variable data  (like a sequence number)

in the  file extension.  If you choose to put  variable data in the

file  extension,  you should choose  the LONG option to display the

entire  file name.  If you choose the LONG option, only the hour of

the time at which the  file was saved is displayed.  If you  choose

the SHORT option, both the hour and the minute of the time the file

was saved are displayed.

�  PromptOnQuit

-  YES - Prompt before leaving EMON

-  NO  - Leave immediately without prompting

Note: If a  quake is  in  progress,  you  will always  be given the

opportunity to save the current data before exiting.

�  BeepOnFileSave

-  NO  - No beep

-  YES - Beep when a quake file is saved

-  ALARM  - Beep every 15 minutes until a  key  is  pressed, once a

file is saved

-  IMMEDIATE  -  Same as ALARM but begin beeping immediately when a

quake is detected, without waiting for a file to be saved.

�  AlarmLevel

A  number between 0 and 100.  Each time  a file is saved to disk, a

"relative strength" value is calculated.  This is  a percentage  of

the range of the A/D card that the input signal reached during that

event.  100 means the A/D card reached its limit.  If you only want

EMON to beep or sound an alarm for larger quakes, set this value to

a higher number.  If  you  want EMON to beep or sound the alarm for

all quakes saved to disk, set this value to zero.

Note: AlarmLevel is activated only if  BeepOnFileSave is set to YES

or ALARM.  If BeepOnFileSave is set to IMMEDIATE, *all* files  will

trigger the audible alarm and AlarmLevel will be ignored.

�  FileFormat

-  NEW - Generate data files in new version 6.00 format

-  OLD - Generate data files in old format.

EMON  V6.00 has the capability  to  write quake data  files  in  an

improved format.  With the new format files, more information about

your  ADC card can  be kept in the file,  which  will  allow better

display  and analysis of the data files.  Also, the new quake  file

format has room for you to insert more information about  the quake

(like depth and USGS  magnitude) in new  reserved  locations in the

Software description  22

file for improved estimates of distance.

To  display  the new-format data files, you will need QUAKEVU v6.00

or higher.  If your version of QUAKEVU is older than this, you  can

tell EMON  to go back to creating old-format  data files by setting

FileFormat to OLD.  However  it is recommended that you  allow EMON

to create  the new format files and use QUAKEVU v6.00 or higher, or

WinQuake v1.6 or higher.  Either of these new display programs will

still work fine on all your old format data files.

If you don't specifically set  the FileFormat  parameter,  it  will

default to NEW.  However, an annoying Note screen will pop up every

time you start EMON.  To  eliminate this screen, add a line setting

FileFormat to NEW (or OLD) to your options file.

�  ActivityLog

-  YES - Write a record to a trace file for significant events

-  NO  - No tracing

The  EMON activity  log  can help you determine what went wrong  if

there is a problem  saving data files  or figuring out which  files

correspond to which event.  A one line entry  with date and time is

appended to the log file each time EMON is started or stopped, or a

data file is saved to disk.

�  ActivityLogFile

If you specifid  YES to ActivityLog, enter the  name of the file to

use for the EMON activity  log.  The default value is EMON.LOG, and

the default path is the current drive and directory.

�  FirstSeismometerMagCorr

�  SecondSeismometerMagCorr

�  ThirdSeismometerMagCorr

The WinQuake  program by Larry Cochrane displays EMON data files in

a Windows environment.  It has  significant advantages over QUAKEVU

in usability, and includes some additional function  (like  Fourier

transforms) as well.  WinQuake  allows  you to define a correlation

factor  for  your  seismometer which relates the peak  value  in  a

datafile with  the  magnitude of the quake.  Once  you  empirically

determine this value you can enter it here and it will be stored in

all data files saved for that device.  (However, this  is only done

if you specify FileFormat = NEW).

�  FileNamePattern

The FileNamePattern parameter  allows  you  to choose how your data

files are named.  You  assemble  the  pattern  you  like  best from

building blocks that  represent items like the  year,  month,  day,

sequence number, time,  location  ID,  etc.  Separate  the building

Software description  23

blocks with either  slash /, plus + or dash - characters.  You  can

also place a  character string  in the file name by enclosing it in

parentheses.  Use the dot . to identify the start of the extension.

Note: do not put separator characters around the dot.

���������������������������������������������������������������������������Ŀ

�            File Name Pattern building blocks to choose from:              �

�                                                                           �

�  YY   = year                            Example:                          �

�  MM   = month                           Quake at 3:03 am on 2 May 94      �

�  DD   = day                             Vertical sensor, 1st quake of day �

�  JJJ  = Julian date (001-366)                                             �

�  HH   = hour (00-23)                    Pattern ----------> File name     �

�  H    = hour (A-X)                      ����������������    �����������-  �

�  TT   = minute (00-59)                  MM-DD-YY-O-S.III    050294Z1.TBJ  �

�  T    = two minute interval (A-Z,0-3)   (QK)+YY+MM+DD.II+S  QK940502.GC1  �

�  O    = Orientation (N, E or Z)         YY/MM/DD/O/S.III    940502Z1.RWC  �

�  S    = seq no. (A-Z,0-9)               YY-MM-DD-II.O-SS    940502RC.N01  �

�  SS   = seq no. (01-99)    "            YY-JJJ-H-M-O.III    9412232Z.MSS  �

�  SSS  = seq no. (001-999)  "                                              �

�  II   = station ID (1st two chars)    * Filename is maximum 8 characters  �

�  III  = station ID (three chars)      * Extension is maximum 3 characters �

�  (cc) = character string, any length  * You must use "O" somewhere in the �

�  .    = . (separates name from ext)     pattern if you have >1 seismometer�

�                                                                           �

�����������������������������������������������������������������������������

Parameters which existed in Version 5.xx

�  OperatingMode

EMON can operate (obtain its data) in one of three ways.

1. Normal

This is the normal mode of operation in which data  is read from

the  Analog  to  Digital  Converter  (ADC).  The  card  must  be

installed and operational or unpredictable results may occur.

2. Random

If "random"  is  specified, no  ADC card is necessary to run the

program.  A random signal will  be generated internally  by  the

program and displayed on the screen as if it were real data.  In

"random"  mode the "trigger" algorithm is overridden.  Every set

of 25000  samples generated is saved on disk as if an earthquake

had occurred.   You can  use this  mode to test the  program and

adjust other parameters.

3. File

If "file"  is specified, data is read from a previously-recorded

quake file.  You will be prompted during EMON initialization for

Software description  24

the  name of  an  existing data file.  This  file will be loaded

during initialization, and EMON  will  get its input, sample  by

sample, from  that set  of data.  This capability allows  you to

test  new trigger settings  on  old  recordings.  When trying  a

radically new trigger setting, you might want to be sure that an

event which was seen and saved at the old settings  would not be

missed at the new settings.

�  FirstSeismometerOrientation

Acceptable values are

-  N for a device whose axis is aligned in a North-South direction,

-  E  for a device whose axis is aligned in an East-West direction,

and

-  Z for a device which is sensitive to vertical motion.

�  FirstSeismometerAddress

This  is the decimal I/O address of the  card in the  PC containing

the   analog-to-digital    converter   chip.   Usually   the   card

manufacturer provides switches on the card with  which the user can

choose one of several different addresses.

�  FirstSeismometerChannel

Some ADC cards  have  input connectors for  several  inputs.  These

separate inputs are called channels.  See  "Addressing"  on page 12

for further information on setting the Address and Channel settings

for your card.

�  SecondSeismometerOrientation, Address, Channel

Same as  for the first seismometer.  Set to 0 if you  do not have a

second seismometer.

�  ThirdSeismometerOrientation, Address, Channel

Same as for the first  seismometer.  Set to 0 if you do not  have a

third seismometer.

�  SeismometerToMonitor

Acceptable  values are N, E or Z.  Choose  which device you wish to

use for the on-screen real time display.  You can change  this from

the keyboard while the program is running.

�  AdcValueAtMaxVolts

�  AdcValueAtZeroVolts

�  AdcValueAtMinVolts

Software description  25

These three parameters tell EMON the range of numbers your ADC card

generates at (1)  its highest acceptable input voltage, (2) at zero

volts input, and (3) at its lowest acceptable  input voltage.  Some

cards  convert  voltages  to  values from  0  to  4096,  with  2048

corresponding  to  0V.  Other  cards  generate values from -2048 to

2047 with a 0V value of 0.  Still  other cards give you a choice by

having switches  on the card that you can  set.  It really  doesn't

matter which kind of card you have.  Just  make  sure you tell EMON

the proper values for your card.

�  NoiseRangeToIgnore

This parameter is  a percentage  of  the total ADC card range above

and  below the value of  AdcValueAtZeroVolts.  Signals  within this

zone will not be counted toward the decision to  save a quake file.

For example if your  ADC returned values from 0 to 4095 with a zero

value of  2048, specifying NoiseRangeToIgnore = 10  would result in

EMON ignoring any points within a range of  10% (10 * 4096 / 100  =

409.6, rounded  to  410) above  or below 2048.  So  any data points

within a range  of (2048-410) to (2048+410) would be ignored, while

data points outside this zone would contribute toward the  decision

to save a quake file.  A typical value in a noisy environment might

be 10-15.  In a quiet area, 1-3 is possible.

�  SampleCountToSaveFile

This is the number of samples  outside the noise zone  that must be

observed before a quake file will  be saved.  This prevents lots of

junk files being saved when only one or two points  above and below

the noise zone are observed.  Typical value might be 50 or 100.

�  SamplesPerDataPoint

Multiple  ADC  samples  can be averaged  to  obtain one data point.

This  provides limited smoothing if you are in a noisy environment,

but its  biggest  benefit is to give you a  way to control how many

samples  are   taken  per  second.  EMON  does  not  delay  between

obtaining  samples, so the only way to slow down  the sampling rate

is  to  tell it  to  make  each data point  an  average  of several

samples.

The fastest sampling rate is achieved  by setting  this value to 1.

If this  is not fast enough you  either need a faster PC, a  faster

ADC card or both.  However, sampling at the  maximum  rate may have

some of the following drawbacks (especially on a faster PC):

-  Each  file  may then contain only 5-10 minutes worth of data, so

you will have many more files to deal with.

-  You  may find  that distant events span  multiple  files,  which

means you will have to use MSPLICE to combine the files later.

Try  to achieve a sampling rate of about 10-20 per  second.  On  an

XT-class machine you may have  to tell EMON  to average 5-8 samples

for each data point to achieve this.  If you are running EMON  on a

Software description  26

286 or faster machine (AT-class) you may need to average as many as

40-50 samples for each data point to achieve a final  sampling rate

of 10-20 per second.  The  actual sampling rate is displayed on the

EMON screen.

�  StationName

Specify up to 15 characters  for  your  station  name.  Only single

blanks  are allowed in the name.  This  information is  placed into

each quake file as it is saved.

�  StationID

Specify a three character ID  identifying your station.  This  will

be used as the file name extension (".xxx") on all files saved.

�  RecordingLatitude

Enter the latitude of your station, with up to three decimal digits

of  accuracy (fewer  is OK  too).  Use  positive values if you  are

north of the  Equator, negative  values if  you are south.  If your

map provides latitude only in degrees, minutes and seconds you will

have  to  convert  to degrees.  For example, 16  degrees 30 minutes

North latitude would be entered  as  16.5,  since 30 minutes is one

half of a degree.

�  RecordingLongititude

Enter the  longitude  of  your  station,  with  up to three decimal

digits of  accuracy (fewer is OK too).  Use positive  values if you

are east of Greenwich, negative values if you are west.

�  DisplayLines

Enter the number of lines of data to display  on  the screen if you

wish  a  real-time  display  of   data.  Entering  0  disables  the

real-time display.  Only  odd numbers are acceptable.  If you enter

an even number, EMON rounds it up to the next higher odd number.

�  DisplayCompression

Enter a number to use as a vertical compression factor for the EMON

real-time  display.  The  higher this  number, the more  compressed

each line of the display will be.  Generally a value of 10-25 works

well.  The  number  you will need to get a good display will depend

on the gain of your amplifier and  the range  of voltages generated

by your seismometer.

�  SavePath

This is the DOS  "path" that EMON  will use  to locate the disk and

directory in which to save quake files.  It can consist of

-  just a  drive letter (for example, C:) in which case the current

directory for that disk is assumed,

Software description  27

-  just  a  directory (in which case the named directory is assumed

to be a sub-directory in  the  current directory on  the current

drive, or

-  a fully qualified combination of drive and directory.

The directory must exist.  If it does not already exist at startup,

EMON  will issue  a warning message  and terminate.  The  following

examples are all acceptable:

SavePath           Meaning

----------         ----------------------------------------------------------

C:                 Use current directory on C drive

C:\                Use root directory on C drive

QUAKES             Use QUAKES (sub-)directory on current drive and directory

C:\QUAKES          Use QUAKES directory on C drive regardless of current dir

C:\QUAKES\         Same as C:\QUAKES

A:                 Use current directory on A drive

B:\                Use root directory on B drive

The following examples are not  acceptable since they are not valid

DOS paths:

A

C::

C\

�  TemporaryBufferSize

If  you  look into the  Appendix  in  which  the program  logic  is

described, you'll  see  how  the Temporary Buffers  "A" and "B" are

used.  Unless you care how often EMON examines the counters related

to  quake  signals,  leave this  at  4000.  Acceptable  values  are

1000-8000.

�  UserCommand1

�  UserCommand2

Here you specify two  commands  which  can be executed  against the

currently marked file on the screen.  Data  collection is suspended

during the execution  of  a  UserCommand.  Place  the "/" character

wherever you  would  like  the name of the file to  be substituted.

For example, if the on-screen file marker were  pointing  at a file

named 93066ACN.TBJ, the following substitutions would be made:

UserCommand             Actual command executed

-----------             -----------------------

QUAKEVU /      ===>     QUAKEVU 93066ACN.TBJ

QUAKEVU / /Q   ===>     QUAKEVU 93066ACN.TBJ /Q

COPY / A:      ===>     COPY 93066ACN.TBJ A:

MYBAT /        ===>     MYBAT 93066ACN.TBJ

In  the first example above, the QUAKEVU program will be invoked to

view  the  file.  Your  PC must  have enough "conventional  memory"

Software description  28

(i.e.  memory  below  640K) to  allow several  large programs to be

active at one time,  or  you  may get  an "out  of memory" message.

"Memory maker" utilities which  move portions of DOS above the 640K

line may help you, but I  have  not tested them.  Examples might be

QEMM and MemMaker.

In the  second  example  above,  QUAKEVU is invoked  with the /Q or

QuickDisplay option.  This tells QUAKEVU to display only every 10th

point in the file.  On slower PCs this can give you a quick idea of

whether the file contains anything interesting.

In  the third example above,  the file is  to be copied to  the  A:

drive.

In  the  last  example  above,  the  user  has written a  BAT  file

containing multiple DOS  commands, and  passes  it the name of  the

marked file as its first parameter.

�  ADCOffset

This  is  a constant  value  to  be  added  to  each  sample taken.

Negative numbers are  acceptable also.  This can  be handy  if your

amplifier does not have a "zero" or "level" adjustment and you want

to center the signal on the screen.

�  ADCProgrammableGain

Certain  ADC  cards  contain  onboard  amplifiers   whose  gain  is

controllable by the software.  The  Real Time  Devices DR110 is one

example.  This card supports  gain of 1, 10 and 100.  This value is

ignored if the ADC card you are using does not support programmable

gain.

EMON MAIN SCREEN

The EMON main screen is shown below.

Software description  29

��������������������������������������������������������������������Ŀ

�                                                                    �

����������������������������������������������������������������������

Figure 9. EMON typical screen display

Header area

The first  line shows  the current date  (in standard and Julian form)

and  time,  the  version of EMON  you are running,  and  the  address,

channel and orientation of the seismometer currently being monitored.

The second  line  shows your  location and station  id,  latitude  and

longitude of  your station, and which of  the  three  internal buffers

("A", "B"  or  "Q") is being filled.  The program  alternates  between

filling  the "A" and "B" buffers until a  quake has been detected.  At

this time it switches to filling the "Q" buffer.

The third line displays the sampling rate in samples per second.  This

line (along with the time of day) is updated only every few seconds to

avoid slowing down the sampling rate.

Seismometer output display area

This area contains  a trace of the output of the seismometer.  You can

specify  how many lines  of trace it is to  show in the OPT file along

with the vertical scale.  The large tick mark on the left  edge of the

display  box shows where  the line  sould  be  when there is no  quake

activity.  The  small tick marks above  and  below this represent  the

upper and  lower  boundaries  of the  "noise zone".  If you change the

size  of the noise zone with  the F4  key  (or editing the  OPT file),

these markers will be adjusted to the new size.

Filename display area

This area shows  the files that  have been  saved  since  program  was

started.  For each file the file name, date and time are shown.  There

is also  a number between  1  and  100 which  describes  the  relative

intensity of the quake  record contained in the file.  (A value of 100

simply means  that the ADC card  recorded a signal  which  reached its

maximum  range.  Lower  numbers  signify  correspondingly  lower  peak

signals.)  This is to give you a rough idea of the size  of  the event

in the file.

A  small marker is always in front  of one of the file names.  You can

move this  marker with the up  and down arrow  keys.  The  UserCommand

keys always operate on the file with the marker in front of it.

All of the actions you can take from the keyboard are described below.

Software description  30

EMON KEYBOARD ACTIONS

Up Arrow / Down Arrow

Use the  up and down arrow keys to select the file you wish to operate

on with the UserCommand  keys.  See  the  description  of the EMON.OPT

UserCommand1and UserCommand2parameters for further information.

F1 - Erase Current File

Erases the file currently  being pointed to on the screen.  You  would

generally use this  key  only  after  first  examining  the  file with

QUAKEVU, and copying all or part of it to another disk.  Copy the file

first if you want to keep it!

F2 - Turn On/Off Data Trace

The data trace shows the digitized value of each sample taken.  It  is

useful for debugging the A/D converter, or understanding noise levels.

If trace is off, pressing this key turns it on, and vice versa.

F3 - A/D Conversions Per Sample

This value is  the number  of separate  analog-to-digital  samples  to

average to get one  data point.  In  noisy environments, some  control

over noise  can be obtained by averaging  several samples, but  at the

cost of  slowing down the sampling  rate.  On  the  other hand, if you

wish to slow down the sampling rate, just specify a number larger than

1 for this parameter.  Initial value specified in EMON.OPT.

F4 - Noise Zone Width

Percentage above and below the "zero volts" value of the A/D converter

that defines  the  "noise zone".  Only samples outside this noise zone

will count toward the decision to save data to disk.

F5 - DOS command

Single DOS commands can be entered, or you can just press ENTER to  be

placed at the DOS prompt.  Typing EXIT returns you to EMON.

F6 - Save path:

Drive and path to be used  when saving  data  files.  A  hard drive or

1.2Mb floppy disk are recommended.  It  is possible to save files to a

5 1/4"  floppy, but only 7 files fit on a  standard 360K disk.  If the

disk  fills  up,  you  will  be prompted  to  insert a  disk with more

available space.  Sampling will  not resume until the data  file  just

obtained has been successfully saved to disk.

F7 - Display Next Device

When more than one seismometer  is  being monitored, pressing this key

will rotate the display among the devices.

Software description  31

F8 - <UserCommand1> <File>

F9 - <UserCommand2> <File>

This key executes a command against  the  listed file.  The command is

specified  in  the  UserCommand  parameters  in EMON.OPT.  The  string

"<File>" is replaced by the name of the file currently marked with the

">" character on the screen.

F10 - Safe Quit

Return to DOS.  If a quake is in progress,  you will be asked if it is

acceptable  to discard  the data currently  being  collected.  If  you

reply "n",  the  current  data  will  be  saved in a "short" data file

before exiting.

ADC BOARDS SUPPORTED

EMON currently supports the following ADC data acquisition boards:

�  IBM Data Acquisition and Control Adapter (12-bit)

�  MetraByte DAS-4 (8-bit)

�  MetraByte DASH-16 (12-bit)

�  A  homemade board based on  the Real Time Devices PD-100  prototype

board  for  the  IBM  PC, and the  Burr-Brown ADC674 analog-digital

converter (12-bit).

�  Alpha Products AN-146 A/D converter (12-bit).

�  A homemade board based on modifying the PC parallel printer port to

be a bi-directional I/O port.

�  PC-Lab 714 SuperCard

�  PC-Lab 711S

�  Real Time Devices DR112 (13-bit)

An  entry in the OPT file allows you to  specify the type of board you

are using.

If  you  have  a  different model of  data  acquisition board,  please

contact me and I will be glad to add support for it if I can.

DISPLAYING QUAKE FILES WITH QUAKEVU

The QUAKEVU  program  displays  the  quake files captured by the  EMON

program.  It uses the  Color Graphics Adapter (CGA) in high resolution

(640x200) mode.  This is equivalent to SCREEN 2 in IBM BASIC.

Software description  32

Note: If you are running QUAKEVU under DOS version 3.1 or higher,  you

must issue the DOS command "GRAPHICS" or "GRAFTABL" once each time you

power-on or re-boot the  machine.  If you do not  execute this command

before running QUAKEVU, the screen will look very strange with lots of

fuzzy square characters in the menus.  DOS  began  to require  this in

version  3  as part of their support for languages other than English.

If you find  you have this problem,  the simplest solution would be to

add  a line  with  the proper command  to your PC's AUTOEXEC.BAT file.

QUAKEVU will remind you when it starts if you need to run the GRAFTABL

command but have not done so.

QUAKEVU FEATURES

Functions supported by QUAKEVU are:

�  Multiple  files  from  the same event (from  the  same or different

recording stations) can be displayed simultaneously

�  Mouse support with pull-down menus

�  Online help

�  Variable scale, on both X (Time) and Y (Amplitude) axes

�  Interactive P- and S-wave identification via graphic pointers

�  Automatic  epicenter distance and travel time calculations as the P

and S pointers are moved

�  Expanded view ("zoom") on a subset of the data in a file

�  Ability to create a new data file containing only a subset  of data

in the original file (and a comment if you wish)

�  Connect data points with lines, or plot as points only

MOUSE INTERFACE TO QUAKEVU

If your  PC  has a mouse,  you  can  use  it  to  do  all of the above

functions via  pop-down  menus.  Several pop-down  menu  items  list a

keyboard function key (like F1  or F2) that does the same thing as the

menu bar.  You can use either one.

ARROW KEYS WHILE ZOOM'ED

If  you  are zoomed in on  a small section of a  file, you can use the

left and right keyboard arrow keys to move left and  right through the

file.  Each keypress moves one screen left or right.

Software description  33

Further help is available in the program.  Enter QUAKEVU ? for details

or select HELP while running the program.

PRINTING THE SCREEN FROM QUAKEVU

Use the Shift-PRTSC key to print the screen if you want to save a hard

copy.  However, the DOS Print Screen function prints only in character

mode  and QUAKEVU runs  in  graphics  mode.  Luckily, DOS  supplies  a

command called GRAPHICS which takes care of this problem.  Execute the

GRAPHICS command  once each  time you power-on your  PC.  Once you run

this command, the  Print  Screen key will  print your QUAKEVU  screens

properly.  As with the GRAFTABL command the easiest solution may be to

add the GRAPHICS command to the AUTOEXEC.BAT file of the  PC where you

usually run QUAKEVU.

QUAKEVU EXAMPLE

Let's use QUAKEVU  to examine a  recording  of  a recent magnitude 6.0

earthquake  in  Hokkaido, Japan  on January  19,  1993.  This  was  an

aftershock  of the large magnitude  6.6  quake  that caused tremendous

damage in Hokkaido on January 15.

For this  example we'll assume we're using an older, slower  model PC,

so we'll invoke QUAKEVU with the QuickDisplay option (/Q) as follows:

QUAKEVU 93019OIN.TBJ /Q

The  /Q option speeds up the display on  slower  PCs  by plotting only

every  10th sample and not connecting the points.  However,  the rough

outline you see is usually enough to  let you zoom in on  the  area of

interest.  Then you can turn  off the QuickDisplay  option to  see all

the details of the quake.

If  you  are  running QUAKEVU on a PS/2 with a 286 or faster processor

you will probably never need to use the /Q option.

Software description  34

Figure 10 on page  36  shows  the QUAKEVU initial  display.  Even with

only the rough outline displayed, we can see which area of the display

contains the  interesting  parts of the  earthquake  record.  We  have

bracketed this area by  choosing  Screen/Left Marker (or  pressing F4)

for  Zoom  and Screen/Right Marker for Zoom (or  pressing F4 a  second

time) and  placing the brackets around  the area of interest.  We  are

about to click on Zoom (or press F4 a third time).

Software description  35

�����������������������������������������������������Ŀ

�������������������������������������������������������

Figure 10. Initial QUAKEVU display  using  QuickDisplay

(/Q) option

In Figure 11 on page 37  we have zoomed  in  on  the area of interest,

and are still using QuickDisplay to speed up  the  drawing.  (We  have

also expanded the vertical scale from 55 to 10  to expand the trace in

the vertical direction.)  Now that we have the interesting part of the

trace on the  screen,  let's  turn  QuickDisplay  off and "connect the

dots" to see the actual record as it would have  been  recorded  on  a

chart recorder.

Software description  36

�����������������������������������������������������Ŀ

�������������������������������������������������������

Figure 11. Zoom display of the area of interest.

In Figure 12 on page  38  we can now see for the first time the actual

earthquake record.  You  can  begin to see  the arrival of the P waves

(not  very  distinct  due  to local noise), the S  waves (first  large

vertical  waves, quite distinct)  and other waves (L waves, the second

large vertical wave group).

Now let's tell QUAKEVU where we think the P and S wave arrivals are.

Software description  37

�����������������������������������������������������Ŀ

�������������������������������������������������������

Figure 12.  QUAKEVU screen after  turning  QuickDisplay

off

Now in Figure 13 on page 39  we have selected P/S Waves and placed the

P and S markers at our best estimate of  the P and S arrivals.  Notice

that as we move the pointers, QUAKEVU calculates  the origin  time and

the distance from  the  recording  station to the  earthquake location

based on our placement of the  P and  S markers.  In this example  the

calculated  origin time of  14:40:23  does not  exactly match the true

(USGS) origin time of 14:39:27 since the PC clock was off by about one

minute.  However the distance estimate (640 km)  matches closely  with

the actual distance to the earthquake location.  (The  actual distance

was  determined by using the  HOWFAR program to calculate the distance

between  the  recording  location  and the  earthquake  location.  The

actual latitude and longitude  of the earthquake was obtained from the

U.S. Geological Survey and newspaper stories.)

Software description  38

��������������������������������������������������������������������Ŀ

����������������������������������������������������������������������

Figure 13. Origin time & distance calc. using P/S pointers

SAVING A SUBSET OF THE ORIGINAL FILE

At  this  point you will probably want to use File/Save (F8) to save a

smaller size file  containing  only the portion of  the  original file

currently  being displayed.  Select  File/Save (or  press F8) and  you

will  be prompted to specify up to 7  characters of a file name.  (The

8th character and the file extension are copied from the original file

name).  You can enter names like the following:

a:*            Saves file on a: drive with same name as large file

b:myquake      Saves file as b:myquake.lll (where lll is station ID)

\quakes\*      Saves file in QUAKES directory with same name

myquake        Saves file in current drive/directory as myquake.lll

*              Replaces input file (large) with subset file (small)

PRINTING THE SCREEN CONTENTS

You might want to print what is on the screen to have a hard copy  for

your  files.  To  print the screen contents, you must have issued  the

DOS  command "GRAPHICS" in the  DOS session  before  entering QUAKEVU.

This command tells DOS that all future PrintScreen  key presses should

assume the screen is in graphics mode (like QUAKEVU uses), rather than

just as text.  If  you  did  not issue  the  GRAPHICS  command  before

invoking QUAKEVU, you  can do so now via  the  DOS  Command key.  Then

just press the Shift-PrtSc key and DOS will print your screen image on

the printer.  Note - some laser printers  will not properly print this

type of screen.  You may  have  to  use an "old-fashioned"  dot-matrix

printer or some other type of screen-capture program.

DATA FILES

Quake data files contain exactly 25,000 data  points when  first saved

to disk by  EMON.  As mentioned in the QUAKEVU example, you can create

a  smaller  file  containing  just  the  interesting  portions   of  a

recording.  First, load  the  file  into  QUAKEVU.  Next, use the Zoom

function,  and perhaps  the right and left arrow keys  (or  CTRL-arrow

keys), to  show just  the interesting part of the  file on the screen.

Finally, use the  SAVE  function  to  save just the  part  of the file

showing  on  the  screen.  It  is  a  good idea  to do this if you are

uploading the  file  to a BBS  - it can save telephone charges  if you

upload smaller  files, and saves other people money when they download

your files to their PCs.  During the save process you will be asked if

you want to add descriptive information which  you may have determined

through other sources to reserved spaces in  the file.  These might be

things like the true origin time, or a comment.  For  files  saved  by

EMON  V6 or  later you will  be asked  if you want  to  add additional

Software description  39

information like depth, latitude and longitude.

DATA FILE NAMES

Data file names for files created by EMON V5

Data file names are designed to  pack as much information  as possible

in  the eleven characters  that DOS allows us to use.  The  format for

quake file names is YYDDDHMO.LLL, where:

�  YY is the year

�  DDD is  the day of the year (YYDDD is  sometimes called the "Julian

date")

�  H is a letter between A and X corresponding  to  the hour in  which

the first sample in  the  file was obtained.  For example,  a  file

with "A"  in this position contains data from sometime in the first

hour after midnight.

�  M  is  a  letter  between A  and  L corresponding to  the  5-minute

interval in the hour during which the first sample in  the file was

obtained.  A file with "C" in this position  was collected starting

at around 10 minutes after the hour.

�  O is the orientation of the seismometer - N for a device whose boom

was oriented North-South, E for one oriented East-West, and Z for a

vertical unit.  You  supply this  description  in  the OPT file for

each device you monitor.

�  LLL  is the 3-character location code you  supply in the  OPT file.

This code identifies  your monitoring station.  It should be unique

among the groups with which you share data.

As an example, a file saved on January 3, 1992 at 1:08am in Wappingers

Falls, NY on a N-S oriented device might be named 92003BBN.WPF.

Data file names for files created by EMON V6

The pattern  for  data file names  created by EMON V6 is chosen by the

station  operator.  Please  see  EMON  Options  File  on page 16 for a

description of the file name patterns which are possible.

Software description  40

BIBLIOGRAPHY

GENERAL BACKGROUND

Bolt, B. A..  Earthquakes.  W. H. Freeman, 1988.

Gere,  J.  M.  and H. C. Shah.  Terra  Non  Firma,  Understanding  and

Preparing for Earthquakes.  W. H. Freeman, 1984.

"Earthquakes & Volcanoes", published bimonthly by the U. S. Geological

Survey.  Available from: Sup't.  of  Documents,  U. S. Gov't  Printing

Office, Washington, DC 20402.

SEISMIC MONITORING STATION PRACTICES

World  Data  Center  A for Solid  Earth Geophysics,  September,  1979.

Manual of Seismological Observatory Practice, Report SE-20.  Available

from:  United States Dept.  of  Commerce,  National  Oceanographic and

Atmospheric  Administration,  National  Geophysical Data  Center,  325

Broadway, Boulder, Co. 80303

HOME SEISMOMETER CONSTRUCTION AND DESIGN

Various older  Scientific American "Amateur Scientist" columns:  June,

1953, July, 1957, May, 1961, and August, 1970.

"The Amateur Scientist: A sensitive mercury tiltmeter that serves as a

seismometer", Scientific American, November, 1973, pp. 124-129

"The Amateur Scientist: Electronic stratagems are the  key to making a

sensitive  seismometer",  Scientific  American,  September,  1975, pp.

182-187

"The Amateur Scientist: How  to  build a  simple seismograph to record

earthquake  waves  at  home".  Amateur  Scientist  column,  Scientific

American, July, 1979,  pp. 152-161 (Plus a correction to the amplifier

schematic in the Amateur Scientist column, August, 1979)

EARTHQUAKE INTERPRETATION

Simon,  Ruth L.  Earthquake  Interpretations,  A  Manual  For  Reading

Seismograms.  Los Altos, Ca., William Kaufman, Inc., 1981.

Jeffreys, H. and K. E. Bullen.  Seismological Tables.  Smith & Ritchie

Ltd., Edinburgh, 1967.

Bibliography  41

PERSONAL COMPUTER DATA ACQUISITION

Webster,  E.,  1986. How To Use  Your  BBC Computer As a Cheap Storage

Oscilloscope.  Phys. Educ. 21, pp. 119-122

Ciarcia, S.  A., "Computers on the Brain:  Clever  signal  amplifiers,

noise rejection and A/D conversion are all part of the HAL EEG", Byte,

June, 1988.

SEISMIC SOFTWARE DESIGN

Allen,  R.,  1982.  Automated Phase Pickers:  Their  Present  Use  and

Future Prospects.  Bull. Seis. Soc. Am. 72(6), pp.S225-S242

Goforth,   T.  and  E.  Herrin,  1981.  An  Automatic  Seismic  Signal

Detection  Algorithm Based On The Walsh Transform.  Bull.  Seis.  Soc.

Am. 71(4), pp.1351-1360

SEISMOMETERS IN THE CLASSROOM

Barker,  G. "A Working Seismograph For The Classroom".  Michigan Earth

Scientist, Michigan  Earth  Science  Teachers  Association, September,

1983.

Barker, G. "Calculating the  Size and Properties of the Earth's Core".

Michigan Earth Scientist, Michigan Earth Science Teachers Association,

June, 1984.

Bibliography  42