VESA DPMS screen saver

����������������������

OVERVIEW

DpmSave is a DOS screen saver for 'green' (power-saving) monitors

that conform to the VESA DPMS standard.

This screen saver does not simply blank the screen, it will

switch your computer monitor into suspend (power-saving) mode!

REQUIREMENTS

You need three things:

- PC compatible computer running MS-DOS

- computer monitor that conforms to the VESA DPMS standard

- VGA compatible graphics card

Optional:

- XMS memory and a driver program for upper memory blocks.

If this is available, DpmSave can load itself into an

upper memory block, using none of the main 640kB memory.

WARNING

DPMS (Display Power Management Signaling) signals the monitor to go to

suspend or sleep mode by switching off one or both sync signals.

THIS MISSING OF SYNC SIGNALS WILL CAUSE *PERMANENT* *DAMAGE*

TO SEVERAL OF THE OLDER COMPUTER MONITORS THAT DO NOT SUPPORT DPMS!

The program DpmSave can not detect whether or not your monitor does

support DPMS. It's your responsibility to find out and not use this

program if your monitor does not support DPMS. Please consult the

manual of your monitor or ask the dealer or manufacturer. If DPMS is

not mentioned in the manual, then the monitor most probably does not

support it.

If you hear a high pitch sound from your monitor when DpmSave

activates or the picture gets squeezed or scrambled, switch the

monitor off immediately! However note that absence of these warning

signals does not tell that your monitor does support DPMS. You can not

try it, you must look it up.

COPYRIGHT and COST

This program and its documentation are fully copyrighted

(c) Copyright 1995 by Klaus Hartnegg, Kleist-street 7,

D-79331 Teningen, Germany.

All rights reserved except as stated below:

Disclaimer: It's almost impossible to write any non-trivial computer

program that does not have some bugs. You use this program entirely on

your own risk. I may not be held liable for any damage. If you don't

like this, don't use it at all.

You might COPY and DISTRIBUTE this program and documentation if both

are UNMODIFIED and distributed together and it's not distributed as

part of or addon to a commercial program.

You might USE this version of DpmSave FREE OF CHARGE.

QUICK START

This section is for experienced users only. Please skip this

if you want to know more about the program before trying it.

1. Make sure that your computer meets all requirements mentioned above,

especially the monitor must conform to the DPMS standard!

2. Execute the command 'DPMSAVE t30s' and wait 20 seconds without

pressing any keys. You should now hear several short beeps.

After another 10 seconds the screen should become blank and

your monitor should switch to suspend mode.

IF IT DOESN'T WORK, PRESS RESET TO REBOOT YOUR PC!

3. If it did work: reboot to get rid of the trial version, then

execute the command 'DPMSAVE t10 umb klr' to install it with more

useful options (activate after 10 minutes, load into upper memory,

define hotkey left+right shift key).

DESCRIPTION

DpmSave monitors all keyboard input. Whenever no key was pressed for a

certain time, it will switch off the vertical sync signal. This causes

a VESA DPMS monitor to go to suspend mode.

DpmSave will issue several short beeps 10 seconds before activation.

If you hear this and don't want it do activate right now, press

any key to restart the counter.

Once DpmSave is activated, it continues to monitor the keyboard.

Any further keypress will cause the program to switch the computer

monitor back on.

All programs should continue running while DpmSave is activated.

DpmSave will not activate while Microsoft Windows is running.

Whenever it wants to activate, it tests for presence of Windows.

If it is found, the time counter is reset. See the section

about LIMITATIONS below for more informations.

INSTALLATION

To install DpmSave, you must specify the time after that power saving

should be activated. If you specify no time, the program will not stay

resident, but display its online help.

Syntax:    DpmSave [tX[s]] [umb] [kXY]

Options:   tX                 install and activate after X minutes

tXs                install and activate after X seconds

umb                load into upper memory if available

kXY                define hotkey

Example:   DpmSave t10 umb klr

To have DpmSave started automatically each time the computer is switched

on, include the above command in the file C:\AUTOEXEC.BAT

IMPORTANT NOTE:

You should choose an activation time that does not cause the monitor

to be switched on and off to often because this can shorten the lifetime

of the monitor. Power-Saving should be activated only a few times per

day and it should usually not be deactivated shortly after activation.

The manual of you monitor should define safe limits for this.

HOTKEY:

The power saving can optionally be activated by pressing a hotkey.

When this key is pressed, power saving will be immediately be

activated without waiting for the activation time to pass.

To enable this feature, a hotkey must be specified when installing

DpmSave.

The hotkey is a combination of two, three or four of the special keys

Alt, Ctrl, Left-Shift, Right-Shift pressed simultaneously.

To define a hotkey, use the option kXY, where X and Y are keys from

the following table:

A: Left Alt

C: Left Ctrl

L: Left Shift

R: Right Shift

Example: to use Alt+Ctrl as hotkey, specify the option KAC.

You could use the command line: DpmSave t10 umb kac

Then whenever you press the Alt and Control-Key together,

the power saving will be immediately activated.

MEMORY USAGE

This is a TSR (terminate and stay resident) program. This means that

part of it will stay resident in memory.

I have made every effort to make the memory requirement as small as

possible. The result is one of the smallest TSR programs that I have

ever seen: the memory-resident part occupies only 416 bytes (plus 16

Bytes for a memory control block of DOS). No, that's not a typo,

it really needs only four-hundred-and-sixteen bytes!!

Additionally DpmSave can load itself into an upper memory block

thus using absolutely none of the lower 640kB of memory.

DO YOU WANT TO BE ABLE TO WRITE SUCH TSR PROGRAMS YOURSELF

USING TURBO PASCAL? PLEASE CONTACT ME FOR INFORMATIONS ABOUT

MY TSR DEVELOPER TOOLKIT!

Loading DpmSave into upper memory should be done with the 'umb'

option of the program, not with the loadhigh-command of a memory

manager. With a memory manager about 17000 bytes of upper memory

are required during installation. If this amount is not available,

loading will fail. The built in umb-installation works even if

only 432 Bytes are available.

If you load DpmSave low (i.e. not into upper memory), DOS will

automatically allocate additional memory for a copy of all environment

variables. DpmSave will release this memory. This creates a memory

hole. This memory will be recycled if another program is loaded that

needs the same amount for its environment. Otherwise the hole stays

empty.

Hint: You can save memory by installing TSR programs before setting

environment variables.

LIMITATIONS

DpmSave currently monitors only keyboard input, no mouse usage.

DpmSave will not activate while Microsoft Windows is running

because inside Windows it cannot detect keyboard input. Windows

stops all memory resident programs that intercept the keyboard

interrupt. If somebody knows how to change this, please tell me!

Removing DpmSave from memory is currently not supported but it's

possible using third party software (mark,release and my freeumb).

Please forgive my poor english, it's not my native language!

KNOWN BUGS

If the display mode changes while the monitor is in power-saving mode

(for example the computer switches into graphics mode), then the mode

might be disturbed when DpmSave de-activates. Use the DOS command

MODE CO80 to reset the display.

Some monitors seem to like better if the other sync signal

is switched off. I will try to offer both in future versions.

If you find another bug or problem, or have suggestions, please

write me an e-mail to my Internet address below.

REVISION HISTORY

v1.0  23-Apr-95

first published version

v1.1  02-May-95

oops: beep forgot to save cx-register, sometimes caused reboot

command line syntax changed

hotkey

load itself into XMS upper memory block

v1.2  13-May-95

needs less memory in upper memory block

v1.3  27-May-95

load itself into DOS 5+ upper memory block

v1.3a docs updated

FUTURE PLANS

optionally require password for deactivation (see my program KeyLock)

do also switch off hard disks (modern IDE and almost all SCSI disks)

change parameters without reinstallation

optionally load into high memory area instead of upper memory block

work inside Microsoft Windows (anybody interested? can anyone help?)

monitor mouse movements and mouse keys (anybody interested?)

These will likely be implemented only if I receive enough feedback.

Further suggestions, comments etc. are welcome.

LEGAL STUFF

All trademarks mentioned in this document are trademarks or registered

trademarks of their respective companies and are hereby acknowledged.

Turbo Pascal is a great Pascal Compiler from Borland. MS-DOS and

MS-Windows are from Microsoft Corp. VGA, VESA and DPMS are likely

registered too. VGA was developped by IBM. VESA is the Video

Electronic Standard Association. DPMS is the Display Power Management

Signalling standard defined by the VESA. Some trademarks may be

missing in this list; this does not mean that they are not protected,

I just forgot to mention them. If I made other mistakes, please

forgive me - I'm not a lawyer, just a programmer.

AUTHOR

Klaus Hartnegg

Kleist-Strasse 7

D-79331 Teningen

Germany

Internet e-mail:

hartnegg@einstein.freinet.de       (home machine)

hartnegg@sun2.ruf.uni-freiburg.de  (university)