DISKIO - Fixed Disk Benchmark, Version 1.9

(C) 1994-1995 Kai Uwe Rommel

Dhrystone 2.1 C benchmark routines (C) 1988 Reinhold P. Weicker

This is a simple fixed disk benchmark program for OS/2. It measures

(approximately) the following numbers:

- data transfer rate (on inner and outer tracks)

- drive cache/bus transfer rate (introduced with version 1.6)

- CPU load caused by continuous disk I/O (introduced with version 1.9)

- average latency time

- average data access time

It also measures CPU speed without disk I/O using the Dhrystone CPU

benchmark which is also used for the CPU load caused by disk I/O

calculation.

Of course all values are influenced by other activity on the system,

so stop all other software if you want reliable results. The results

will also differ slightly between repeated runs.

The benchmark needs about 65 seconds to complete. The benchmark is run

for every fixed disk in the system, if started without arguments. To

run it only for a specific disk, pass the number of the disk (or

multiple numbers for multiple disks) to the program as argument. Disks

are counted starting with 1.

The CPU load caused by the disk I/O of the benchmark is "measured" by

first calculating the Dhrystone benchmark result without any disk I/O

and then the number of Dhrystone's the CPU could run in the background

during drive cache/bus transfers. The lower the percentage number, the

less CPU time is (ab)used by the disk interface. This depends on the

quality and technology of the disk interface (i.e. if it uses PIO or

DMA).

A few sample results, numbers listed are the data transfer rate on

track 0 (k/sec), latency time (ms), access time (ms), cache/bus speed

(k/sec), CPU usage (last two numbers only if result was available):

- Pentium PCI/SCSI systems:

Quantum Atlas + AHA-2940 + P-133:		6800, 1.3, 13, 6900, 20%

HP C3323 + NCR 53c810 + P-100:			4569, 1.2, 18, 7016, 21%

IBM DPES-31080 + NCR 53c810 + P-100:		4186, 1.1, 18, 7008, 15%

DEC DSP-3160 + NCR 53c810 + P-120:		3930, 2, 16, 6370, 15%

DEC DSP-3133L + NCR 53c810 + P-100:		4393, 1.5, 17, 6124, 22%

DEC DSP-3133L + AHA-2940 + P-66:		4350, 2, 15

- Pentium PCI/EIDE (Intel Triton) systems:

Quantum LPS-540A + IBM1S506.ADD + P-100:	2177, 1.3, 19, 2227, 92%

Quantum LPS-540A + TRIOS2.ADD + P-100:		3532, 1.5, 20, 6083, 9%

- 486 PCI/SCSI systems:

HP C3323 + NCR 53c810 + 486-100:		3945, 2, 18, 5045, 54%

IBM DPES-31080 + NCR 53c810 + 486-100:		4239, 1.3, 18, 6613, 46%

DEC DSP-3160 + NCR 53c810 + 486-66:		3930, 2, 16

DEC DSP-3053L + NCR 53c810 + 486-66:		4130, 3, 16

Quantum LPS-340S + NCR 53c810 + 486-66:		1900, 5, 23

Maxtor 7345S + NCR 53c810 + 486-66:		1881, 10, 17

- 486 PCI/IDE systems:

Quantum LPS-540A + IBM1S506.ADD + 486-100:	1796, 1.6, 20, 1820, 94%

- 486 EISA/SCSI systems:

DEC DSP-3210 + AHA-2740 + 486-50:		3650, 2, 17

DEC DSP-3160 + AHA-1742 + 486-66:		3050, 3, 18

Quantum LPS-270S + AHA-2740 + 486-66:		?

- 486 ISA systems:

Quantum LPS-340S + AHA-1542CF + 486-50:		1508, 7, 26

Quantum LPS-540A + IDE + 486-66:		1492, 4, 21

There is no warranty. Use this software on your own risk. Due to the

complexity and variety of today's hardware and software which may be

used to run this program, I am not responsible for any damage or loss of

data caused by use of this software. It was tested very well and is

expected to work correctly, but nobody can actually guarantee this for

any circumstances. And because this software is free, you get what you

pay for ...

This program can be used freely for private or educational purposes. If

you want to use it for commercial purposes or find any bugs or have

suggestions about further enhancement, please contact the author.

Author:   Kai Uwe Rommel

Muenchen, Germany

Fax:      +49 89 324 4524

E-Mail:   rommel@ars.de (preferred)

rommel@leo.org