DRD60ASP.SYS INFORMATION

------------------------

This README file contains some additional information to supplement that

contained in the User's Manual.

-----------------------------  ATTENTION  -----------------------------

1. Please make sure that the DRM-600/604X drive's SCSI ID number, which is

set by the dip switches at the rear of the changer, and the other SCSI

device's (eg. SCSI hard disk or MO disk) ID numbers are not the same.

This is to ensure that there are no undesirable effects when power is

turned on. For more detailed information on the SCSI device's ID number,

please consult your SCSI device's user's manuals.

2. WARNING, in the interest of safety and of possible harmful side

effects to the devices that are to be interconnected. Please ensure that

power to the devices is turned off before attempting to connect DRM-600/604X

to a computer.

----------------------------  VERSION UP  -----------------------------

1. The CD-ROM device version has been upgraded to 1.10.  A bug in the

LDP/CDP interface has been fixed.  When the driver is used with the

MPC.COM utility, the proper information will be displayed.

2. Version 1.10 of the DRD60ASP device driver will auto-search the first

SCSI host adapter for a Pioneer CD-ROM drive if the /S: parameter is not

specified.

3. Multiple host adapter support has been added back in to version 1.12

����������������������������������������������������������������������������

Dip Switch Settings (ROM version 0804)

������������������ͻ

� 1 2 3 4 5 6 7 8  �

� ڿڿڿڿڿڿڿڿ �

� ���������������� �

��ͳ��ͳͳͳͳͳͳͼ

����� � � � � �

�   � � � � �

SCSI ID ����������   � � � � �Disable Eject Switch

� � � �Disk Check

1    2   3   ID          � � �Reserved

����������������         � �Reserved

off off off�  0          �2048/512 Default Sector Size (off = 2048)

on  off off�  1

off on  off�  2

on  on  off�  3

off off on �  4

on  off on �  5

off on  on �  6

on  on  on �  7

����������������������������������������������������������������������������

Dip Switch Settings (ROM version 0801, 0803)

������������������ͻ

� 1 2 3 4 5 6 7 8  �

� ڿڿڿڿڿڿڿڿ �

� ���������������� �

��ͳ��ͳͳͳͳͳͳͼ

����� � � � � �

�   � � � � �

SCSI ID ����������   � � � � �Disable Eject Switch

� � � �Disk Check

1    2   3   ID          � � �Reserved

����������������         � �Reserved

off off off�  0          �Reserved

on  off off�  1

off on  off�  2

on  on  off�  3

off off on �  4

on  off on �  5

off on  on �  6

on  on  on �  7

����������������������������������������������������������������������������

DRD6ASP.SYS command line switches

Switch                Use                              Example / Default

����������������      ���������������������������      ������������

/D:(device name)      Specifies the device name        /D:MSCD000

for MSCDEX.

/S:n1(;b1)(,n2(;b2))  SCSI ID of the target            /S:0

drive(s).  Set to the same

ID as the CD-ROM drive.

/N:n1,(n2)            Specifies the number of CD-      /N:6

ROM's in the cartridge.

/C:(device name)      Specifies the device name of     /C:CDP

character device for CDP

control.

/B: or /E:            Specifies whether or not the

device driver allocates a

buffer for prefetch.

/B: -> if EMS not available use conventional memory

/E: -> EMS memory

����������������������������������������������������������������������������

MSCDEX command line switches

Switch                Use                              Example

����������������      ���������������������������      ������������

/D:(device name)      Specifies the device name        /D:MSCD000

of the CD-ROM device

driver.

/E                    Use EMS memory for buffers

/L:(drive letter)     Tells MSCDEX which drive         /L:M

letter to start from when

assigning drive letters.

/K                    Tells MSCDEX to use any

Kanji (Japanese) file

structures if present.

/S                    Tells MSCDEX to patch DOS

to allow sharing of CD-ROM

drives on MS-NET based

networks.

/M:(value)            Tells MSCDEX how many cache      /M:10

buffers to allocate.  Default

is 4 (8 Kbytes).

/V                    Provides memory usage

statistics of how much memory

is used by buffers, resident

data, and resident code.

#