Device Driver INFORMATION Ver 2.03

----------------------------------             Nov 1994

This README file contains some additional information to supplement that

contained in the User's Manual.

------------------------------------------------------------------------------

DRD600.SYS , DRD60ASP.SYS, DRD60PS.SYS

------------------------------------------------------------------------------

1. SCSI interface borad

DRD600.SYS is the device driver for FUTURE DOMAIN SCSI interface

board (8bit, 16bit and microchannel) or it's compatible (PIONEER DD-C5002).

DRD60PS.SYS is the device driver for IBM PS/2 SCSI Adapter /A.

Please use the appropriate driver for your interface board.

DRD60ASP.SYS is an ASPI device driver.  It will work with any host

adapter that support the ASPI protocol. See later information.

2. MPC.COM utility

This utility is a remote control software to reproduce an audio CD for

Pioneer's 6 mini changer CD-ROM drive, DRM-600, DRM-604X, and DRM-602X.

It dose not work with DR-S104X.

3. DISC.EXE utility

The DRD600 series device driver can handle a maximum of 12 Discs.

Some of the currently available applications can only work on systems

that have a desiginated drive letter and subunit number. The DISC.EXE

utility program allows the desigination of the CD-ROM subunit No. to

be in any desired order, but only in conjunction with the DRD600

series Device Driver.

The use of this program is explaind below;

To see the current order of the Discs.

>DISC cd-rom_device_name

>DISC cd-rom_drive_letter:

The drive letter is required ':'.

To change the current order of the Discs.

>DISC cd-rom_device_name [-R] no1 no2 .....

The position of the number to be changed is switched with the

position of the number its replacing. Using the -R option reseting

of the order from 0 upwards is possible.

To exchange two drive letters.

>DISC cd-rom_drive_letter: cd-rom_drive_letter:

4. DRDUTL.EXE utility

The DRD600 series device driver and the MSCDEX.EXE allows the MS-DOS

to add multiple CD-ROM drives. Currently, some applications will waste

time do useless actions or can't handle the device driver error messages,

if multiple CD-ROM drives are connected. This utility changes the driver's

behavior (mode) to permit the optimum performance of this type of

application. Use this utility only when your application program requests

this kind of change of mode.

The use of this program is explained below:

To see the current mode.

>DRDUTL CD-ROM_device_name

>DRDUTL CD-ROM_drive_letter:

The drive letter is required ':'.

To change the current mode.

>DRDUTL CD-ROM_device_name switch [switch] ..

>DRDUTL CD-ROM_drive_letter: switch [switch] ..

switch

DSKCHK:ON(OFF) Enables or disables the disc check operation.

SEEK:ON(OFF)   Enables or disables the seek operation.

VDRTRY:ON(OFF) Enables or disables "Invalid Medium Change"

error message when reading the Volume Descriptor.

Follow the setting of these switches as the application request.

****************************** NEW UTILITY *******************************

5. XDISC.EXE utility

This utility allows selection of an active disc for the assigned CD-ROM

Drive letter when the drive has been assigned only one drive letter.

This permits up to eight (8) DRM-602X or DRM-604X to be on line and

occupying only one drive letter each. The driver line in the CONFIG.SYS

would be as follows:

DEVICE=C:\PIONEER\DRD600.SYS /D:MSCD000 /N:0,0,0,0 /S:0,1,2,3

The /N:0,0,0,0 will assign only one drive letter to each unit. In the above

example four CD-ROM drive letters are assigned. The /S:0,1,2,3 is the

SCSI address and each drive and must be set accordingly.

NOTE: The DRD600.SYS driver is for Future Domain SCSI cards only.

The XDISC utility is then used to switch the disc required to be read.

XDISC Drive: <Disc Number>

To select the third disc in the magazine in drive F:.

XDISC F: 3

To see the number of the currently active disc.

XDISC Drive:

XDISC F:

For the above example this would return the number 3.

Note: This utility will not work with earlier versions of the drivers.

------------------------------- Attention ---------------------------------

The IBM SCSI Adapter /A interface board checks the SCSI devices when

the computer is booted up. If the SCSI device doesn't answer correctly,

the device isn't available once the computer has booted up. Please turn

the power of the drive on before or at the same time as that of the

computer.

---------------------------------------------------------------------------

DRD60ASP.SYS

---------------------------------------------------------------------------

DRD60ASP.SYS is an ASPI device driver for Pioneer's CDROM drives.

It supports Multisession PhotoCD, and will autosearch for drives.

ASPI

---------------------------------------------------------------------------

YOU MUST HAVE THE ASPI MANAGER FOR YOUR SCSI CARD!!!!!!!!!!

The ASPI manager software is provided by the SCSI card vendor, not Pioneer.

Examples:

Adaptec	ASPIxDOS.SYS

Future Domain	ASPICAM.SYS or ASPIFD.SYS

UltraStor	USPI.SYS

NOTE:  Many ASPI managers require the "/L" parameter to support Logical

Units.  Without Logical Unit support, only the top disc can be accessed.

The ASPI manager must be in CONFIG.SYS  *BEFORE*  DRD60ASP.SYS.

INSTALLING DRD60ASP.SYS

---------------------------------------------------------------------------

(Release version will come with installation software)

Create a directory on your hard drive:

"md c:\pioneer"

Copy the floppy to that directory:

"copy a:\*.*  c:\pioneer"

Edit CONFIG.SYS

Add the DEVICE statement BELOW the ASPI manager

"DEVICE=C:\PIONEER\DRD60ASP.SYS /D:MSCD000"

^^^^^^^^^^^  MATCH

EDIT AUTOEXEC.BAT

Add mscdex.exe BEFORE windows or any menu program:

"C:\PIONEER\MSCDEX.EXE  /D:MSCD000"

^^^^^^^^^^^  MATCH

NOTE: The "/D:" parameters do not have to be "MSCD000", but they must match.

Use something else ("MSCD001", for example) if another driver is using

"MSCD000".