PC-RESON version 1.00

Command and Element Reference

February 1992

By John A. Priller and John Vincent

Edited by Becky Palmer-Scott

National Superconducting Cyclotron Laboratory

Michigan State University

East Lansing, MI 48824-1321

TABLE OF CONTENTS

COPYRIGHT INFORMATION & PROGRAM DESCRIPTION..................... 1

REQUIRED SOFTWARE & HARDWARE ................................... 2

RUNNING RESON .................................................. 3

CONVENTIONS & TIPS ON USING RESON FOR AC ANALYSIS .............. 4

Nomenclature: Elements, Terminals, and Nodes ................. 4

Circuit Nodes ................................................ 4

Voltage Loops ................................................ 4

Linear AC Analysis ........................................... 5

Current Flow ................................................. 5

Output Dimensions ............................................ 5

Tips ......................................................... 5

INPUT FILE FORMAT .............................................. 6

Text Format .................................................. 6

Upper and lower case ....................................... 6

Numeric fields ............................................. 6

Text strings ............................................... 7

Line Format .................................................. 7

Comment format ............................................. 7

Command format ............................................. 7

Element statement format ................................... 7

Sample Input Files ........................................... 8

OUTPUT FUNCTIONS ............................................... 9

V -- Voltage ................................................. 9

VD -- Device Voltage ......................................... 9

I -- Current ................................................ 10

ID -- Device Current ........................................ 10

IS -- Source Current ........................................ 10

IN -- Node Current .......................................... 10

P -- Power .................................................. 11

U -- Energy ................................................. 11

Q -- Quality Factor ......................................... 11

N -- Node Number at Given Terminal .......................... 11

RESON COMMANDS ................................................ 12

Define Data Parameters ...................................... 12

.MAT ...................................................... 12

.UNIT ..................................................... 12

.VSCALE ................................................... 12

Invoke Circuit Analysis ..................................... 13

.AC ....................................................... 13

.REPORT ................................................... 13

.INCLUDE .................................................. 13

Specify Output Appearance...................................  14

.COLUMNS .................................................. 14

.MARGIN ................................................... 14

.PRECISION ................................................ 14

.TITLE .................................................... 14

Specify and Generate Output Topics .......................... 15

.PRINT ...................................................  15

.OUTPUT ................................................... 16

.COUPLE ................................................... 17

.SHUNT .................................................... 17

(Continued on next page)

TABLE OF CONTENTS

(cont.)

RESON ELEMENT STATEMENTS: GENERAL FORMAT ...................... 18

NAME ........................................................ 18

Terminal Connections ........................................ 18

Required Parameters ......................................... 18

Optional Parameters ......................................... 18

RESON ELEMENT STATEMENTS: INDIVIDUAL FORMATS .................. 19

Components .................................................. 19

C -- Capacitor ............................................ 19

K -- Coupled coil ......................................... 19

L -- Inductor ............................................. 19

R -- Resistor ............................................. 19

Transmission lines ........................................ 20

T -- General transmission line .......................... 20

TC -- Coaxial transmission line ......................... 20

TR -- Rectangular (plate) transmission line ............. 20

Voltage Sources ............................................. 22

V -- Independent voltage source ........................... 22

H -- Current-controlled voltage source .................... 22

E -- Voltage-controlled voltage source .................... 22

Current Sources ............................................. 23

I -- Independent current source ........................... 23

F -- Current-controlled voltage source .................... 23

G -- Voltage-controlled voltage source .................... 23

ERROR MESSAGES ................................................ 24

RESON Errors ................................................ 24

Compiler Errors ............................................. 25

DOS errors ............................................... 25

I/O errors ............................................... 27

Critical errors .......................................... 28

Fatal errors ............................................. 28

- 1 -

COPYRIGHT INFORMATION

The copyright ((c) 1991, 1992) on this software product is held by

John Vincent, John A. Priller, and Michigan State University. All

rights are reserved. The above individuals and institutions

reserve the right to limit distribution of this software, within

the bylaws of Michigan State University, and within the copyright

laws of the State of Michigan and of the United States of America.

The authors of this software have used their best efforts in

preparing it. These efforts include development, research and

testing of the theories and algorithms to determine their

correctness and effectiveness. The authors of this software and

Michigan State University make no warranty of any kind, expressed

or implied, with regard to this software or to its documentation.

The authors of this software and Michigan State University shall

not be liable in any event for incidental or consequential damages

in connection with, or rising out of, the furnishing, performance,

use or misuse of this software.

PROGRAM DESCRIPTION

RESON is a enhanced AC circuit-analysis program modeled loosely on

SPICE. "Enhanced" means that beyond solving for voltages and

currents, RESON computes circulating and dissipated energy, and

allows for material properties and geometry. RESON also has

additional output functions and formats.

All of RESON's input and output files are standard ASCII, and can

be created and read by most computer systems. Input files look

similar to SPICE input files. Input files contain statements and

comments which describe the circuit and the analysis to be

performed. (Since the program currently does AC analysis only,

SPICE-like statements for DC or transient analysis are not

included). Output files have quote-delimited strings and comma-

delimited numeric fields, so that they may be easily imported into

spreadsheet programs (such as Lotus 123, or Borland's Quattro).

All user I/O and data management is performed by routines written

by John Priller, while the computation and analysis engine was

written by John Vincent. Please direct questions pertaining to

RESON to the appropriate individual.

- 2 -

REQUIRED SOFTWARE

RESON is shipped with the following files:

RESON.EXE -- The RESON program

RESON.DOC -- This document

MATERIAL.RSN -- The materials data file

UNITS.RSN -- The units data file

README.DOC -- Last-minute notes and information

*.DAT-- Sample input files

RESON requires the MS-DOS or PC-DOS version 2.1 (or higher)

operating system to run. It may also be possible to run RESON from

a Microsoft Windows 3.0 or OS/2 "DOS window", though this has not

yet been verified.

REQUIRED HARDWARE

RESON requires the following minimum hardware setup:

- An IBM-compatible PC with an Intel 8088 or higher CPU; an 80386

or i486 is preferred.

- A math coprocessor chip, such as an 80387 (the i486 chip

includes a math coprocessor) is recommended but not necessary.

- 640K of RAM

- A hard disk is recommended but not necessary.

- 3 -

RUNNING RESON

RESON reads an input file (e.g. RFSTUFF.DAT) and creates an *.OUT

file (e.g. RFSTUFF.OUT) and any other output files requested

in the input file (e.g.  RFSTUFF.0...RFSTUFF.n). If errors are

encountered, error log RFSTUFF.LOG is created instead of the

output files.

RESON files consist of command statements and element statements

(described in this manual), and any desired comment statements.

To run RESON, type

RESON filename.ext         ; RESON accepts any filename

or

RESON filename             ; ".DAT" is assumed if no extender is

given

or

RESON filename.ext > nul   ; "nul" prevents program status from

RESON filename > nul          appearing on screen during the

run.

With each new RESON run, new output files are created -- RESON

will not append data from a new run onto an old output file. Old

output files with identical names will be destroyed. However,

during the same run, data can be appended to an existing output

table -- see the .PRINT, .OUTPUT, .COUPLE, and .SHUNT commands in

the section, "Specify and Generate Output Topics."

- 4 -

CONVENTIONS AND TIPS ON USING RESON FOR AC ANALYSIS

Nomenclature: Elements, Terminals, and Nodes

In RESON, "elements" are components or power sources, which

have "terminals" connecting them to a circuit. "Nodes" are the

circuit locations the terminals connect to.

In most statements, you replace each terminal designator with

the number of its circuit node. Terminals are numbered left to

right, so in this resistor definition,

RSHUNT 15 4 100k

terminal 1 connects to node 15, and terminal 2 to node 4.

However, terminal numbers are used explicitly in some .OUTPUT

and .PRINT functions. For the resistor defined above, the

following commands request the same voltage sets:

.PRINT AC V(15) V(4)                ; Voltages at terminal 1

and terminal 2 requested

=                                      with corresponding nodes

.PRINT AC VD(RSHUNT,1) VD(RSHUNT,2) ; Voltages at terminal 1

and terminal 2 requested

with terminal numbers

Circuit Nodes

Node numbers are always positive integers. They do not have to

be used sequentially -- node numbers may be skipped if desired.

Node 0 is reserved for ground. The circuit must have

connections to this node.

Each node (including ground) must have at least two

connections. RESON will check the circuit for this requirement

before attempting to process it. Nodes do not require a DC path

to ground.

Voltage Loops

Loops of voltage are illegal. RESON does not currently check

for this condition, so the user must take care that it does not

occur.

- 5 -

Linear AC Analysis

RESON's AC analysis solves for the small-signal frequency

response. All non-linear elements are made linear about the

operating point by user input model parameters or by idealized

default parameters. For example, if the model parameters

specify a gain of ten, then the gain remains ten no matter what

the input level. An input of 1 would result in an output of 10,

an input of 10 results in 100, etc. The gain has become linear.

Current Direction

1) Positive current flows in the direction positive charges

would go.

2) Positive current flows into component terminals.

3) Positive current flows through sources from the positive to

the negative terminal.

Output Dimensions

All output dimensions are in meters, regardless of input units.

See also Tips below.

Tips

-- To get average power and energy values from the output

functions, enter all source values in RMS units. To get peak

power and energy values, enter all source values in peak

units.

-- In most RESON statements, you need to replace the terminal

number with the corresponding circuit node.

-- RESON sweeps the frequency of the source(s) to determine the

circuit's response. If your circuit has only one AC input,

you can set the AC input equal to 1 with a phase angle of 0;

the output at any node will then conveniently reflect the

transfer function.

-- If you have a single input source set to 1 volt or 1 amp,

you can scale the resulting computed energies for some new

source value 'V' or 'I' by multiplying all energies by V*V

or I*I. (Also see .VSCALE command.)

- 6 -

INPUT FILE FORMAT

The circuit that RESON will analyze is described by a netlist.

This is a group of statements, each of which identifies a circuit

element, its interconnecting nodes, and its properties. In

addition to the netlist, a RESON input file also contains commands

that direct the analyses to be performed and the output to be

created. An input file may also include comment lines, which

describe the circuit and analyses for future reference. All of

these are described individually below.

Text Format

Upper and lower case

With the exception of 'm' and 'M' metric suffixes for

numeric fields, RESON is not case-sensitive; that is,

RTOP = Rtop = rTop

Numeric fields

Numeric fields in RESON can be either integer, floating

point 'F' or floating point 'E' format:

Integer            : 0        7         91          8M

'F' floating point : 17.1m    35.88k    1641.002

'E' floating point : 1E-8     4.02E+6

As shown above, metric suffixes are supported for all real-

number fields (everything except terminal and node numbers).

The suffixes RESON recognizes are:

a,A (atto) = 1.0E-15          k,K (kilo) = 1.0E+03

p,P (pico) = 1.0E-12          M (mega) = 1.0E+06

n,N (nano) = 1.0E-09          g,G (giga) = 1.0E+09

u,U (micro) = 1.0E-06         t,T (tera) = 1.0E+12

m (milli) = 1.0E-03

No spaces or other characters may occur between the final

character of the number and the suffix, and any characters

beyond the initial suffix character are ignored. All formats

below are acceptable for denoting 4 megahertz (or 4 meg of

anything):

4000000.0   * Straight 'F' floating point with no suffix

4.0E+6   * 'E' floating point

4.0M   * 'F' floating point with 'M' (mega) suffix

4Mhz   * Integer with 'M' suffix -- the 'hz' is ignored

Note also that for milli- and mega-, the uppercase and

lowercase letter 'm' is distinct.

- 7 -

Text strings

Text strings can be at most 64 characters; element names are

limited to 10 characters. Strings enclosed within quotes may

contain any characters. Those not within quotes:

- Cannot begin with a number    - May include percent (%)

- Cannot contain spaces         - May include ampersand (&)

- May include dollar sign ($)   - May include underscore (_)

Examples:      A_STRING

"This is another string"

'So is this'

Line Format

Input file syntax is free-format, with fields separated by one

or more blanks, tabs, commas or parentheses. Carriage returns

mark the end of each input line. Blank lines are ignored.

Each RESON element or command must be defined on a single line,

although a line continuation character, the vertical bar (|),

can be placed at the end of a line to denote that the following

line is a continuation of the previous one:

* This comment looks like |

a single line to RESON!

Comment format

Comments begin with an asterisk (*) or a semi-colon (;), and

are ignored during input file processing. Comments may occur

at the end of command and element lines.

Command format

RESON command lines begin a period, just as SPICE commands

do. In the following descriptions, items in angle brackets

(<>) are mandatory, those within square brackets ([]) are

optional, and options separated by vertical bars (|) mean

that one and only one of the set may be chosen. (Do not

include brackets in actual RESON statements.)

Example:   .REPORT <AC | DC> <frequency> [TITLE=string]

In this .REPORT command, AC or DC must be chosen; the

frequency must be given; there needn't be a title string.

Element statement format

No characters precede the element statement. See the RESON

ELEMENT STATEMENT sections for details on statement format.

- 8 -

Sample Input Files

These files can be found on the RESON program disk.

QWAVE2.DAT

Notice that element statements beginning "ISRC," "T1" and

"TR1" are disabled by the comment * preceding them.

;

; A quarter-wave resonator

;

; This example is from "Engineering Electromagnetics" by

; William H. Hayt, Fourth edition. pp 474 - 477.

; TC1 is the line described by Hayt while the rest of the

; lines are equivalent to TC1 using the available RESON

; models. Only one line per run should be uncommented.

;

; RLC.DAT is the equivalent shunt circuit of this resonator

;

; coupling has been added

.MAT SILVER

.Columns 6

.UNIT Centimeter

*ISRC 1 0 53.2uA 0

V1 999 0 61.912mV 0.0

Cc 999 1 1.368pF

TC1  1 2 0 Ri=1 Ro=2.72 L=37.5 E=4 QE=1000

*T1 1 2 0 Z0=30.0 TD=2.5nS E=4 QE=1000 RABAC=4.026uOhms

RCOMAC=1.48uOhms

*TR1 1 2 0 Z0=30.0 Wa=6.2832 Wc=17.0903 L=37.5 E=4 QE=1000

*TR1 1 2 0 H=1.86 Wa=6.2832 Wc=17.0903 L=37.5 E=4 QE=1000

R1   1 0 100Mohm

VShort 2 0 0.0 0.0

.AC LIN 25 98.310Mhz 98.560Mhz

.PRINT AC V(1) I(VShort)

.Report AC 98.378940Mhz

.Output 1 "" ALL $ VDm(1) IDm(1) Pc Pe Ue Uh

*.Couple 1 "Test Capacitive Couple" Node=1 Z0=50

*.Couple 1 "Test Tline Couple" Tname=TC1 Z0=50

;

;

RLC.DAT

;

; A RLC equivalent shunt circuit model of QWAVE2.DAT

;

.Columns 6

V1 999 0 62.890mV 0.0

Cc 999 1 1.368pF

L1 1 0 60.74nH

C1 1 0 41.64pF

R1 1 0 26.56kOhms

.AC LIN 25 98.310Mhz 98.560Mhz

.PRINT AC V(1) I(VShort)

.Report AC 98.4700Mhz

.Output 1 "" ALL $ VDm(1) IDm(1) Pc Pe Ue Uh

- 9 -

OUTPUT FUNCTIONS

Output functions, used in the .OUTPUT and .PRINT commands, compute

or return results based on RESON analysis. They begin with an

identifier, such as "V" or "P," which specifies the type of

information desired, and which is sometimes followed by an

operator: "i," "r," "m," or "p" to describe a complex number

component or "c," "e," or "h" to describe the type of power or

energy. Most functions also list the circuit nodes, element names,

and/or terminals the information is about. Element names are

always omitted in .OUTPUT function statements.

This section describes the functions available. In the following

descriptions, items in angle brackets (<>) are mandatory, those

within square brackets ([]) are optional, and options separated by

vertical bars (|) mean that one and only one of the set may be

chosen. In addition, items in {} brackets must be in .PRINT

commands but should not be in .OUTPUT commands. Include

parentheses but not brackets in actual RESON statements.

V -- Voltage (volt):

Determines the potential difference between two nodes, where

V = V(nodeX) - V(nodeY).

V[i|r|m|p](nodeX[,nodeY])

i = imaginary part            Example: VM(11) for .PRINT

r = real part

m = magnitude

p = phase

nodeX = a node number

nodeY = another node number; if not entered, node 0 is

assumed

VD -- Device Voltage (volt):

Determines the potential difference between the element

terminals or between an element terminal and ground, where

VD = VD(terminalX) - VD(terminalY).

VD[i|r|m|p]({element,}terminalX[,terminalY])

i = imaginary part      Examples: VD(1,2)    for .OUTPUT

r = real part                     VD(1)      for .OUTPUT

m = magnitude                     VD(C16,1,2) for .PRINT

p = phase                         VD(C16,1)   for .PRINT

element = element name

terminalX = a terminal number

terminalY = another terminal number; if not entered, the

potential difference between terminalX and

node 0 is assumed.

- 10 -

I -- Current (ampere):

Determines the current through a voltage source.

I[i|r|m|p](vname)

i = imaginary part           Example: IM(V1)  for .PRINT

r = real part

m = magnitude

p = phase

vname = voltage source name

ID -- Device Current (ampere):

Determines the current entering a device terminal.

ID[i|r|m|p]({element,}terminal)

i = imaginary part         Examples: ID(1)   for .OUTPUT

r = real part                        ID(C8,1) for .PRINT

m = magnitude

p = phase

element = element name

terminal = terminal #

IS -- Source Current (ampere):

Determines all current from current source(s) attached to the

node.

IS[i|r|m|p](node)

i = imaginary part            Example: IS(11) for .PRINT

r = real part

m = magnitude

p = phase

node = node number

IN -- Node Current (ampere):

Determines the total current crossing a node.  (Currently

doesn't work properly.)

IN[i|r|m|p](node)

i = imaginary part             Example: IN(4) for .PRINT

r = real part

m = magnitude

p = phase

node = node number

- 11 -

P -- Power (watt):

Determines the energy dissipated due to conduction currents,

electric energy, magnetic energy, or the energy delivered by

any source. Power delivered is negative; power dissipated is

positive. If no subscript modifier is given, the total power

(Pc + Pe + Ph) is reported.

P[c|e|h]{(element)}

c = conductive losses       Examples: P      for .OUTPUT

e = electric                          P(R17)  for .PRINT

h = magnetic                          PH(R44) for .PRINT

element = element name

U -- Energy (VAR):

Determines the magnetic or electric reactive energy. If no

subscript modifier is given, the total energy (Ue + Uh) is

reported.

U[e|h]{(element)}

e = electric              Examples: U       for .OUTPUT

h = magnetic                        Ue      for .OUTPUT

element = element name              Uh(LTOP) for .PRINT

Q -- Quality Factor:

Determines the quality factor Q, which equals energy/power;

(Q = U/P).

Q{(element)}

element = element name     Examples: Q      for .OUTPUT

Q(CAP1) for .PRINT

N -- Node Number at Given Terminal:

Determines the node number to which the specified terminal is

connected.

N({element,}terminal)

element = element name    Examples: N(1)     for .OUTPUT

terminal = terminal #               N(CAP1,1) for .PRINT

- 12 -

RESON COMMANDS

In the following descriptions, items in angle brackets (<>) are

mandatory, those within square brackets ([]) are optional, and

options separated by vertical bars (|) mean that one and only one

of the set may be chosen. (Do not include brackets in actual RESON

statements.)

Define Data Parameters

The following commands provide default values for the input

file. See RESON ELEMENT STATEMENTS for individualized parameter

definitions.

.MAT <material>

Defines the material that RESON should use in calculating

default properties for subsequent elements. Supported

materials are listed in the MATERIAL.RSN data file, and the

first material in this file (usually COPPER) is the default

material if none is specified. Additional materials may be

added to the MATERIAL.RSN file by the user. Multiple .MAT

commands may be used to define parts of the circuit which

are made of differing materials.

Example: .MAT SILVER

.UNIT <unit_name>

Defines the units that RESON should use in size

specifications and in calculating default properties for

subsequent elements.  Supported units are listed in the

UNITS.RSN file, and the first unit in this file (usually the

METER) is the default unit if none is specified. Additional

units may be added to the UNITS.RSN file by the user.

Multiple .UNIT commands can be used to define different

parts of the circuit.

Example: .UNIT INCH

.VSCALE <node> <value>

Scales the matrix solution so that the voltage at the given

node is the given value. Only one .VSCALE command is allowed

per case.

Example: .VSCALE 17 100

- 13 -

Invoke Circuit Analysis

.AC LIN <NumIntervals> <Start-Freq> <End-Freq>

Performs an AC analysis of the circuit defined by the input

file, over the region of frequencies specified. LIN

indicates a linear division of the frequency range.

Example: .AC LIN 10 8.9MHz 9.1MHz

.REPORT AC <frequency> [TITLE=string]

Performs a circuit analysis. Multiple .REPORT commands in an

input file will be processed in the order encountered.

Example: .REPORT AC 1.0MHz TITLE="Test circuit at 1Mhz"

.INCLUDE <filename>

Processes the specified input file for RESON commands and

elements. Any number of nested .INCLUDE files may be

specified (.INCLUDEd files may .INCLUDE other files).

Example: .INCLUDE "SUBCIRC.DAT"

- 14 -

Specify Output Appearance

.COLUMNS <number>

Specifies the maximum number of data columns for .PRINT,

.OUTPUT, .COUPLE, and .SHUNT commands. Default is 5. Only

one .COLUMNS command per input file will be obeyed; if more

than one is encountered, the last one will be used.

Example: .COLUMNS 8

.MARGIN <number>

Defines left margin of RESON output. Default is 6 spaces.

Example: .MARGIN 12

.PRECISION <freq-precision> <other-precision>

Specifies the number of significant digits to use in

floating-point output. "freq-precision" specifies the number

of significant digits for frequency output; "other-

precision" specifies the number of significant digits for

all other floating-point output.  Only one .PRECISION

command per input file will be obeyed; the last one will be

used. The default for .PRECISION is 6, 4.  The width of a

floating point data column is:

column width = 6 + significant digits + comma + space

= significant digits + 8

6 significant digits = +0.00000E+00      ; width = 14

4 significant digits = +0.000E+00        ; width = 12

Commas are added to the output to insure compatibility with

spreadsheet import formats.

Example: .PRECISION 8 6

.TITLE <string>

Specifies the title for the RESON case. "string" is a text

string and must be within quotes if more than one word.

Example: .TITLE "Test Resonator"

- 15 -

Specify and Generate Output Topics

.PRINT AC <function-list>

Writes data from the previous .AC analysis command to a

filename.OUT file. ("filename" is the same as the input

"filename".) Multiple .PRINT commands can exist in an input

file; the data will be appended to filename.OUT. Function

lists consist of one or more functions from the following

table:

Function                    Description              Example

------------------------------------------------------------

P[c|e|h](element)         ; Power                     P(R44)

U[e|h]                    ; Energy                  Ue(CAP1)

Q(element)                ; Quality factor           Q(CAP1)

N(element,terminal)       ; Node # at terminal     N(CAP1,1)

V[r|i|m|p](node1[,node2]) ; Voltage                   VM(11)

I[r|i|m|p](vname)         ; Current                   IM(V1)

IN[r|i|m|p](node)         ; Node current               IN(4)

IS[r|i|m|p](node)         ; Source current            IS(11)

VD[r|i|m|p](element,      ; Device voltage       VD(C16,1,2)

terminalX[,terminalY])

ID[r|i|m|p](element,      ; Device current          ID(C8,1)

terminal)

------------------------------------------------------------

where                               and

P = Power                            c = conductive losses

U = Energy                           e = electrical energy

Q = Quality factor                   h = magnetic energy

N = Node # at given terminal         r = real part

V = Voltage                          i = imaginary part

IN = Node current                     m = magnitude

IS = Source current                   p = phase

VD = Device voltage

ID = Device current

Examples:    .PRINT AC V(15) V(4)

.PRINT AC VD(RSHUNT,1) VD(RSHUNT,2)

.PRINT AC ISR(11) VM(11) I(12) V(12)

- 16 -

.OUTPUT <output-file#> <title> <element-list> $ <function-list>

Generates a table from a previous .REPORT analysis, with

functions in the columns and elements in the rows. Multiple

.OUTPUT commands may exist for each .REPORT .

.OUTPUT command format is below. Note the required '$'

between the element list and function list. Also, the number

of functions requested must be less than the number of

columns requested by the .COLUMNS command.

output-file# :

The numeric extender of the output file: filename.0 to

filename.9.  ("filename" is automatically the same as

the input "filename".) With each new RESON run, a brand

new output file is opened, if requested. If .OUTPUT

commands repeat the file extender during the RESON run,

the data will be appended to the output file.

title :

Text-string title for the output.

element-list :

The elements to include in the output rows. Elements

may be specified individually, in groups, or in a

combination of the two: Examples:

TR17  : Rectangular transmission line TR17

TR    : All rectangular transmission lines

C     : All capacitors

C1-R12: Input file elements from C1 to R12, in order

ALL   : All elements in input file order

function-list :

The data to be displayed in the output columns.

Includes any of the following:

VD[i|r|m|p](tx[,ty]) : Device Voltage

ID[i|r|m|p](t)       : Device Current

Q                    : "Q" quality factor

P[c|e|h]             : Power (W)    ; If no subscripts,

U[e|h]               : Energy (VA)  ; total is given.

where                             and

P = Power                         c = conductive losses

U = Energy                        e = electrical energy

Q = Quality factor                h = magnetic energy

VD = Device voltage                r = real part

ID = Device current                i = imaginary part

m = magnitude

p = phase

Examples: .OUTPUT 1 "Parallel resonator Response" R L C $ Ue P

.OUTPUT 0 "" TR R1-C11 $ VD(1,2) ID(1)

- 17 -

.COUPLE <output-file#> <title> <node=# | Tname=xxx> <Z0=R>

Generates data from the last .REPORT analysis. Multiple

.COUPLE commands may exist for each analysis. Matches a feed

transmission line of characteristic impedance "Z0" to the

circuit. If "node=#" is requested, reports the series

capacitance and frequency shift needed at that node. If

"Tname" is requested, reports the distance from terminal t2

of "Tname" transmission line where the feed transmission

line can be tied directly. The result is valid only if the

circuit is resonant.

output-file# : The numeric extender of the output file:

filename.0 to filename.9.  ("filename" is

automatically the same as the input "filename".) With

each new RESON run, a brand new output file is opened,

if requested.  If .OUTPUT commands repeat the file

extender during the run, the data will be appended to

the output file.

title : Text-string title for the output.

node=# : The statement "node=#" where "#" is the node

number of the circuit to couple to.

Tname : The statement "Tname=xxx" where "xxx" is the name

of the transmission line to couple to.

Z0=R : The statement "Z0=R", where "R" is the

characteristic impedance (in ohms) of the feed line to

match to the circuit.

Examples: .COUPLE 1 "Capacitive Coupler" node=6 Z0=50

.COUPLE 0 "Direct Feed" Tname=T11 Z0=50

.SHUNT <output-file#> <title> <node#>

Generates data from the last .REPORT analysis. Multiple

.SHUNT commands may exist for each analysis. Determines the

equivalent shunt circuit at the specified circuit node. The

result is only valid if the circuit is resonant.

output-file# : The numeric extender of the output file:

filename.0 to filename.9.  ("filename" is

automatically the same as the input "filename".) With

each new RESON run, a brand new output file is opened,

if requested. If .OUTPUT commands repeat the file

extender during the run, the data will be appended to

the output file.

title : Text-string title for the output.

node# : The node number at which to determine the

equivalent shunt circuit.

Example: .SHUNT 0 "Shunt circuit at node 10" 10

- 18 -

RESON ELEMENT STATEMENTS: GENERAL FORMAT

Element statements define the circuit that RESON will analyze.

Statement structures vary, but have several parts in common:

NAME <t1>..<tn> <required_parameter=#>..[optional_parameter=#]...

NAME

An element NAME can be up to 10 characters long. The first

character (and second character, in the case of transmission

lines) determines what type of element is being defined. The

rest of the name is determined by the user. Element names in a

circuit must be unique! First characters of names are:

C...   : capacitor

K...   : coupled coil

L...   : inductor

R...   : resistor

V...   : independent voltage source

H...   : current-controlled voltage source

E...   : voltage-controlled voltage source

I...   : independent current source

F...   : current-control current source

G...   : voltage-control current source

T...   : general transmission line

TC...  : coaxial transmission line

TR...  : rectangular transmission line

Terminal Connections

t1..tn specify the terminal connections of the element. To

insert an element into a circuit, you simply replace each

terminal designator with the number of the circuit node it

connects to.  Terminals are numbered left to right, so in this

resistor definition:

RSHUNT 15 4 100k

terminal 1 connects to node 15, and terminal 2 to node 4.

Required Parameters

Most required parameters will be a single number, but some take

the form of XX=value, where XX is a one-or-more character

identifier string, followed by an optional =, followed by a

value (almost always a number).

Optional Parameters

Optional parameters always have the XX=value form. They must

appear AFTER the required element parameters, but may appear in

any order among themselves.

- 19 -

RESON ELEMENT STATEMENTS: INDIVIDUAL FORMATS

Components

RESON follows the standard engineering convention that positive

current flows in the direction that positive charges would go,

and flows into component terminals.

C -- Capacitor:

C... <t1> <t2> <value> [Q=value]

Specifies a capacitor, with value specified in farads. If no

Q is specified, it is assumed to be infinite. Replace t1 and

t2 with the circuit nodes they connect to.

K -- Coupled coil:

K... <L1> <L2> <value>

Couples two inductors, L1 and L2. Value is the coupling

coefficient, and must be between 0 and 1. Establishes a

mutual inductance, M, between L1 and L2, where M =

value*SQRT(L1*L2).

L -- Inductor:

L... <t1> <t2> <value> [Q=value]

Specifies an inductor, with value given in henries. The

default Q value is infinite, if none is specified. Replace

t1 and t2 with the circuit nodes they connect to.

R -- Resistor:

R... <t1> <t2> <value>

Specifies a resistor. Value is in ohms. Replace t1 and t2

with the circuit nodes they connect to.

- 20 -

Transmission lines:

A transmission line consists of two conductors of equal

length. t1 and t2 are the terminal connections to the

primary conductor, while both ends of the return conductor

are tied to terminal t3. The voltage on the primary

conductor varies according to the "unsimplified" lossy

transmission line equations, while the voltage on the return

conductor is held constant to the node voltage where t3 is

tied. The current on the return conductor mirrors the

current on the primary conductor.

All length/width/height/radius parameters are assumed to be

given in the currently selected units defined by the .UNIT

command. All resistances are calculated using the default

material parameters defined by the .MAT command. Si, So, Sa

and Sc override these conductivity parameters.

All dimensions are output in meters, regardless of input.

General transmission line:

T... <t1> <t2> <t3> <Z0=value> <TD=value> <RABDC=value>

<RABAC=value> [RCOMDC=value] [RCOMAC=value] [E=value]

[M=value] [QE=value] [QM=value]

Coaxial transmission line:

TC... <t1> <t2> <t3> <Ri=value> <Ro=value> <L=value>

[Si=value] [So=value] [E=value] [M=value] [QE=value]

[QM=value]

Conductor resistance default values are computed from the

conductor materials defined by the .MAT command. Si= and

So= parameters will override these defaults.

Rectangular (plate) transmission line:

TR... <t1> <t2> <t3> <Wa=value> <Wc=value> <L=value>

[Z0=value | H=value] [Sa=value] [Sc=value] [E=value]

[M=value] [QE=value] [QM=value]

If height (H) is specified, Z0 is computed assuming the

width of each conductor is the average of Wa and Wc. If

Z0 is specified, Wa and Wc are simply used for power

calculations and height is irrelevant. At all times the

current density of the primary and return conductors is

assumed to be uniform.

See parameter definitions on next page.

- 21 -

t1       : Terminal 1; replace with corresponding circuit

node.

t2       : Terminal 2; replace with corresponding node.

t3       : Terminal 3; replace with corresponding node.

E=n      : Relative permittivity of media between

conductors.  Default = 1.0.

M=n      : Relative permeability of media between

conductors. Default = 1.0.

QE=n     : Electric material quality factor (1/loss_tangent)

computed from electric energy and losses (a

measure of fill material losses due to electric

fields).  Default = infinity.

QM=n     : Magnetic material quality factor (1/loss_tangent)

computed from magnetic energy and losses (a

measure of fill material losses due to magnetic

fields).  Default = infinity.

Z0=n     : Characteristic impedance in ohms.

Td=n     : Time delay from t1 to t2, in seconds.

RABDC=n  : DC resistance per meter of conductor spanning t1

to t2. Default = 0.0.

RABAC=n  : AC resistance per meter per sqrt(Hz) of conductor

spanning t1 to t2. Default = 0.0.

RCOMDC=n : DC resistance per meter of return conductor.

Default = 0.0.

RCOMAC=n : AC resistance per meter per sqrt(Hz) of return

conductor. Default = 0.0

Ri=n     : Radius of inner conductor in current units.

Ro=n     : Radius of outer conductor in current units.

Sa=n     : Conductivity of primary conductor (mhos/meter).

Sc=n     : Conductivity of return conductor (mhos/meter).

Si=n     : Conductivity of inner conductor (mhos/meter).

So=n     : Conductivity of outer conductor (mhos/meter).

Wa=n     : Width of primary conductor in current units.

Wc=n     : Width of return conductor in current units.

L=n      : Length of both conductors in current units.

H=n      : Height between conductors in current units.

- 22 -

Voltage Sources

For sources, RESON presumes that current flows from t1

(positive terminal) to t2 (negative terminal).

Replace t1 and t2 with the circuit nodes they connect to.

V -- Independent voltage source:

V... <t1> <t2> <magnitude> <phase>

Specifies an independent voltage source, with positive

current flowing from terminals t1 to t2. Magnitude is

specified in volts, phase in degrees.

H -- Current-controlled voltage source:

H... <t1> <t2> <Vname> <transresistance>

Specifies a current-controlled voltage source between

terminals t1 and t2. Vname is any independent voltage source

defined elsewhere in the circuit. The voltage source has the

value of I(Vname)*transresistance.

E -- Voltage-controlled voltage source:

E... <t1> <t2> <NC1> <NC2> <gain>

Specifies a voltage-controlled voltage source between

terminals t1 and t2. NC1 and NC2 are the positive and

negative controlling nodes, respectively. The voltage source

has the value of (V(NC1)-V(NC2))*gain.

- 23 -

Current Sources

For sources, RESON presumes that current flows from positive

to negative nodes.

Replace t1 and t2 with the circuit nodes they connect to.

I -- Independent current source:

I... <t1> <t2> <magnitude> <phase>

Specifies an independent current source, where t1 and t2 are

the positive and negative terminals, respectively. Current

is assumed to flow from t1 through the source to t2.

Magnitude is specified in amps, phase in degrees.

F -- Current-controlled current source:

F... <t1> <t2> <Vname> <gain>

Specifies a current-controlled current source placed between

terminals t1 and t2. Vname is any independent voltage source

defined elsewhere in the circuit. The current source the

value of I(Vname)*gain.

G -- Voltage-controlled current source:

G... <t1> <t2> <NC1> <NC2> <transconductance>

Specifies a voltage-controlled current source placed between

terminals t1 and t2. NC1 and NC2 are the positive and

negative controlling nodes, respectively. The current source

has the value of (V(NC1)-V(NC2))*transconductance.

- 24 -

ERROR MESSAGES

There are two types of error codes encountered during RESON use:

1. those detected by the RESON program, and

2. compiler run-time errors detected by Turbo Pascal. This type

of error will have the format:

Run-time error nnn at xxxx:yyyy

All errors that don't look like that are RESON errors.

RESON Errors

Error # : Code Description

---------------------------------------------------------

0 : No errors encountered

1 : Matrix size is less than 1

2 : Matrix size cannot exceed CNarraysize (200)

3 : Matrix has not been opened

4 : No solution exists

5 : Source or element not found

6 : Problem not solved

7 : Index out of range

8 : User node not found

9 : Maximum number of elements or sources exceeded

10 : No elements or sources defined

11 : Illegal node connection(s)

12 : Element specifications not complete enough for

attempted operation

13 : Using default parameters

14 : Illegal element value(s)

15 : Problem nodes not defined

16 : Invalid terminal specifications

17 : Element name already exists

18 : No circuit owner, or circuit owner is nil

19 : Circuit not post-processed

20 : Unknown element type

21 : Not enough memory

--------------------------------------------------------

- 25 -

Compiler Errors

(The following was copied from Appendix A of the Turbo Pascal

Programmer's Guide, v.6.0.)

Run-time errors cause the program to terminate and display an

error message:

Run-time error nnn at xxxx:yyyy

where "nnn" is the error number and "xxxx:yyyy" is the error

address (segment and offset).

Run-time errors are divided into four categories: DOS errors 1

through 99; I/O errors, 100 through 149; critical errors, 150

through 199; and fatal errors, 200 through 207.

-----------------------------------------------------------

DOS errors

1 : Invalid function number

You made a call to a non-existent DOS function.

2 : File not found

Reported by Reset, Append, Rename, or Erase if the

name assigned to the file variable does not specify

an existing file.

3 : Path not found

-- Reported by Reset, Rewrite, Append, Rename, or

Erase if the name assigned to the file variable is

invalid or specified a nonexistent subdirectory.

-- Reported by ChDir, MkDir, or RmDir if the path is

invalid or specified a nonexistent subdirectory.

4 : Too many open files

Reported by Reset, Rewrite, or Append if the program

has too many open files. DOS never allows more than

15 open files per process. If you get this error with

less than 15 open files, it may indicate that the

CONFIG.SYS file does not include a FILES=xx entry or

that the entry specifies too few files. Increase the

number to some suitable value, for instance, 20.

- 26 -

5 : File access denied

-- Reported by Reset or Append if FileMode allows

writing and the name assigned to the file variable

specifies a directory or a read-only file.

-- Reported by Rewrite if the directory is full or if

the name assigned to the file variable specifies a

directory or an existing read-only file.

-- Reported by Rename if the name assigned to the

file variable specifies a directory or if the new

name specifies an existing file.

-- Reported by Erase if the name assigned to the file

variable specifies a directory or a read-only

file.

-- Reported by MkDir if a file with the same name

exists in the parent directory, if there is no

room in the parent directory, or if the path

specifies a device.

-- Reported by RmDir if the directory isn't empty, if

the path doesn't specify a directory, or if the

path specifies the root directory.

-- Reported by Read or BlockRead on a typed or

untyped file if the file is not open for reading.

-- Reported by Write or BlockWrite on a typed or

untyped file if the file is not open for writing.

6 : Invalid file handle

This error is reported if an invalid file handle is

passed to a DOS system call. It should never occur;

if it does, it is an indication that the file

variable is somehow trashed.

12 : Invalid file access code

Reported by Reset or Append on a typed or untyped

file if the value of FileMode is invalid.

15 : Invalid drive number

Reported by GetDir or ChDir if the drive number is

invalid.

16 : Cannot remove current directory

Reported by RmDir if the path specifies the current

directory.

17 : Cannot rename across drives

Reported by Rename if both names are not on the same

drive.

- 27 -

-----------------------------------------------------------

I/O errors

These errors cause termination if the particular statement

was compiled in the {$|+} state. in the ($|-} state, the

program continues to execute, and the error is reported by

the IOResult function.

100 : Disk read error

Reported by Read on a typed file if you attempt to

read past the end of the file.

101 : Disk write error

Reported by Close, Write, WriteIn, Flush, or Page

if the disk becomes full.

102 : File not assigned

Reported by Reset, ReWrite, Append, Rename, and

Erase if the file variable has not been assigned a

name through a call to Assign.

103 : File not open

Reported by Close, Read, Write, Seek, Eof, FilePos,

FileSize, Flush, BlockRead, or BlockWrite if the

file is not open.

104 : File not open for input

Reported by Read, ReadIn, Eof, Eoln, SeekEof, or

SeekEoln on a text file if the file is not open for

input.

105 : File not open for output

Reported by Write and WriteIn on a text file if the

file is not open for output.

106 : Invalid numeric format

Reported by Read or ReadIn if a numeric value read

from a text file does not conform to the proper

numeric format.

- 28 -

-----------------------------------------------------------

Critical errors

150 : Disk is write-protected

151 : Unknown unit

152 : Drive not ready

153 : Unknown command

154 : CRC error in data

155 : Bad drive request structure length

156 : Disk seek error

157 : Unknown media type

158 : Sector not found

159 : Printer out of paper

160 : Device write fault

161 : Device read fault

162 : Hardware failure

Refer to your DOS programmers' reference manual for more

information about critical errors.

-----------------------------------------------------------

Fatal errors

These errors always immediately terminate the program.

200 : Division by zero

The program attempted to divide a number by zero

during a 1, mod, or div operation.

201 : Range check error

This error is reported by statements compiled in

the {$R+} state when one of the following

situations arises:

-- The index expression of an array qualifier was

out of range.

-- You attempted to assign an out-of-range value to

a variable.

-- You attempted to assign an out-of-range value as

a parameter to a procedure or function.

202 : Stack overflow error

This error is reported on entry to a procedure or

function compiled in the {$S+} state when there is

not enough stack space to allocate the subprogram's

local variables. Increase the size of the stack by

using the $M compiler directive.

This error may also be caused by infinite

recursion, or by an assembly language procedure

that does not maintain the stack project.

- 29 -

203 : Heap overflow error

This error is reported by New or GetMem when there

is not enough free space in the heap to allocate a

block of the requested size.

204 : Invalid pointer operation

This error is reported by Dispose or FreeMem if the

pointer is nil or points to a location outside the

heap, or if the free list cannot be expanded due to

a full free list or to HeapPtr being too close to

the bottom of the free list.

205 : Floating point overflow

A floating-point operation produced a number too

large for Turbo Pascal or the numeric coprocessor

(if any) to handle.

206 : Floating point underflow

A floating-point operation produced an underflow.

This error is only reported if you are using the

8087 numeric coprocessor with a control word that

unmasks underflow exceptions. By default, an

underflow causes a result of zero to be returned.

207 : Invalid floating point operation

-- The real value passed to Trunc or Round could

not be converted to an integer within the

Longint range (-2,147,483,648 to 2,147,483,647).

-- The argument passed to the Sqrt function was

negative.

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 4,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

Orders only:

1-800-2424-PSL

MC/Visa/AmEx/Discover

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394