===========================================================================

Pipx                                          Parallel Technologies, Inc.

Parallel Port Networking                                         PO Box 7

for DOS ODI Workstations                           Redmond, WA 98073-0007

(C) Copyright 1993 Jay Lowe

Lab  206-869-1136

Use of this program is subject to the                   Fax  206-869-9767

Restricted License Agreement                     Compuserve     76640,203

contained in Pipx.Doc                   Internet 76640.203@COMPUSERVE.COM

===========================================================================

README for the Pipx TEST DRIVE Release

Version 1.10 Test Drive

���������������������������������������������������Ŀ

� This is a TEST DRIVE release. It is limited to    �

� a maximum of 60 minutes of network operation.     �

�                                                   �

� You have a 15 day trial license to use the Pipx   �

� TEST DRIVE release for evaluation purposes only.  �

�����������������������������������������������������

1. Program Release Information

This documentation accompanies TEST DRIVE release of Pipx, an ODI

workstation driver which provides convenient parallel port networking

for Netware Lite and other ODI-capable network software.

Pipx supports NDIS software, such as Windows for Workgroups,

through the ODINSUP shim, with some limitations.

Pipx support TCP/IP and other software which uses the packet driver

interface through the ODIPKT shim.  These are available on Compuserve.

The TEST DRIVE release allows you to use Pipx for EVALUATION PURPOSES

ONLY for a LIMITED PERIOD OF TIME and includes certain features

which will help you remember you are using evaluation software which

is subject to a limited license agreement.

Pipx brings the extreme convenience and simplicity of parallel port

networking to the ODI environment. Pipx is part of family of parallel

port networking drivers; version for NDIS and Packet Driver interfaces

will be released in the near future.

Pipx is different from other parallel port connectivity products:

-  it allows you to use first-class networking software

-  it allows integrated networking with Ethernet and Token Ring

-  it supports all types of ports and four types of cables

-  it offers low-end or high-end performance choices

2. Contents of the distribution package

LICENSE  DOC      3803 04-04-93   6:28p   The License Agreement

ORDER    TXT      3232 04-22-93   2:08p   Copy of Pipx Order Form

PIPX     COM     29454 05-24-93  11:33a   Pipx ODI driver

PIPX86   COM     29486 05-24-93  11:33a   Pipx ODI driver for PC/XT's

PIPX     DOC     56537 05-10-93   5:22p   Pipx Documentation

PIPXLITE BAT      2231 04-04-93   6:51p   Sample NW Lite Batch File

PIPX_NET CFG       470 04-04-93   7:00p   Sample NET.CFG file

PARALLEL DOC     86933 03-11-93  10:09p   Parallel Port Info System

PARALLEL EXE     13717 03-21-93   1:00p   Documentation for Parallel

README   DOC     12169 05-10-93   5:37p   This file

3. License Agreement

Your right to use this program is governed by your acceptance of

the terms and conditions stated in the License Agreement which is

set forth in the files License.Doc and again in Pipx.Doc.

We offer inexpensive licensing arrangements for those vendors who

would like to include Pipx or Parallel in their distributed software.

4. Limitations of the Test Drive release

The test drive release has a number of limitations which are not

present in the standard release.

a.  The documentation has been abbreviated [ PIPX.DOC ]

b.  TURBO support is omitted

c.  A reminder screen is present (NagWare)

d.  Certain LIGHTS functions are omitted

e.  The automatic menu configuration program is omitted [ PSETUP.EXE ]

f.  The driver will only operate for a limited period of time

g.  Technical support is available only via Compuserve (76640,203)

5. Program Operation

Pipx is a TSR program which provides an ODI software interface which

supports IPX or other protocols operating over parallel port cable

connections between two PC systems.

Thus, Pipx allows you to use your favorite network operating system

using just parallel port cables, instead of network adapters.

Pipx supports all types of parallel ports and (currently) three types

of parallel port cable connections: 4BIT, 8BIT, and TURBO.

The 4BIT cable type works with all ports and is laplink-compatible

The 8BIT cable type works only with 8BIT ports and is 50% faster

The TURBO cable type works with all ports and is extremely fast.

TURBO cables can achieve transfer rates of up to 800 Kbytes/Sec

between EPP ports, which translates into NW Lite Perform3 results

ranging up to 200 Kbytes/sec.

6. Pipx Configuration

a.  Netware Lite Clients and Servers

Installing:             SHARE           ; load first - not unloadable

LSL

PIPX

IPXODI

SERVER          ; [optional]

CLIENT

NET LOGIN username

Optional Removal:       CLIENT U        ; this requires that you

SERVER U        ; have NOT installed any

IPXODI U        ; other TSR programs above

PIPX U          ; any of these drivers

LSL U

b.  Windows for Workgroups and Lan Manager

Pipx may be also used with the ODINSUP driver provided in the

Novell DOSUP7.ZIP workstation update available on Compuserv.

The ODINSUP driver provides an NDIS interface working through

the Pipx driver. This allows you to run LanMan workstation

software and WorkGroups for Windows through Pipx if desired.

e.  Using Pipx Other Protocols

Refer to the protocol documentation.  Pipx is a fully

compatible ODI driver with "version 1.2 LSL capability".

7. Program Parameters (in the NET.CFG file)

When you run PIPX, either at the DOS command line or from a

batch file, it looks for a NET.CFG file and scans it for parameters

which may be used to control various features. The parameters are

placed in the LINK DRIVER PIPX section of NET.CFG.

PORT            3BC, 378, or 278                [ required ]

INT             5 or 7                          [ required ]

CABLE           4BIT, 8BIT, or TURBO            [ required ]

NODE ADDRESS    12 hex digits                   [ required ]

may be used to override the adapter's Node Address

with a locally administered Ethernet address.

Note: Be sure you use different addresses on each

Pipx station, otherwise Pipx will not operate

LIGHTS          Transient or Persistent         [ optional ]

The Display Lites are five characters displayed

in the upper right hand corner of the display - only

when the display is in a Text mode. It is not active

in any graphics mode, and has no effect on graphics

mode programs like Windows. There is a slight (5%)

performance penalty for using the Display Lites.

In Persistent Mode, the display is not cleared and

the state of the Lites reflects the last state.

In Transient Mode, the display is auto-cleared and

the state of the Lites reflects the current state.

The meaning of the Display Lites is:

+---------------- ISR     State

|  +------------- Timeout State

|  |  +---------- Rxcv    State

|  |  |  +------- Txmit   State

|  |  |  |  +---- Driver  State

V  V  V  V  V

I                 ISR is active

R                 ISR is serving Rx Complete

T                 ISR is serving Tx Complete

A                 ISR is initiating a queued Txmit

i                 ISR is inactive

O              a Tx Timeout is being handled

F              a forced interrupt has been handled

K              a timed out Tx is being aborted

T              a queued Txmit is being initiated

R           Rx in progress

r           Rx complete

T         Tx in progress

t         Tx complete

i     Driver is starting to initialize

I     Driver has been initialized

D     Driver has been Shutdown

SOUNDS          Transmit or Receive

Causes a PC speaker click once per packet

8. Known Bugs or Misfeatures

a.  Pipx is currently incompatible with SmartDrv write caching,

at least in some configurations. We are investigating this.

b.  It is possible for Pipx v1.10 to experience high retry rates when

undergoing intense stress testing in symmetrical server-server

configurations, i.e., when the servers are performing intense

continuous read/write testing on each other. This condition may

create protocol timeout events which may appear as occasional lost

connections, file open failures, and so forth. These symptoms only

appear in cases of intense server-server load testing.

c.  It has been reported that Pipx does not co-exist well with

a Soundblaster in it's default IRQ7 configuration. Changing

the Soundblaster to another IRQ level solves the problem. It

appears that the Soundblaster is creating a high number of

spurious IRQ7's which causes Pipx to operate slowly. Thanks

to Ray Johnson for reporting this anomaly.

9. Hardware and Software Compatibility

a.  Pipx has been extensively tested on a wide range of PC systems

with a variety of different parallel port implementations. Pipx

is compatible with all parallel ports with the exception of those

which do not have normally functioning IRQ lines. This limitation

will be lifted in a future release.

b.  Pipx v1.10 has been tested with the following network software:

(1)  Netware Lite, v1.00 and v1.1

(2)  Windows for Workgroups        (using the ODINSUP shim)

(3)  PCTools Drivemap              (using SPX and NETBIOS connections)

(4)  PCTools Commute               (using NETBIOS connections)

(5)  PC Anywhere/Lan               (using NETBIOS connections)

(6)  SilverNet                     (using NetBIOS connections)

(7)  WebNet                        (using IPX connections)

(8)  FTP Software's PC/TCP         (using ODIPKT)

Our in-house compatibility testing is proceeding apace, so feel

free to call us for the most recent compatibility information.

10. Feedback

We are very interested in your comments and suggestions regarding Pipx.

You may order your copy of Pipx by calling us at 206-869-1136.

Thank you for your interest in our products.