HATMAN and SOLVE

HATMAN and SOLVE are both utility extensions for COPAN/II.

The function of these programs is to analyze stiffened

composite plates, or in the case of COPAN/II, stiffened .PRP

files.

HATMAN - The Hat Section Manufacturer

=====================================

To use HATMAN, you must have created at least one .PRP file

using LAMCON.   Having done this, it is now possible to build

a hat section using this single .PRP file.   A stiffened

plate may be made up with as few as one and as many as six

.PRP files.   This is the type of hat sections that you will

be able to build:

[D]

==========

==        ==

[C] ==          == [E]

==            ==

==              ==

===========               ===========

[B]   ===========                 ===========  [F]

-------------------------------------------------------------

[A]

-------------------------------------------------------------

A LAMCON (.PRP) file can be specified for each of the

locations as shown above referenced with A-F.    Not all six

need be used however they must be used in proper sequence.

For example, a section having .PRP files specified for A-D

would be considered valid.   A HATMAN file should not be built

with out of order files such as B, E and F.  Just use common

sense.

When entering dimensions in HATMAN, you must keep track of

whether the dimension is width or height.  For A,B,D,F, you

will be entering the width.  For the remaining two, you must

enter the height of the section.

#

Note that heights (or thicknesses) are assumed in this

fashion:

-----------------------

|        [D]          |

-----------------------

----               ----

|  |               |  |

[C] |               | [E]

|  |               |  |

----               ----

--------------               --------------

-----[B]------               -----[F]------

-------------------------------------------------------------

[A]

-------------------------------------------------------------

After you have finished specifying the PRP files and the

associated dimensions within HATMAN, a transformation stiffness

value is input.  This will normalize all plies to that

stiffness base during calculations.   If this is not desired,

then enter the value 1.0.

SOLVE - Analyzing the HATMAN files.

To obtain the sectional report on the HATMAN file, type in

SOLVE from the DOS prompt as follows:

A> SOLVE file#1 file#2 S

Where file#1 is the name of the .HAT file that you created in

HATMAN.  Do not include the .HAT as it is assumed that you will

be running only .HAT files.   The second filename (file#2) is

the output name that in which you wish the results to be

stored on disk.   Printing can take place only after the

results have been written to disk.   Use your word processor to

print or edit the result files.

The trailing S is an optional switch that can be used if you

wish to see the overall results of the analysis on the screen.

If this is not specified, you will have to read the disk file

to obtain the results.