SECPRO - Section Properties of Composite Plates

SECPRO is used to analyzed a single .PRP file for sectional

properties.   It can be called via ACCESS or loaded directly from

the DOS prompt.

Before running SecPro, it is necessary to create at least one

.PRP file using LAMCON.   Assuming you have done this, we can get

started.

Booting SecPro

From the ACCESS menu, move the pointer to ScPro or enter SECPRO

at the DOS prompt.

If the ACCESS version does not cause SecPro to load, see the

section on TroubleShooting.

Operational Details

SecPro is very easy to operate.  You should be able to generate

output without having to reference this manual.  However, just in

case you do have problems, a sample problem is presented.

#Default Section Width#

When SecPro loads, the opening screen displays the title and

copyright notice.  Just below this, the section width option is

shown.

SecPro *  Version F22   (c) 1987, Parados Technical Services

Default section width is 1.0

Change this? (Y/N):   N

If you wish to keep the default setting of 1.0, then press the

<CR>.  Otherwise, the [Y] key should be used.  This will allow

you to enter a new section width.

#

#Base Theory#

When computing sectional properties of composites, it is common

to use a transformational theory to adjust for the varying levels

of stiffness within the ply groups.   Because more than one

theory can be found in use in industry, SecPro gives you the

choice.

Currently: Max Thickness

1 = Low Stiffness

2 = Max Thickness

3 = High Stiffness

4 = User defined base

? = Explanations

X = Continue (Produces INPUT prompt)

[ ]

This menu appears the first time that SecPro is booted.  This is

kind of a reminder that you should set the base initially as the

default setting may not be appropriate for your use.

The default setting is Maximum Thickness which means that the

base stiffness is determined based on the thickest ply.  This is

the method that is used in the Gibbs and Cox design manual.

The other settings can be selected by pressing the appropriate

number key (1-4).  The user defined base is commonly used as this

permits direct comparison between dissimilar lamintes.  Usually,

a user defined base is set at about 1.0E6.

Some explanations are available if you need some additional help.

When the selection has been made, press the [X] key and the input

prompt will be displayed.

The Input Prompt

SecPro is a typical loop type of program.  You remember those

don't you?  The program runs from top to bottom and then starts

over again.   The good thing about this type of design is that it

is very easy to learn.  The bad news is that it is somewhat

resrictive and boring to watch.  Oh well.

Input Prompt --> Read.PRP File

Assign Output Name

Calculate Results ----> Write Disk Report

The SecPro Design Loop

#

The following is what you will see the first time that the Input

Prompt is displayed.  Of course the directory of .PRP files will

not be the same.

* INPUT *

Enter filename  (Do not include .PRP extension)

[?] for some assistance

BENCH.PRP     BTI.PRP       BTI001.PRP    BTI002.PRP    CDB200.PRP

FARR1.PRP     FARR2.PRP     GIBBS.PRP     KOV001.PRP    KOV002.PRP

UNI.PRP       UNI2.PRP      UNI3.PRP

>

The input prompt can be easily recognized by the shape of the

cursor bracket.  The cursor bracket is always the bright white >

symbol that rests just to the left of the cursor.

#Commands and FileNames#

While at the input prompt, both commands and filenames can be

entered.   A listing of the available commands can be displayed

by entering a [?] at the input prompt.  The following screen will

appear.

While at the Input prompt, the following options are available:

*   Enter a valid .PRP filename to obtain section property report

*   Press the [RETURN] key with nothing entered to exit SecPro

*    DIR  will produce a directory listing of all .PRP files on disk

*    HELP  shows a bunch of text that explains SecPro

*    CLS  is used to removed any screen clutter - Try it and see.

*    NEWB  can be entered if the section width is not equal to 1

*    UPDATE  reads a text file called UPDATE.  Last minute stuff.

*    REVIEW  displays a .SEC output file on the screen.

*    NEWBASE  controls what theory is being used to transform plies.

*    PLY STIFFNESS  allows you to select which ply stiffness is used.

*    STATUS?  tells you what the current defaults settings are.

*    STRENGTH  produces the .STN report giving ultimate moments &loads.

*    SHOW DETAIL  writes the .DTL file which is an expanded .SEC

Obviously, if SecPro accepts these commands, you are not allowed to

use these as laminate file names.  If you have, you must return to

DOS and REName it.

#

#.PRP FileNames#

Most of the time, the input prompt is used to enter .PRP names.

When doing this, do not include the .PRP as it is assumed by

SecPro.

Just below the Input Prompt, the directory on the default drive

has been displayed showing all .PRP files.  If no files appear

when the Input Prompt is displayed the first time, this will

indicate that you must precede the filename with a drive

designation.  ie.  A:filename  or  B:filename

The second time that the Input Prompt is shown, the directory

will not automatically be visible.  To produce it, you must enter

DIR.  Other DIR commands consist of  DIR *.*  and  DIR *.SEC

If a valid .PRP filename is entered, SecPro will attempt to read

it.  The following file summary should appear:

Attemping to open file: BTI.PRP in C:\COPAN

< File Summary >

05-19-1987 / S glass-Kevlar epoxy laminate

Groups=  8

Plies=   8

Tot.t=   .4285

Prop=   PSI

Dist=   IN

Dens=   LBFT3

Total ply groups in laminate: 8

No core plies found

Is this the correct file? (Y/N):

SecPro will report what was in the file and will then wait for

you to confirm that this was the correct laminate.  [Y] or [N]

must be pressed in order to continue.  If [N] then the Input

Prompt will return allowing you to enter another file.  The [Y]

of course will continue the process which produces the next

prompt...

#

The OutPut Prompt

Before the calculations begin, an output name must be assigned.

The reason for this is because all reports are written directly

to the disk.   All names must be given without an extension as

SecPro will add the .SEC extension to all output reports.

* OUTPUT *

? for an explanation

>

Creating and Appending

If the name that you give is not already in use, then it will be

created.  A message will indicate such.   However, in the event

that the filename already exists, SecPro will not automatically

overwrite it.   Another message/prompt will appear informing you

that the file was found.  You then have the option of  OV e R writing

or  ADD ing to it.

To do one or the other, press the appropriate keys. (OVR or ADD)

For example, you could collect the results into a single file for

all runs pertaining to a single project.  Then, when reviewing,

you would not have to edit/print multiple separate reports.

If you wish to use the same name as used at the Input Prompt, all

you have to do is press the <CR> with nothing entered.  SecPro

will confirm the name before continuing.

Calculation Results

When the calculations are complete, a brief screen listing will

be shown so that you have an idea of the sectional properties.

The file will automatically be written to the default drive

unless you specified a drive designator in the output name.

#

Immediately following the calculations, this type of screen will

be visible.

SecPro  Version F22   (c) 1987, Parados Technical Services

-------------------------

Summary of Analysis

-------------------------

Using Max thickness theory     Base stiffness=1.2E+06

Neutral Axis located 0.223 above bottom of ply 8

Laminate Inertia=0.007392    Z=0.0332

G/C Total  EI=1.02E+04

Mixtures   E =1.33E+06

<< Output document has been successfully written to disk >>

REVIEW can be used to screen list  BTI.SEC

* INPUT *

Enter filename  (Do not include .PRP extension)

[?] for some assistance

>

Again the Input Prompt is produced which means that you have

completed a program loop.  You are now back at the entry and exit

point.   To run another .PRP file, enter it and the entire

process begins once again.  To exit, press <CR> with nothing

entered.

Reviewing the Output Reports

As you may have noticed in the previous screen a line of

information indicated that to screen list a .SEC file, the

command REVIEW must be used.  To use REVIEW, enter it from the

Input Prompt.

* INPUT *

Enter filename  (Do not include .PRP extension)

[?] for some assistance

>REVIEW

#

A small review prompt will appear reminding you not to include

the .SEC:

Do not include the .SEC

File to review:

To look at the file that you have just run, enter the output name

that you assigned.

If the file is found, a speed menu will appear that will

determine how quickly the information should be scrolled on the

screen.   Medium is a good scanning speed.

--------

Fast

Medium

Slow

--------

After you have decided on the speed, any key should begin the

output.   The output can not be paused during the scroll however

it can be stopped using the [Esc] key.

When the file has finished scrolling, the review prompt will

return.  SecPro assumes that you wish to see another file.  To

cancel the review, press <CR> with nothing entered.  SecPro will

then return to the Input Prompt.

Exiting SecPro

Press <CR> at the Input Prompt with nothing entered.