SECPRO - Extensions and Enhancements

As of this printing, Version H14  of SecPro has been

released.  This release of SecPro includes some features

not mentioned on the previous pages.

#STRENGTH#

The keyword STRENGTH should be entered from the input

prompt.  This will produce a message that will enable you

to activate or deactivate the strength report.  The report

contains information pertaining to a specific PRP cross

section and it's theoretical load performance across a

predetermined span.

After entering STRENGTH from the input prompt, the

following will appear:

> STRENGTH

> Include the .STR file (Y/N)

If the [Y] key is pressed, a span must also be included:

SPAN:

The span should be entered in units compatible with those

in use within the PRP files that you wish to analyze.  Most

commonly, these will be inches.

Immediately after the span is entered, the input prompt

will reappear.

#SHOW DETAIL#

Another input prompt command available is SHOW DETAIL.

This command will cause another form of the section

property report to be written to disk.  The difference

being that some of the intermediate results will be

included within the report.   The file extension is .DTL

If you will be submitting the results of SecPro to some

outside firm or agency, it is suggested that you run the

.DTL version as well for backup.  This way, the chance of

not knowing where the results came from is reduced.

> SHOW DETAIL

> Include the .DTL file (Y/N)

Immediately after this prompt is answered, the input prompt

will be redisplayed.

#

#STATUS?#

This new keyword is used to check the settings of SecPro.

All of the current defaults will be listed.  As additional

SecPro features are added that make use of variable

defaults, this list will continue to grow.

> STATUS?

Section width =  1.0

Ply E Code    =  1

Trans Theory  =  User Defined

Base E        =  1.1E6

STRENGTH file =  FALSE

DETAIL REPORT =  FALSE

LAST FILE RUN =  DEMO

>

The current settings of SecPro will be listed as shown

above.  Immediately after the list is complete, the input

prompt is redisplayed.

#TXT Files#

SecPro uses .TXT files when producing help or explanations

from within various portions of the program.  The reason

for this is to keep the code more compact but more

importantly, to allow you to update these files.

The .TXT files are text (ASCII) formatted so any word

processor can be used to edit.   Simply load them as you

would any letter.

To determine which file is used at which portion of the

program, simply pay attention to the top line of the screen

when the file is displayed.   Then, if you find that the

information presented is unclear or incomplete, you can go

into that file and make whatever changes you feel

necessary.

#A Better REVIEW

When reviewing a file from the input prompt, you will find

SecPro more flexible than previously.  To send the file to

the screen, use the SPACE BAR to initiate the process.

During the output, the scroll may be paused by pressing the

SPACE BAR again.   To restart the scroll, press any other

key.   If you press the SPACE BAR to restart the scroll,

you will see only a single line appear.  Each subsequent

press of the SPACE BAR will produce another line.

The [Esc] key is still used to cancel.

#

#Sending to the printers#

REVIEW can also write to either LPT1 or LPT2.  Keys [1] and

[2] are used for this purpose.   If you are using only a

single printer, then use printer port #1  (LPT1).    It is

suggested that you use the Fast setting when sending to the

printer.

#PLY STIFFNESS#

This command includes two additional choices for selecting

the base stiffness.

Averages of 0/90 for tension or compression are available.

We are not sure when this is applicable so no

recommendations are given regarding when this is valid.

Also, note that within the section property report, the

BASE: is preceded by a value of 1-6.  It looks something

like this:

1/BASE: User defined @ 1.1E6

This 1 indicates that the ply stiffness used was the first

choice in the options list.    The options list is produced

by entering PLY STIFFNESS from the input prompt.

> PLY STIFFNESS

> 1=TE0  2=TE90  3=CE0  4=CE90  5=AvgTE  6=AvgCE  [ ]

This is given so that when viewing the .SEC report, it will

be possible to quickly determine which ply stiffness was

used in making the transformations.

Another way of determining this is to run the .DTL report

using the SHOW DETAIL command.   The .DTL report gives a

listing of what the actual ply stiffness values used.

#.STF Files#

When running PRP files that contain unidirectionals having

ply angles other than 0 degrees, a .STF will automatically

be written.  This file shows the results of SecPro

calculations solving for off-axis stiffness values.

The .STF file must be reviewed/printed with an editor or

LINER.

#

#Appending / Overwriting#

When analyzing PRP files, the .SEC file is created.  If

SecPro finds that the output name you assign already

exists, you will have the option of adding to it or writing

over it.   To do either, all that is needed is a three key

response of either OVR or ADD.   Which performs the

appropriate function should be obvious.

As for the remaining files written by SecPro, the above

does not apply.

If SecPro finds the file, it adds to it.  So all .STN, .DTL

and .STF files will continue to grow each time that the

same name is used.  If you wish to start a new file, you

will have to use a new name or go back to DOS and DELete

the old one.