1

VAPOR-LIQUID ACTIVITY COEFFICIENTS

ESTIMATED BY THE  UNIFAC GROUP CONTRIBUTION METHOD

Pete Goffinet -- January 1987

INTRODUCTION

These MSDOS programs provide an estimation of vapor-

liquid activity coefficients by the UNIFAC group contribution

method for a system of up to ten components, each formed from a

maximum of ten different subgroups.  Recently published group

interaction parameters and subgroup constants (1985) are supplied

and are automatically loaded and accessed by the program.

The following files are included on the distribution

disk:

UNIFAC.EXE   Compiled UNIFAC Program with Printer Output

UNIFACS.EXE  Compiled UNIFAC Program with Screen Output

UFR2.DTA     Sub Group Rk Constants

UFQ2.DTA     Sub Group Qk Constants

UFA2.DTA     Main Group Interaction Parameters

UFN2$3.DTA   Sub Group Names

UFN1$3.DTA   Main Group Names

UFC92.DTA    Main Group Numbering File

UFCC92.DTA   Sub Group Numbering File

UNIFAC.DOC   Documentation (TYPE to screen or printer)

README.1ST   Brief Instructions for UNIFAC.DOC

Documentation is not meant to be tutorial in scope, and

familiarity with the procedure for using the UNIFAC method is

assumed.  Articles by Gmehling, Rasmussen, and Fredenslund and

the text by Reid, Prausnitz, and Sherwood (1) are particularly

recommended for detailed information.  Since various tables of

constants have been published, it is important to check group and

subgroup structures with the identification table provided by

these programs to insure that consistent structure identification

numbers are assigned.

Program operation is controlled by the function keys F1

through F6.  Each input is prompted, and provisions for error

checking have been included.  Improper entries are signaled and

re-entry is prompted.  Each group of entries is displayed or

printed depending on the program selected and may be checked and

re-entered if mistakes are detected.

Because of the number of iteration loops involved,

calculation of activity coefficients may routinely require from

several seconds to more than one minute depending on the number

of components and the number of different groups composing each

component.

2

Since one of the principal uses of this program is to

estimate infinite dilution activity coefficients for binary pairs

to be used in determining constants in less complex equilibrium

correlations, entering concentrations of "0" and "1.0" for the

first component of a binary pair sets minimal concentrations for

such an estimation.

OPERATING INSTRUCTIONS

A disc containing the selected program version and all

the data files should be inserted in the default drive or made

the default directory of a hard disk drive. The program is

initiated by entering UNIFAC or UNIFACS at the operating system

prompt.  The data files will be loaded automatically, and the

introductory screen which labels the function keys F1 through F6

will be displayed.  If the program directing output to the screen

is selected, intermediate screens may be difficult to read

because the pause key is not recognized during program execution.

F1 - The group and subgroup identification list can be

displayed or printed depending on the selected program by keying

F1 whenever the function keys are active.

F2 - Input of component information is begun by keying

F2.  For initial data entry, keys F2 through F4 should be

employed in sequence.  After data have been entered, changes can

be input by keying the appropriate function key.  The requested

component names are arbitrary and limited to a maximum of twelve

characters.  They are used for identification purposes only.  For

each component, entry of the subgroup numbers making up its

structure and their frequency of occurrence is next prompted.

Entry of subgroups for each component is terminated by entering

"0,0".  Entry of components is in turn concluded by keying ENTER

without a component name.  Logically incorrect entries result in

an error message and a re-entry prompt.

After all component data are entered, a summary of the

input together with the appropriate subgroup constants and main

group interaction parameters to be used in estimating

coefficients is listed, and verification is requested.  "Y"

should be entered if corrections are needed or "N" if input is

correct.  Any entry other than "Y" is interpreted as a no.  If

this information is displayed, it may be examined using the CTRL

Num Lock keys if necessary as described above.

3

*************** NOTE WELL ***************

Since interaction parameters have not been determined

for all pairs of main structural groups, an interaction parameter

of zero for an unlike pair may mean that the value is unavailable

and that "ideal" interaction will be assumed.  In such cases,

Table I should be checked to determine whether the value is in

fact zero or unavailable and assumed ideal.  The worth of the

calculated activity coefficients should be judged accordingly.

*************** NOTE WELL ***************

F3 - Entry of concentration data for each component is

initiated by keying F3.  Concentrations are entered in mole

fraction units.  Entry of "0" for the first component of a binary

pair sets its concentration to 10E-6 mole fraction for

calculation of its "infinite dilution" activity coefficient.

Entry of "1.0" sets the concentration of the second component to

10E-6 for the same calculation.  An entry greater than unity

results in an error message and a re-entry prompt.  After data

for all but the last component are entered, its concentration is

calculated by difference, and the input composition is displayed

or printed for verification. The last component should be checked

for a negative value indicative of an input error.

F4 - Temperature in degrees Celsius is input by keying

F4.

F5 - Calculation of activity coefficients is begun by

keying F5, and the result is displayed or printed together with

temperature and composition.  Coefficients at other temperatures

and concentrations can be determined by entering new data with

keys F3 and F4.  Components can be changed by keying F3.

F6 - Keying F6 returns control to the operating system.

4

TABLE I

MAIN GROUP INTERACTION PARAMETERS

x Available

o Not Available

No.

2   x

3   x  x

4   x  x  x

5   x  x  x  x

6   x  x  x  x  x

7   x  x  x  x  x  x

8   x  x  x  x  x  x  x

9   x  x  x  x  x  x  x  x

10   x  x  x  x  x  x  x  o  x

11   x  x  x  x  x  x  x  x  x  x

12   x  x  x  x  x  x  o  o  x  x  x

13   x  x  x  x  x  x  x  o  x  x  x  o

14   x  x  x  o  x  x  x  o  o  o  o  o  o

15   x  x  x  x  x  x  x  o  o  o  x  o  x  x

16   x  x  x  x  x  x  x  o  x  o  o  o  o  x  x

17   x  x  x  x  x  x  x  x  x  o  x  o  o  x  o  o

18   x  o  x  x  x  x  x  x  x  o  o  o  o  o  o  o  o

19   x  x  x  x  x  x  x  o  x  o  x  o  x  o  o  o  x  x

20   x  x  x  x  x  x  x  o  x  o  x  x  x  o  o  o  o  x  o

21   x  x  x  x  x  x  x  o  x  x  x  o  x  o  o  o  x  o  x  x

22   x  x  x  x  x  x  x  o  x  o  x  o  x  o  o  x  o  x  x  x

23   x  x  x  x  x  x  x  o  x  x  x  x  x  o  o  x  o  x  x  x

24   x  x  x  x  x  x  x  x  x  o  x  x  x  x  x  x  x  x  x  x

25   x  x  x  x  x  x  x  o  x  o  x  o  x  x  x  x  x  o  x  o

26   x  x  x  x  x  x  x  o  x  o  x  o  x  o  o  o  o  o  x  o

27   x  o  x  x  x  o  x  o  o  o  o  o  o  o  o  o  x  o  o  o

28   x  x  x  x  x  x  x  o  x  o  x  o  x  o  o  o  o  o  x  o

29   x  o  x  o  x  x  o  o  x  o  o  x  x  x  o  o  o  o  x  o

30   x  o  x  x  x  o  x  o  x  o  x  o  o  o  o  o  o  o  o  o

31   x  o  x  x  x  o  x  x  o  o  x  o  x  o  o  o  x  o  o  o

32   x  o  x  o  x  o  o  o  x  o  x  o  x  o  o  o  o  o  o  o

33   x  o  x  x  x  x  o  o  x  o  x  o  x  o  o  o  o  o  o  x

34   x  x  o  o  o  o  o  o  x  o  o  o  o  o  o  o  o  o  x  o

35   x  x  x  x  x  x  x  o  x  o  x  o  x  o  o  o  o  o  o  o

36   x  o  o  o  o  o  x  o  o  o  x  o  o  o  o  o  o  o  x  o

37   x  x  x  o  x  x  o  o  x  o  x  o  x  o  o  o  o  o  x  x

38   x  o  x  x  x  x  o  o  o  o  o  o  x  o  o  x  o  o  o  o

39   x  x  x  x  x  x  x  o  x  o  o  o  x  o  o  o  x  o  o  o

40   x  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o

41   x  x  x  x  x  x  x  x  x  o  x  x  x  o  x  o  x  o  x  x

42   x  o  x  o  x  o  o  o  x  o  o  o  o  o  o  o  o  o  o  o

43   x  o  x  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o

44   x  o  x  x  x  o  x  o  o  o  o  o  o  o  o  o  o  o  o  o

No.  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15 16 17 18 19 20

5

No.

22   x

23   x  x

24   x  x  x

25   x  o  o  x

26   x  o  o  x  x

27   o  o  o  x  o  x

28   x  o  x  x  o  o  o

29   x  o  o  o  o  o  o  o

30   o  o  x  x  o  o  o  o  o

31   o  o  o  o  o  x  o  o  o  o

32   o  x  x  x  o  x  o  o  o  o  o

33   x  o  o  x  x  x  o  o  o  o  o  o

34   o  o  o  o  o  x  o  o  o  o  o  o  o

35   o  x  x  x  o  o  o  o  x  o  x  o  o  o

36   o  o  o  x  o  o  o  o  o  o  o  o  o  o  o

37   x  o  x  x  o  x  o  x  o  o  o  o  o  o  o  o

38   o  o  o  x  o  o  o  o  o  o  o  o  o  o  o  o  o

39   o  o  o  x  o  o  o  o  x  o  x  o  o  x  x  o  o  o

40   o  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o  x  o

41   x  x  x  x  x  x  o  x  o  o  o  x  o  o  x  x  x  o  o  o

42   o  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o

43   o  o  o  x  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o

44   o  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o  o

No. 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40

No.

42   o

43   o  x

44   o  o  o

No. 41 42 43

Program and documentation copyright 1984, 1986, 1987

Edward P. Goffinet, Jr., Wilmington, Delaware

REFERENCE

(1) Robert C. Reid, John M. Prausnitz, and Thomas K. Sherwood

"The Properties of Gases and Liquids", 3rd Edition, McGraw-Hill,

New York, 1977.

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

The Public (Software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  The P(s)L cannot de-

bug programs over the telephone.

Disks in the P(s)L are updated monthly, so if you did not get

this disk  directly from the P(s)L,  you should be aware that

the files in this set may no  longer be the current versions.

For a copy of the latest monthly software library newsletter

and a list of the 1,000+ disks in the library, call or write

The Public (Software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

(713) 721-6104 or (713) 721-5205