WORLD29

Version 2.9H

Purpose: WORLD29 version 2.9H is designed to provide rapid, graphic

representation of the area surrounding any location on Earth.

Rapid graphic location determination by several methods is

provided. Tables are provided for U.S. State capitals, National

capitals, and locations provided by the user. The location can be

entered as a latitude and longitude. The user may select a

location by typing the name, or a portion of the name, and

selecting from the possibilities presented.New locations can be

added to the database.

The Continents of the world may be displayed.

A pointer can be moved to a location on a projection of the

world and a new projection centered on that location is created.

Distances between selected points can be determined.

Storm or other element tracking throughout the world is

provided. Data can be entered within the program for future

viewing. Single storm or other track plotting is provided.

1

Table of Contents

Purpose. . . . . . . . . . . . . . . . . . . . . . . . . . . .  1

Starting the Program . . . . . . . . . . . . . . . . . . . . .  4

The Opening Screen . . . . . . . . . . . . . . . . . . .  4

The Main Menu. . . . . . . . . . . . . . . . . . . . . .  4

The Details. . . . . . . . . . . . . . . . . . . . . . . . . .  6

Option 1 . . . . . . . . . . . . . . . . . . . . . . . . . . .  6

Entry by Latitude and Longitude. . . . . . . . . . . . .  6

CITY VIEW OPTION . . . . . . . . . . . . . . . . . . . .  8

WORLD VIEW OPTION. . . . . . . . . . . . . . . . . . . . 12

OPTION 2 . . . . . . . . . . . . . . . . . . . . . . . . . . . 15

The Continents . . . . . . . . . . . . . . . . . . . . . 15

OPTION 3 . . . . . . . . . . . . . . . . . . . . . . . . . . . 16

Hemispheric views. . . . . . . . . . . . . . . . . . . . 16

Option 4 . . . . . . . . . . . . . . . . . . . . . . . . . . . 17

Find using CITY or COUNTRY name. . . . . . . . . . . . . 17

Option 5 . . . . . . . . . . . . . . . . . . . . . . . . . . . 20

Capitals of the USA. . . . . . . . . . . . . . . . . . . 20

Option 6 . . . . . . . . . . . . . . . . . . . . . . . . . . . 21

National Capitals. . . . . . . . . . . . . . . . . . . . 21

OPTION 7 . . . . . . . . . . . . . . . . . . . . . . . . . . . 22

Other CITIES of note . . . . . . . . . . . . . . . . . . 22

OPTION 8 . . . . . . . . . . . . . . . . . . . . . . . . . . . 24

Enter new info on Tracks . . . . . . . . . . . . . . . . 24

OPTION 9 . . . . . . . . . . . . . . . . . . . . . . . . . . . 26

The Global view. . . . . . . . . . . . . . . . . . . . . 26

OPTION 10. . . . . . . . . . . . . . . . . . . . . . . . . . . 27

Point at a location. . . . . . . . . . . . . . . . . . . 27

OPTION 11. . . . . . . . . . . . . . . . . . . . . . . . . . . 29

Approximate distance between Cities. . . . . . . . . . . 29

OPTION 12. . . . . . . . . . . . . . . . . . . . . . . . . . . 31

Plot any one Track . . . . . . . . . . . . . . . . . . . 31

OPTION 13. . . . . . . . . . . . . . . . . . . . . . . . . . . 33

Name the City. . . . . . . . . . . . . . . . . . . . . . 33

2

PREPARING A WORKING DISK. . . . . . . . . . . . . . . . . . . 34

HARD DISK. . . . . . . . . . . . . . . . . . . . . . . . 34

FLOPPY DISKS . . . . . . . . . . . . . . . . . . . . . . 36

CUSTOM CONFIGURATION . . . . . . . . . . . . . . . . . . . . . 37

MONITOR SELECTION. . . . . . . . . . . . . . . . . . . . 37

CHECK FOR FILES. . . . . . . . . . . . . . . . . . . . . 37

SPECIAL SUBDIRECTORY . . . . . . . . . . . . . . . . . . 38

SUPPLEMENTAL FILE NAME . . . . . . . . . . . . . . . . . 38

COLOR SELECTION. . . . . . . . . . . . . . . . . . . . . 38

FILE FORMATS . . . . . . . . . . . . . . . . . . . . . . . . . 40

CUSTOM.LOC files . . . . . . . . . . . . . . . . . . . . 40

TRAKxxxx. files. . . . . . . . . . . . . . . . . . . . . 41

SUP.DAT files. . . . . . . . . . . . . . . . . . . . . . 41

REGISTRATION . . . . . . . . . . . . . . . . . . . . . . . . . 43

3

Starting the Program

The Program is designed to run on any graphics PC compatible

system with more than 256kb of memory without modification. The

single exception is the case of a HERCULES or Hercules Compatible

system. For these systems a CGA SIMULATOR must be run before

running WORLD29. The program is started by typing WORLD29 and

hitting ENTER.

The Opening Screen

You are presented with a stylized depiction of the world. This screen is

on a timer. If you do nothing the main menu will appear. If you are in a

hurry just hit ENTER to go directly to the main screen.

The Main Menu

Once an overlay and location file have been selected, the screen will

clear and you will be presented with a Menu of 14 options. You will also see

a report of the CURRENT .DAT and .LOC files and a way to end the program. To

end an Option early type END or QUIT at a text prompt.

The top of the screen lists the program name and version

number.

Option 1 -   1 - Select by LAT and LONG

Permits entry by latitude and longitude. Once entered

you will be presented with further options to enable the creation

of a map centered on the latitude and longitude you have entered.

Option 2 -   2 - Select by large area name

Permits entry by Name. If you want a map of South America you type

S. AM and hit ENTER. A map will be created based on the name you select from

a list.

Option 3 -   3 - Select by Hemisphere

Permits entry by Hemisphere. A map will be created of the selected

hemisphere.

4

Option 4 -   4 - Find using CITY or COUNTRY name

Permits the user to create a map centered about a

location identified in the data base. Searches may be made based

on partial names. Search includes the LOCation file selected.

Option 5 -   5 - Capitals of the USA

Selection of a state capital is made by number from a

list of the capitals. A map is created centered on that capital.

Option 6 -   6 - National Capitals

Selection of a national capital is made by number from a

list of over 200 national capitals. A map is created centered on

that capital.

Option 7 -   7 - Other CITIES of note

Selection of a location from the data base maintained by

the user is made by number. A map is created centered on that

location. This data base is the Location file selected.

Option 8 -   8 - Enter new info on Tracks

Allows the addition of information to tracking data

Option 9 -   9 - The Global view

Permits entry by latitude and longitude. Once entered

you will be presented with further options to enable the creation

of a global depiction centered on the latitude and longitude the

user has entered.

Option 10 - 10 - Point at a location

A depiction of the world allows you to move a pointer

and create a map centered on the location of the pointer.

Option 11 - 11 - Approximate distance between Cities

Calculates Great Circle distance between any two

locations in the data base, including the LOCation file.

Option 12 - 12 - Plot any one Track

Permits the isolation of any single track.

Option 13 - 13 - Name the City

A simple game randomly creates a map and challenges the

user to name the central location.

Option 14 - 14 - Select new DAT or LOC file. Allows you to change OVERLAY

and or  LOCATION files.

To end the program type 99 or END and hit the ENTER key.

5

The Details

Option 1

Entry by Latitude and Longitude

On the Main menu type 1 and hit ENTER.

Purpose: This option provides a variety of maps centered on user

entered latitude and longitude. It permits the rapid display of

locations in proximity to a given latitude and longitude.

This selection presents the user with an input module.

The user is reminded that

"Just ENTER will re-enter the last position".

This means that, in general, if you want the same latitude

(or longitude) as the last time, when asked for a lat, you can

just hit ENTER.

The first question

LAT (-for SOUTH) ?

is what latitude is the user interested in. (He is also

reminded that a southern latitude may be entered as a negative

number).

The user can respond to the question in the form DD MM H or

DD.FF.

DD stands for whole degrees

MM for minutes

H for Hemisphere (N or S)

FF for fractional degrees (1 minute =.0167 degrees)

DD can be from -90 to 90 (if less than 10 degrees a 0 must

precede the degrees)

MM from 0 to 59

H can be any of s, S, n, or N.

Entries over 90 will be changed to 90 degrees.

A SPACE MUST separate degrees from minutes. For the Northern

hemisphere the N may be left off, if present, it MUST be

separated from the minutes by a SPACE. The minutes may be

eliminated if whole degree entry is desired.

6

Alternatively, the user may enter the latitude as a decimal

number of degrees (I.E., 45 30 S could be entered as -45.5)

Acceptable entries are:

34 15 N

34 15    (Northern hemisphere assumed)

34 N

03 02 n    (3 degrees 2 minutes North)

34         (34 deg. Northern assumed)

34.25      ( 34 15 N)

34 15 S

34 s

-3.5        (3 deg 30 min. South)

The user is then asked to enter a longitude.

LONG (-for EAST) ?

The response should be in the form DDD MM H or DDD.FF.

DDD may be from -179 to 179 degrees

MM from 0 to 59 minutes

H can be e, E, w or W.

FF is fractional degrees

A negative number will be assumed to be Eastern hemisphere.

Formats are the same as for latitude with the exception that

there are 3 digits in the degree field.

DEFAULTS

The following questions all have DEFAULT answers represented

by what is enclosed in the [] symbols. If the default choice is

made (hitting ENTER in response to each question) for all the

questions in this segment the resulting map will:

Be a Mercator type projection in which the selected latitude

and longitude is at the center of the screen.

It will use a scale of 3 (the medium size area).

The names of all appropriate locations in the internal and

external data base will be printed on the map.

These defaults were chosen to provide the most rapid display

of the most likely desirable information. The alternatives are

presented to cover those cases where other information is

desired.

7

WORLD or CITY VIEW

You are to choose "World view or nearby [C]ities"

You type the letter W or C followed by ENTER or just hit ENTER.

This prompt is to remind you that there are two types of

maps available. The World view is a spherical map (as if on a

globe) and the City view is more appropriate for viewing smaller

areas (Mercator type projections).

As noted by the [] surrounding the C in Cities, Cities (or

Mercator) view is the default and will be selected if you just

hit the ENTER. We will examine this choice first.

CITY VIEW OPTION

Purpose: To provide the quickest display of a relatively small

section of the earth for rapid identification of locations in

close proximity.

SCALING

The next question

Scaling Factor 1,2,[3],4,5 ?

has to do with scale of the map to be presented. A response

of 1 provides approximately 60 degrees of longitude (about the

size of the U.S.A). A response of 5 shows about 5 degrees (a

little bigger than Massachusetts.) The range is as follows:

1 - 60 degrees of longitude

2 - 30

3 - 15

4 -  8

5 -  5+ degrees of longitude

Display level selection allows you to control the amount of information

(words) printed on the screen. The following display is intended as a

reminder to the user of the variations permitted.

1=level 1 only

5=level 5 only

A= accumulate level + lower levels eg. A5

NAMES= only locations with xxxxx in them

Y or ENTER= only Capital cities

N= no locations

Do you want City names [Y]NA(1-5)?

The first 6 lines are to remind you of the options you have

available.

1=level 1 only - is to remind you that if you answer the question

with a 1 you will see only the capital cities and the locations from the

location file you selected in the opening sequence which are identified with

a 1.

8

5=level 5 only - reminds you that only capitals and locations with an

identifier of 5 will be displayed.

A= accumulate level + lower levels eg. A5 - is to remind you that if

you select A5 you will see all capital cities and all locations from the

location file selected in the opening sequence that are on the display.

eg: A1 displays all capitals and all with the 1 identifier.

A2 displays all capitals and all with the 1 or 2 id.

A2 displays all capitals and all with the 1 or 2 id.

A3 displays all capitals and all with 1, 2, or 3.

A4 displays all capitals and all with 1, 2, 3, or 4.

A5 displays all capitals and all locations on the display.

------ ONLY AVAILABLE ON EGA/VGA SYSTEMS -------

NAMES= only locations with xxxx in them - is used only after a search

for a city name (OPTION 4 on the main menu).  The xxxxx will be replaced by

what you searched for. ONLY AVAILABLE ON EGA/VGA SYSTEMS

eg. You used option 4 to search for SAN. The program answers with

SANTA FE, NM. When asked if you want City names you answer with NAMES. Select

a scale of 1, 2, or 3. You will see the mark for SANTA FE and you will see

some other marks in TEXAS. THESE are the locations from your location file

which have the letter SAN in their name. (using scale one you will also see

marks at SAN FRANCISCO, SAN DIEGO, and CABO SAN LUCAS if using the CUSTOM.LOC

file that comes with WORLD29).

------ ONLY AVAILABLE ON EGA/VGA SYSTEMS -------

SHOW TRACKS

You will be asked:

"Show Tracks y[n]"

The default response, NO, is designated by []. If chosen no

tracking data will be displayed.

If  response of Y, y, Yes, or yes is received a menu will

appear. It will consist of all files that begin with the letters

TRAK that exist in the specified subdirectory (see CUSTOM

CONFIGURATION) will appear.

The first name will be highlighted. If you wish to plot the

tracks stored in this collection hit ENTER. If you want to plot

tracks stored in another collection of tracks use the up or down

arrows to highlight the appropriate collection name.

NOTE: Only 5 collection names are displayed at one time. If more

exist a down arrow will appear at the bottom of the box. The down

arrow will scroll the display until all have been seen. You can

scroll back through the list with an up arrow until you select a

collection by highlighting it and hitting the ENTER key.

Once you have selected a collection of tracks the map will

be created with the appropriate tracks.

9

MAPPING

The program will create a map centered on the latitude and

longitude chosen. Colors will be determined based upon the

capabilities of the system and the mode selected in the

configuration process. (Configuration should only be changed by

knowledgeable personnel)

A circle in the center of the screen represents the entered

latitude and longitude. The latitude will be printed at the

center right of the screen and the longitude at the center

bottom.

IF YOU ANSWERED AFFIRMATIVELY the question

"Do you want City names [Y]NC(1-5)?"

the appropriate data bases will be searched.  At any time the ENTER

key can be depressed to end the search. SEE PAGES 8 & 9.

National and State capitals will be identified by a small circle. The

name of the city and the state name or nations name will be displayed to the

right of the circle.

All locations identified by a display level of 1 will be identified by a

small box with the name to the right.

All locations identified by a display level of 2 will be identified by a

chevron with the point up.

All locations identified by a display level of 3 will be identified by a

chevron with the point left.

All locations identified by a display level of 4 will be identified by a

chevron with the point right.

All locations identified by a display level of 5 will be identified by a

chevron with the point down.

Selecting 1 will display capitals and all locations at display level 1.

Selecting 2 will display capitals and all locations at display level 2.

Selecting 3 will display capitals and all locations at display level 3.

Selecting 4 will display capitals and all locations at display level 4.

Selecting 5 will display capitals and all locations at display level 5.

10

To display a map with no locations identified select N in response to

the prompt:

"Do you want City names [Y]NC(1-5)?"

To display only the capital cities enter a 0.

There is no way to display any given display level without displaying

the capitals.

Note: Eliminating a capital

If there is an error or change in the name or location of a capital

(Berlin or Bonn???) please notify the author. As an interim fix you can add a

location to the CUSTOM.LOC file which will overwrite the erroneous entry.

Find the location of the capital you wish to eliminate (using

WORLD29, of course). Convert the minutes to decimals (1 minute =.0167

degrees).

Using your editor or word processor (in ASCII mode - see CUSTOM

below) add an entry consisting of:

"------,-----" (The number of - before the , equals the number of

letters in the city name. Those after the , equals the number of letters in

the country name)

a comma

The latitude of the city (in decimal degrees. Negative numbers represent

the southern hemisphere)

a comma

The longitude of the city ( in decimal degrees. Negative numbers

represent the eastern hemisphere)

a comma

0 ( the number zero)

This should look like:

"-------,------",23.6,-120,0

If you use spaces the capital name will be overwritten by some other

name.

11

WORLD VIEW OPTION

Purpose: To provide the user with a spherical (or global) view of

the data. This representation, although slower than others,

displays shapes and areas as they are on the earth. It is the

best choice when a very large area is required.

If you chose W, w, or World and hit ENTER you will be asked

for specifics about the global projection you want.

DEFAULTS

The following questions all have DEFAULT answers represented

by what is enclosed in the [] symbols. If the default choice is

made (hitting ENTER in response to each question) for all the

questions in this segment the resulting map will:

Be a global (spherical) type projection in which the

selected latitude and longitude is in the center of the screen.

It will use a scale of 1 (the entire hemisphere).

No tracks will be plotted.

No state borders will be plotted.

No political borders will be plotted.

Latitude and longitude lines will be plotted at 10 degree

increments.

These defaults were chosen to provide the most rapid display

of the most likely desirable information. The alternatives are

presented to cover those cases where other information is

desired.

GLOBAL SCALING

You will be asked a scaling question.

Scaling Factor [1],2,3,4,5?

The default [1] is a scale of 1. This scale will show an

entire hemisphere.

Scale 1 = hemisphere

Scale 2 = area equal to North America

Scale 3 = area slightly smaller than USA

Scale 4 = area about the size of the mid west

Scale 5 = area about the size of Iowa, Illinois, and Indiana

12

The screen will clear and you will be asked a series of

questions to determine which of the available information you

wish displayed. Your selection will determine the absolute time

necessary to create the final display. The less data you require

the faster the process.

TRACKING

You will be asked.

Do you want tracks Y[N]?

The default is N for No, do NOT plot any tracks.

A response of Y, y, or Yes will cause a selection menu to

print to the screen. It will have a heading consisting of the

preselected subdirectory for TRAKS files (selected in the

WORLD.CNF file) and the word TRAK*. Below will be listed up to 5

TRAKxxxx files (xxxx can be any valid character), which have been

previously created. If there are more than 5 TRAKxxxx files the

list can be scrolled until all have been displayed.

To select one of the files to be displayed, press the down

arrow until the desired filename is highlighted. If you go too

far, you can use the up arrow to retrace your steps. When the

desired TRAKxxxx file is highlighted, press the ENTER key.

After the selected section of the globe is displayed, the tracks

stored in the chosen TRAKxxxx file will be displayed.

STATE BOUNDARIES

Do you want state boundaries Y[N]?

The default is N for No, do not plot state boundaries.

A response of Y, y, or Yes will result in the final display

including the appropriate state borders.

POLITICAL BOUNDARIES

Do you want political boundaries Y[N]?

The default is N for No, do not plot political boundaries.

A response of Y, y, or Yes will result in the final display

including the appropriate political borders.

Political boundaries are the generally agreed to borders of

the nations of the world as opposed to the physical edge of the

continents. These borders change from time to time and will be

reflected in updates when the new borders become generally

accepted by the majority of countries of the world.

13

GRID LINES

Do you want grid lines [Y]N?

The default is Y for Yes, plot latitude and longitude lines

at 10 degree intervals.

A response of N, n, or No will omit the latitude and

longitude lines from the final display.

The screen will clear and a spherical plot of the selected

area will be created with the selected latitude and longitude in

the center of the screen. When the display is completed, the

program will go into pause mode and display a "?" in the upper

left corner of the screen. When you are ready to return to the

main menu, hit the ENTER key.

14

OPTION 2

Select by large area name

On the Main menu type 2 and hit ENTER.

Purpose: To provide rapid access to certain preselected large

area maps by name of the area.

This selection presents the user with an input module. The user is

presented with a list of names of preselected views of various areas.

The program selects upper case automatically and the

response to the question:

WHICH AREA ?

should be in upper case and should only include the upper case

letters used in the listing. If you wish to see a map of South America you

would type S. AM and hit the ENTER.

When the ENTER key is depressed, the screen will clear and

the preselected map will be drawn.

If desired, you can add the current TRAKS file to the map by appending

the letters TRACKS to your entry. In the above example you would type S.

AMTRACKS.

15

OPTION 3

Select by Hemisphere

On the Main menu type 3 and hit ENTER.

Purpose: To provide rapid access to hemispheric views

This selection presents the user with an input module. The

user is presented with a list of preselected views. Selection is by number.

When the ENTER key is depressed the screen will clear and

the preselected map will be drawn.

16

Option 4

Find using CITY or COUNTRY name.

On the Main menu type 4 and hit ENTER.

Purpose: To provide rapid access to maps of areas surrounding

locations when the user knows the name or part of the name.

This selection presents the user with an input question:

What combination of letters are you looking for?

This is designed to be as flexible an entry as possible.

Case is ignored. You can type any letter or combination of

letters you desire. If you type END you will be returned to the

main menu.

WORLD29 will search its data base and the current .LOC file for the

first occurrence of the letter or letters that were entered.

When the program finds the combination of letter in the name

of a location in the data bases it will present you with the name

it identified and the following choices:

[W]orld

Area map

Select again

Look some more?

NOTE: The [] denote the default W for World (or global

projection). If you hit ENTER at this time a spherical map

centered on the location identified will be created based upon

further inputs.

Magnification .25-10 [1] ?

Do you want tracks Y[N]?

Do you want State boundaries Y[N]?

Do you want Political boundaries Y[N]?

Do you want grid lines [Y]N?

These questions are the same as described above under OPTION

1, WORLD VIEW OPTION.

A for Area map selects a mercator type projection. The

follow on questions and reactions will be as described above

under OPTION 1, CITY VIEW OPTION.

17

S for Select again will send you back to the

"What combination of letters are you looking for?"

prompt.

L for Look some more continues the search for the next match

in the data bases.

NOTE: This is a two edged sword! If you enter a single letter

there is a high probability that many, many location names in the

data bases will include that letter. If you enter a misspelled

name of a city or country it is likely that the program will not

be able to identify the location.

The advantage of this system is that if you are only sure of

a part of the name you can enter only that part. You will have a

high likelihood that the program will identify the location you

are searching for.

EXAMPLE of LETTER SEARCH:

Suppose we would like to create a mercator type map centered

on San Diego, California. From the main menu choose OPTION 4 by

typing 4 and hitting ENTER.

At the

"What combination of letters are you looking for?"

prompt we decide that we will only type the letter S.

The program will respond with Annapolis, since this is the

first entry in the data base with the letter s in it. Since this

is not what we want select L for Look some more.

The program will respond with Augusta, the next name in the

data base with the letter s in it.

Obviously, this will take too long, so type S for Select

again. This sends you back to the:

What combination of letters are you looking for?

prompt. This time we decide to really narrow the search and type

SAN DIEGO, CALIFORNIA and hit the ENTER key. Unfortunately, this

is too much information for the program to find a proper match.

The program responds with:

I can't identify that!!

18

When you press ENTER you are returned to the main menu. To

continue the search you can either type 4 and hit ENTER or just

hit ENTER to return to the:

What combination of letters are you looking for?

prompt.

If we now respond with SAN, the program will step you

through all locations it can find with the letters SAN in the

name. This might be reasonable or we can try DIEGO, which should

find the correct location rapidly.

ALL the letters you enter must be in the location name

stored in the data base in the order you type them including any

spaces.

This is helpful if you do not know the exact spelling of a

location but are sure of at least a few letters. Suppose that you

want to find DHAKA,BANGLADESH but don't know how to spell it. It

is very likely, from the sound of it, that the letters BANG are

in the name. Using OPTION 4 we can type BANG and after 3 or 4

come up with the correct response. If we had entered DAKA, as we

might guess, we would not come up with the correct answer.

19

Option 5

Capitals of the USA

On the Main menu type 5 and hit ENTER.

Purpose: To provide a rapid method for accessing maps of areas

surrounding capitals of the U.S.A..

The screen will clear and you will be presented with an

alphabetized listing of all 50 states. Each state is identified

with a number. You are prompted to:

SELECT BY NUMBER?

Enter any number from 1 to 50 inclusive. Typing END and

hitting enter will return you to the main menu. A number (1-50)

will result in the question:

World view or nearby Cities W[C]?

This question and later questions and will function as

described above under OPTION 1 starting at WORLD or CITY VIEW.

This option facilitates learning the state capitals in

relation to surrounding states and can be used to rapidly find

the shortest distance between capitals.

20

Option 6

National Capitals

On the Main menu type 6 and hit ENTER.

Purpose: To provide rapid access to maps of areas surrounding

capitals of the nations of the world.

The screen will clear and you will be presented with an

alphabetized listing of 60 of the worlds capitals and the prompt:

SELECT BY NUMBER (M FOR MORE)?

You can select any of the listed names as the center of a

map by typing the number associated with the capital and hitting

ENTER. If you type END and hit ENTER, you will be returned to the

main menu.

If you type M and hit ENTER, you will be presented with

capitals 61 to 120 and the same prompt.

The procedure will be repeated until all capital names have

been displayed. If you type M on the last page, you will be

returned to the first page.

At any prompt you can type any VALID number to create a map.

If you type a number greater than the number of the last national

capital in the data base, you will be returned to the first page.

At any prompt you can type END to return to the main menu. M

and ENTER brings up the next page of capitals.

Valid numbers will result in the prompt:

World view or nearby Cities W[C]?

and later questions and responses as above in OPTION 1,

WORLD or CITY VIEWS.

21

OPTION 7

Other CITIES of note

On the Main menu type 7 and hit ENTER.

Purpose: To provide rapid access to maps of areas surrounding

locations chosen by the user.

This is YOUR section. The files CUSTOM.LOC and USSR.LOC are provided as

sample files and are to be maintained by the user. (See CREATOR.EXE and FILE

FORMATS below)

CUSTOM.LOC is the inclusive list of Names, latitudes,

longitudes, and display levels each on its own line and stored as

an ASCII file. This file is readable by most word processors such

as EDLIN, PC-WRITE, WORD PERFECT (use CTRL-F5, DOS text, to save

and load), and WORDSTAR (use N for NON DOCUMENT mode). The file,

CREATOR.EXE, provided with WORLD29 can also be used to add

locations to CUSTOM.LOC (discussed below).

A sample CUSTOM.LOC file is included with the program.

Option 7 displays the names of all files in the current .LOC file and

associates each one with a number as in option 6. The names are listed with

no more than 60 on a display page. The prompt:

SELECT BY NUMBER (M FOR MORE)?

is used as discussed in option 6.

The limits on the number of items a the .LOC file is technically over

30,000 but practically much lower. The program functions properly and option

7 has been tested with over 1500 items in CUSTOM.LOC.

A greater limiting factor may be associated with the speed to search the

larger .LOC files. This speed will be dependant not only on the length of the

file, but also, on the type of media (hard disk is faster than floppy) and

the speed of the machine.

22

If you find the time to search the CUSTOM.LOC data base is

excessive, you can physically split the file into two or more

separate files.

WORLD29 now supports LOCation files of over 1000 names. Paging is now

handled with the letter M (for More). This provides access to LOCation 999 as

well as all others. The display has been adjusted for proper display of up to

9,900 locations.

You can page back and forth in the file using the "+" sign before a

number. For example if you are on the page displaying LOCation 121 through

180. You know the LOCation you are interested must be a long way from where

you are. Instead of repeatedly typing M ENTER, M ENTER, etc. type +1000. If

possible WORLD29 will jump to the page containing LOCation number 1000. If

you jump too far you can jump back with a +900. You can switch between the +

and M as desired.

Option 14 now makes it practical to utilize more than one LOCation file

in a given session. You may type any filename at the "Location file to use"

prompt. The filename may include a drive and a completet path.  For

simplicity I find keeping all of my LOCation files in the same directory as

WORLD29. I also find that having all of my LOCation files retain the .LOC

extention helps. WORLD29, however, doesn't care about the name, only the

contents. SEE PAGE 40

This flexability also applies to the OVERLAY files. WORLD29 will accept

any file that is in the proper format. SEE PAGE 41

Both of the overlay and location questions provide DEFAULT ANSWERS

(the answer if you just hit the ENTER key). The DEFAULT ANSWER will be

printed above the question inside square brackets []. As in prior version you

may answer NONE to the "OVERLAY to use?" question to eliminate all overlays

from the program. The "Location file to use?" question, however, will NOT

accept NONE - you MUST have a LOCATION file. If you don't have one use the

CREATOR program to creat CUSTOM.LOC. Each question will accept a complete

filename, including a path to allow you to use files in directories other

than the one WORLD29 is in.

When you enter a filename WORLD29 will check to be sure it exists and

if not give you a chance to re-enter the filename. WORLD29 does NOT, however,

check for the proper format at this point. You must insure that the file your

tell WORLD29 to run conforms to the standards specified in  WORLD.DOC. If the

program later finds an incorrect file format the results are unpredictable.

NOT DANGEROUS just NOT what you were expecting.

All of the features of Option 7 will function in the same

manor as in Option 6.

23

OPTION 8

Enter new info on Tracks

On the Main menu type 8 and hit ENTER.

Purpose: To allow the user to create or make additions to the

file TRAKS. The file TRAKS is used as the current or default

collection of tracks in option 2 and 3 and must exist before

those options are used.

The file TRAKS and any files created by the program and

renamed to filenames beginning with TRAK are all ASCII files.

ASCII can be modified by word processors as discussed in option 7

under the discussion of CUSTOM.LOC. For further information see

FILE FORMATS below.

The prompt:

SHALL I CONTINUE [Y]N?

give you the option to return to the main menu without changing

or creating the TRAKS file. If you type N, or n, or No you will

be returned to the main menu.

The DEFAULT response is Y for YES (noted by []) will ask

questions to determine what should be added to the TRAKS file.

This response will create a file named TRAKS if it doesn't exits.

TRACK IDENTIFIER STRING ?

This is the request for a name or something to tie

sequential entries together. (Originally used for the Name of a

hurricane). If later sequential entries have the same IDENTIFIER

STRING, they will be linked together as a track.

DATE <MMDD> ?

The date of the entry. Originally the month and day of the

fix in two digit month and two digit day form. This can be any

numeric entry. This number is not displayed but can be used for

sorting or historical purposes.

TRACK STRENGTH OR CONFIDENCE 1-4 1>4 ?

A numeric representation of the strength or confidence in

the fix. Originally used to represent storm strength as:

1 = Hurricane

2 = Tropical Storm

3 = Tropical Depression

4 = Tropical Wave

24

This number is translated to the size of the circle plotted

around the fix with a 1 being represented by the largest circle.

LAT?

Requests the latitude of the fix. This MUST be in the form

of decimal degrees. (44 degrees 30 minutes must be entered as

44.5) Southern latitudes are represented by a negative sign (-

44.25 would represent 44 degrees 15 minutes SOUTH latitude)

LONG?

Requests the longitude of the fix. This MUST be in the form

of decimal degrees. (80 degrees 45 minutes must be entered as

80.75). Eastern longitudes are represented by a negative sign

(-123.5 would translate to 123 degrees 30 minutes EAST)

DO YOU HAVE MORE DATA Y[N]?

The DEFAULT entry is N for No more data. A quick mercator

type map will be created centered on the last entered fix. It

will show any other tracks in the TRAKS file that occur on the

map.

A response of Y, y or Yes will take you back to the prompt:

SHALL I CONTINUE [Y]N?

If you have additional entries you can go on. If you type N, or

n, or No you will be returned to the main menu.

NOTE: This option can be used to track anything that can be given

a latitude and longitude. The key to making continuous track

plots is to keep all fixes with the same name in sequence. The

TRAKS file is an ASCII file and can be loaded into, manipulated

by, and saved from word processors as discussed in option 7. The

file can also be sorted with the SORT program from the

supplemental programs disk that comes with DOS to keep names

together.

When some logical collection of tracks is completed, the

file named TRAKS should be renamed to something beginning with

TRAK. For example each year I rename TRAKS to TRAKSxx where xx is

the last two digits of the year. These file (TRAKS89, TRAKS88,

etc.) represent collections of hurricane tracks for the

appropriate year and can be retrieved and plotted with several

options.

25

OPTION 9

The Global view

On the Main menu type 9 and hit ENTER.

Purpose: This option is provided to permit direct entry into a

global or spherical representation. It provides the same features

as OPTION 1, WORLD VIEW OPTION but allows the user to select a magnification

factor. It is retained only for the convenience of certain customers.

Once a global plot has started, it can be interrupted at any

point by hitting the ENTER key. This will return you to the pause

prompt. ("?" in the upper left of the screen) Hitting the ENTER

again returns you to the  main menu.

26

OPTION 10

Point at a location

On the Main menu type 10 and hit ENTER.

Purpose: To provide access to expanded views of areas of the

world selected by a pointer. Secondarily, it provides a crude

distance calculator.

The screen is cleared and a mercator type representation of

the world is drawn. A pointer arrow is created in the northern

Atlantic Ocean.

The pointer can be moved as follows:

Up             using the UP arrow or the 8 key

Down           using DOWN arrow or the 2 key

Left           using the LEFT or the 4 key

Right          using the RIGHT or 6 key

Up and Right     using the 9 key

Up and Left      using the 7 key

Dn and Right      using the 3 key

Dn and Left    using the 1 key

The program has manipulated the NumLock key to allow this

flexibility. It will restore the NumLock key and the CapsLock key

to the condition that existed when the program started upon exit

by the 99 menu entry.

You may move the pointer arrow to any point on the map. Do

NOT try to wrap the pointer around one end of the display or the

other. As you move the pointer, the latitude of the point will be

displayed at the center right of the screen. The longitude will

be shown at the bottom center of the screen. The arrow will only

move in 1 degree increments. Hitting the E for Escape or End at

any point will return you to the main menu.

When the arrow is pointing at a place you are interested in,

hit the ENTER key. The tip of the arrow will become the center of

a new, expanded map. This procedure can be continued through a

total of five magnifications.

On magnification 5 the CUSTOM.LOC file will be searched and

any locations on the map will be plotted.

On any EXCEPT magnification 5 you can do some rough (to the

nearest whole degree) distance measurement. Move the arrow point

to the first location you are interested in and hit the F1 key.

At the top of the screen it will print:

27

First fix  xx   xxx

xx will be the latitude of the point of the arrow. xxx will be

the longitude of the point of the arrow. You can reposition the

arrow and hit F1 again if this was not the point you expected.

Remember, the latitude of the pointer is displayed at the right

center and the longitude at the bottom center.

Now move the arrow until the point is at the second point

you are interested in and hit F2. The program will print:

Second fix  xx   xxx

xxxx miles

just below the First fix. Again the xx will be the latitude of

the pointer and xxx the longitude. xxxx will be replaced with the

number of statute miles between the first fix and the second fix.

You can now move the pointer and hit F2 again for another

distance measurement. F1 has remained the same. This can be

repeated any number of times.

If you want a new first point, place the cursor and hit F1

again. You must select a new second point once you have used F1

or the distance will not be relevant.

28

OPTION 11

Approximate distance between Cities

On the Main menu type 11 and hit ENTER.

Purpose: To provide rapid calculation of the shortest distance

between any two known points on earth.

The screen clears and you are asked for a first name. The

input is similar to that in option 4. Case is not considered and

the entire data base is searched for the letters you enter. The

prompt is:

Ok or Look more ([O]L,Q)?

O for OK is the default. (indicated by [])

L for Look more continues the data base search

Q for Quit sends you to the pause prompt and then the main

menu.

Once you have accepted a location you are asked for a second

location. The prompts and actions are the same as for the first

name.

Upon acceptance of a second location the screen clears and a

box opens with prompt as below:

Approximate distance between FIRST NAME

and SECOND NAME is  XXXX  miles  YYY Deg

Would you like a map?

FIRST NAME is replaced by the name selected above.

SECOND NAME is replaced by the name selected above.

XXXX is replaced by the approximate number of statute miles

between the two locations on a GREAT CIRCLE running through the

two.

YYY is an approximate bearing on the GREAT CIRCLE route from

the first location to the second location.

The distance calculator can also be used to calculate the

distance between two pair of latitude and longitude coordinates

or a named location and a latitude and longitude.  When a number

is entered instead of a location name it is assumed to be a

latitude and you will be prompted for a longitude.

Either or both the first and second entry may be either a

name or a location.

29

If you respond to the

"Would you like a map?"

prompt with a N or NO or ENTER you will return to the main menu.

If you respond with a Y or y a representative map will be

created.

NOTE: The map request will provide a rectangular projection

displaying both the first point and the second point. The only

purpose for this map is to show the relative locations of the two

points. The map is often distorted. It is always created from

West to East and the line connecting the two points does NOT

represent the GREAT CIRCLE route between the points. The

connecting line is only to identify the points at each end of the

line as the first point and the second point. The first point and

the second point can be at either the left or right of the line

and no attempt is made to identify which is where.

When tracking an item you can rapidly find the distance

between any point and the most recent location.  Enter the

identification of the point in response to the First Name prompt.

Enter the word CANE or the word TRACK at the Second Name prompt.

You will be presented with the list of track collections. Select

the appropriate file.  WORLD29 will then tell you the distance

from the point to the last entry in the file.

30

OPTION 12

Plot any one Track

On the Main menu type 12 and hit ENTER.

Purpose: To allow the user to isolate a single track on a map

containing the initial position of the track.

The screen will clear and a small window will appear listing

up to 5 TRAKxxxx files.

If more files exist you may scroll through the list using

the UP or DOWN arrows. On most systems a highlight will move

through the list as you scroll (certain monochrome systems do not

show the highlight and will require the user to keep track). When

the highlight is on the TRAKxxxx collection name you are

interested in, hit the ENTER.

The screen will clear and all unique track names in the

collection will be displayed. Up to 100 names can be displayed at

once.

If there are more than 100, they will scroll out of sight.

The tracks that scroll are still accessible but breaking the

TRAKxxxx file into two separate names is recommended.

The prompt:

Type the name of the track you want to plot?

NOTE: This search, unlike all before, is for an exact match.

It is case insensitive, however spelling DOES matter. This

is to reduce the chance of plotting the incorrect track.

If the program cannot make a match it responds with:

I didn't see that one

Any response will return you to the main menu where you can

start over with the default re-entry into option 12.

Once a match has been made you will be prompted:

World view or nearby Cities W[C]?

Default, questions, and responses are as in option 1 above.

Scaling Factor 1,2,[3],4,5?

Default, answers and responses are as in option 1 above.

31

Do you want nearest City [Y]N?

NOTE: The default is Y for yes. You must type N or n or No to

avoid this calculation.

NOTE: The Nearest City feature calculates the distance from the

initial point of the track to the nearest location.

The distance calculation takes place between the plotting of

the initial point of the track and the plotting of the second and

subsequent points, if any, in the track. In this case the second

and subsequent points will not be plotted until you have exited

the nearest location routine by responding with an ENTER.

32

OPTION 13

Name the City

On the Main menu type 13 and hit ENTER.

Purpose: To provide a simple geographic knowledge challenge.

Maps are created of the area surrounding randomly chosen

world and U.S. capitals. The user is challenged to name the city

and country or state. The map is always scale 3, includes the

latitude and longitude of the capital, and any other capitals on

the map.

The user must respond with the exact spelling of both the

city and the country as entered in the data base. (After all, you

don't like it when someone misspells your name!) Case does not

matter. U.S. states will be identified by their two letter postal

abbreviation.

Responding with END or QUIT at any prompt will take you back

to the main menu.

A running session score of the number of correct responses

versus the number attempted is maintained. This permits the user

to exit the game to study the maps some more. When you reenter

the challenge your score is based upon what you did earlier in

the session as well as this time.

OPTION 14

Requests new DAT and LOC file names. ENTER, by itself makes no change.

You must include the path if the file IS NOT in the current directory.

PRINTING A MAP

The creators of the operating system chose not to support

the printing of GRAPHIC images within DOS. This requires an

external program to be run before running the WORLD29 program.

For CGA screens the supplemental disk that comes with DOS

contains a program named GRAPHICS. This program is discussed in

chapter 7 of the DOS manual. There are restrictions on the

printers supported and options that can change the appearance of

the output. The GRAPHICS program that comes with DOS version 4.01 or later

supports CGA, EGA, and VGA screen prints and many printers.

33

PREPARING A WORKING DISK

Neither WORLD29 nor any of its associated files are copy

protected and a backup of the original disk should be created

before the program is used.

Creating a Backup disk

Start the computer with the Disk Operating System (DOS).

If you are presented with a menu of options, see your

supervisor about making backups.

HARD DISK

If you have a hard disk and are presented with a C:> prompt

or some variation of this:

Place the original copy of WORLD29 in the A: drive.

This is generally the Top or Left most Floppy diskette drive.

Close the door on the A: drive. Find a diskette to make the copy

on. This diskette will effectively be erased and any data or

programs on it will be unusable once a copy of WORLD29 is placed

on it. Place a label on this disk telling you that it is the

BACKUP copy of WORLD29.

IF YOU HAVE ONE FLOPPY DRIVE  type DISKCOPY A: B: and

hit enter.

Don't worry, after the computer reads some of the data

from the original disk it will tell you to place the second (or

TARGET) disk in the drive. The computer may ask you to place the

disk with COMMAND.COM on it back in the drive. That means the DOS

disk. If it asks, put the DOS disk in the drive. If it asks for

the SOURCE disk, it wants the Original WORLD29. If it asks for

the target, it wants the BACKUP disk. When it does so, remove the

original and place the disk that will be your backup in the drive

and close the door. When you are ready, press the ENTER key and

the computer will write what it read to the new disk.

When it has finished, it MAY ask you to put the

original disk (SOURCE) back in the drive. This will depend on how

much memory the computer can find to work with. If it asks for

the original, take the backup disk out and put the original back

in the drive. Close the door and press ENTER when you are ready.

Continue this process until the computer asks you if you want to

copy another disk. Answer this question by pressing the N key and

hitting ENTER. Remove the disks.

Place the original copy of WORLD29 in a safe place. If

you ever have a problem with the BACKUP copy, you can retrieve

the original and follow the same procedure to make another

BACKUP.

34

IF YOU HAVE TWO FLOPPY DRIVES place the BACKUP disk in

the second drive, close the door, type DISKCOPY A: B: and hit

enter. The computer will read the information from the A: drive

and write it to the B: drive.

When the computer asks if you want to copy another

disk, answer by pressing N and hitting ENTER. Remove the disks.

Place the original copy of WORLD29 in a safe place. If

you ever have a problem with the BACKUP copy you can retrieve the

original and follow the same procedure to make another BACKUP.

COPY THE FILES TO THE HARD DISK. Place the BACKUP copy

of WORLD29 in the A: drive. Now create a special subdirectory to

keep all the files belonging to WORLD29 together.

Type MD\WORLD29 and hit ENTER. This means Make a directory

named WORLD29. (Notice the \ slants from upper left to lower

right and you can give the subdirectory any name you want after

the \)

Now to get to the subdirectory type CD\WORLD29 and hit

ENTER. This means Change location to the subdirectory named

WORLD29.  (if you used a different name be sure to type that name

instead of WORLD29 after typing the \)

Now type copy A:*.* and hit ENTER. This means to copy each

and every file from the disk in the A: drive to the subdirectory

you are now in.

35

FLOPPY DISK SYSTEM

Type DIR DISKCOPY and hit Enter. If the computer does not

print things that include the words "DISKCOPY.COM" and the words

"1 File(s)", you will have to find the disk with the program

DISKCOPY on it. This is generally the second disk that came with

the DOS.

IF YOU HAVE ONE FLOPPY DISK place the disk with

DISKCOPY.COM on it in the drive and type DISKCOPY A: B: and hit

enter.

Don't worry, after the computer reads some of the data

from the original disk it will tell you to place the second (or

TARGET) disk in the drive. The computer may ask you to place the

disk with COMMAND.COM on it back in the drive. That means the DOS

disk. If it asks, put the DOS disk in the drive. If it asks for

the SOURCE disk, it wants the Original WORLD29. If it asks for

the target, it wants the BACKUP disk. When it does so, remove the

original and place the disk that will be your original in the

drive and close the door. When you are ready press the ENTER key

and the computer will write what it read to the new disk.

When it has finished, it MAY ask you to put the

original disk (SOURCE) back in the drive. This will depend on how

much memory the computer can find to work with. If it asks for

the original, take the backup disk out and put the original back

in the drive. Close the door and press ENTER when you are ready.

Continue this process until the computer asks you if you want to

copy another disk. Answer this question by pressing the N key and

hitting ENTER. Remove the disks.

Place the original copy of WORLD29 in a safe place. If

you ever have a problem with the BACKUP copy, you can retrieve

the original and follow the same procedure to make another

BACKUP.

IF YOU HAVE TWO FLOPPY DISKS place the disk with

DISKCOPY in drive a:, the BACKUP disk in the second drive, close

the door, type DISKCOPY A: B: and hit enter. The computer will

read the program from the A: drive, then it will ask you to put

the SOURCE disk in drive a: and the TARGET disk in drive B:.

Source is the original WORLD29 and TARGET is the BACKUP disk. The

computer will then read the information from the A: drive and

write it to the B: drive.

When the computer asks if you want to copy another disk

answer by pressing N and hitting ENTER. Remove the disks.

Place the original copy of WORLD29 in a safe place. If

you ever have a problem with the BACKUP copy, you can retrieve

the original and follow the same procedure to make another

BACKUP.

Use the Backup for your daily use.

36

CUSTOM CONFIGURATION

Although WORLD29 is designed to be run upon receipt, you may

want to change certain things to take advantage of your taste or

system. These SPECIAL changes will be stored in a file called

WORLD.CNF. WORLD.CNF is presently set up for operation on a

computer with a CGA graphics board. It will have WORLD29 check to

insure certain files are where they should be. It expects to find

all TRAKSXX file in the same subdirectory as WORLD29. The colors

are set the way I normally use them on a CGA.

To change any of these settings the CREATOR.EXE program has

been provided. This is the WORLD FILE CREATION ASSISTANT

(CREATOR.EXE). It will present you with a menu of options. To

change the WORLD.CNF file press W and hit ENTER (Note the capital

W).

MONITOR SELECTION

The program will ask "Do you have CGA, EGA, or VGA

([C],E,V)? The Default answer (the answer you will get if you

just hit ENTER is enclosed in the square brackets []) is C for

CGA. This is used internally by CREATOR only (2.9G on)

If you have a CGA or Hercules compatible system you

MUST select C and hit ENTER (or just hit ENTER). The program will

not function properly if you enter E ENTER or V ENTER.

If you have an EGA or VGA you can type C or E or V. If

limit the colors to those avaiyou select C the program will limit

to colors available on a CGA.

If you type E and ENTER you will maximize the

capabilities of WORLD29. You will be allowed to select the colors

of various line segments and display all maps optimally.

If you type V and ENTER you will be running the program

at the limits of its capabilities. This is, in fact, the EGA

mode. The V is provided for convenience. I have experimented with

VGA modes and have found that the line strength does not warrant

its use. (IE. the data does not support higher resolution and the

line strength is too weak)

CHECK FOR FILES

The program will ask "Do you want to check for the presence

of correct files ([Y]N)? The Default answer is Y for Yes. I

recommend only selecting N if you know that a required file is

missing and that the fact that it is missing does not concern

you. When you leave this answer Y, WORLD29 will check to see if

certain files are where they are supposed to be. If it finds

37

something wrong, the program will give you a report when you

start the program.

Depending on which file is missing you will be told what to

do if anything. If, for example, this file (WORLD.DOC) is missing

WORLD29 will simply print a list of files that should be present.

If the program cannot find the TRAKS file, it will report that

fact and remind you to use option 8 to create a new TRAKS file.

SPECIAL SUBDIRECTORY

Since the data in the TRAKS files is in ASCII (no special

characters), the file can be utilized by other programs. To

facilitate the use by other programs I have provided the ability

to place these files (TRAKSXX - where XX can be any legal

character) in a different subdirectory. The usual location is in

the same subdirectory as WORLD29. This is the default (Current)

subdirectory and is chosen by simply hitting the ENTER key. If

you wish to keep the TRAKSXX files in another subdirectory, type

the name of the subdirectory and hit enter.

NOTE: Do NOT include the first \ when typing the name)

SUPPLEMENTAL FILE NAME

WORLD29 can read in supplemental files that act as overlays

for the appropriate maps. Two sample supplemental files are

included with the program (SUP.DAT - the boundaries of the

provinces of Canada and USSR.DAT - the Republics of the Soviet

Union -courtesy of Brian Cook). You can create your own overlays

using these files as examples. They are ASCII files and can be

read by word processors or editors.

The default supplemental file is SUP.DAT. If you hit

ENTER, this file will be loaded each time WORLD29 is run. If you

want another supplemental file, simply type its name and hit

enter. If you want no supplemental file, type NONE in capitals

and hit enter.

COLOR SELECTION

You will be prompted for a number that translates to a color

that depends on your system. Unfortunately, what I call and see

as yellow on my system may not be what you call and see as yellow

on your system. I have provided defaults for each color that I

find pleasing or useful. If you do not like my choices feel free

to experiment!

The first selection is for the Primary text color - default is

7 (generally white)

38

The next is for Primary background color - default is 0 (black)

Color for box 1 - default 4 - recommend 0 for mono systems.

Frame for box 1 - default 3 - recommend 7 for mono systems.

Color for box 2 - default 4 - recommend 0 for mono systems.

Frame for box 2 - default 11 - recommend 7 for mono systems.

Header color box 2 -default 11 - recommend 0 for mono systems.

Header frame box 2 -default 4 -           7 for mono

IF YOU HAVE SELECTED C FOR A CGA SYSTEM above you will be

asked "Screen 1 color pallet" the default is

0 Green, Red, Yellow

1 Cyan, Magenta, White

The program will now print on the screen what you have

entered for the configuration. This will also include a line of

zeros. These are place holders and should be of no concern to

you. It will then ask " is this ok ?"

If it is the way you want it, type Y and hit ENTER. Any

other response will NOT write the new WORLD.CNF file to the disk.

If you respond with Y and ENTER, the program will confirm

that it has written the new WORLD.CNF file to the disk.

IF YOU HAVE SELECTED EITHER E FOR EGA OR V FOR VGA, you will

be asked for more color selections. Each selection will include a

default that can be chosen by simply hitting the ENTER key. You

may enter any number between 0 and 63 for any of these

selections. Each number represents a color that can be reproduced

on the EGA or VGA system. You can experiment with the choices

until you achieve the desired effect. Remember, if you have

chosen a 0 (black) background, drawing lines in color 0 (black)

will create invisible lines or erase portions of lines that are

overdrawn.

The program will now print on the screen what you have

entered for the configuration. It will then ask " is this ok ?"

If it is the way you want it, type Y and hit ENTER. Any

other response will NOT write the new WORLD.CNF file to the disk.

If you respond with Y and ENTER, the program will confirm

that it has written the new WORLD.CNF file to the disk.

39

FILE FORMATS

As discussed above, all files created by WORLD29 are ASCII

files. They can be maintained, changed, or created by a vast

array of editors and word processors.

When using                use mode

EDLIN                     standard commands

PC-WRITE                  standard commands (no dot commands)

WORD PERFECT              load with CTRL-F5, Dos Text

WORDSTAR                  Non Document mode

For any other editor check the manual for DOS or ASCII file

manipulation.

NOTE: in the following discussions FF represents fractional

degrees (1 minute =.0167 degrees). Do not confuse this with

minutes. (sng) - represents single precision numbers and (int)

represents integers.

CUSTOM.LOC files.

Each line represents one location or record. Each element of the

record must be separated by a comma ",".

The format for each line is:

Quoted string - Used as the location name.

Number(sng)   - Used as latitude.(South Hemisphere -)

Number(sng)   - Used as longitude.(West Hemisphere -)

Number(int)   - Used for display level (1-5)

NOTE: The Quoted string MUST contain a ","(comma) or a " "

(space). Without either a comma or space the displays will not be

properly identified.

Limitations: Technically the Quoted string may be up to 250

characters. Practically it should be limited to 32 characters for

display in the nearest location field. When displayed in the

Option 7 table it will be truncated to 19 characters.

The latitude must be less than 90 degrees and greater

than -90 degrees. They must be in the form DD.FF. Leading and

trailing 0s are not required. Negative indicates SOUTHERN

hemisphere.

The longitude must be less than 180 and more than -180

degrees. It must be in the form DDD.FF. Leading and trailing 0s

are not required. Negative indicates EASTERN hemisphere.

The display level must be an integer from 1 to 5

inclusive. 0 is reserved for internal locations and numbers

greater than 5 will not be displayed.

40

TRAKxxxx. files

Each line represents one fix, position, or record.  Each element

of the record must be separated by a comma ",".

The format for each line is:

Quoted string - used as the track identifier

Number(int)   - used as a date identifier

Number(sng)   - Used as latitude.(South Hemisphere -)

Number(sng)   - Used as longitude.(West Hemisphere -)

Number(int)   - Used for display level (1-4)

Limitations: Technically the quoted string is limited to 250

characters. Practically It should be small to avoid screen

clutter when displayed (generally less than 20 characters)

The latitude must be less than 90 degrees and greater

than -90 degrees. They must be in the form DD.FF. Leading and

trailing 0s are not required. Negative indicates SOUTHERN

hemisphere.

The longitude must be less than 180 and more than -180

degrees. It must be in the form DDD.FF. Leading and trailing 0s

are not required. Negative indicates EASTERN hemisphere.

The strength or confidence level must be an integer

from 1 to 4 inclusive. These numbers are translated into circle

diameters with 1 being the largest and 4 the smallest.

SUP.DAT files

The name SUP.DAT is representative of a class of files I call

supplemental or OVERLAY files. These file allow you to add

whatever graphic information you wish to the maps created by

WORLD29.

The program comes with two sample OVERLAY files:

SUP.DAT, which includes the province boundaries for CANADA

USSR.DAT, created by Larry Cook, the republics of the USSR

The format of this file is somewhat more flexible than the

others.

Essentially, WORLD29 reads pairs of numbers from the OVERLAY

files and deals with them on the following basis.

If the NUMBER PAIR is 99,99 it is treated as a PEN UP

command. Any line being drawn is ended.

If the PEN IS UP and THE NUMBER PAIR IS (int),99 and int is

less than 15 on an EGA or VGA system the color associated with

that attribute in the WORLD.CNF file is used to color subsequent

line segments. For CGA systems the int should be 3 or less.

Larger numbers will be indistinguishable.

41

If the PEN IS UP and THE NUMBER PAIR is 99,99 WORLD29 ceases

reading the file (This is its EOF marker - 99,99,99,99)

If the NUMBER PAIR consists of (sng),(sng) and the first

number is a valid latitude (90 to -90) and the second a valid

longitude (180 to -180) WORLD29 will attempt to plot the point in

the current color.

If the PEN is UP it will be lowered and a line will be drawn

from this point to the next valid point unless the PEN UP command

(99,99) is received.

The first (sng) will be considered a latitude, the second a

longitude.

NOTE: In OVERLAY files the WESTERN hemisphere is indicated by a

negative (-sng). This is inconsistent with the entries made

within the program and in the TRAK files. This is due to the

internal manipulation of data.

The number of elements on any line is not critical. Each

element on any line must be separated from the next by a comma

",". It IS critical that the numbers be paired.

I find it most convenient to type sequential lat and long

points to a convenient line length, continue on the next line, if

necessary, and end a line with the PEN UP (99,99).

I isolate color changes to a single pair on a line

((int),99). This, however, is not necessary.

42

REGISTRATION

Registration is buying the program! It's as simple as that.

When you pay a distribution service a few dollars you have bought

a disk and paid for the service to search through many many

programs to find programs that are of value to their customers.

The person or company that created the program receive nothing

from the distributor.

If you receive the program from a friend or BBS, the

creator of the program again receives nothing.

Shareware is like test driving an automobile. You have a

100% operational, real program to test drive! You can take the

car out of the showroom and, in some cases, even take it home for

a while. With shareware you can even take it to work and see if

it does what you want it to do. But, until you register the

program it is not yours!

Once you find that you are USING the program for something, not

just trying it out to see what it will do, it is your

responsibility to pay for the program. That alone is enough

reason to register programs but in most cases there are other

benefits that accrue to those who do register.

The Cost of registering a single copy of this program is

$10 in U.S. funds.

When you register this program you will:

Be entitled to the latest update of the program on

your disk for free and all future updates at $5 each.

Updates will be the result of user suggestions and/or

corrections. (an additional $5 handling fee if I am to

supply the disk and mailer)

You will receive an acknowledgement from the author

and a single page summary of other programs he has

written.

ALL FEES MUST BE IN U.S. FUNDS. INTERNATIONAL MONEY ORDERS IN

U.S. FUNDS ARE GLADLY ACCEPTED.

43

Multiple copies are offered as follows:

Single copy price is $15.........Disk/Envelope/on disk Doc

2 to 10 .............$10/disk....       "               "

11 to 50 ............$8/disk.....       "               "

51 to 100 ...........$6/disk.....       "               "

Additional units can be negotiated.

Companies or other entities who use more than one copy of the program MUST

register the appropriate number of copies.

Minor modification to suit the needs of specific users can

generally be made for $200 or under.

44

INDEX

256kb

Memory. . . . . . . . . . . . . . . . . . . . . . . . . . .4

999 FOR MORE

Next Page . . . . . . . . . . . . . . . . . . . . . . . . 21

A.S.P.. . . . . . . . . . . . . . . . . . . . . . . . . . . . 44

Acceptable entries. . . . . . . . . . . . . . . . . . . . . . .7

Area Name . . . . . . . . . . . . . . . . . . . . . . . . . . 15

Areas . . . . . . . . . . . . . . . . . . . . . . . . . . . . 15

Arrow keys

Pointer . . . . . . . . . . . . . . . . . . . . . . . . . 27

Arrows

TRAKxxxx. . . . . . . . . . . . . . . . . . . . . . . . . 31

ASCII . . . . . . . . . . . . . . . . . . . . . . . . . . . . 38

Backup. . . . . . . . . . . . . . . . . . . . . . . . . . . . 34

Backup Disk

Backup. . . . . . . . . . . . . . . . . . . . . . . . . . 34

Benefits

REGISTER. . . . . . . . . . . . . . . . . . . . . . . . . 43

Brian Cook. . . . . . . . . . . . . . . . . . . . . . . . . . 38

Buying the program

REGISTERING . . . . . . . . . . . . . . . . . . . . . . . 43

Case. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 17

CD\WORLD. . . . . . . . . . . . . . . . . . . . . . . . . . . 35

CGA . . . . . . . . . . . . . . . . . . . . . . . . . . . 37, 39

Monitor . . . . . . . . . . . . . . . . . . . . . . . . . 37

CGA COLORS. . . . . . . . . . . . . . . . . . . . . . . . . . 39

CHECK FILES

CHECK . . . . . . . . . . . . . . . . . . . . . . . . . . 37

CHECK FOR FILES

Missing files . . . . . . . . . . . . . . . . . . . . . . 37

CITY VIEW OPTION. . . . . . . . . . . . . . . . . . . . . . . .8

COLOR

Selection . . . . . . . . . . . . . . . . . . . . . . . . 38

COLOR SELECTION . . . . . . . . . . . . . . . . . . . . . . . 38

Combination of letters. . . . . . . . . . . . . . . . . . . . 17

COMMAND.COM . . . . . . . . . . . . . . . . . . . . . . . . . 36

Configuration

Custom. . . . . . . . . . . . . . . . . . . . . . . . . . 37

CONFIRMATION

CONFIGURE . . . . . . . . . . . . . . . . . . . . . . . . 39

CONTINUE. . . . . . . . . . . . . . . . . . . . . . . . . . . 24

Copy Protection

copy. . . . . . . . . . . . . . . . . . . . . . . . . . . 34

Copy to Hard Disk . . . . . . . . . . . . . . . . . . . . . . 35

Cost. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 43

CREATOR.EXE

Copyright Robert L. Lloyd . . . . . . . . . . . . . . . . 22

Custom

Customize . . . . . . . . . . . . . . . . . . . . . . . . 37

CUSTOM CONFIGURATION. . . . . . . . . . . . . . . . . . . . . 37

CUSTOM.LOC. . . . . . . . . . . . . . . . . . . . . . . . . . 17

Custom. . . . . . . . . . . . . . . . . . . . . . . . . . 40

Your list . . . . . . . . . . . . . . . . . . . . . . . . 22

DATE. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 24

DEFAULTS. . . . . . . . . . . . . . . . . . . . . . . . . .7, 12

DISKCOPY

backup. . . . . . . . . . . . . . . . . . . . . . . . . . 36

Display Levels. . . . . . . . . . . . . . . . . . . . . . . . 10

Distance

Pointer . . . . . . . . . . . . . . . . . . . . . . . . . 27

Distances . . . . . . . . . . . . . . . . . . . . . . . . . . .1

DOS . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 34

Eastern hemisphere. . . . . . . . . . . . . . . . . . . . . . .7

EDITING

CUSTOM.LOC. . . . . . . . . . . . . . . . . . . . . . . . 22

EDLIN . . . . . . . . . . . . . . . . . . . . . . . . . . 22, 40

EGA . . . . . . . . . . . . . . . . . . . . . . . . . . . 37, 39

COLORS. . . . . . . . . . . . . . . . . . . . . . . . . . 39

Monitor . . . . . . . . . . . . . . . . . . . . . . . . . 37

END

Exit. . . . . . . . . . . . . . . . . . . . . . . . . . . 21

Quit. . . . . . . . . . . . . . . . . . . . . . . . . . . 33

Exact spelling

Game. . . . . . . . . . . . . . . . . . . . . . . . . . . 33

FILE FORMATS

ASCII . . . . . . . . . . . . . . . . . . . . . . . . . . 40

FLOPPY DISKS. . . . . . . . . . . . . . . . . . . . . . . . . 36

GAME

Random. . . . . . . . . . . . . . . . . . . . . . . . . . 33

GLOBAL SCALING. . . . . . . . . . . . . . . . . . . . . . . . 12

Graphics compatible

Requirement . . . . . . . . . . . . . . . . . . . . . . . .4

GRID LINES. . . . . . . . . . . . . . . . . . . . . . . . . . 14

HARD DISK . . . . . . . . . . . . . . . . . . . . . . . . . . 34

HARD. . . . . . . . . . . . . . . . . . . . . . . . . . . 34

Hercules. . . . . . . . . . . . . . . . . . . . . . . . . . . 37

Monitor . . . . . . . . . . . . . . . . . . . . . . . . . 37

SIMULATOR . . . . . . . . . . . . . . . . . . . . . . . . .4

LAT . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .6

Latitude and Longitude

Entry . . . . . . . . . . . . . . . . . . . . . . . . . . .6

Limitations: CUSTOM.LOC

Custom. . . . . . . . . . . . . . . . . . . . . . . . . . 40

Limitations: TRAKxxxx files

Custom. . . . . . . . . . . . . . . . . . . . . . . . . . 41

Limits

CUSTOM.LOC. . . . . . . . . . . . . . . . . . . . . . . . 22

TRAKxxxx. . . . . . . . . . . . . . . . . . . . . . . . . 31

LONG. . . . . . . . . . . . . . . . . . . . . . . . . . . . . .7

Look more . . . . . . . . . . . . . . . . . . . . . . . . . . 18

MAPPING . . . . . . . . . . . . . . . . . . . . . . . . . . . .9

MD\WORLD. . . . . . . . . . . . . . . . . . . . . . . . . . . 35

MEMBER

A.S.P.. . . . . . . . . . . . . . . . . . . . . . . . . . 44

Menu. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 34

Missing File. . . . . . . . . . . . . . . . . . . . . . . . . 38

MONEY ORDERS. . . . . . . . . . . . . . . . . . . . . . . . . 43

MONITOR . . . . . . . . . . . . . . . . . . . . . . . . . . . 37

MONITOR SELECTION . . . . . . . . . . . . . . . . . . . . . . 37

MORE DATA

TRAKS . . . . . . . . . . . . . . . . . . . . . . . . . . 25

Multiple Copies . . . . . . . . . . . . . . . . . . . . . . . 44

National capitals

Tables. . . . . . . . . . . . . . . . . . . . . . . . . . .1

NEAREST CITY. . . . . . . . . . . . . . . . . . . . . . . . . .9

New Locations . . . . . . . . . . . . . . . . . . . . . . . . .1

NOTE: CODES

File formats. . . . . . . . . . . . . . . . . . . . . . . 40

NOTE: CUSTOM.LOC. . . . . . . . . . . . . . . . . . . . . . . 40

NOTE: Distance Map. . . . . . . . . . . . . . . . . . . . . . 30

NOTE: ELIMINATE A CAPITAL . . . . . . . . . . . . . . . . . . 11

NOTE: Global. . . . . . . . . . . . . . . . . . . . . . . . . 17

NOTE: Nearest city. . . . . . . . . . . . . . . . . . . . . . 32

NOTE: Nearest track . . . . . . . . . . . . . . . . . . . . . 32

NOTE: One track . . . . . . . . . . . . . . . . . . . . . . . 31

NOTE: OVERLAY

Custom. . . . . . . . . . . . . . . . . . . . . . . . . . 42

NOTE: SEARCH. . . . . . . . . . . . . . . . . . . . . . . . . 18

NOTE: TRAKS . . . . . . . . . . . . . . . . . . . . . . . . . 25

NOTE: TRAKxxx SUBDIRECTORY. . . . . . . . . . . . . . . . . . 38

NOTE: Traks . . . . . . . . . . . . . . . . . . . . . . . . . .9

Number. . . . . . . . . . . . . . . . . . . . . . . . . . . . 38

ONE FLOPPY. . . . . . . . . . . . . . . . . . . . . . . . 34, 36

Floppy system . . . . . . . . . . . . . . . . . . . . . . 36

Hard disk . . . . . . . . . . . . . . . . . . . . . . . . 34

Option 1

Latitude and Longitude. . . . . . . . . . . . . . . . . . .6

OPTION 10

Point at a location . . . . . . . . . . . . . . . . . . . 27

OPTION 11

Distance between Cities . . . . . . . . . . . . . . . . . 29

OPTION 12

Plot one Track. . . . . . . . . . . . . . . . . . . . . . 31

OPTION 13

Name the City . . . . . . . . . . . . . . . . . . . . . . 33

OPTION 2

Large area name . . . . . . . . . . . . . . . . . . . . . 15

OPTION 3

Europe. . . . . . . . . . . . . . . . . . . . . . . . . . 16

Option 4

City or Country name. . . . . . . . . . . . . . . . . . . 17

Option 5

Capitals of the U.S.A.. . . . . . . . . . . . . . . . . . 20

Option 6

National Capitals . . . . . . . . . . . . . . . . . . . . 21

OPTION 7

Cities of note. . . . . . . . . . . . . . . . . . . . . . 22

OPTION 8

new info on Tracks. . . . . . . . . . . . . . . . . . . . 24

OPTION 9

Global view . . . . . . . . . . . . . . . . . . . . . . . 26

OVERLAY file. . . . . . . . . . . . . . . . . . . . . . . . . .4

Overlays. . . . . . . . . . . . . . . . . . . . . . . . . . . 38

PC-WRITE. . . . . . . . . . . . . . . . . . . . . . . . . . . 40

Copyright Quicksoft . . . . . . . . . . . . . . . . . . . 22

Pointer . . . . . . . . . . . . . . . . . . . . . . . . . . . .1

POLITICAL BOUNDARIES. . . . . . . . . . . . . . . . . . . . . 13

PREPARING WORKING DISK

BACKUP. . . . . . . . . . . . . . . . . . . . . . . . . . 34

PRINTING a map

Hardcopy. . . . . . . . . . . . . . . . . . . . . . . . . 33

Printing EGA. . . . . . . . . . . . . . . . . . . . . . . . . 33

Purpose . . . . . . . . . . . . . . . . . . . . . . . . . . . .1

REGISTRATION

SHAREWARE . . . . . . . . . . . . . . . . . . . . . . . . 43

Requirements

Hercules. . . . . . . . . . . . . . . . . . . . . . . . . .4

Sample Letter Search. . . . . . . . . . . . . . . . . . . . . 18

SCALING . . . . . . . . . . . . . . . . . . . . . . . . . . . .8

Score . . . . . . . . . . . . . . . . . . . . . . . . . . . . 33

Select again. . . . . . . . . . . . . . . . . . . . . . . . . 18

SELECT BY NUMBER. . . . . . . . . . . . . . . . . . . . . . . 20

SHALL I CONTINUE

TRAKS . . . . . . . . . . . . . . . . . . . . . . . . . . 25

Shareware

REGISTERING . . . . . . . . . . . . . . . . . . . . . . . 43

SHOW TRACKS . . . . . . . . . . . . . . . . . . . . . . . . . .9

SIMULATOR

Hercules. . . . . . . . . . . . . . . . . . . . . . . . . .4

SOURCE

(SOURCE). . . . . . . . . . . . . . . . . . . . . . . . . 34

Southern hemisphere . . . . . . . . . . . . . . . . . . . . . .6

SPECIAL SUBDIRECTORY. . . . . . . . . . . . . . . . . . . . . 38

Spherical map . . . . . . . . . . . . . . . . . . . . . . . . .8

Started . . . . . . . . . . . . . . . . . . . . . . . . . . . .4

Starting the Program

Starting. . . . . . . . . . . . . . . . . . . . . . . . . .4

STATE BOUNDARIES. . . . . . . . . . . . . . . . . . . . . . . 13

Subdirectory. . . . . . . . . . . . . . . . . . . . . . . 35, 38

SUP.DAT . . . . . . . . . . . . . . . . . . . . . . . . . 38

SUP.DAT

SUP.DAT . . . . . . . . . . . . . . . . . . . . . . . . . 38

SUP.DAT files

Custom. . . . . . . . . . . . . . . . . . . . . . . . . . 41

SUPPLEMENTAL. . . . . . . . . . . . . . . . . . . . . . . . . 38

SUPPLEMENTAL FILE NAME

OVERLAY . . . . . . . . . . . . . . . . . . . . . . . . . 38

Tables

State Capitals. . . . . . . . . . . . . . . . . . . . . . .1

Tailored to your needs. . . . . . . . . . . . . . . . . . . . 44

TARGET

Backup. . . . . . . . . . . . . . . . . . . . . . . . . . 36

test. . . . . . . . . . . . . . . . . . . . . . . . . . . . . 39

The Details . . . . . . . . . . . . . . . . . . . . . . . . . .6

The Main Menu . . . . . . . . . . . . . . . . . . . . . . . . .4

The Opening Screen

OVERLAY file? . . . . . . . . . . . . . . . . . . . . . . .4

TRACK IDENTIFIER STRING

TRAKS . . . . . . . . . . . . . . . . . . . . . . . . . . 24

TRACK STRENGTH or CONFIDENCE

TRAKS . . . . . . . . . . . . . . . . . . . . . . . . . . 24

TRACKING. . . . . . . . . . . . . . . . . . . . . . . . . . . 13

Storms. . . . . . . . . . . . . . . . . . . . . . . . . . .1

TRAK* . . . . . . . . . . . . . . . . . . . . . . . . . . . . 13

TRAKSXX

Storms. . . . . . . . . . . . . . . . . . . . . . . . . . 37

TRAKxxxx. files

Custom. . . . . . . . . . . . . . . . . . . . . . . . . . 41

TWO FLOPPY. . . . . . . . . . . . . . . . . . . . . . . . 35, 36

Floppy system . . . . . . . . . . . . . . . . . . . . . . 36

Hard disk . . . . . . . . . . . . . . . . . . . . . . . . 35

Two point distance. . . . . . . . . . . . . . . . . . . . . . 29

Typing END

EXIT. . . . . . . . . . . . . . . . . . . . . . . . . . . 20

U.S. FUNDS

Please. . . . . . . . . . . . . . . . . . . . . . . . . . 43

USING the program . . . . . . . . . . . . . . . . . . . . . . 43

USSR.DAT. . . . . . . . . . . . . . . . . . . . . . . . . . . 38

VGA . . . . . . . . . . . . . . . . . . . . . . . . . . . 37, 39

COLORS. . . . . . . . . . . . . . . . . . . . . . . . . . 39

limitation. . . . . . . . . . . . . . . . . . . . . . . . 37

Monitor . . . . . . . . . . . . . . . . . . . . . . . . . 37

WHICH AREA. . . . . . . . . . . . . . . . . . . . . . . . . . 15

WORD PERFECT. . . . . . . . . . . . . . . . . . . . . . . . . 40

Copyright WordPerfect . . . . . . . . . . . . . . . . . . 22

WORDSTAR. . . . . . . . . . . . . . . . . . . . . . . . . . . 40

Copyright MicroPro International. . . . . . . . . . . . . 22

WORLD or CITY VIEW. . . . . . . . . . . . . . . . . . . . . . .8

WORLD VIEW OPTION . . . . . . . . . . . . . . . . . . . . . . 12

WORLD.CNF

Configuration . . . . . . . . . . . . . . . . . . . . . . 37

DISCLAIMER

THIS PROGRAM IS PROVIDED AS A GENERAL REFERENCE.

I WILL NOT BE HELD LIABLE FOR ANY DAMAGE CAUSED BY THE USE

OF THIS PROGRAM. THE PROGRAM CONTAINS INACCURACIES AND ERRORS

IN FACT THAT HAVE SLIPPED BY ME! I WILL TRY TO ELIMINATE THESE

INACCURACIES WHEN NOTIFIED IN WRITING OF THEM. THIS PROGRAM

SHOULD NOT BE USED FOR NAVIGATION OR STORM WARNING OR AS AN

AUTHORITY FOR ANY OTHER PURPOSE. IT IS ONLY PROVIDED TO ASSIST

IN LEARNING AND FOR FUN!

#