-------------------------------------------

KBDINT - A KEYBOARD INTERRUPT DEMONSTRATION

-------------------------------------------

In recent days, I've been trying to write an Interrupt Service Routine (ISR)

for the keyboard interrupt INT 9, which will respond to a keypress of any key

on the keyboard, including [Caps Lock], [Num Lock] and [Scroll Lock].

After several attempts, using snippets of code from books, I discovered that

the code cited in all the books I had referred to, would not work for the

[Caps Lock], [Num Lock] and [Scroll Lock] keys (on my computer at least).

My computer would either just lock up and not respond to any keypress -

requiring a cold boot - or the ISR would act in a way I didn't expect - in

my particular case, a variable, which should have been incremented only once,

was incremented eight times.

I concluded that the original ISR, which must be called by my own ISR (to

prevent a system crash !), probably executes the INT 9 instruction when the

[Caps Lock], [Num Lock] or [Scroll Lock] key is pressed.  So I adjusted the

code of my own ISR to allow for this.

Thinking that it might be of interest to others how I did this, I decided to

write a small program to demonstrate the results of each of the three

versions of my ISR.  The main purpose of the ISR is to increment (once) the

variable 'bKbhit' in the main program, each time a key is pressed.  The main

program, in this case however, exits after one keypress.

KBDINT.EXE uses the successful version.  To run it just type KBDINT.

To see how KBDINT works with the other versions just change line #26 of

KBDINT.ASM:

from

INT09_ROUTINE   =       INT09_RIGHT

to

INT09_ROUTINE   =       INT09_WRONG1

or

INT09_ROUTINE   =       INT09_WRONG2

and re-assemble and link.  It would be easiest to reassemble and link using

the included makefile, KBDINT.MAK, and MAKE version 3.6 or later, with the

command:

make -f kbdint  (for earlier versions use: make -fkbdint)

If you replace INT09_RIGHT with INT09_WRONG1 in the equate above, you'll have

to cold-boot your computer (I had to cold-boot mine anyway).  If you use

INT09_WRONG2 in the above equate, the system doesn't crash, but you must wait

about a minute for the program to end and return control back to DOS.

The output displayed by KBDINT is identical using all three versions of the

int 9 ISR (all named Int09) unless the [Caps Lock], [Num Lock] or

[Scroll Lock] key is pressed.  That is:

The old interrupt address was: XXXXH:XXXXH

The new interrupt address is : XXXXH:XXXXH

Press any key...

'bKbhit' variable was incremented 1 time(s).

The interrupt address is restored to: XXXXH:XXXXH

With INT09_ROUTINE set to INT09_RIGHT, the output is the same as above when

the [Caps Lock], [Num Lock] or [Scroll Lock] keys are pressed.  However, when

set to INT09_WRONG1, the system locks up at line 3 (Press any key...).  When

set to INT09_WRONG2, line 4 (on my system) changes to:

'bKbhit' variable was incremented 8 time(s).

If there is anyone who might have some better ideas for making a better ISR

for the keyboard interrupt 9, or if you just want to make some comments or

ask some questions, please contact me via CompuServe Mail.

My CompuServe ID number is:     1000236,2477

ABOUT ASMS.LIB

--------------

KBDINT.ASM includes some procedures in ASMS.LIB - a library of procedures

(for the small memory model) which I wrote, and which I have frequently found

useful in my programs.  If you would like a description of how to use these

procedures, or the original source code, I would be happy to send it to you

via CompuServe Mail, but I'll probably have to charge a small fee for the

service (sorry - but I have to try to make a living from writing my

programs).  A complete list of the procedures is in ASMS.LST.