VOICE DIGITIZATION AND REPRODUCTION ON THE

IBM PC/XT AND PC/AT BUILT-IN SPEAKER

--------------------------------------------

Alan D. Jones	 July 1988

The speaker on the PC and its associated driver circuitry is quite

simple and crude, having been designed primarily for creating single

square-wave tones of various audio frequencies. This speaker is typically

driven by a pair of transistors used as current amplifier which is in turn

driven directly by the output of a TTL gate. This results in only two

possibilities of voltage across the voice coil: 0 volts and 5 volts. Any

sound to be reproduced by this system must be reduced to an approximation

in the form of a stream of constant-amplitude, variable-width rectangular

pulses.

Examination of a speech waveform on an oscilloscope display quickly

tells us that it is not going to be possible to even remotely mimic this

waveform under the above restrictions. Much of the information contained

in the waveform is in the form of amplitude variations, and this is the

one attribute we cannot reproduce. It is initially tempting to try to

use the technique of the "class D" amplifier to create the waveform, using

high-speed pulse width modulation and depending on the mechanical

characteristics of the speaker and those of the human ear to provide the

missing low-pass filtering. Assuming the sampling rate to be 8 KHz (based

on the Nyquist criterion) and, to conserve memory, assuming the samples

to contain only 4 bits of amplitude information (16 levels), we can see

that data accumulates at a rate of 4k bytes per second, which is certainly

acceptable. The problem comes when we try to play back the sound. Pulses

occur at intervals of 125 microseconds, which doesn't seem too bad, but

since each pulse can have 16 possible widths, it is necessary to time the

pulses with a resolution of well under 8 microseconds. This is only a

couple of instruction times on a 4.77 MHz XT, and even on a fast 80386

it doesn't give the CPU much time between bits to shift bits, read and

increment a pointer, check the pointer to see if it's done yet, etc., not

to mention the difficulty of servicing unrelated interrupts.

The search for simpler (but still usable) and less CPU-intensive

methods of reproducing speech leads to the question of what information

in the waveform we can discard without an unacceptable loss of

intelligibility. My experiments with running speech signals through

a graphic equalizer revealed that the lower-frequency components, those

which are most visible to the eye on the oscilloscope, are actually of

minimal importance in understanding speech. This is also demonstrated by

the fact that a whisper is just as understandable as normal speech, but

does not make use of vibrating vocal chords, which are the primary source

of low-frequency components in the voice.

The digitizer circuit consists of two stages of voltage amplification with

some high-pass filtering built into the coupling capacitors, followed by a

differentiator.  The output of the differentiator is fed to a voltage

comparator, thus producing an output which has approximately the following

relationship to the input from the microphone:	If the derivative of the speech

waveform if positive, then the output is logic zero; If the derivative of the

speech waveform is negative, then the output is logic one.  The transition

timing at the output is entirely analog in nature; there is no synchronizing

clock signal anywhere in the circuit.

If the output of this circuit is connected directly to a speaker, the

resulting sound will still be an understandable version of the input.

Since the output consists of nothing but a digital bit stream, the job

of the computer becomes that of simply recording and accurately reproducing

this bit stream.

The program operates by reprogramming the 8253 time chip to produce

hardware interrupts at the 16.5 KHz rate. The interrupt service routine then

manipulates the NAND gate driving the speaker based on bits read from the

file. The 16.5 Khz rate was chosen by trial-and-error; this is the audible

"point of diminishing returns", where a further increase in sampling rate

didn't produce enough of an improvement to warrant the increased memory

usage.

This technique is somewhat limited in its usefulness. It necessitates

the writing of a "badly behaved" program which not only reprograms the timer

chip but also totally hogs the CPU for the duration of the voice output.

Nevertheless, it demonstrates a few interesting things about how humans hear

speech. I first developed this circuit over a year ago as a rebuttal to

someone who said "it couldn't be done". Not only can it be done, it is

actually quite simple. Certainly the circuit could be improved, at the

possible expense of increased complexity. I'm waiting to hear from some of

you. If anyone has questions, especially about my sloppy code, I check

for messages on CIS every three or four days.

- Alan

74030,554