PortaCall(c)

User's Guide

------------

Introduction:

If you regularly use your cellular phone to check your answering

machine messages, then PortaCall is the program for you.

With PortaCall, your computer will call you whenever a new message is

on your machine.  PortaCall lets you know there is a message, by

dialing your cellular phone, pager or any other type of phone,

leaving a few harmless rings and beeps, and then hanging up.

When you receove a PortaCall call, you can then dial up your answering

machine and retrieve your new message.

PortaCall will make you more responsive to your customers and will save

you money in phone charges-  You'll only call your answering

machine when you KNOW there are messages.

Hardware Requirements and Setup

-------------------------------

To use PortaCall you'll need an answering machine and a Hayes

compatible, auto-dial modem.  PortaCall will dial out on either

a PULSE or TONE phone system.  The default setting is TONE,  if you

want PULSE you must put a 'P' before the phone number (see examples

below).

For PortaCall to work properly, you must connect your phone equipment

so that your answering machine and your modem sense incoming calls at

the same time.

One way to do this is to connect your phone line from the wall jack

into the LINE jack on the modem and then connect the answering machine

to the modem's PHONE jack.

When properly connected your answering machine should pick up calls as

usual and (without any communications software running) your modem

lights should blink whenever you receive phone calls.  If you have an

internal modem, obviously, you cannot check the lights.  In this case

just make sure the answering machine is working properly.

(1)

Running the Program

--------------------

1.      Before you leave your office, turn on your PC and modem

2.      Load PortaCall by entering:

PCALL PhoneNumber WaitTime RingCount C[Comm. Port]

For example:

PCALL 555-1212 2 3 C2

or

PCALL P555-1212 2 3 C2

(this option uses PULSE dialing).

(Hint: store your PortaCall startup command in a batch file, or

use DOSSHELL to invoke PortaCall with your startup information)

The above examples will set PCALL to dial 555-1212 two minutes

after your answering machine receives a message.  PortaCall will

leave three rings on the cellular phone and then hang up.

PortaCall will look for your modem on COM2.

Things to Consider

------------------

The phone number can be up to 24 characters long.  It can include

dashes and commas(pauses).  It can also include any special Hayes

dialing commands.  Look at PORTACALL USAGE EXAMPLES at the end of

these intructions.

The WAIT TIME is in minutes.  It begins as soon as your office phone

rings.  You should use a WAIT TIME that is long enough for someone to

leave a nice long message.  If the wait time is too short, then

PortaCall may begin dialing your cellular phone before the caller

finishes his message.  The WAIT TIME must be between 1 and 9 minutes.

The RING COUNT is based on the standard '6 seconds per ring' timing

on most phones.  On cellular phones there is sometimes a long delay

before the first ring occurs.  It may be necessary to add 1 to the ring

count.  For example, if you want PortaCall to leave 2 rings on your

cellular phone, use 3 as the ring count.  The RING COUNT must be

between 1 and 9 rings.

The COM port tells PortaCall where your modem is connected.  If you're

not sure what COM port to use, start with COM1 and work your way up to

COM4.  You'll know if you've found the right port when you call into

your answering machine and PortaCall begins counting down; preparing

to dial out.

(2)

Checking Your Messages

----------------------

Whenever your remote phone rings a few times and then stops, you'll

know there are messages waiting for you. In the mean time, PortaCall

will have looped back to wait for the next phone call to your answering

machine.  Keep in mind that when you call up to get your messages you

will be activating  PortaCall again and because PortaCall doesn't know

that it's you checking your messages, it will, again, call your cellular

phone a few minutes after you've dialed in. If you are certain that this

second PortaCall call is due to your message checking, then just ignore

it.

The PortaCall Screen

--------------------

The PortaCall screen shows the values you chose when you started

PortaCall.  To modify these values, press ESC to exit PortaCall and

then start the program again with new values:

Dial        Indicates the cellular phone number

Wait        Indicates the delay time between when a

message is left on the answering machine and

when PortaCall dials the cellular phone

And Leave   Indicates the number of rings to be left on

the cellular phone

Status      Indicates the status of PortaCall

COM Port    Indicates the communications port where the

modem is connected.

When PortaCall is loaded the status will say 'Waiting for a Call', as

soon as a call is received the status will change to 'nnn Seconds Until

Dial'.  You will see the counter changing.  When the counter reaches 000

PortaCall will dial the indicated phone number and leave the specified

number of rings.

(3)

PortaCall Usage Examples

------------------------

Usage 1:   Dial, Ring and Hangup

Example startup command: PCALL 555-1234 2 4 C2

This method tells PortaCall to dial the phone number and hangup after

4 rings.  This method is good for cellular phone users who don't

give out their phone number.  When you hear the cellular phone ringing

you'll know its a PortaCall phone call.  Simply allow the phone to

finish ringing and don't even pick it up.

----------------------------------------------------------------------

Usage 2:  Dial, Ring, Leave a few beeps (or dial an extension), Hangup

Example startup command: CALL 555-1234@1111 2 6 C2

This method tells PortaCall to dial the phone number, pause 5 seconds

and then leave a few beeps.  It could be used when you're working in

a remote office and you want to distinguish PortaCall calls from other

phone calls.  You'll know it's a PortaCall call if you pick up your

phone, hear 5 seconds of silence and then a few beeps.

This same method could be used on VOICE MAIL systems to dial a telephone

extension after PortaCall has dialed the main number.  It could also be

used when dialing a PAGER.  To have PortaCall dial a pager, just replace

the '1111' with the pager access number.

Registration

-------------

PortaCall is a SHAREWARE program.  It can be copied and distrubted

freely.  However, it would be greatly appreciated if you registered

PortaCall with its author.  PortaCall should save you a great deal

of time and money if you use it regularly.  For this I ask just $10.00.

Please register by sending a check or money order for $10.00 to Bill

Dineen, 455 Hope Street #3F, Stamford, CT  06906.

If you have any questions about PortaCall, my CIS ID# is 76370,2451.

(4)