===========================================================

USRATI6.EXE   v1.0               (c)1994 Brian E. Kelly

===========================================================

What it does:

-------------

USRATI6 is a small program which sends the "ATI6" command

to a U.S. Robotics modem, filters its reply with a list in

a configuration file and writes the result to a file.

Why I wrote it:

---------------

I run a multi-node Wildcat! BBS under OS/2. Sometimes I get

complaints that the system is running slow or the predicted

time for file downloads is inaccurate. I have not been able

to replicate the problem myself and therefore have to assume

that poor telephone lines are responsible. I use U.S.Robotics

modems which have an "ARQ" indicator on their front panels

and I note that sometimes this flickers while a user is

connected. The flicker tells me that an error has been

detected and is being corrected. These errors are hidden from

the user because the modem asks for a block of data to be

repeated and only passes it through its DTE connector when

all the errors have been removed. However, this resending

takes time and gives the user the impression that the BBS

is running slower than it should. All USR modems have an

in-built facility to report details of the last call by

sending the ATI6 command to them. By sending this command

and disecting the reply before the BBS resets the modem it

is possible to see what really did happen during the call.

How to use it:

--------------

The program was written to be used with Wildcat! BBS but will

quite happily work with any comms program as well. I did the

initial trials by using QmodemPro and running the program

after every call.

The program expects to run at a DOS prompt, at the prompt in

an OS/2 DOS session or in a batch file. The syntax is:

USRATI6 <com port> <baud rate> <TERM _or_ logging file name>

In its present incarnation the program only works with COM1

and COM2 both using the standard DOS interrupts. The next

release will have the option to use more ports and to select

non-standard IRQs.

The baud rate can be 300, 600, 1200, 2400, 4800, 9600, 19200

38400 or 57600. If a value of zero is entered, the divider

register in the UART is left untouched and the port remains

at whatever speed it was previously used at. For BBS use,

leave the value	at zero as the DTE speed should be kept locked

anyway.

Any valid logging file name can be used. You can enter a full

path and name or just the name itself. In no path is given,

the file will be written in the current directory. The file is

used in 'append' mode so the log can be added to the end of

an existing file. If the file doesn't exist it will be created.

I give the name of the activity log for the Wildcat! nodes it

runs on so the information is written after the call details.

If you find the activity log becomes too large to handle, you

can log to a different file. The date, time and port number

are in the log to allow matching against the activity log.

If the word "TERM" is entered instead of a file name, the

program becomes a very simple ASCII terminal. This is to help

when configuring the modem or checking its responses. I do not

recommend using it to make outgoing calls via the modem, it

may work but the risk is yours. You can exit from terminal

mode by pressing the [Esc] key.

The Configuration file:

-----------------------

This is a text file containing all the parameters returned by

the modem. The file must be in the same directory as USRATI6

is executed from. On a Wildcat! system this will probably be

the "wcwork\node*" directory for the node being logged. By

using this location, a different configuration can be set for

each node. If the program is run without a configuration file

it will create a default one and then use it. A warning is

given that the file may need editing but this may not be seen

if the program is run from a batch file. Use any text editor

to change the files contents. Any line starting with a ";"

(semi-colon) is ignored, delete the ";" to make the line

active. Only one parameter is allowed on each line. Note there

are lines for "Equalisation" and "Equalization" as both

spellings are used depending on the modems intended country

of use. If new parameters are introduced in later models

they can be added to the config file, there is no limit to the

files length.

What you should get:

--------------------

something like this-

� Connection report from USRATI6 v1.0 on Sun Jul 03 00:10:52 1994 Ŀ

� Chars sent          4697                                         �

� Chars Received      2442623                                      �

� Chars lost          0                                            �

� Octets sent         4697                                         �

� Octets Received     2442623                                      �

� Blocks sent         3763                                         �

� Blocks Received     83970                                        �

� Blocks resent       0                                            �

� Retrains Requested  0                                            �

� Retrains Granted    0                                            �

� Line Reversals      0                                            �

� Blers               13                                           �

� Link Timeouts       0                                            �

� Link Naks           0                                            �

� Data Compression    NONE                                         �

� Equalisation        Not Reported                                 �

� Equalization        Long                                         �

� Fallback            Enabled                                      �

� Protocol            LAPM                                         �

� Speed               9600                                         �

� Last Call           Not Reported                                 �

� Disconnect Reason   is Loss of Carrier                           �

�--------------------------------------------------[Port 2]--------�

This is taken from an actual call and with all the lines in

the config file active. The modem used was a Dual Standard

HST/V32 model. The call was in V32 mode, be aware that some

lines are meaningless unless in HST mode.

The report is probably much longer than you would ever need

but editing the config file would of course make it smaller.

Feedback:

---------

I hope you find USRATI6 meets your requirements. I know the

limit of two com ports may present problems but steps are

being taken to overcome this deficiency.

I have tested the program and used it on my own BBS, it seems

to run fine. If you find any bugs I would like to hear about

them as quickly as possible. My BBS and CIS numbers are later

in this text.

Registration:

-------------

The program didn't take long to write and doesn't employ any

nifty programming tricks so I'm not asking a fortune for its

registration. Try it out and if you think it deserves some

of your hard earned cash, send a small donation toward my

BBS running costs. I suggest $10, 5 UK pounds or its equivalent

in your own currency. I can accept cheques (checks) from UK

banks only. For other currencies, put paper cash in an envelope

and mail it to me. Make sure the contents can't be seen through

the envelope. I will try to confirm its arrival by Email if

you give me a return address. If you make a donation it will

also cover any future releases of the program.

Disclaimer:

-----------

(sorry about this but I have to say it)

The program is released without warranty or guarantee of any

kind. The author will not be held liable for damage caused by

this program or as consequence of it being used on any system.

It is released in good faith and is believed to be bug free.

Acknowledgements:

-----------------

The program is written in Borland Turbo C++ version 3.0.

Serial routines are from PCL4C by Marshallsoft Computing Inc.

Contacting me:

--------------

My BBS details are -

BetWiXt BBS. +44 633 614765  (from the UK it is 0633 614765)

U.S. Robotics Dual Standard V32terbo modems - 2 lines.

My CIS number is -  71141,2757

My mail address is -

Brian E. Kelly.

12 Cotswold Way

Risca

Gwent.  NP1 6QT

United Kingdom.

BBS note:

---------

BetWiXt has been running since 1990. It started in Palatine, IL.

but moved with me to South Wales, UK in 1991.

It concentrates on programming files and utilities and has sections

covering Ham radio and television. It is adopted by the B.A.T.C.

(British Amateur Television Club.)

Limited access on the first call but full access is given within

48 hours subject to confirmation of log-in details.