VK3UM INTERFACE BOARD DIAGNOSTIC CHECK-OUT

Firstly it cannot be more strongly emphasized that the printed circuit board

is 100% and should you find that the interface does not work then it is

something other than the board.

In all but one case, (a component placed in a wrong hole) most faults have

been caused by faulty IC's. The most common have been the 74LS00, 74LS05 and

74LS08. One board I have repaired was due to solder bridges between tracks!

The guy must have used a kilo of solder and a 1kW soldering iron!!!

I have also been asked about the timing aspects of the board. No it is NOT

critical and the components need not be precise. 5% is more than adequate

as the design is quite broad. Accordingly the pulse widths as described

here under are typical and a 5% variation (apart from the clock) may be

seen.

I have been at a loss just why some people have bought faulty IC's. In this

country I have been fortunate? to have not run into the problem. It seems to

be associated with European suppliers but all I can say is to be careful that

you obtain your IC's from a reliable source. They (the 74LSxx) only cost cents.

The design of the board is NOT repeat NOT critical of IC tolerances. Just

make sure that the IC's you get work in the first place!!!!!!

The decoding is fairly complex and should you run into a problem a single

faulty IC will totally upset the logic and the board will not work. Two

faulty IC's make the task even more complex.

Note    It is NOT necessary to install Relays RL7-8, transistors T7-8,

Diodes D7-D8, Resistors R7-8 and R15-16 to make the board function.

Here is an overview of how to go about isolating your problem.

Note IC11/1 refers to Integrated circuit number 11 pin 1 for example.

1. Double check your IC orientation.

All of the vertical IC's, pin 1 should be to the top/left of the board.

(relays are at the top).

All of the horizontal IC's, pin 1 should be to the left/bottom of the

board.

2. The relay driver transistors. T1-T8. Make sure that the flat is pointing

to the bottom of the board.

3. The diodes across the relays (D1-D8) have all their bands to the top of

the board.

4. Check the polarity of the tantalum capacitors with the layout. Note that

C20 and C21 run in opposite directions.

5. Check the polarity orientation of the remainder of the diodes. D9 and D10

have their bands to the top of the board whilst with D10 the band is

closest to the bottom of the board.

6. Check for the correct location of components between IC10 and IC11 as well

as between IC11 and IC12.

7. When applying power check the total current drawn as measured WITHOUT the

regulator (ie the +5v rail) will be between 170-190mA with NO RELAYS

operated. If it is greater than this you most likely have IC problems and

a quick check with your finger will locate the hot faulty chip and burn

your digit!! Be careful!!

NO RELAYS WILL BE OPERATED WHEN THE BOARD IS FIRST SWITCHED ON.

8. Run the software and select the Pulse Mode.

Again NO RELAYS SHOULD BE OPERATED UNTIL the Up/Down and CCW/CW keys are

selected. Each additional relay will increase the current drawn by

approximately 200mA. ie if two relays are operated the total current drawn

will be 570-590mA. Again this is a function of the relays chosen.

9. If the relays do not operate you have a problem!!! You will now need a

logic probe or a Cathode Ray Oscilloscope, and most assuredly a

replacement IC or two !!!!!!!

10. Baud Rate Oscillator.

IC19/7 (pin 7). You should see 5v square wave with a pulse width of

2.75uS. This will also appear on IC1/40, IC1/17 and IC20/10. If it is

not present then check around IC19 as it is not oscillating!

11. Are you sending data to the interface?

Have you selected the correct COM port and plugged it into the right

socket of your computer?  Now look on IC2/13 and you should see Data

coming from the computer of approx 26v p-p positive going pulses.

If you cannot see the data then you have the wrong RS232 connection or

the wrong COM port selected.

12. Is the MAX232 IC outputting the data?

Check IC2/12 and you should see the reverse of 11 above but at 5v p-p.

The pulses will be going negative. If not then you have a faulty IC2 or

the components around this chip maybe incorrectly placed.

13. IC3/5. You should see +5v positive going pulses 47uS wide repeated as

seen in 12 above. If this is not true then IC3 is faulty or even IC1

(very unlikely). Check around IC3 for problems.

This is where it becomes tricky as all the following IC's are interlocked and

it is not possible to diagnose the exact IC if one is faulty. However the

following checks will help to a considerable extent. In any case the faulty

IC or IC's will be one or more of  IC's 7-14.

14. IC10/13, IC4/1 and IC11/1 should all have the same wave form. This is in

the form of +ve going pulses 25uS wide similar to that seen in 12 above,

HOWEVER each time the Up/Down or CW/CCW keys are pressed you will see an

additional pulse.

15. IC11/3. In the normal state there will be 0v raising to a +5v pulse 25uS

wide each time a key (refer 14) is pressed.

16. IC13/12 will be the reverse of 15 above. That is +5v normally, with a

negative going pulse.

17. Relay selection and decoding.

IC11/11 and IC6/9 you will see a +ve 5v pulse 25uS wide each time the up

or down keys are selected. IC11/8  and IC5/9 will show the same pulses

when the CW or CCW keys are selected.

18. Relay drivers.

The normal state of the following conditions is 0v raising to +5v and

holding at this voltage whilst the motor direction is on. The respective

relays should operate.

CW   IC5/2   0 ---> +5v   seen also on RHS of resistor R1

CCW  IC5/15  0 ---> +5v   seen also on RHS of resistor R2

Up   IC6/15  0 ---> +5v   seen also on RHS of resistor R3

Down IC6/2   0 ---> +5v   seen also on RHS of resistor R4

Failure to see the above indicates a faulty IC 5 or 6 or transistors T1-T4.

Well that's about all I can do to help at this distance. I include this file

for those unlucky enough not to have the board run first go.

Remember that if you are using a Pulse Mode input then it has to be held high

(+5v) and go low to count.

It's no use me saying that I and many others have not had any problems if YOU

are reading this and HAVE a problem!!!

Hopefully this document will enable you to find the trouble fast!  I know I

am repeating myself when I say the fault is most likely an IC!!

If all else fails post it back to me (with sufficient return postage!) and I

will fix it for you !...................

Regards

Doug McArthur (VK3UM)

February 1993