The ______.ENG files included with this release of NX3 are

samples of engines we have simulated.  These files are simply the

report output for these engines.  You can review them using

whatever method you use to read this, or you can print them.

The following engine simulations are included:

353 cid Ford SVO SuperModified drag engine          SM_FORD .ENG

338 cid Chev SuperModified drag engine              SM_CHEV .ENG

"Grumpy" Jenkins 330 cid Pro Stock of the '70s      GRMPYTOY.ENG

SCCA Formula 5000, 302 Chev, road racer             F5000CHV.ENG

305 cid Ford Cobra wedge head for vintage racing    COBRA302.ENG

Typical NASCAR single 4 bbl before restrictors      NASCAR  .ENG

500 cid Pro-Stock Chevrolet, Low block deck         PRSTKCHV.ENG

A tunnel port, stroked Ford FE built for

"street" currently happily in a '74 Pantera      TPFORDFE.ENG

1275cc Mini-Minor British sedan road racer          BMC_MINI.ENG

Ford 351c Oval Track race engine                    BOSS351C.ENG

Ruggles-style Buick V-6, 4.5 liter NASCAR           V6_BUICK.ENG

Speculation about Lamborghini-Chrysler

Formula 1, 3.5 liter V-12                    F1_LAMBO.ENG

350 Chev engine you get if you accept all defaults  ES3SMPL .ENG

The Lambo is known to be an 80 degree bank, naturally aspirated,

V-12 of 3.5 liter displacement.  Both 4 and 5 valve cylinder heads

are being developed, and over 600 horsepower will probably be

required to be competitive.  Interestingly, this class of engine is

not limited by the strength of parts required to cope with nearly

turbine level engine speeds with short rod ratios, but rather by

piston ring flutter.  Enormous piston acceleration across TDC com-

pression/ignition will throw all but the lightest rings off the bottom

ring land, losing gas pressure loading off the top of the ring, and

allowing blow-by at the most critical part of the combustion cycle.

Knife thin motorcycle style rings are required, high compression for

greater pressure on top of the ring, and Grumpy Jenkins style gas ports

should help.

The BOSS351C was an iron head, flat tappet cam, Edelbrock Torker, and

850 Holley 351c we built and dyno-ed in 1977. The car was competitive

with aluminum head Rat Chevrolet engines, but as you may see had a narrow

power band.  If we had this program in '77, we would have redesigned the

cam and headers, run up to 7500 RPM and done much better.

Among the SuperModified and NASCAR engines it is interesting to

note the tremendous performances being attained with restrictive

carburetors.  The SM engines are limited to 750 cfm Holleys with

choke horn intact, but no choke blade.  Holley says the latest

models actually flow 800 cfm, which in Denver amounts to 890 (a

carburetor is a mass flow device, not a volume flow device).  The

SM Ford uses a large plenum, fabricated tunnel ram manifold; and

the SM Chev uses the latest Bow-Tie single plane manifold with a

1-1/2+ inch tall open spacer.  In any case, these engines must

fight carb size limits to be competitive.

The other engines have no such intake limits.  The F5000 engines

used timed port injection and achieved excellent wide range

driveability.  The pro-stocks use 2 x 660 cfm for Grumpy's Toy

and 2 x 1150 on tunnel ram manifolds for the 500 cid Pro Stocks.#