MSCDEX.DOC

QUOTE...

...the files CONFIG.SYS and AUTOEXEC.BAT need to be modified to complete

the installation of MSCDEX.

To do this, connect to your bootable drive and edit the file

CONFIG.SYS. If the CONFIG.SYS file is not there, you will have to

create it. First, scan the contents of CONFIG.SYS to see if there

is a LASTDRIVE command line in it. This line will either have to

be modified or added to read:

LASTDRIVE=Z

This lets MS-DOS know that it will have to allocate enough drive

records to account for the additional CD-ROM drive letters. It's

a good idea to have this line as the first entry in the

CONFIG.SYS file. The last drive in fact only needs to be the

largest drive letter that will be used on the system. For

example, a PC with 2 floppy drives (drives A and B), a hard disk

(drive C), and two CD-ROM drives (drives D and E) would only need

LASTDRIVE=E. A PC that is on a network though will typically

allocate more drive records than are used so that new network

drives can be added after booting.

Next, add entries for each device driver that is going to be

installed. The entry will read:

DEVICE=<driver.sys> /D:<device_name> <driver specific

switches>

The name of the device driver, driver.sys, will be replaced by

the full path name of the device driver that is being installed.

This will include the drive letter of the bootable drive followed

by the pathname of the device driver if the device driver was

installed in a subdirectory. If the above directions were

followed, then the device driver will be located in the root

directory. For example, on a hard disk system that boots off

drive C, a sample entry might read:

DEVICE=C:\DRIVER.SYS /D:MSCD001

For the Hitachi driver, this should read:

DEVICE=C:\HITACHI.SYS /N:1 /D:MSCD001

The next parameter /D:device_name is the name MSCDEX will use to

find the device driver. When the device driver is installed, this

name is what will be used by the device driver to name itself.

Every installed device driver must have a unique name. In

addition, due to the way that MS-DOS handles file opening, if

there is a device driver with the same name as a file in a

directory, then a file open call on that name will open a handle

to the device driver and not to the file. For this reason, you

will want to chose a device_name that will not likely be chosen

as a file name later. We recommend using names of "MSCDXXX" where

XXX is three digits. This will allow CD-ROM device driver entries

to be located in config.sys by the setup program at a later date.

The remaining fields for driver specific switches are device

driver dependent and will be specified in the documentation that

describes that particular device driver.

Next, AUTOEXEC.BAT needs to be edited to include an entry to

invoke MSCDEX each time the system is booted. If your PC is also

on a network, the network software must be installed before

MSCDEX. Otherwise the network will not install, since MSCDEX will

not allow the network to be installed second.

If your AUTOEXEC.BAT file starts up a 'shell' program such as

DOSSHELL or Windows, or runs another BAT file, you will need to

make sure the line that starts MSCDEX is ahead of the line that

starts your 'shell' or BAT file. Otherwise MSCDEX will not have a

chance to start the CD-ROM drives once your 'shell' or BAT file

begins.

A sample entry in AUTOEXEC.BAT to install MSCDEX would be:

C:\MSCDEX.EXE /D:MSCD000 /D:MSCD001 /M:20 /V /E

Include the drive letter of the bootable disk and the full

pathname for MSCDEX. If the above directions were followed,

MSCDEX will be in the root directory. Each device driver will

have a device_name listed on the command line to MSCDEX following

'/D:'. MSCDEX will use this parameter to locate each of the

device drivers and this name must be the identical to each of the

device_name fields used for each device entry in CONFIG.SYS.

The /M:<value> switch determines how many sector buffers MSCDEX

will allocate when it installs itself. The larger this value is,

the more sector cache entries are available and the less MSCDEX

will have to read directly from the CD-ROM drive. It is

especially important that there be enough entries to cache the

path table, and the more entries available for directory sectors,

the less MSCDEX will have to reread the directory files.

Typically, each drive should have at a minimum about 4-5 buffers

per drive but the larger this value is, the better the

performance will be.

Both the CD-ROM device driver and MSCDEX both require small

amounts of additional memory to run which should not cause any

problems. But if you do find after you have installed MSCDEX that

some programs no longer run because there is not enough memory,

you may want to review your CONFIG.SYS and AUTOEXEC.BAT files to

locate other entries that consume memory such as GRAPHICS or

FASTOPEN that you may not require and can remove.

There is an additional parameter /E which will tell MSCDEX to use

expanded memory if it is installed and available. The verbose

switch /V will ask MSCDEX to print additional information about

memory usage during initialization.

For software that requires that the CD-ROM drive be on a

particular drive letter, you can use the /L:<drive letter>

switch. The SETUP program will not set this switch so you will

have to edit your autoexec.bat file if this switch is needed. For

example, the following will begin assigning CD-ROM drives at

drive letter L:

MSCDEX /D:MSCD001 /L:L

There are also two other switches: /K and /S. /K causes MSCDEX to

look for a Supplementary Volume Descriptor that identifies a

shift-JIS Kanji volume for Japanese and /S specifies that MSCDEX

patch DOS to allow CD-ROM drives to be shared on MS-NET based

servers.

MSCDEX - Microsoft MS-DOS CD-ROM Extensions Version 2.20

Installation Instructions - Copyright (C) Microsoft Corp. 1989.

All rights reserved - page {page|1}

...ENDQUOTE