QWIK Ansi  -  Standard I/O Device Driver

(C) Copyright by Michael J. Acker.

MJA Software      Oct 12, 1986

DESCRIPTION:

QWIK Ansi is a replacement for the DOS ANSI.SYS device

driver.  The purpose of Qwik Ansi is for full support of the

ANSI.SYS functions with some added enhancements.  There seems

to be a large amount of interest in 'FANSI' screen creation

and display programs.  That is why I have decided to put my

own device driver into the public domain.   I wrote this

device driver some time ago, simply to satisfy my own desires

to have a much faster display routine.  To get the maximum

speed increase possible I suggest that you use QWIK Ansi with

QWIKSCRN, a screen handling interrupt routine also written by

your's truely.  I have included another small program in the

QWIK* series called QWIKMODE, which can be used with just

about any ANSI type device driver I have tested.  If you can

forgo the advantages of Ctrl_Break during screen I/O and do

not mind having the end of file characters displayed at the

end of text, you will certainly find QWIKMODE to be a very

usefull program.  DOS as you may or may not know treats all

screen I/O devices as a character device and not a block

device.  QWIKMODE simply tells DOS to treat the driver as a

block device, therefore increasing screen update speeds by

at least 10 fold.  If you decide to use QWIKSCRN, QWIK Ansi,

and QWIKMODE for your screen I/O handling, and you are not

Evelynn Woods, then I suggest you get a copy of BACKSCRL

from just about any BBS in the nation; because after using

the TYPE command on any text file larger than 2K you will

need to be able to page back and read what just flew by at

about warp 10.

Just a few words to 'FANSI' screen users.  Try the three

programs supplied in this package, and remember to change

the screen into block mode with QWIKMODE.  You will find that

a full screen, such as those found in ANSIDRAW27, can be put

up in under a second; verses the slow line by line updating

that you have been suffering through for so long.  Remember

that QWIKMODE works with other ansi type device drivers that

you might prefer to use.

Refer to the documentation on QWIKSCRN to understand it's

uses and how to implement the program.  QWIKMODE can be invoked

at the command line, simply by typing QWIKMODE.  If used in

this manner the screen device driver will be toggled between

character mode and block mode each time you invoke QWIKMODE.

If you prefer you can turn block mode on or off by specifing

/ON or /OFF at the command line.

You may use the QWIK* series of programs however you

feel necessary and feel free to distribute them in whole,

unmodified form, for no charge.  I reserve the exclusive

rights to any and all revenues collected in the distribution

of these programs, in whole or part, by anyone; whether the

revenues were collected for these programs, the techniques

or media by which they were transferred, or any other

software or documentation that they may be distributed in

conjunction with.  Your distribution of these programs, in

whole or part, for the generation of revenue or in the

connection with the generation of revenue is your given

acknowledgement of might rights to those revenues.

Please direct any suggestions, criticisms, or monies to:

MJA Software

Michael J. Acker

4206-45th Street

Lubbock, Texas  79413

QWIK Ansi Escape Control Sequences

CUP         ecs[#;#H                    Cursor position

CUU         ecs[#A                      Cursor up

CUD         ecs[#B                      Cursor down

CUF         ecs[#C                      Cursor forward

CUB         ecs[#D                      Cursor backward

HVP         ecs[#;#f                    Horizontal and vertical position

DSR         ecs[6n                      Device status report

CPR         ecs[#;#R                    Cursor position report

SCP         ecs[s                       Save cursor position

RCP         ecs[u                       Restore cursor position

ED          ecs[2J                      Erase in display

EL          ecs[k                       Erase in display

SGR         ecs[#;...;#m                Set graphics rendition

SM          ecs[=#h  or esc[?#h         Set mode

RM          ecs[=#I  or esc[?#I         Reset mode

KKR         ecs[#;"string";#;#;...;#p   Keyboerd key reassignment

KKD         ecs[#;#;...;#d              Keyboard key assigment delete

IL          ecs[#L                      Insert lines at cursor line  (scroll dn)

DL          ecs[#M                      Delete lines at cursor line  (scroll up)

SWD         ecs[#;#;#;#;#W              Scroll window down (u_r,c l_r,c #_lines)

SWU         ecs[#;#;#;#;#V              Scroll window up   (u_r,c l_r,c #_lines)

TON         ecs[#z                      Tabs expansion on      (emulate DOS)

TOF         ecs[#y                      Tabs expansion off     (binary mode)

SAP         ecs[#a                      Select active display page

----------------end-of-author's-documentation---------------

This disk copy provided as a service of

The Public (Software) Library

Disks in the library are updated monthly.

For a copy of the latest monthly software library newsletter

and a list of the 700+ disks in the library, call or write

The Public (Software) Library

P.O.Box 35705, Rm. F

Houston, TX 77235-5705

(713) 721-6104

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions.