�������������������Ŀ

� ABOUT THE PROGRAM �

���������������������

The program MAPS.EXE converts latitude and longitude coordinates into an

AutoCAD script file format. The AutoCAD script file can be run within

AutoCAD to automatically draw a cylindrical equal-distance projection

map of any portion of the world.

The program is designed to used the data digitized by John B. Allison in

1986.

Any other Latitude and Longitude data could be used as long as it is in the

same ASCI format as Mr. Allison's.

Mr. Allison's data is in the the form of a Latitude coordinate and a Longitude

coordinate in decimal degrees in an ASCI format. The data is available on many

bulletin boards around the country. The data is the same as that used by the

program MAPVIEW 2.0

A file containing the digitized data for the coast of South and Central

America is included with this package as a demo. Data files for the entire

world are available.

Two known sources for the world coordinate data area the Southwest Lincoln

Wildcat BBS at 402-421-1963 or Northeast Lincoln Wildcat BBS at 402-467-2170.

The program MAPS.EXE reads the ASCI text file containing the Latitude and

Longitude coordinates. An AutoCAD script file is created that draws a

continuous line connecting all data points until a blank line is encountered

in the ASCI text file. When a blank line is encountered a new line is drawn

beginning with the next coordinate in the file.

���������������Ŀ

� INSTRUCTIONS  �

�����������������

The following is a list is steps to follow in creating an AutoCAD map of the

coast of Africa using the Africa coastline data.

1)  Enter the command "MAPS" at the DOS prompt.

2)  For the name of the map file enter "SA0" (.MP1 extension is assumed)

3)  For the output file name enter "SA0"

(The .SCR extension will be added)

4)  Watch as the Latitude and Longitude coordinates are converted to AutoCAD

X and Y drawing coordinates.

5)  When all data is converted exit the MAPS program.

6)  Begin a new AutoCAD drawing

7)  Set the limits of the drawing to be -200,-200 by 200,200

8)  Zoom ALL

9)  Enter the AutoCAD command "SCRIPT"

10) For the script file name enter "SA0"

(The script file must be in the same directory)

11) AutoCAD will draw the coast of South and Central America

on the current layer. The AutoCAD coordinates for the map

will match the actual Latitude and Longitude coordinates

in the real world.

Plot additional maps within the same AutoCAD drawing to produce and entire map

of the world or any portions you desire. Enhance the maps with text, grid

lines etc. as you wish.

This conversion program is being distributed in the public domain for

AutoCAD users who might want to generate some detailed world maps easily,

using some accurate available digitized coordinate data.

Any comments or questions are welcome.

Direct to: ��������������������Ŀ

� TDM Software       �

� Terry Majewski     �

� 3620 L Street      �

� Lincoln, Ne. 68510 �

����������������������