���������������������������������������������������������������Ŀ

�            Copyright (c) 1991 by Bruce W. Colletti            �

�                                                               �

� You may copy and distribute intact the contents of this disk. �

� Monetary contributions for PDNNET are not solicited.          �

� Although PDNNET may not be sold for profit, fees may be       �

� charged to cover minimal distribution costs such as disks,    �

� postage, etc.                                                 �

�                                                               �

� The author does not guarantee the accuracy of the code and    �

� documentation, but does believe that both are correct.        �

� PDNNET is used at one's own risk.  Users may modify the       �

� source code to suit a nonprofit need, and may not "alter and  �

� distribute" this document file (MANUAL.DOC) without the       �

� author's permission.                                          �

�����������������������������������������������������������������

Public Domain Neural Network 1.1

User's Manual

�������������

This is the user's manual to PDNNET, a backpropagation neural network

development package.  PDNNET was created in Winter 1990 to stimulate interest

in and understanding of neural networks.  The distribution package includes the

PDNNET C source code so that users may customize PDNNET to address specific

needs.

Once neural nets are better understood, one may choose from among several more

complete commercial neural network packages.  These are advertised in such

magazines as "AI Expert" and "PC AI", have prices starting near $100, and may

implement other types of neural networks besides backpropagation.

Those who find PDNNET useful are welcome to join--at no cost--the PDNNET User's

Group found on the ShadeTree BBS (412-244-9416 2400N81, Pittsburgh PA  USA;

Bill Keller, SysOp; shade.tree@f124.n129.z1.fidonet.org).  ShadeTree is a forum

for anyone interested in artificial intelligence applications and has many

files that are available for downloading.  ShadeTree is also a node on the

FIDONET and carries the NEURAL_NET and AI Echoes.

This package consists of the files:

- PDNNET.EXE (executable code)

- PDNNET.C (TURBO C 2.0 source code)

- MANUAL.DOC (this document)

- TRAIN.DAT, EXAMPLE.DAT (sample training sets)

- PROPGAT1.DAT, PROPGAT2.DAT (sample propagation sets)

- OUT.DAT, OUT2.DAT (results from propagation sessions)

- ARCH.DAT,EXAMPLE1.WTS, EXAMPLE2.WTS, EXAMPLE3.WTS (files representing

different network architectures)

AUTHOR:  Bruce W. Colletti

03 DEC 92

Richmond VA  USA (until 31JAN93), Alexandria VA (01MAR93 onwards)

Internet     :  71121.1452@compuserve.com

CompuServe   :  71121,1452

GEnie        :  b.colletti

ShadeTree BBS:  412-244-9416 2400N81 (Pittsburgh PA, USA)

************

* CONTENTS *

************

I  Overview of the Backpropagation Neural Network

II  Description of the Public Domain Neural Network (PDNNET)

III  Training a Neural Net

IV  Propagating an Input Set

V  Example Application

VI  Creating Non-Standard Transfer Functions

Appendix A  The Mathematical Development of Backpropagation

Appendix B  Bibliography and Neural Network Companies

Appendix C  Neural Network Information Resources

**************************************************

I.  Overview of the Backpropagation Neural Network

**************************************************

1.  There are many types of neural networks and the one discussed here is the

popular backpropagation neural net. Simply put, this type of neural network

tries to fit a curve using observed data.  A statistical analogy is that of

performing a regression to link independent and dependent variables.  Once a

neural network has adequately fit the data, the net is used to compute output

values for inputs not seen before.

2.  The data which the neural net analyzes is called the "training set" and

consists of input/output values which the user knows to be correct.  It is from

this training set that the neural network will try to generalize a mathematical

relationship.  For example, the training set

independent      dependent

----INPUT----    --OUTPUT--

0.5  0.2  0.1    .32    0.9

0.4  0.6   20    0.9    0.7

may reflect experimental results:  settings on three environmental input

parameters result in the observed temperatures at two locations.  A "training

pair" is defined to be a particular input/output pair, e.g., (.5,.2,.1) paired

with (.32,.9) above.

3.  How does a backpropagation neural net look?  It looks like a transportation

network.  In the latter, cities in each state have roads to all cities in

border states, and each road has its own transportation cost.  Likewise, in the

neural net, neurons in each layer have synaptic connections to all neurons in

the layers above and below, and each synapse has its own weight.  As a

transportation strategy may be found by altering costs, a neural net adjusts

weights until the data is acceptably fitted.

4.  A user determines the number of layers and neurons within layer.

Guidelines suggest that without enough of either, the data may be ill-fitted (a

statistical analogy is omitting an important variable).  Too many of either may

result in memorization and not generalization (an analogy is memorizing tables

rather than learning to add).  How a neural net is configured permits it to

take a more general approach to data fitting than would a statistical method

which assumes linearity, normality, etc.  Unfortunately, this generality can be

offset by the neural net's increased running time (if it finishes at all).

5.  The neural net's strategy of data fitting is to change weights according to

"controlled brute force":

- controlled because weights change with the mathematical rationing of

error throughout the network

- brute force because the training set keeps getting re-read until all

training pair errors are acceptably small

The process of calculating an error and then adjusting the weights is called

"training the network."  Details of the error rationing quickly become

technical and so are omitted here.  The interested reader is referred to the

bibliography, or if familiar with partial differential calculus and matrix

algebra, to Appendix A where the mathematical development of backpropagation is

presented.

6.  Let's recap details thus far.  In a neural network:

- there are several layers

- each layer has several neurons

- each neuron is connected to all neurons in the layers just above and

below

- each connection has a number assigned to it called a "weight" that is

initially a random number

The neural net trains itself by:

- reading a training pair from the training set

- sending the pair through the network (forward propagating)

- computing an error

- adjusting all weights by rationing the error throughout the network

(backpropagating)

This process repeats itself with the next training pair and continues until all

errors are small enough (even if this means re-reading the training set).  Once

done, the network is "trained" and the existing weights represent the neural

network's "knowledge."  A statistical analogy is that the coefficients of

regression have been computed.

7.  It remains to describe how a training pair is processed and the error

computed.  When a training pair is selected, each value in the input member

becomes the "output signal" of a first layer neuron.  This initializes the

processing step.  For example, in paragraph 2, the input member of the first

training pair is (0.5, 0.2, 0.1).  Thus, the output signal of the 1st neuron in

the first layer is 0.5, the signal of the 2nd neuron is 0.2, and the signal of

the 3rd neuron is 0.1.

8.  EACH neuron in the next layer receives an "input signal" from the previous

layer.  This signal is the weighted sum of the:

- neuron's connection weights to EACH previous layer neuron

- output signals of each neuron in the previous layer

9.  This input signal goes through a "transfer function" whose output signal is

between zero and one.  The commonly used transfer function is:

OutputSignal = 1 / [1 + exp(-InputSignal)]

After all the neurons within a layer have had their input and output signals

computed, attention advances to the next layer where the actions of paragraph 8

are repeated.

10.  Eventually, the last layer is reached, and the output signals of its

neurons are subtracted from what they should've been (given by the output

member of the training pair).  If any difference is too large, then network

weights are adjusted based on the computed errors.  This adjustment is quite

technical and is described in Appendix A.

11.  After all weights have been adjusted, the next training pair is selected

and the process repeats from paragraph 7.  Once all errors (for every training

pair) are within tolerance, training ceases and the network is considered

"trained."  To use the neural network, enter an input for which the correct

output is unknown.  The computed output is the neural net's estimate for the

true output.

*************************************************************

II.  Description of the Public Domain Neural Network (PDNNET)

*************************************************************

12.  PDNNET trains a neural network using backpropagation without momentum and

will use a trained network for an application.  When training a neural network,

the user specifies:

- the training set (an ASCII file)

- how many layers the network has

- the number of neurons within each layer

- the learning rate (not discussed yet)

- the error tolerance

- the random number seed initializing network weights

Training may be interrupted to monitor learning, change parameters, or save the

current network state.  By saving the network's state, users can resume

training later on.

13.  When using a trained network in an application, the user specifies the

file:

- which defines the neural network (saved during a previous training

session)

- which contains the observations whose outputs need to be estimated

- to write these inputs and their calculated outputs

14.  To run the program, enter PDNNET.  Up comes the menu:

(T)rain Neural Network

(P)ropagate observations

(Q)uit

Training the network is like computing regression coefficients:  the user

provides the raw data from which computations proceed.  The second option uses

a trained network to estimate outputs.  A statistical analogy is that the

regression coefficients have been computed and one plugs in new values to see

what pops out.  The next two chapters address these propagation processes.

***************************

III.  Training a Neural Net

***************************

15.  Training requires that a network's architecture be known.  Users can start

from scratch or use one that already exists.  Thus the menu:

(M)anually create architecture

(R)ead architecture/weights from external file

(Q)uit to main menu

16.  Users who choose the first option will be asked for the number of layers

in the network and the number of neurons in each layer (see paragraphs 3 and 4;

there are no specific rules to choose an architecture).  In a four layer

network, each layer consisting of 3, 12, 5 and 2 neurons respectively, the

prompts would be:

Number of Layers �4

Number of entries in the input member of an I/O training pair�3

Number of neurons in hidden layer 1�12

Number of neurons in hidden layer 2�5

Number of entries in the output member of an I/O training pair�2

Note that the number of entries in the first and last layers have the same

sizes as the I/O members of the training set (for those familiar with neural

nets, PDNNET automatically creates a bias node in the input layer).  Next,

synaptic weights are assigned from among random numbers between -1 and 1.

Users will be asked for the random number seed starting this process:

Random # seed used to initially establish synaptic weights�7

The neural network architecture has now been manually created.

17.  To re-use a previously saved architecture (not yet described), the user

would choose the (R) menu option.  A prompt for the ASCII file describing the

architecture will appear:

File containing architecture/weights �EXAMPLE1.WTS

The network's architecture will be re-established, and previous training can

resume where it left off.

18.  After the architecture has been established, values are assigned to the

"learning rate" and to the "error tolerance."  The former affects how quickly

the changing weights settle down.  The latter will halt training when the

computed and correct outputs are close enough.  The prompts are:

Learning rate   (default is 0.700000)�.6

Error tolerance > 0 (default is 0.000000)�.2

Default values exist if the architecture was established from a previously

saved network.

19.  The learning rate can be any real number and is often chosen between zero

and one.  A rule of thumb is to start large (.8+) and lower the learning rate

throughout the training.

20.  Here's an example of the role of the error tolerance.  Choose an error

tolerance of .20 and suppose that a training I/O pair is

(.1, .7, -.2) & (.3, .1)

i.e., the correct output for (.1, .7, -.2) is (.3, .1).  Assume that the neural

net computes an output of (.27, .5). Comparing these outputs, we find that

although the first ordinates are within .2 of one another, the second ordinates

aren't.  Thus, the neural net will adjust its synaptic weights based on the

error (.03, -.4) and then grab the next I/O training pair.  If the errors are

all within tolerance, network weights aren't adjusted and training skips to the

next training pair.

21.  Once the architecture, learning rate and error tolerance are set, it

remains to establish the training set:

Training Set�train.dat

Number of observations in training set�2

A training set is an ASCII file, e.g.:

.5  .2  .1     .32  .9

.4  .6  20     .9   .7

The first three columns are inputs and the last two are outputs For example, an

I/O training pair is (.5, .2, .1) & (.32, .9).

22.  WARNING.  In the distribution version of PDNNET, output values must fall

between zero and one.  This is due to the mathematics of the transfer function

(paragraph 9).  However, users can relax this restriction by defining their own

transfer functions (Chapter VI).  In general, output values don't have to be

between zero and one for backpropagation neural networks.  It's just that with

the distribution version of PDNNET this should be observed (but isn't checked

for).  As an aside for the mathematically minded, the bibliographic reference

to Kolmogorov's work addresses input tuples whose ordinates lie between zero

and one.

23.  Training now begins and this menu appears:

****** Network is currently training ******

B change learning rate

E change error tolerance

W save weights to file

Q abort and exit

A abort to main menu

Any other key gives status of network

*******************************************

Training is interrupted by hitting a key.  The (W) option permits the user to

save the current architecture, weights, and learning rate of the neural

network.  Saving the network permits one to resume later training (paragraph

17).  The layout of a saved network file is the "@" character, learning rate, #

of layers, # of nodes within each layer, and then tables of the weights between

consecutive layers (1-2, 2-3, etc.).  ARCH.DAT is an example of a saved network

file.

24.  Hitting other keys displays the network status:

LastCycleMAXERR=0.247033 cycle=297 �=0.900000 �=0.200000

Continue?

This says that in the last pass through the training set (pass #297), the

largest output error was .247033 obtained with a current learning rate of .9.

The current error tolerance is .20.  To change a factor, hit the appropriate

key found in the menu.  As an aside, the learning rate will probably be reset

often during training.

25.  SUMMARY:  Here are the steps in the training process:

- create the architecture

- initialize weights with random numbers between -1 and 1

- provide the learning rate

- provide the error tolerance

- identify the training set

- monitor training by "peeking in" (hit a key)

26.  When the network detects that all computed outputs are within tolerance,

training ceases and this message appears:

***** Training Completed in XX cycles *****

File to save weights (return if no save)�ARCH.DAT

Continue training?

where XX gives the number of complete passes through the training set that

occurred during training.  In this example, the user has saved the "trained"

network to the file ARCH.DAT.  Training can resume by hitting 'y'.

*****************************

IV.  Propagating an Input Set

*****************************

27.  A time will come when a trained network has been saved and new data is

obtained for which outputs need to be found.  The (P) option in paragraph 14

will accomplish this by printing each observation and its computed output to an

ASCII file.

28.  For example, suppose a trained network has been saved to the file ARCH.DAT

(paragraph 26).  A new survey is taken and its observations stored in

PROPGAT1.DAT:

7.2   -4.2   0.9

0.1    5.5   6.2

The question now is "what should the outputs be for the observations (7.2, -

4.2, 0.9) and (.1, 5.5, 6.2)?"  The answers are found by choosing the (P)

option whose prompts will be:

File containing architecture/weights �a:arch.dat

File containing observations to be propagated �a:propgat1.dat

File to write results of propagation �out.dat

29.  The resulting output file OUT.DAT would be:

3 2

7.200000 -4.200000 0.900000          0.660900 0.707811

0.100000  5.500000 6.200000          0.558043 0.643487

That is, the estimated output for (7.2, -4.2, .9) is (.66,.7) and the estimated

output for (.1, 5.5, 6.2) is (.558, .643).  (NOTE:  the 3 and 2 on the first

line indicates that the input member has three values and the output member has

two.  These dimensions may be needed in another application, e.g., reading

OUT.DAT into a spreadsheet, or into a FORTRAN program that uses the dimensions

to allocate arrays for the input and output values.

***********************

V.  Example Application

***********************

30.  Consider the challenge of transforming a rectangle into the surface of a

cone.  This is a mapping from two dimensions (the rectangle) into three

dimensions (the cone).  A neural network will be trained to answer the

question, "Given a point in the rectangle, what is its corresponding image on

the cone?"

31.  The rectangle has opposite vertices at (0,0) and (1,2pi), and the cone

opens upward with vertex at (.5,.5,0) and height of one unit.  The

transformation which maps the rectangle into the cone is defined by:

f(u,v) = (.5*[1+u*cos(v)], .5*[1+u*sin(v)], u)

For example, f(.2,.3) = (.5*[1+.2cos(.3)], .5*[1+.2sin(.3)], .2).

32.  A neural network will be trained to estimate this mapping given the

twenty-five observations in the file EXAMPLE.DAT.  Each observation consists of

a point in the rectangle and its corresponding image on the cone.  For example,

the first two observations in the training set are:

--- rectangle---     --------- cone ----------

0.01056  0.02380     0.50528  0.50013  0.01056

0.33515  0.19959     0.66425  0.53323  0.33515

The point (.01056, .02380) in the rectangle has the point

(.50528, .50013, .01056)

as its conic image.

NOTE:  reread paragraph 22 and observe that each ordinate of a conic point lies

between zero and one.  If one uses the standard transfer function in paragraph

9 (as does the distribution version of PDNNET), then EVERY output value in the

training set MUST be between zero and one.  Otherwise, users must either

rescale the output values (thus creating a new training set while leaving the

code unchanged) or establish non-standard transfer functions, as discussed in

Chapter VI (thus leaving the training set intact while modifying and

recompiling PDNNET).

33.  The first requirement of paragraph 12 has now been met:  the training set

has been identified.  The next step establishes the network architecture, bound

only by the restrictions that the input layer have two neurons and the output

layer has three (because the mapping is from two dimensions into three

dimensions).  Let the network consist of 4 layers with two, five, five, and

three neurons in each layer respectively.  Set the learning rate to .8 and

error tolerance to .3.  The random number seed of 14 will be used to initialize

all synaptic weights.

34.  Per paragraph 14, enter the command PDNNET.  Respond to subsequent prompts

(Chapter III) with the entries:

T/M/4/2/5/5/3/14/.8/.3/example.dat/25

�������������������������������������������

(NOTE:  depending on where PDNNET is installed, you may have to provide the

training set's complete pathname).  Training will begin, and this menu will

appear:

****** Network is currently training ******

B change learning rate

E change error tolerance

W save weights to file

Q abort and exit

A abort to main menu

Any other key gives status of network

*******************************************

35.  After waiting a few moments, interrupt training by hitting the space bar.

This message may appear:

LastCycleMAXERR=0.507911 cycle=15 �=0.800000 �=0.300000

Continue?y

What this means is that when training was interrupted, the neural network had

read the training set EXAMPLE.DAT fifteen times, and that with a learning rate

of .8, the current maximum error is .507911.  In other words, the largest

absolute value (in cycle #15) for a difference between a calculated output and

its correct value was .507911 (see paragraph 20).  Since this exceeds the error

tolerance of .3, the user is asked if training should continue.

36.  Subsequent training interruptions may produce the sequence:

LastCycleMAXERR=0.453133 cycle=39  �=0.800000 �=0.300000

LastCycleMAXERR=0.428176 cycle=68  �=0.800000 �=0.300000

LastCycleMAXERR=0.427601 cycle=90  �=0.800000 �=0.300000

LastCycleMAXERR=0.358746 cycle=117 �=0.800000 �=0.300000

LastCycleMAXERR=0.340847 cycle=151 �=0.800000 �=0.300000

LastCycleMAXERR=0.332438 cycle=193 �=0.800000 �=0.300000

LastCycleMAXERR=0.334492 cycle=210 �=0.800000 �=0.300000

Although further training generally leads to lower errors, error can increase,

as it does from cycle 193 to 210.  If errors keep growing, the user can:

- interrupt training, change the learning rate, resume training, and "see

what happens"

- abort training and restart using the same architecture but use a

different random number seed

- abort training and try an entirely new architecture and totally

different parameters

37.  Fortunately for this example, the error resumes its decline and eventually

this message appears:

***** Training Completed in 232 cycles *****

Thus, the absolute value of the difference between each calculated output and

its correct value is less than or equal to the error tolerance of .30:  the

mapping has been estimated within tolerance.

38.  Suppose that the user wants to save the current network so that training

can later resume to achieve a .25 error tolerance.  Saving the trained network

is accomplished by giving a filename to the following prompt:

File to save weights (return if no save)�example1.wts

Continue training?n

Paragraph 23 describes the layout of this saved network file.  NOTE:  in

EXAMPLE1.WTS, the number of neurons in the input layer is three and not two (as

might be believed because the rectangle is a two dimensional object).  This is

because PDNNET automatically adds a bias node to the input layer (paragraph

16).

39.  What follows is a session which resumes training with an error tolerance

of .25:

C:\>pdnnet

(T)rain Neural Network

(P)ropagate observations

(Q)uit

T

(M)anually create architecture

(R)ead architecture/weights from external file

(Q)uit to main menu

R

File containing architecture/weights �example1.wts

Learning rate   (default is 0.800000)� (return to use .8)

Error tolerance > 0 (default is 0.000000)�.25

Training Set�example.dat

Number of observations in training set�25

****** Network is currently training ******

B change learning rate

E change error tolerance

W save weights to file

Q abort and exit

A abort to main menu

Any other key gives status of network

*******************************************

***** Training Completed in 206 cycles *****

File to save weights (return if no save)�example2.wts

40.  Thus, after restoring the previous network (EXAMPLE1.WTS) and using the

old learning rate of .8, training ran another 206 cycles to achieve an error

tolerance of .25.  The resulting "better trained" network was saved in the file

EXAMPLE2.WTS for future use.  Training may have been quicker if the learning

rate had been set to a value less than .8.

41.  Here's an interesting side note:  if the above application ran from

scratch (using the parameters of paragraph 34) with a tolerance of .25, and if

the learning rate remained unchanged throughout the training session, then the

session would've lasted 467 cycles (EXAMPLE3.WTS) as opposed to the combined

438 cycles of paragraphs 37 and 39.  This demonstrates that there are many ways

to train a network, that interim errors don't always decrease, and the manner

in which parameters are reset throughout training may dramatically affect the

length of the training session.

42.  Finally, suppose the trained network given by EXAMPLE2.WTS represents the

conclusion of the entire training effort.  To estimate the conic images for the

new points:

0.02112  5.98444

0.66634  4.38862

0.67818  0.47206

this session of PDNNET would proceed:

C:\>pdnnet

(T)rain Neural Network

(P)ropagate observations

(Q)uit

P

File containing architecture/weights �example2.wts

File containing observations to be propagated �propgat2.dat

File to write results of propagation �out2.dat

43.  OUT2.DAT contains the estimates of the conic images of these points:

-Rectangle Input-          ---Conic Image Estimates--

2 3

0.021120 5.984440          0.344757 0.247143 0.316217

0.666340 4.388620          0.488448 0.303899 0.645308

0.678180 0.472060          0.589334 0.725256 0.696703

where the first line reflects that inputs have two values (a rectangle's

coordinates) and outputs have three (a conic point in 3-space).  The correct

values achieved under the transformation of paragraph 31 are:

0.02112  5.98444           0.51009  0.49689  0.02112

0.66634  4.38862           0.39401  0.18414  0.66634

0.67818  0.47206           0.80201  0.65419  0.67818

To achieve better estimates, training would continue with tighter error

tolerances and the estimates recomputed.  However, it's possible that a given

tolerance may be unreachable with the current network architecture and the

initial random assignment of weights.

********************************************

VI  Creating Non-Standard Transfer Functions

********************************************

44.  PDNNET can employ user-defined transfer functions if the user has a C

compiler.  PDNNET.C is the TURBO C 2.0 source code for PDNNET, and this chapter

shows how to modify the code to employ different transfer functions.

45.  First, a summary.  Recall that each neuron has a transfer function.  The

function changes an input signal from the previous layer into an output signal

which is sent to the next layer.  Usually, each neuron uses the transfer

function of paragraph 9.  However, different transfer functions may be needed

because of some known property about the phenomena under study.  For example,

it may be known that signals assume only two values, thus suggesting the use of

a step function.

46.  By changing each of three areas of PDNNET.C, users are free to assign

transfer functions, even if neuron-unique.  These procedures are the first

found in PDNNET.C and follow the initial program documentation and

procedure/variable declarations.

47.  THE FIRST AREA establishes the functional form for a transfer function:

/*********************************************************/

/* FIRST  USER INPUT:                                    */

/*                                                       */

/*             Define Transfer Functions Here            */

/*********************************************************/

xtype sigmoid00(xtype x) {return(1.0);}

xtype sigmoid01(xtype x) {return(1.0 / (1.0 + exp(-x)));}

48.  In this segment, two transfer functions are created:

- sigmoid00 always returns the value 1

- sigmoid01 returns a value per paragraph 9

The sigmoidXX convention doesn't have to be used.

49.  Now suppose you instead want these functions:

- sigmoid00(x) = sin(x) + cos(x)

- sigmoid01(x) = 4*ln(x) + 7

- the step function

sigmoid02(x) = 4  x <= 9

7  x  > 9

The resulting code would read:

xtype sigmoid00(xtype x) {return(sin(x) + cos(x));}

xtype sigmoid01(xtype x) {return(4*log(x)+7);}

xtype sigmoid02(xtype x)

{if (x <= 9) return(4);

else

return(7);

}

50.  THE SECOND AREA establishes the derivatives of each transfer function.

Backpropagation uses these to adjust network weights, and so you'll need to

either dust off those calculus books or find someone to find the derivatives:

- derivative of sigmoid00 is sigmoid00D = cos(x) - sin(x)

- derivative of sigmoid01 is sigmoid01D = 4/x

- derivative of sigmoid02 is sigmoid02D = 0

The modified code would be:

/***************************************************/

/* SECOND USER INPUT                               */

/*                                                 */

/*      Define Transfer Function Derivatives Here  */

/***************************************************/

xtype sigmoid00D(xtype x) {return(cos(x)-sin(x));}

xtype sigmoid01D(xtype x) {return(4/x);}

xtype sigmoid02D(xtype x) {return(0);}

51.  THE THIRD AREA assigns the transfer functions and their derivatives to

each neuron.  In this procedure, the user MUST use the predefined variables

"transfer" and "derivative" because the rest of the code assumes these:

- transfer[i][j] is the transfer function for neuron j in layer i

- derivative[i][j] is the derivative of transfer[i][j]

52.  The procedure below assigns the above transfer functions and their

derivatives to the neurons in a neural network.  This neural net has N layers

where the number of neurons in layer i is given by the variable

nodes_in_layer[i].  Also, all neurons:

- in layer 1 use the sigmoid00 functions

- in layer N use the sigmoid03 functions

- in the middle layers use the sigmoid02 functions

Also recall that C arrays are zero relative:  although the layers run from 1 to

N, their respective C array indices run from 0 to N-1.

/****************************************************/

/* LAST USER INPUT                                  */

/*                                                  */

/*    establish transfer functions and derivatives: */

/****************************************************/

void create_transfers_derivatives(void)

{int i,j,k;

for(j=0; j<nodes_in_layer[0]; j++)

{  transfer[0][j] = sigmoid00;

derivative[0][j] = sigmoid00D;

}

for(i=1; i<N-1; i++)

{for(j=0; j<nodes_in_layer[i]; j++)

{  transfer[i][j] = sigmoid02;

derivative[i][j] = sigmoid02D;

}}

for(j=0; j<nodes_in_layer[N-1]; j++)

{  transfer[N-1][j] = sigmoid03;

derivative[N-1][j] = sigmoid03D;

}

return;

}

53.  Once these changes to PDNNET.C have been made, the user compiles the new

source code to create a new executable version of PDNNET.  As stated in the

program's documentation, PDNNET.C was written in TURBO C 2.0 and compiled in

compact mode on an 80286-12 w/math chip running under MS-DOS 3.3.

APPENDIX A

The Mathematical Development of Backpropagation

(PDNNET by Bruce Colletti, 71121.1452@compuserve.com)

(ShadeTree BBS, 412-244-9416 2400N81)

�����������������������������������������������������

This appendix presents the mathematical development of the backpropagation

algorithm, first developed by Dr. Paul Werbos in his doctoral dissertation.

The reader is assumed to be familiar with elementary matrix algebra and partial

differential calculus.  After briefly sketching backpropagation neural nets,

this appendix will state some basic theorems, establish notation, and then

conclude with the mathematical development of backpropagation.

First, some preliminary comments.  This appendix was revised in November 1992

following discussion on the NEURAL_NET Echo, a FIDONET electronic discussion

forum (see Appendix C).  A forum member had correctly pointed out that the

grueling math found in the appendix ended with the last equation mistakenly

dropping a minus sign.  The author responded by reviewing this appendix for

accuracy (last done in March 1991) and found this dropped sign to be the only

mistake.  However, the author also found it difficult to follow his own writing

and so rewrote the appendix into this (hopefully) more understandable form.

�����������������������������������������������Ŀ

� BACKPROPAGATION NEURAL NETWORKS IN A NUTSHELL �

�������������������������������������������������

The backpropagation neural net is one the best known members of the large

neural net (NNet) family.  A darling of the popular press, it is often drawn to

show what a NNet looks like.

A backpropagation NNet consists of many stacked "layers" which successively

refine calculations.  Input data is entered at the first "input" layer and the

observed results are seen at the last "output" layer.  The in-between "hidden"

layers mathematically contort the input data in a way whose end result is seen

at the output layer.  Simply put, data is passed from layer 1 to layer 2.

Layer 2 refines the data and passes its results to layer 3.  Likewise, layer 3

refines the data it receives and passes its results to layer 4.  The process

ends at the output layer where no more such refinements occur.

Each layer consists of many little processors called "neurons" and each neuron

is connected to all the neurons in the two layers immediately beneath and above

it.  Each connection is called a "synapse" and has a numeric weight assigned to

it.  A neuron operates by first doing a weighted sum of its inputs.  It then

uses a "transfer function" to mathematically transform this sum into another

number which it transmits to all the neurons in the layer above it.

A NNet's initial weights are simply random numbers which will change during

training.  This consists of presenting actual experimental data (i.e.,

observations) to the NNet and using a mathematical algorithm--the

backpropagation algorithm--to adjust the weights.  Adjustments are based on the

difference between the correct and computed outputs.  Once each observation's

computed and actual outputs are close enough, training stops and the NNet is

ready for use:  give it a new input observation and it will estimate what the

corresponding output values should be.

����������������������Ŀ

� FUNDAMENTAL THEOREMS �

������������������������

The mathematics of backpropagation will rely on the following theorems.  First,

though, some notation:

- the partial derivative of f with respect to x is denoted by Df/Dx.

- the matrix with entries x(i,j) is represented by [x(i,j)].

THEOREM1 (From "Elementary Numerical Analysis", 3rd Edition; Conte, deBoor;

McGraw Hill; 1980; ISBN 0-07-012447-7; pgs 209-210):

Let f be differentiable in Euclidean n-space.  Then the direction of greatest

decrease in the value of f at the point P is in the direction of -[Df/Dx(i)]

evaluated at P, where [Df/Dx(i)] is the gradient of f.

THEOREM2 (From "Advanced Calculus", 2nd Edition; Buck; McGraw Hill; 1965; pg

250):

Let w = f(s,v), s = g(x,y), v = h(x,y), g and h differentiable at the point P

and w differentiable at (g(P), h(P)).  Then w = f(g(x,y),h(x,y)) = F(x,y) is

differentiable in a neighborhood of P and in this neighborhood,

Dw/Dx = (Dw/Ds)(Ds/Dx) + (Dw/Dv)(Dv/Dx)

����������������������������Ŀ

� DISCUSSION ON THE THEOREMS �

������������������������������

1.  Theorem 2 offers an observation that is critical to the development of

backpropagation.  In the diagram below, arrows flow from s and v into w

because--according to Theorem 2--w is a function of s and v.  Likewise, arrows

flow from x and y into s because s is a function of x and y.  Arrows likewise

flow into v.

s <--- x

/ \    /

/   \  /

w <-     \/         (flow is right to left)

\    /\

\  /  \

v <--- y

(This flow diagram looks like a neural network, doesn't it?  The input layer is

x and y, the hidden layer is s and v, and the output layer is w).

2.  Now look carefully at the conclusion of Theorem 2 and the above diagram.

How is Dw/Dx calculated?  Theorem 2 says:

Dw/Dx = (Dw/Ds)(Ds/Dx) + (Dw/Dv)(Dv/Dx)

Now, what are all the paths from w to x (since we're computing Dw/Dx) going

AGAINST the flow?  There are two:

w  ->  s  ->  x

w  ->  v  ->  x

How many summands are there in the computation of Dw/Dx?  There are two (just

like the number of paths from w to x):  (Dw/Ds)(Ds/Dx) and (Dw/Dv)(Dv/Dx).

3.  Compare the first flow and the first summand:

w ->  s  ->  x           (flow)

�     �\     �

�     � \    �

�     �  \   �

(Dw /  Ds)(Ds/Dx)         (summand)

See the connection?  There is a relationship between the flow diagram and the

corresponding summand!  The relationship also holds between the second flow and

summand.  In fact, it can be proven (but not here) that a similar pattern holds

for longer paths!  This observation is key to unravelling the mathematics of

backpropagation, as will be demonstrated.

4.  Let's summarize how to find the partial derivative of a function with

respect to one of its variables:

- build a "flow diagram"

- find all the paths "against the flow" from the function to the variable

that the derivative is with respect to

- re-express each path in its functional form

- add the summands to obtain the sought-after partial derivative

5.  Here's another example.  Suppose that a path in a hypothetical diagram is:

w -> a -> b -> x

The corresponding summand would be

(Dw/Da)(Da/Db)(Db/Dx)

If another path from w to x is

w -> c -> x

then the second summand would be

(Dw/Dc)(Dc/Dx).

Finally, if the last path from w to x is

w -> p -> x

then the third summand would be

(Dw/Dp)(Dp/Dx)

Add the three summands to obtain

Dw/Dx = (Dw/Da)(Da/Db)(Db/Dx) + (Dw/Dc)(Dc/Dx) + (Dw/Dp)(Dp/Dx)

6.  If the reader follows this discussion about flow diagrams and partial

derivatives, then backpropagation has been licked.  As will be shown below, the

mathematical development of backprop is simply an extension of the above

discussion.

�������������������������Ŀ

� NEURAL NETWORK NOTATION �

���������������������������

7.  Before proceeding, define network variables:

- N is the number of layers in the neural network

- Layer i has n(i) neurons

- w(L,i,j) is the synaptic weight between neuron i in layer L+1 and neuron

j in layer L < N

- W(L) = [ w(L,i,j) ] is the n(L+1) x n(L) matrix of weights between

layers L and L+1

- net(L,i) is the net input into neuron i in layer L > 1

- NET(L) = [ net(L,i) ] the column vector of net inputs to Layer L neurons

- x(L,i) is the output of neuron i in Layer L

- X(L) = [ x(L,i) ] is the column vector of outputs from Layer L neurons

- f(L,i) is the differentiable transfer function of node i in layer L

-- net(L,i) = SUM  w(L-1,i,j) * x(L-1,j)

j

-- x(L,i) = f(L,i)(net(L,i))

- F(L) = [ f(L,i) ] is the column vector of Layer L transfer functions

- the training set consists of column vector pairs {X(1),V}, where

the X(1)'s have n(1) rows and the V's have n(N) rows

8.  Furthermore, define these operations:

- F(L)(X(L)) is defined to be the matrix [ f(L,i)(x(L,i)) ]

- if A = [ a(i,j) ] and B = [ b(i,j) ] are two matrices with the same

dimensions, then A @ B is the matrix obtained by pairwise multiplication

of elements in A and B, i.e.,

A @ B = [ a(i,j)*b(i,j) ]

���������������������Ŀ

� FORWARD PROPAGATION �

�����������������������

9.  Now to begin the mathematical development of backpropagation.  What follows

is an extension on what is found in the text "Adaptive Pattern Recognition and

Neural Networks" by Yoh-Han Pao (Addison-Wesley; 1989; ISBN 0-201-12584-6; pp

120-129).  The mathematics presented thus far, as well as aspects of what

follows, were what I needed to understand Dr. Pao's presentation.

10.  Recall that the training process requies taking an input/output (I/O)

pair, entering its input component at layer 1, and then "forward propagating"

the refinements of each layer to the next layer.  Thus, if the training I/O

pair is (X(1),V), then forward-propagation implies that for all L > 1:

NET(L) = W(L-1)*X(L-1)                       [1]

and

X(L) = F(L)(NET(L))                         [2]

11.  After forward propagation is done, the observed output is found in X(N).

The next step is to compute the error vector V - X(N).  If the absolute value

of EACH component is less than some predefined positive tolerance epsilon, then

simply grab the next I/O training pair and repeat forward propagation anew.  If

all observations are within tolerance, then the NNet has been trained and is

ready for use.

12. However, if the error vector V - X(N) has a component that busts the

tolerance, then all the weights in the NNet must be adjusted.  This is done by

the backpropagation algorithm developed below.

�����������������Ŀ

� BACKPROPAGATION �

�������������������

13.  Let the components of V be v(i), i = 1..n(N).  Define the error function

to be:

e = .5 * SUM  (v(i) - x(N,i))^2                   [3]

i

where x^2 means "x squared", e.g., 3^2 = 3*3 = 9, 5^2 = 5*5 = 25.

14.  We now aim to decrease e by changing all of the network's synaptic

weights.  For the moment, let's focus on the weights connecting layers L and

L+1, i.e., the matrix W(L).  According to Theorem 1, e will be reduced by

adjusting the existing value of W(L) in the opposite direction of the gradient

of e with respect to these weights.  This gradient is given by:

G(L) = [ De / Dw(L,i,j) ]                      [4]

and when found, W(L) will be adjusted according to the formula:

W(L) = W(L) - beta*G(L)                       [5]

where beta is a positive scalar called the "learning rate."  In practice, the

user may have to continually modify beta during the training session.

15.  With some thought, the structure of the NNet can be seen--with one

exception--to be represented in this "Monster Diagram", so named because it's

large, complicated, and gives rise to the mathematical nightmare that follows.

The exception is that while the x(N,i) variables on the left would normally be

the output layer, we will let the error function e be the output layer for

notational convenience.

x(N,1) <-- net(N,1) <-- x(N-1,1) <-- net(N-1,1) <-- x(N-2,1) ... <-- net(L+2,1) <-- x(L+1,1) <-- net(L+1,1)

/                \       /                  \       /                      \       /   .          .            w(L,j,1)

/    .          .  \     /    .           .   \     /     .              .   \     /    .          .          /

/     .          .   \   /     .           .    \   /      .              .    \   /     .          .         /        .

/      .          .    \ /      .           .     \ /       .              .     \ /                          /         .

e <-       .          .     /       .           .      /        .              .      \     x(L+1,j) <-- net(L+1,j)         .

\      .          .    / \      .           .     / \       .              .     / \      .          .        \         .

\     .          .   /   \     .           .    /   \      .              .    /   \     .          .         \        .

\    .          .  /     \    .           .   /     \     .              .   /     \    .          .          \

\               /       \                  /       \                      /       \                           w(L,j,_)

x(N,_) <-- net(N,1) <-- x(N-1,_) <-- net(N-1,_) <-- x(N-2,_) ... <-- net(L+2,_) <-- x(L+1,_) <-- net(L+1,_)

16.  Interpreting the diagram is easy:

- Data enters at the right and flows left.

- e depends directly on each x(N,i), and so arrows flow from these into e

- Only one arrow flows from net(L,i) to x(L,i) because the latter is

obtained by passing the former through the transfer function f(L,i).

- Because net(L,i) is a function of the x(L-1,j)'s, arrows flow into

net(L,i) from each x(L-1,j)

- The far right of the diagram states that net(L+1,j) depends on each

w(L,j,i)

- The variables w(L,i,j) appear on the far right because the gradient with

respect to the synaptic weights between layers L and L+1 is sought.

17.  To compute the gradient G(L), extend Theorem 2 in light of the diagram:

De / Dw(L,i,j) = SUM  De/Dnet(L+1,k) * Dnet(L+1,k)/Dw(L,i,j)

k

But as is clear from the far right of the diagram:

Dnet(L+1,k) / Dw(L,i,j) = 0,  k not equal to i

Thus,

De/Dw(L,i,j) =     De/Dnet(L+1,i) * Dnet(L+1,i)/Dw(L,i,j)

=[1]= De/Dnet(L+1,i) * x(L,j)                   [6]

(NOTE:  =[1]= means that a substitution will occur from equation [1])

18.  Next, we define an important variable whose use will make the development

of backpropagation easier.  Define the variable d(L+1,i) to be:

d(L+1,i) = De/Dnet(L+1,i)                      [7]

But from the diagram,

De/Dnet(L+1,i) = SUM  De/Dx(L+1,j) * Dx(L+1,j)/Dnet(L+1,i)

j

But [2] implies that

Dx(L+1,j)/Dnet(L+1,i) = 0, j not equal to i

and so

De/Dnet(L+1,i) =     De/Dx(L+1,i) * Dx(L+1,i)/Dnet(L+1,i)

=[2]= De/Dx(L+1,i) * f'(L+1,i)(net(L+1,i))       [8]

19.  From the diagram,

De/Dx(L+1,i) =       SUM De/Dnet(L+2,k) * Dnet(L+2,k)/Dx(L+1,i)

k

=[2,4]= SUM d(L+2,k)*w(L+1,k,i)                      [9]

k

By substituting [9] into [8] and noting that [7] = [8], we get:

d(L+1,i) = f'(L+1,i)(net(L+1,i)) * SUM d(L+2,k)*w(L+1,k,i)     [10]

k

������������������������������� INTERIM SUMMARY �����������������������������Ŀ

�                                                                             �

� From [4], we want to find G(L) = [ De/Dw(L,i,j) ]                           �

�                                                                             �

� From [6] and [7], De/Dw(L,i,j) = d(L+1,i)*x(L,j)                       [11] �

�                                                                             �

� From [10], d(L+1,i) = f'(L+1,i)(net(L+1,i)) * SUM d(L+2,k)*w(L+1,k,i)  [12] �

�                                               k                             �

�������������������������������������������������������������������������������

20.  Before continuing, let's represent the above summary in matrix algebraic

form for notational convenience.

- From [11], G(L) = [ d(L+1,i)*x(L,j) ]

= delta(L+1) * 1(L) * diag(X(L))                   [13]

where

-- delta(L+1) = [ d(L+1,i) ] is a column vector with n(L+1) rows

-- 1(L) is a row vector that consists entirely of n(L) ones

-- diag(X(L)) is a square matrix whose diagonal is X(L) and

whose off-diagonal entries are zero

- From [12],

delta(L+1) = [ d(L+1,i) ]

= [ f'(L+1,i)(net(L+1,i)) * SUM d(L+2,k)*w(L+1,k,i) ]

k

= [ f'(L+1,i)(net(L+1,i))] @ [SUM d(L+2,k)*w(L+1,k,i) ]

k

= F'(L+1)(NET(L+1))  @   W"(L+1)*delta(L+2)          [14]

where W"(L+1) is the transpose of W(L+1).

������������������������������� INTERIM SUMMARY �����������������������������Ŀ

�                                                                             �

� From [5], W(L) = W(L) - beta*G(L), beta > 0                                 �

�                                                                             �

� From [13], G(L) = delta(L+1) * 1(L) * diag(X(L))                       [15] �

�                                                                             �

� From [14], delta(L+1) = F'(L+1)(NET(L+1))  @   W"(L+1)*delta(L+2)      [16] �

�                                                                             �

�������������������������������������������������������������������������������

21.  From [15], all we need to compute G(L) is delta(L+1) since X(L) is already

known.  But [16] says that to compute delta(L+1), you must know delta(L+2).  By

extension, to compute delta(L+2), you must know delta(L+3), etc.  Eventually

this will stop at delta(N), i.e., the delta vector of the output layer.

22.  According to [7],

delta(N) = [ d(N,i) ]

= [ De/Dnet(N,i) ]

=(diagram)= [ De/Dx(N,i) * Dx(N,i)/Dnet(N,i) ]

=[3]= [ (x(N,i) - v(i))*f'(N,i)(net(N,i)) ]

= (X(N) - V) @ F'(N)(NET(N))

This concludes the mathematical development of the backpropagation algorithm.

������������������������������������������Ŀ

� SUMMARY OF THE BACKPROPAGATION ALGORITHM �

��������������������������������������������

Using the notation defined in paragraphs 7 and 8, the backpropagation algorithm

can be described as follows:

1.  W(L) = W(L) - beta*G(L), beta > 0, L < N

2.  G(L) = delta(L+1) * 1(L) * diag(X(L)), L < N

3.  delta(L+1) = F'(L+1)(NET(L+1))  @   W"(L+1)*delta(L+2)

4.  delta(N) = (X(N) - V) @ F'(N)(NET(N))

where

5.  X(L) = F(L)(NET(L))

6.  NET(L) = W(L-1)*X(L-1), L > 1

7.  W(L) = [ w(L,i,j) ], where w(L,i,j) is the synaptic weight from node i

in layer L+1 to node j in layer L

8.  1(L) is a row vector that has n(L) ones

9.  (X(1),V) is the input training I/O pair

10.  A @ B is the matrix obtained by elementwise multiplication of A and B

APPENDIX B

Bibliography and Neural Network Companies

�����������������������������������������

***** Bibliography *****

AI Expert (DEC89, JUN90), a monthly magazine.

Caudill, Maureen, "Neural Networks Primers, I-VIII", AI expert:  (DEC87, FEB88,

JUN88, AUG88, NOV88, FEB89, MAY89, AUG89).

IEEE Communications (NOV89).

Kolmogorov, Andrei Nikolaevic, "On the Representation of Continuous Functions

of Many Variables by Superposition of Continuous Functions of One Variable and

Addition", Dokl. Akad. Nauk USSR, 114:  953-956 (1957).

Lippmann, Richard P., "An Introduction to Computing with Neural Nets", IEEE

Association of Signal Processing Magazine:  4-22 (APR87).

Lorentz, G.G., "The 13th Problem of Hilbert", Proc. of Symposia in Pure

Mathematics, 28, American Mathematical Society (1976). Omidvar, Omid.

"Progress in Neural Networks, Volumes One and Two."  Ablex Publishing Corp.

(1990; 0-89391-610-2).

Nelson, Marilyn N. and Illingworth, Bill.  "Neural Networks:  What Are They?"

This is a small pamphlet ($1) that can be obtained from Marilyn Nelson,

Associated Consultants, 1046 County Rd 500, Bayfield CO  81122; 303-884-2220.

Pao, Yoh-Han.  "Adaptive Pattern Recognition and Neural Networks."  Reading MA:

Addison Wesley (1989; 0-201-12584-6).

Simpson, Patrick K.  "Artificial Neural Systems:  Foundations, Paradigms,

Applications, and Implementations."  New York:  Pergamon Press (1990; 0-08-

037894-3).

Wasserman, Philip D.  "Neural Computing:  Theory and Practice."  New York:  Van

Nostrand Reinhold (1989; 0-442-20743-3).

***** Neural Network Companies *****

November 1992

The following companies are among those who market neural network products.

These are far more complete than this public domain package which has only

implemented a simple backpropagation neural network without momentum.

Although these companies have agreed to be listed in this manual, they were not

involved in the creation of this public domain neural network package.  A

company's permission to be listed does not imply the company's endorsement of

this public domain product.

California Scientific Software         COSMIC

10024 Newtown Rd                       The University of Georgia

Nevada City CA  95959 USA              382 East Broad St

800-284-8112                           Athens GA  30602 USA

916-478-9040                           404-542-3265

DAIR Computer Systems                  NeuralWare, Inc.

3440 Kenneth Drive                     Penn Center West

Palo Alto, CA  94303 USA               Building IV, Suite 227

415-494-7081                           Pittsburgh PA  15276 USA

412-787-8222

Neural Systems Incorporated            Ward Systems Group, Inc.

2827 W. 43rd Avenue                    245 W. Patrick Street

Vancouver, British Columbia            Frederick, MD  21701 USA

V6N 3H9 CANADA                         301-662-7950 FAX 5666

604-263-3667

APPENDIX C

Neural Network Information Resources

������������������������������������

Electronic Bulletin Board Systems (BBS)

Central Neural System, 509-627-6267; Wesley Elsberry, SysOp.  This BBS has one

of the largest collection of public domain and shareware neural net related

files and information available.  Is also the home of the FIDONET's NEURAL_NET

Echo.

Interocitor, 214-258-1832; Steve Rainwater, SysOp.  This BBS focuses on natural

language processing and is the home of the FIDONET's AI_ECHO.  Steve is also

the author of the AI CD-ROM disk, a gargantuan collection of public domain and

shareware software relating to artificial intelligence.

ShadeTree BBS, 412-244-9416 2400N81; Bill Keller, SysOp; 417 Peebles St,

Pittsburgh PA  15221  USA (shade.tree@f124.n129.z1.fidonet.org).  Dedicated to

educating on artificial intelligence, ShadeTree also publishes an inexpensive

bi-monthly newsletter that discusses AI matters in layperson terms.  It also

hosts the PDNNET User's Group.

Other Electronic Information Sources

The FIDONET NEURAL_NET and AI Echos.  These Echos carry international

discussion on neural nets and artificial intelligence.  Anyone can participate

in these and other forums by simply locating a local BBS that is a node on the

FIDONET.  The FIDONET is collection of BBS that form a loose, informal, and

free information network.

The USENET newsgroups comp.ai.neural-networks and comp.ai

COSMIC--which distributes NETS, a neural net package developed by NASA--has

NETS-LIST, an Internet forum that can be subscribed to by sending Email to

nets-list@cossack.cosmic.uga.edu

CompuServe:  GO AIEXPERT

International Neural Network Society:  send Email having the message "SUBSCRIBE

INNS-L your name here" to listserv@umdd.bitnet

Periodicals

"Advanced Technology For Developers", a monthly newsletter; High-Tech

Communications, 103 Buckskin Ct, Sewickley PA  15143-9946; 412-741-7699

FAX 6094

"AI Expert", a monthly magazine; Miller Freeman Inc, 600 Harrison St, San

Francisco CA  94107; 415-905-2200

"AI Magazine", a quarterly magazine; American Association for Artificial

Intelligence, 445 Burgess Dr, Menlo Park CA  94025-3496; 415-328-3123;

membership@aaai.org

"NASA Tech Briefs", a monthly magazine; Associated Business Publications Co, 41

E 42nd St, NY NY 10017-5391; 212-490-3999 FAX 212-986-7864

"Neural Networks", a quarterly journal; International Neural Network Society,

1250 24th St NW, Suite 300, Washington DC  20037; 202-466-4667 FAX 2888;

70712.3265@compuserve.com

"PC AI", a bimonthly magazine; PC AI, 3310 W. Bell Rd, Suite 119, Phoenix AZ

85023; 602-971-1869 FAX 2321

"ShadeTree Newsletter", a bimonthly newsletter; ShadeTree, 417 Peebles St,

Pittsburgh PA  15521; 412-244-9416; shade.tree@f124.n129.z1.fidonet.org