WELCOME to PLUX

COARSE SCHEMATIC

of

HOW "PLUX" WORKS

|----------|

||--------||

||        ||

||        ||                                         RELAY

||--------||                                        MODULES

|----------|                     ==========| -->  |---------|

|------------|                      =             |-----------|

|       ---- |     |-----|            =           |-----------|

|------------| <-- |     | <-- |========          .....    .....

PC            |-----|          RIBBON       inputs   outputs

CARDS           CABLE

(1) the I/O CARDS plug into the computer's expansion slots.

(2) the RIBBON CABLES connect the CARDS to their conjugate

RELAY MODULES.

(3) the Ladder Logic Program in memory receives input configur-

ations FROM the relay modules VIA the I/O cards.  It then

runs a program scan and sends the resulting output config-

uration TO the relay modules VIA the I/O cards.  This pro-

cedure constitutes ONE(1) scan.  These scans are repeated

indefinitely.

(4) Thus "PLUX" performs Process Control.

"WHAT IS PLUX"

The  word "PLUX" is something of a corrupted acronym  formed

from  the first letters of the title "Process Logic  Control  and

Simulation".  It is  so named because it performs these functions

with just your PC.

The PLUX system is a fully enabled Ladder Logic Programmer &

Editor which provides for program simulation both as bits and  on

Page 1 of 4

the  screen ladder.  The PLUX software/driver also  performs  the

function  of  driving 24-point I/O computer cards  which  receive

input from and send output to their conjugate relay modules.

It  may be clarifying to consider the PLUX-SYSTEM as  analo-

gous  to  an  internal modem system  (see  the  coarse  schematic

above).  An internal modem system consists of a computer card,  a

software  driver and an external device from which  the  computer

receives  and sends(telephone line).  "PLUX" works the  same  way

except  that the exterior I/O device is a relay module(s).   When

the  input  sensors of your process are connected  to  the  input

relays of the Relay Module, the output relays of the Relay Module

are connected to the controlling devices of your process, and the

logic  of  your process has been programmed by use  of  the  PLUX

editor/compiler,   your  PC  will  communicate  with  the   Relay

Module(s)  via  the I/O cards and conduct your  process'  control

according to the dictates of your program.

"YOUR PC as a PLC"

The  files in this package contains a file named  "PLC.EXE".

This file converts your PC into a PLC; see the "USR_PGMR.MNL" for

full details.

"SAMPLE PROGRAMS"

Three sample ladder logic programs are included among  these

files.   For  PLUX to use them, however, (and you  may  not  want

them)  they must reside in a sub-directory named "programs".   To

set  them up simply run the "GET_XPLS.BAT" file and  the  samples

will be set up.

Page 2 of 4

"DOCUMENTATION"

The  two  files having file extensions "mnl"  are  the  PLUX

documentation.  The "inst_ref.mnl" is the Instructions and Refer-

ence Manual.  It contains full explanations of the PLUX  instruc-

tion set, both logically and alphabetically.  The  "usr_pgmr.mnl"

is the User and Programmers Manual.  It contains detailed  infor-

mation on all aspects of PLUX that are necessary for full  utili-

zation and understanding.

Any document among these files will dump to your printer  in

a form suitable for a 3-point binder.  To list one of these  doc-

ments on a printer, at the DOS prompt type:

type [file name]>prn <CR>

"EXECUTING PLUX"

************************

To run PLUX, simply go to the DOS prompt and type:

PLUX<CR>

************************

"I/O CARDS and RELAY MODLUES"

PLUX  is  designed to work with  KEITHLEY  DataAcquisition's

PIO-12 I/O cards and their SSIO-24 relay module boards and relays

OR THEIR CLONES.  There are many sources of these devices, two of

which are listed below.

KEITHLEY METRABYTE

Data Acquisition

440 Myles Standish Blvd.

Taunton, MA 02780

ph (508) 880-3000  fax (508) 880-0179

Page 3 of 4

CyberResearch, Inc.

P.O. Box 9565

New Haven, CT 06535-0565

ph (800) 341-2525  fax (203) 483-9024

________________________________________________________________

<<< QUESTIONS and COMMENTS >>>

Send your questions or comments to any of the outlets below:

MAIL:     Jack Chasteen

c/o GISToBYTE, Inc.

P.O. Box 20221

Dayton, OH 45420-0221

COMPUSERVE E-mail:  75212,26

Be sure to include in your communique' an outlet for return-

ing an answer, i.e., an E-mail handle or a mailing address.

<<< IF YOU LIKE PLUX >>>

After you have had PLUX for a while and find yourself  using

the  software for whatever purpose, you are then obliged  to  pay

for it or erase it from your files.  The price, however, is  very

reasonable  considering PLUX's qualities.  Just send a  check  or

money  order in the amount or $49 to the above address.  Be  sure

to  include your name and mailing address in order that  you  may

receive  a  receipt  and that you will be  recognized  in  future

inquiries.

Page 4 of 4

###################################################################