Turbo Pascal Access to the Covox Speech Thing

Kim Kokkonen

TurboPower Software

October 1990

The Covox Speech Thing is a high-quality, cost effective system for adding

speech capabilities to PC programs. For only about $80, you get an interface

plug that connects to a PC's parallel port, an auxiliary speaker that connects

to the interface plug, and the SmoothTalker TSR, which converts ASCII text

into output instructions that generate good quality speech from the speaker.

The Speech Thing package also includes capabilities for playing digitized

(prerecorded) sounds as opposed to synthesized speech; STHING.DOC focuses on

the speech synthesis capabilities.

Although Covox supplies a version of the SmoothTalker TSR that will run on

4.77MHz 8088-based PC's, the quality of the speech on such a machine is not

very good. Realistically, you need an 8Mhz 8088 or a 286 or 386 machine to get

good speech quality. When running on a 16MHz 286 machine (my development

system), the Speech Thing produces speech quality that rivals that of $1000

dedicated speech processing systems.

The programming interface that Covox supplies for the Speech Thing is aimed

toward Basic and C programmers. This document describes a Turbo Pascal unit,

STHING, which makes it easy for Turbo Pascal (version 5.0 or later)

programmers to add synthesized speech capabilities to their programs. It also

describes two programs, STRES and TPTALK, which demonstrate some of the

capabilities of the unit. We assume throughout that you have the Covox Speech

Thing User Manual to refer to as needed.

If you don't already have a Speech Thing, you can contact Covox as follows:

Covox, Inc.

675 Conger St.

Eugene, OR 97402

tel: 503-342-1271

fax: 503-342-1283

bbs: 503-342-4135

The Turbo Pascal code provided here is Copyright (C) 1990 TurboPower Software,

all rights reserved. We hereby grant rights to other programmers to use this

code in their own programs, for personal or commercial use, at no charge. You

may freely distribute the files contained in this archive, including

STHING.DOC, STHING.PAS, STHING.ASM, STHING.OBJ, STRES.PAS, STRES.EXE, and

TPTALK.PAS. You may *not* sell any of the files (with the exception of nominal

handling fees charged by bulletin board operators and shareware software

distributors) without the express written consent of TurboPower Software. The

SPEECHV2 and SPEECHV3 programs and other parts of the Speech Thing itself are

the property of Covox, Inc. See the Speech Thing license agreement for

information about using these programs.

Overview of the STHING Unit

---------------------------------------------------------------------------

STHING is a Turbo Pascal unit that provides procedures and functions for

accessing all of the speech synthesis capabilities of the Speech Thing. In

order for STHING to work, you must first load either the SPEECHV2 or SPEECHV3

memory resident program prior to calling the routines in STHING. It is not

necessary for you to load Covox's STALK memory resident program, which is

designed to make it easy for Basic programmers to access SPEECHVx. STALK is

not necessary and in fact adds time and memory overhead for Turbo Pascal

programs.

STHING's initialization block searches for the signature of the SPEECHVx TSR.

If the signature isn't found, STHING's StLoaded function will return False and

none of the other routines will do anything. (It's still safe to call them,

however.) If the signature is found, STHING initializes the Speech Thing as

follows:

- speech output is sent to the LPT1 port

- speech tone, volume, pitch, and speed are set to default values

- the SPEECHV3 hook routine that determines which keystrokes can interrupt

speech is redirected to the STHING unit

All of these defaults may be changed by calling other routines provided by

STHING. For explanation of the third item, see the section "Interrupting

Speech in Progress" below.

The simplest program that should produce speech is the following:

program Speak1;

uses

SThing;

begin

StSpeak('hello world');

end.

If your Speech Thing interface plug is attached to a parallel port other than

LPT1, you need to add one more statement:

program Speak2;

uses

SThing;

begin

StSetLptPort(2); {Sets up for LPT2}

StSpeak('hello world');

end.

The supplied programs STRES and TPTALK demonstrate other features of the

STHING unit.

The routines in STHING can be organized in three groups:

Basic Functions

- set output port

- set speech parameters

- speak an ASCII string

- modify ability to interrupt speech in progress

- unload SPEECHVx from memory

Phonetic Speech Functions

- convert a text string to its phonetic representation

- speak a string in phonetic representation

Dictionary Functions

- initialize or clear the dictionary

- add or remove an entry in the dictionary

- list the elements in the dictionary

- write or read a dictionary file

Detailed descriptions of each routine follow in the "STHING Reference Guide."

Speech Dictionaries

---------------------------------------------------------------------------

As Covox points out in its manual, the English language is notoriously tricky

about pronunciation rules. The speech dictionary provides a means to improve

the quality of synthesized speech for words that don't follow standard rules

of pronunciation. Each entry in the dictionary contains a text string

representing the normal spelling of the word, and a phonetic text string

(using the SmoothTalker phonetic codes shown in Appendix A of the Covox

manual) representing how you'd like the word to sound. SPEECHVx automatically

substitutes the specified pronunciation whenever it encounters a dictionary

word in the text stream passed to it.

Here's a short program that demonstrates the use of a speech dictionary:

program Dict;

uses

SThing;

var

txst : string;

phst : string;

begin

if not StLoaded then

halt;

{Clear the dictionary. Redundant here}

StInitDict(True);

{Speak a hard-to-pronounce word}

writeln('ASCII before adding to dictionary');

StSpeak('ascii');

readln;

{Add the correct pronunciation to the dictionary}

if not StInsertDict('ascii', 'AEskIY') then

writeln('out of memory');

{Speak the word again}

writeln('ASCII after adding to dictionary');

StSpeak('ascii');

readln;

{Display what's in the dictionary}

writeln('Contents of dictionary');

StInitDict(false);

StDumpDict(txst, phst);

while txst <> '' do begin

writeln('"', txst, '", "', phst, '"');

StDumpDict(txst, phst);

end;

readln;

{Remove the word from the dictionary}

StRemoveDict('ascii');

{Speak the word again}

writeln('ASCII after removing from dictionary');

StSpeak('ascii');

readln;

end.

When you add a dictionary entry, SPEECHVx allocates a block of memory to hold

the entry. Normally SPEECHVx allocates this memory directly from the DOS free

memory pool. Although that makes sense for C and Basic programs, it often

doesn't work for Turbo Pascal because all available DOS memory is usually

allocated for the Turbo Pascal heap. To deal with this issue, the STHING unit

takes over the DOS interrupt ($21) and converts requests for DOS memory into

equivalent requests to the Turbo Pascal heap manager. There is only one

disadvantage to the approach: it isn't possible to deallocate the memory

allocated for dictionary entries on the Turbo Pascal heap. If you have an

application that needs to add and remove lots of dictionary entries, you

should set STHING's boolean typed constant StAllocFromHeap to False as soon as

your program starts. Note that in this case your program must use a $M

directive to assure that there is free DOS memory space from which SPEECHVx

can allocate dictionary entries.

One other point to keep in mind: a dictionary remains active for only so long

as the program which created it remains active. That is, as soon as your

program halts, the dictionary is cleared. (A dictionary created by a TSR

remains active as long as the TSR remains loaded.) For this reason, it's

usually important to save dictionaries to a dictionary file before an

application halts and to reload them when the program starts.

Determining the best phonetic codes for a word is not a job for the impatient.

However, in some cases it is well worth the effort. The StTextToPhonetic

procedure will generate a first attempt at the codes by using SPEECHVx's own

internal rules. Then you can modify the phonetic string by referring to

Appendix A of the Speech Thing manual.

Interrupting Speech in Progress

---------------------------------------------------------------------------

When SPEECHV3 is loaded, you can interrupt speech in progress by pressing any

key, with the exception of the Shift and Alt keys. SPEECHV2 does not offer

this interrupt capability. SPEECHV3's behavior is a little annoying, at least

for people who are accustomed to using WordStar editing keystrokes, because

speech is interrupted when the Ctrl key is held down.

SPEECHV3 provides a hook to modify its behavior in this regard. Whenever it

checks whether speech should be interrupted, it issues an interrupt $7E. If

the int $7E handler returns AX <> 0, speech output will be aborted and control

will return to the calling application.

The STHING unit takes over int $7E with a handler that provides the following

behavior: any key that inserts a real keystroke into the keyboard buffer will

interrupt speech. (This definition excludes the Ctrl, Alt, Shift, NumLock, and

ScrollLock keys.) The keystroke causing the interruption remains in the

keyboard buffer where it can be processed by the underlying application.

You can restore SPEECHV3's default behavior by calling STHING's StRestoreInt7E

procedure. If you need different behavior, you can install your own int $7E

handler. Be sure to study the handler in STHING.ASM before writing your own.

STHING Reference Guide

---------------------------------------------------------------------------

The procedures and functions of STHING are described here in alphabetical

order.

-------------------------

procedure StDumpDict(var TextSt : string; var PhonSt : string);

Call this routine repeatedly to retrieve all entries in the dictionary. The

text strings are returned in alphabetical order, always in uppercase. When

there are no more entries, TextSt returns an empty string. Call StInitDict

first to reset the internal dictionary position pointer. Example:

StInitDict(False);

StDumpDict(TextSt, PhonSt);

while TextSt <> '' do begin

writeln(TextSt:40, PhonSt:40);

StDumpDict(TextSt, PhonSt);

end;

-------------------------

procedure StGetParams(var Tone, Volume, Pitch, Speed : Word);

Returns the last speech parameters set. See StSetParams for more

information.

-------------------------

procedure StGrabInt7E;

Modifies the abort-speech-on-keypressed behavior as described in the section

"Interrupting Speech in Progress" above. Since STHING calls this routine

automatically during initialization, you don't need to call it yourself

unless you are restoring and grabbing int $7E repeatedly, as in a TSR. See

STRES.PAS for an example. StGrabInt7E affects the behavior of SPEECHV3 only.

-------------------------

procedure StInitDict(Clear : Boolean);

Reset the dictionary. If Clear = True, all entries are removed. If Clear =

False, the dictionary's internal position pointer is reset to prepare for an

alphabetical scan of the dictionary. See StDumpDict for an example.

-------------------------

function StInsertDict(TextSt : string; PhonSt : string) : Boolean;

Insert a new entry in the dictionary. Returns False if insufficient memory.

If STHING's global typed constant StAllocFromHeap is True, StInsertDict

allocates memory space from the Turbo Pascal heap; otherwise it allocates

memory from the DOS free pool by using DOS function $48.

-------------------------

function StLoaded : Boolean;

Returns True if SPEECHVx is loaded, False otherwise. The other STHING

routines don't do anything if StLoaded returns False.

-------------------------

procedure StPhoneticSpeak(St : string);

Speak the specified phonetic string. See Appendix A of the Speech Thing

manual for information about acceptable phonetic codes. You should not

surround strings passed to StPhoneticSpeak with << and >> strings, which are

used only when mixing phonetic codes into normal text.

-------------------------

function StReadDictFile(FName : string) : Word;

Read dictionary from a text file, returning status. The status value is a

standard Turbo Pascal IoResult code, with the following additions:

8    StInsertDict returned False

106    Invalid dictionary file

65535    SPEECHVx is not loaded

See StWriteDictFile for more information about the file format.

-------------------------

procedure StRemoveDict(TextSt : string);

Remove an entry previously added to dictionary. If TextSt is not found in

the dictionary, StRemoveDict does nothing. If the STHING typed constant

StAllocFromHeap is True, heap space is NOT reclaimed by this call.

-------------------------

procedure StRestoreInt7E;

Restore abort-speech-on-keypressed behavior to the default for SPEECHV3.

-------------------------

procedure StSetLptPort(LPTNumber : Byte);

Set Speech Thing peripheral port for LPT 1, 2, or 3. STHING automatically

initializes for LPT1. Also see StSetPort, which allows specification of an

arbitrary port not associated with an LPT device.

-------------------------

procedure StSetParams(Tone, Volume, Pitch, Speed : Word);

Set parameters, as follows:

valid range  default

-----------  -------

Tone     0..1          0

Volume   0..9          5

Pitch    0..9          5

Speed    0..9          5

StSetParams does not range-check the values it receives.

-------------------------

procedure StSetPort(Port : Word);

Set peripheral port where speaker output goes. Note that the value passed

here is a hardware port number, not an LPT number. StSetPort interprets

Port=0 as a special case meaning the port associated with LPT1. StSetPort

should be called only when using a Voice Master or another Covox product to

emulate a Speech Thing.

-------------------------

procedure StSpeak(St : string);

Speak the specified text string using the previously defined speech

parameters for tone, volume, pitch, and speed. Note that you can pass a

maximum of 255 characters at a time. If the text stream to speak exceeds 255

characters it is your responsibility to parse it into shorter chunks.

StSpeak doesn't return until the string has been completely spoken, unless

the speech is interrupted by a keystroke.

-------------------------

procedure StTextToPhonetic(TextSt : string; var PhonSt : string);

Returns the phonetic codes for the given text string, using SPEECHVx's

internal conversion rules. Note that the length of the phonetic codes often

exceeds that of the text string. It's your responsibility to limit TextSt's

length so that PhonSt doesn't overflow.

-------------------------

procedure StUnload;

Unloads the resident copy of SPEECHV2 or SPEECHV3 from memory. After calling

StUnload, StLoaded will always return False. Note that you can unload

SPEECHV3 from the DOS command line by entering SPEECHV3 /Q. SPEECHV2 does

not offer this command line capability, but it may still be unloaded by

calling StUnload.

-------------------------

function StWriteDictFile(FName : string) : Word;

Write current dictionary to a file, returning status. The status value is a

standard Turbo Pascal IoResult code, with the following addition:

65535    SPEECHVx is not loaded

A dictionary file is a standard text file, with each line representing one

dictionary entry or a comment. Blank lines and lines beginning with a

semicolon are treated as comments. Other lines must contain a text word

followed by one or more spaces followed by the phonetic representation

(which may contain embedded spaces). The case (upper or lower) of the text

word isn't important, but the case of the phonetic representation must

exactly match the codes given by Covox. Here's an example of a valid

dictionary file:

;these entries just encode the standard pronunciations

ARBITRARILY    AArbIXtrEHr4IXl3IY

ARBITRATOR     AArbIXtr4EYDX3ER

COMPUTER       kAXmpy4UWDX3ER

DICTIONARY     dIHkSHAXn4EHr3IY

TPTALK - simple demonstration of STHING

---------------------------------------------------------------------------

TPTALK is a simple program that demonstrates the STHING unit. If you specify

the name of an ASCII text file on the command line when you call TPTALK, it

will read the file for you. If don't specify a file name, you can enter text

and TPTALK will speak each line as you enter it and write the phonetic

representation of the line.

In general, TPTALK does the same thing as the TALK program supplied with the

Speech Thing. TPTALK accepts the following command line options:

/?         show a list of options

/T n       specify tone (0 or 1)

/I n       specify pitch (0..9)

/S n       specify speed (0..9)

/V n       specify volume (0..9)

/L n       specify LPT port (1..3)

Unlike TALK, TPTALK won't load the SPEECHVx program if it isn't already

loaded. Note that TPTALK needs only SPEECHV2 or SPEECHV3; it is *not*

necessary to load STALK.

TPTALK is provided only in source code format. You can compile it by running

the Turbo Pascal compiler and doing a make in the same directory where

STHING.PAS and STHING.OBJ reside.

STRES - a memory resident screen reader

---------------------------------------------------------------------------

STRES is a memory resident program for reading text screens and speaking them

via the Covox Speech Thing. Before loading STRES you must first load SPEECHV2

or SPEECHV3. Note that it is *not* necessary to load Covox's STALK memory

resident program.

STRES is loaded with the following command line:

STRES [options] [dictionaryfile]

You may specify the following command line options:

/?            display a help screen but don't make STRES resident

/L n          specify line printer port (1..3)

/S            speak installation messages

/U            unload STRES from memory

The optional dictionary file must be in the format described under the

StWriteDictFile function above.

The following hot keys activate STRES once it has been loaded:

[Ctrl-RightShift-R] for screen reader mode

[Ctrl-RightShift-S] to speak whole screen

In screen reader mode the following keys are supported:

[Up]            read next line up

[Down]          read next line down

[Left]          read next character left

[Right]         read next character right

[Ctrl-Left]     read next word left

[Ctrl-Right]    read next word right

[Home]          move cursor to left edge of screen

[End]           move cursor to right edge of screen

[PgUp]          move cursor to top of screen

[PgDn]          move cursor to bottom of screen

[Enter],[Esc]   exit from reader mode

Although the cursor is moved while STRES is reading from the screen, its

position is restored when you press [Enter] or [Esc].

You can interrupt the full screen speech by pressing any key.

If you activate STRES while in graphics mode, it will say "not in text mode."

You can type STRES /? for a summary of options and commands. Or type

STRES /S /? for a spoken summary.

STRES.PAS requires TurboPower Software's Object Professional library (see

below) to compile. STRES.EXE is provided in this same archive.

About TurboPower Software

---------------------------------------------------------------------------

TurboPower Software develops and markets add-on packages that make Turbo

Pascal programmers more productive. Here's a brief description of current

products:

Object Professional

An object-oriented library of over 100 object types and 2000 methods for

writing sophisticated text mode user interfaces, for creating memory

resident programs, and for solving many other common programming problems.

Requires Turbo Pascal 5.5. List price $150.

Turbo Professional

The non-object-oriented predecessor to Object Professional. Has most of the

same capabilities. Requires Turbo Pascal 4.0 or later. List price $125.

B-Tree Filer

A library of routines for writing single or multi-user database applications

in Turbo Pascal. Also includes routines for direct access of network

capabilities such as message passing and print spooler control. Supports all

common PC networks. Requires Turbo Pascal 4.0 or later. List price $125 for

the single user version, $175 with network support.

Turbo Analyst

A collection of utilities for documenting, analyzing, and improving Turbo

Pascal programs. Includes cross-referencing, program indexing, program

listing, pretty printing, execution profiling, and an integrated environment

to make the whole process easy and pleasant. Supports Turbo Pascal 4.0 or

later. List price $99.

TurboPower also publishes the BIGED ("Big Ed") text editor, which displays

text in graphics mode at larger than normal sizes for vision-impaired computer

users. We will soon release a shareware version of BIGED that also provides

speech output using the Covox Speech Thing.

For further information, contact TurboPower at:

TurboPower Software

P.O. Box 66747

Scotts Valley, CA 95067

tel: 408-438-8608

fax: 408-438-8610

CompuServe: 76004,2611

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 2,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

1-800-2424-PSL

MC/Visa/AmEx/Discover

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.