This is a windows 8 and 24 bit and GIF  color bitmap viewer,

image processor and printer.  You can negate, posterize, high and

low pass filter, edge detect, histogram  equalize, mirror image,

zoom by non-integer factors more or less than 1.0  independently

in the x and y direction, pixelate, convert to grayscale, color

separation, rotate and many others (over 50) functions including

combinations of  two bitmaps, all in a MDI environment for

interactive display of before and  after. Complete TPW source is

included!

BMTLKT20 T.W. Fulton Compuserve 100015,565

Is an upgrade to my BMPTLKIT, a windows program for

displaying,modifying, and printing Windows 256 color

bitmaps. It is an MDI application allowing simultaneous display

of several bitmaps

(MDI documents)  so for example one can display the "before" and

"after version of

a bitmap.

This program can read and write the Windows 1,2,4,8 and 24 bit

BMP formats and can read only the GIF format. The 24 bit images

are displayed with a grayscale palette though their true color is

preserved. It cannot directly image process 24 bit images but it

can decompose them into three 8 bit images (red,green,blue) which

can be separately processed in this program and recombined into a

24 bit image, also with this program.

I think this program will fill a gap in the area of interactive

image processing, bringing to the user a large range of basic

tools which can be combined to do quite powerful processing,

hence the "toolkit" moniker.

A few words about the design philosophy. Because of the nature of

256 color bitmaps, most of the tools in this toolkit work best on

grayscale bitmaps. This is because a 256 color bitmap's pixel

values are in fact indexes into a palette. So changing a pixel

value on a color bitmap has in general unpredictable results

whereas changing the pixel value of a 256 grayscale bitmap has a

clear meaning, it changes the intensity. Fortunately, its easy to

convert a color bitmap to grayscale. Because it is so essential

for good results with the tools in this program, color to

grayscale conversion is a main menu option.

However, it is possible to process color images with this

program. You must first separate an image into its Y,I and Q

components (luminance,inphase, and quadrature phase components)

using the color separation menu, then process the Y

component(e.g. with a Sobel filter) and finally recombine the

processed Y with the original I and Q components. If you change

the size of the Y component, then you must also change the size

of the I and Q components before recombining.

Operations can be done on a single bitmap in memory (BM modify),

which writes the results back to the input bitmap in memory or

one can perform operations using two different bitmaps (BM & BM),

storing the result in a third bitmap in memory. At any time the

active bitmap(MDI document) can be saved to a file. Most

operations can be restricted to a user specified rectangular area

of the bitmap..

Summary of features on Main menu:

File:

To create empty bitmap or read in from a file,print, and

save modified bitmap files

Ascii I/O can read and write text files in the format

row col value

The values are automatically scaled to the range 0-255

BM Info:

Here you can display the bitmap's histogram, display

minimum,maximum, and average values of a bitmap and it's

entropy or average bits/pixel,

draw the active rectangle on the bitmap or display the list

of processes that the bitmap has been through during the

session.

Grayscale:

Selecting this option converts a 256 color bitmap to a 256

grayscale bitmap. This converts the palette to grayscale and

each pixel value so that it represents the intensity of the

original image. On most color bitmaps, this should be the

first operation performed, although you can perform any of

the operations on color bitmaps though probably not with the

intended results. Do not confuse with the gray palette under

palette modify which only modifies the palette.

BM Modify: This is the core of the toolkit. These are all single

bitmap processes which write the output back to the input bitmap

(in memory)

Refresh:Redraws the bitmap, e.g. after histogram display

Cutout: Retains the portion of the bitmap inside the active

rectangle and discards the part outside the rectangle. Not

to be confused with cutting to clipboard which can also be

done.

Fill:fills a bitmap with a constant value. Useful for BM &

BM operations such as mask.

Bias: adds a constant to every pixel value. This brightens a

gray scale image.

Cmin: sets all pixel values less than cmin to cmin.

Cmax: sets all pixel values greater than cmax to cmax.

Invert: makes the photographic negative of a grayscale image

if threshold=0.

Threshold: sets pixel values less than threshold to 0.

Bitplane slice: for each pixel, if the user-specified-bit is

on then a white(255) pixel is displayed, otherwise black is

displayed. Indicates which bits are most significant in an

image.

N bits/pixel: converts an 8 bit image to N bit resolution

(though it remains an 8 bit/pixel format bitmap.

Histogram equalization: attempts to remap the pixel values

so that in the entire bitmap, all possible gray levels are

represented equally.

Local histogram equalization: same as above but equalization

is done in a user specified rectangle which moves over the

bitmap. This is a VERY SLOW process, even on a 486 50 MHz

machine.

Xpandgray levels: remaps a user specified sub range of

possible gray levels to the complete range 0-255.

Stretch histogram: similar to xpandgray but automatically

finds the range from the histogram and a user specified

threshold for the histogram.

Gray map: changes the linear relation between pixel value

and gray level to a two piece-wise linear relationship.

Kodalith: changes each pixel value above the threshold to

white(255) and each pixel value below the threshold to

black(0). Sometimes called posterization.

Log intensity: Takes the natural logarithm of each pixel and

scales it to 8 bits. It is a form of dynamic range

compression. If you have an image with a few very high

intensity pixels in a largely low intensity background, this

will allow you to bring up the background without clipping

the high intensity pixels.

Vert mirror: rotates the image about a vertical axis, i.e..

the mirror image

Horz mirror: rotates the image about an horizontal axis.

Rotate 90: rotates the image 90 degrees clockwise.

Translate: Moves the whole image a user specified number of

pixels to the right, and wraps the image around to the left

side. To translate to the left or up or down, first mirror

or rotate the image, translate, then invert the rotation

etc.

Shear: holds lower left corner fixed and moves upper right

corner to right by user specified amount. Not strictly

speaking a true shear. Rotate the image first to shear in

other directions.

Zoom: increases or decreases size of image by user specified

amount, horizontal and vertical scales can be fractional and

different to change the aspect ratio, i.e vert scale=3.5,

horz scale=0.75 for example. With interpolation it is slower

but better.

Dither4,3,16, halftone, etc. are five different dithering

methods to convert a grayscale image to binary, i.e either

black or white pixels in order to print on a monochrome

device.

Pixelate: converts all pixels in a user specified moving

rectangle to the value in the upper left corner of the

rectangle. Makes the pixels "blocky" , i.e. TV programs use

this technique over the face of an individual to conceal his

identity.

High Pass: accentuates the high frequencies of an image. Can

give very striking improvement in detail, especially if

followed by histogram equalization.

Low Pass; smooths out the high frequencies of an image, very

similar to smoothing. This is a nine nearest neighbors

filter.

Edges: highlights the edges of objects in an image, i.e

where there is a large and sudden changes in the intensity

of the image.

Emboss: Creates a bas relief look by calculating gradient of

image and superimposing on a mid intensity background.

Sobel H,V and HV: These are high frequency enhancing

filters. H enhances horizontal line, V vertical and HV does

both.

Blur: This simulates the effect of camera movement on a

picture. The image is moved a user specified number of

pixels to the right, adding to the original after each pixel

move.

Smooth: Averages over the 4 nearest neighboring pixels.

Vert smooth: smooths out variations in the vertical

direction only.

Horz smooth: likewise for horizontal directions. These

techniques are useful for minimizing scanning artifacts.

Dip: calculates the magnitude of the maximum change in

intensity in any direction. If the image represented the

elevation of a surface such as ground elevation, then dip

would give the steepness of the slope at each point.

Median: a filter which replaces each pixel with the median

of values in a neighborhood. Useful for removing random

spiky noise without smoothing out detail.

Pwr Spec: computes the 2 dimensional Fast Fourier spectrum,

actually displays the log of the power spectrum. Works

correctly only if only one document is open. I'm working on

this one.

Grab_video: Not implemented, designed for my particular

frame grabber

(Micromint IMWPC)

Pal Modify:These processes change only the palette, not the

associated bitmap values.

Display active: Shows the 256 colors of the palette as a 16

x 16 matrix of rectangles overlaying the bitmap. Use BM

modify refresh to restore the display.

Gray scale: Replaces active palette with a gray scale

palette. Gives a preview of what grayscale conversion would

look like without changing pixel values of bitmap

Rainbow: obvious

Green: shades of green, useful because the human eye can

discriminate more shades of green than any other color

including gray.

Brighten:increases the intensity of each color in the

palette.

Dim: opposite of brighten

Color adjust: change the red, green, or blue component of

each palette entry

Negative: complements each color in the palette.

Update palette: makes any of the above changes a permanent

part of the palette associated with a bitmap

Restore palette: undoes the above temporary changes, does

not undo update palette

Color separation: Separates a color image into three new

images or bitmaps for further processing.

Red,green and blue components: Self explanatory. These can

individually be processed as if they were gray scale images

though the Y,I,Q method is preferred.

Y,I,Q: The color TV method of storing colors. The Y

component is the intensity of the color image, I and Q are

used to hold color information. These are useful for color

image processing. See introductory paragraphs.

Options:

Mouse: These control the meaning of mouse button presses. If

you double click the left button on a bitmap, the cursor

changes to a cross hair and depending on the options set

here you can by holding down the left button and dragging,

form a rectangle which will be used to do one of the

following:

zoom: Will enlarge the bitmap in the rectangle to fill the

window. This is temporary, use BM Modify zoom to make it

permanent.

cutting: cut the rectangle to the clipboard

pasting: paste the clipboard contents to the bitmap when the

left button is clicked anywhere on any bitmap in any MDI

window

windowing: the rectangle becomes the active area for most

processes so one can change only a rectangular subset of a

bitmap. This is the default meaning of a left button double

click.

N.B. holding down the right mouse button gives a continuous

readout of the row,col and pixel value under the cross-hair

printing:depending on the option set, the printed image will

be a

1:one-to-one pixel correspondence with the bitmap

2:the same size as the screen image

3:adjusted to fill the printed page

BM & BM: These are operations using two or three bitmaps(MDI

document windows) as input and storing the output in a third or

fourth bitmap. You should have three documents(bitmaps) open

before trying this. You can create an output bitmap with the New

option under File. For images of different dimensions, note that

the processes start in the lower left corner of each bitmap and

work to the upper right of the largest bitmap.

Add: adds the first and second bitmap, pixel by pixel,

output=input1 + input2

Sub,Mul,Div:obvious

AND:logically ands each pixel of input1 with each pixel of

input2

Or,XOR: likewise

Min:output is minimum of two inputs,pixel by pixel

Max:likewise

Ave:output is average of two inputs

Compare:is absolute value of difference between two inputs

Mask:if input2 > 0 then output= input 1 else output=0

Overlay: if input2 > 0 then output = input2 else

output=input1

Copy palette: copies palette from source to destination

YIQ2RGB: Combines the Y,I and Q component bitmaps (8 bit)

into a normal windows 256 color bitmap. From the Y, I and Q

component, it computes an RGB color which is then matched to

the color palette of the Y bitmap. You must select the

bitmaps in the order Y,I and Q to get the correct results.

This procedure automatically creates a fourth new bitmap

with the correct palette.

Decomp24: From a 24 bit image, creates three 8 bit images

holding the red, green and blue values of the 24 bit image.

These 8 bit images can then be processed as if they were

gray scale images. Recombine them after processing with

Comb24.

Comb24: Takes three 8 bit images corresponding to red,green,

and blue components and combines into a 24 bit image.

Animate: Will alternately display two bitmaps in the same

window as fast as possible. Useful for detecting small

differences between two nearly identical images.

A word about palettes. Windows can display 256 different

colors at any one time. If you have more than one bitmap

open with different palettes, only one can be displayed

correctly with 256 color windows at any time. Clicking on an

inactive bitmap will cause the entire windows display to be

updated for the palette of the now active bitmap, causing

all other windows to temporarily be displayed with the wrong

colors.

Average processing times on a 486 50 MHz machine for a 640 X 480

image:

Most processes run in < 10 secs

Histogram,shear,high,low pass filters take about 20 secs

Dip takes about 25 secs

Median filter takes about 33 secs.

Local histogram equalization of a 256 X 256 image using a 30

pixel block size takes 3.5 minutes. On a 386 20 Mhz machine, this

could take over 15 minutes !

A 2.0 zoom of a 256 X 256 image in both directions with

interpolation takes about

25 secs. To zoom a 640 X 480 image by a factor of 2 could take

over 5 minutes on a 386 20 Mhz machine.

Each image is stored twice, once as a large array on the global

heap and again as a bitmap in a memoryDC. Opening two 640 X 480

bitmaps will use about 1.2 megabytes of memory.

24 bit images are quite slow in loading because the true colors

must be matched to the active palette before displaying.

Remember to set mouse and print options prior to using the mouse

or printing.

If you have trouble reading the histogram on small images,

enlarge the window prior to selecting histogram.