SWANFIX.SYS, A SOLUTION TO USING THE SWAN 486DB WITH QEMM-386

The Swan Computers 486DB, a machine which can run at 66

MHZ, has a bug in its BIOS.  When you use Function 87 of

Interrupt 15, the BIOS call which you use to move data into

extended memory, it does not wait long enough for the 8042 (the

keyboard controller chip, a port of which you use to

enable/disable address line A20, the address line you must enable

to use any memory above one megabyte) to enable the address line.

Because the 8042 is typically slow, QEMM-386 always enables it

twice when it must enable it.  When you run this Swan computer at

the 66 MHz speed, it does not give the 8042 enough time to honor

the BIOS's request to enable line A20 so when you try to use any

of the memory you need to enable line A20 to use (all even

megabytes), then try to use this memory, you crash.

This device driver, Swanfix.sys, hooks Int 15, and, when a

function 87 comes in, does an enable of A20 so that by the time

the BIOS enables A20, it will be the second time, and, by the time

the Swan expected to have enabled A20, it will be ready.

Load this driver in the CONFIG.SYS before QEMM386.SYS:

device=c:\qemm\swanfix.sys

device=c:\qemm\qemm386.sys ram st:m

************************************************************************

*This technical note may be copied and distributed freely as long as it*

*is distributed in its entirety and it is not distributed for profit.  *

*          Copyright (C) 1993 by Quarterdeck Office Systems            *

************************ E N D   O F   F I L E *************************