D I G I - R O U T E

v3.3

16.05.1992

How Digi-Route evolved in time

------------------------------

Originally, Digi-Route had been written by DG6KK. DF3VI had that very good idea

to save the generated link table to disk so that it had no more to be created

at each start-up. He did also some other important improvements. DB1HZ changed

the way data are stored in memory by using the pointer technique. This, and the

support of EMS, helped to increase the amount of stations in memory.

Meanwhile, OK2PEN got a copy of a somewhat changed VGA version from LX1NO to

convert it for CGA. Quite a lot of different versions existed in the mean time,

and after version 3.1 by DB1HZ, LX1NO put both, the CGA and VGA versions

together, changed it for the overlay technique and version 3.2 appeared.

After having sent v3.2 through the Packet Radio net, the next step was v3.3,

because an awfull lot of requests for an EGA implementation reached LX1NO.

So, EGA support was added as well as a new map, provided by DB1HZ, showing much

more of Europe.

The main points of Digi-Route

-----------------------------

The idea of Digi-Route is to give an overview of the existing packet radio

network in Europe. You may choose between getting a map of Europe showing all

stations known by Digi-Route or just pointing out a certain part of Europe and

therewith, getting a closer look on the situation.

In case you do not want all the different kinds of stations to be displayed,

you may display digipeaters, BBSs or PacketCluster-nodes, only.

Furtheron, if you are looking for a possible link route between two stations,

the auto-router provides you with this opportunity.

Additionally, you may ask for information on a particular station or you may

want to display all stations running on a given frequency.

A help screen displaying all available functions provides you with the

necessary information how to use Digi-Route.

Getting started with Digi-Route

-------------------------------

In order to keep all necessary files together, create a sub-directory

(i.e. DIGIROUT) and copy all files into this directory.

The main files are: DIGI.EXE

DIGI33.OVR

DIGIMAP.DAT

Now, you are ready to launch Digi-Route. Type in DIGI and press the Return key.

Digi-Route will now check for the graphic card installed. The programme

recognizes three types of cards, CGA, EGA and VGA. If none of these can be

found, Digi-Route will abort giving an error message. Otherwise, it will

install itself for one of the three modes, always choosing the highest

resolution mode available if you do not explicitly specify for which mode

to install.

I'll give you an example. You have a VGA card, so starting Digi-Route without

command-line parameters will run it in VGA mode. Using the switch /E will start

it in EGA mode and switch /C starts it in CGA mode.

Logically, you cannot start it in a higher mode than you have on your card.

If a mouse driver is found, Digi-Route installs the mouse routines.

Then it checks for the memory available. If sufficient memory is not available,

it will abort. Pre-installed values for v3.3 are 2000 stations and 4000 links.

This will run also on machines having just 512kB RAM.

If you have EMS installed, it will automatically use it.

Having done all installations, Digi-Route looks for a file called DIGIMAP.FST.

This file keeps the link table Digi-Route is going to work on. So, DO NOT

touch this file. If the file does not yet exist, it will be created.

On a first start-up, Digi-Route shows what it is doing on the right bottom

corner on the start-up screen. First, it reads all data kept in DIGIMAP.DAT,

then it will build a link table in memory which will be saved onto disk.

Each time, you are going to change DIGIMAP.DAT, Digi-Route recognizes changes

have been done and the procedure described above is going to be repeated.

From next time on, Digi-Route will just read-in the link table.

Finally, you get the "working" screen, showing a map and some information on

the right side.

F1 or Alt-H gives you a help screen in case you forgot how to use a command.

All commands should be straight-forward and do not need further explanation.

If you have a mouse driver installed and after you have displayed some stations,

you may move the mouse-pointer onto one of the small circles and click on it.

You will get displayed all information found on this station on the bottom line.

You may interrupt displaying stations by pressing a key.

Command-line switches

---------------------

You may use just one of the switches described below at start-up.

/?  gives you a help screen showing all switches available.

Digi-Route is not going to be started.

/V  Starts Digi-Route in VGA mode.

/E  Starts Digi-Route in EGA mode.

/C  Starts Digi-Route in CGA mode.

How to update DIGIMAP.DAT

-------------------------

As described above, DIGIMAP.DAT keeps all data concerning a packet radio

station. You may edit the file with your favourite editor. But be sure to

respect the imposed syntax, otherwise Digi-Route is likely to crash.

The day you are adding or deleting link information, be sure not to delete it

just at one of the two stations concerned. It MUST be deleted at both stations.

If you are not sure, you may check DIGIMAP.DAT with the CHKMAP.EXE programme.

This programme will check if you respected the syntax and if all links have

their counterpart.

Syntax to be respected in file DIGIMAP.DAT:

LX0PRG-8 ,JN39AO,2, 438.025,,(LX0PAC#), Strassen LX9EG FBB 144.625

^       ^    ^    ^    ^    ^   ^          ^

�       �    �    �    �    �   �          ������������ information

�       �    �    �    �    �   �                       (not more than

�       �    �    �    �    �   �                        64 characters)

�       �    �    �    �    �   �

�       �    �    �    �    �   ����������������������� speed indicator

�       �    �    �    �    �                            nothing: 1k2

�       �    �    �    �    �                                  $: 2k4

�       �    �    �    �    �                                  #: 9k6

�       �    �    �    �    �                                  %: interlink

�       �    �    �    �    �                                  &: planned

�       �    �    �    �    �

�       �    �    �    �    ��������������������������� link list

�       �    �    �    �                                (digi1,...,digiN

�       �    �    �    �                                 SSID's might be

�       �    �    �    �                                 used)

�       �    �    �    �

�       �    �    �    �������������������������������� DO NOT forget this

�       �    �    �                                     comma. It's from

�       �    �    �                                     former times.

�       �    �    �                                     Might be eliminated

�       �    �    �                                     in a future version

�       �    �    �

�       �    �    ������������������������������������� working frequency

�       �    �

�       �    ������������������������������������������ typ of station

�       �                                                2: BBS

�       �                                                3: digipeater

�       �                                                4: PacketCluster

�       �

�       ����������������������������������������������� WW-locator

�

������������������������������������������������������� station call sign

(on a 6 character

call sign you may

append a 2 digit

SSID including

a dash ("-") )

In case you want to make a commentary somewhere in the file, just put the "#"

character in coloumn 1. If you are using free text behind, DO NOT put a comma

in it. CHKMAP will recognize this as an error.

Run CHKMAP /? to get a short help text.

Appendix ... Notes  by G8AMG Feb/92

Editing DIGIMAP.DAT.. Take great care with this file, it needs to be edited

with a NON-Document editor capable of handling long lines (at least 132 chars)

without truncating. (A good suitable Shareware editor is "QEDIT")

Your editor should place an EOF (0x1A) (Ctrl+Z) at the end of the file, DON'T

leave any blank lines after the last entry, this will result in an error

report ',' missing, it can also inhibit the full reporting of link errors in

the file which may cause DIGIMAP to lockup.

Another important point to watch, DO NOT leave a spare comma after the last

entry in a node call list thus .. (NODE1,NODE2,NODE3,) this WILL cause the

program to freeze during the link table build process (Needs 3 finger trick)

This last error is NOT reported by CHKMAP.

Meanwhile, have fun using Digi-Route. It's been made for you.

Doc written by LX1NO.