��������������������������������Ŀ

�   Documentation for RITETIME   �

�          Version 1.5           �

�        January 24, 1994        �

����������������������������������

(c) Copyright 1994, Al Sharpe  CIS 72677,3641

Table of Contents

�����������������

Overview................................................1

Calling RITETIME........................................1

The Legal Bits

Who May Use RITETIME...............................2

Who May Not!.......................................2

Modem Initialization....................................3

What's Happening........................................4

Disconnecting...........................................5

Resetting the Modem.....................................5

The MODEMSET Program....................................5

XT Multi-Function Clocks................................6

In Case of Difficulty...................................6

Hayes Compatibility.....................................6

Acknowledgements........................................6

History.................................................7

RITETIME  Version 1.5   01/24/94

Page 1

Overview

��������

RITETIME is a program that can operate manually or unattend-

ed  on any IBM compatible computer and will call via modem for

the exact time of day and date. The remote system operates  24

hours per day and connects using only a 300 baud or higher Hayes

compatible modem.

In Canada, RITETIME connects to a service operated by Leitch

Video International. This service in turn ties into the NRC

(National Research Council of Canada) which controls standard

time for Canada. This program was not commissioned by Leitch and

they therefore bear absolutely no responsibility for it's opera-

tion or support. Any questions regarding it's operation should be

directed only to the author of RITETIME.

Calling RITETIME

����������������

The program is called with two (2) minimum command line

arguments and two (2) optional command line arguments. One option

allows for a Time Zone offset from EST (this number can be from

-9 to +9 (or just 9)), and the other for IRQ2 and IRQ5 hardware

support. The first 2 arguments set up the port and phone number.

If no command line arguments are given, a help page will be dis-

played.

The program should be called as follows:

RITETIME <port> <phone #> [<offset>] [<i2|i5>]

where...

port   : 1,2,3,or 4   (COMM 1 -> COMM 4)

phone# : 445-9408 (Canada) area code 416

745-3900 (Canada) area code 613

424-5631 (USA)    area code 804

offset : optional Time Zone Offset from EST (-9 to 9 hours)

i2|i5 : optional for Serial Ports That Can Use IRQ2 or IRQ5

REMEMBER to include "1" and area code as necessary

for long distance calling.

RITETIME  Version 1.5   01/24/94

Page 2

The Legal Bits

��������������

Who may use RITETIME

��������������������

The program "RITETIME" in executable form and the source

code are copyright. RITETIME is not, nor has never been, in the

public domain. No warranties to it's operation, or lack of it,

are expressed or implied. The author takes no responsibility for

any losses be they financial or other due to operation or mis-

operation of RITETIME. This program has been tested over, and

over again without fail.

The author retains ownership of this program and grants li-

cense to anyone, who agrees to these terms, to use the program

and share it with others. No rights should be assumed other than

those specified here for it's use. Use of this program consti-

tutes acceptance of the terms specified herein.

Who may Not!

������������

RITETIME and it's associated files may be distributed freely

to anyone including public and private BBS's in Canada and the

United States. It's use is controlled as per the information in

the previous paragraphs and anyone using the program must abide

by these rules.

RITETIME may also be distributed by registered user groups

such as CIS, PCSIG and NYACC as long as there is only a nominal

charge which should take care of only the media and it's actual

distribution costs. A typical distribution cost of $7.00 or less

per disk is deemed as nominal.

RITETIME and it's associated files may NOT be included in

disk software packages with other software that has been set up

by money grabbing organizations to support get rich quick schemes

with disk prices in the range of $20. Prices like these are

considered by most people to be outright gouging. I'm not out to

condemn these people... everyone has to make a living but it's

not going to be at my expense. I'm not making a penny on this

personally and I'll be damned if someone thinks they're going to

make a buck on me. Anyone attempting this will find themselves in

court!... so be warned!!

If you acquired RITETIME and Modemset through the type if

disk distribution system I've just discussed here, I'd like to

hear about it. No renumeration may be accepted by anyone for

RITETIME other than for the nominal distribution costs.

RITETIME  Version 1.5   01/24/94

Page 3

Modem Initialization

��������������������

In order for the modem to communicate with the time BBS, it

must first be initialized by RITETIME. This is done by sending

what's called an 'initialization string' to the modem before the

modem is allowed to place the call. This is a multi-step proce-

dure in order to maximize performance in the modem and yet have

complete compatibility with all 'AT' type modems.

The first string is as follows:

AT E0 V0 Q0 S7=60

The first command 'E0' instructs the modem not to echo

commands back to the program. This prevents the input buffer from

filling up unnecessarily.

Next, 'V0' tells the modem to respond with numeric result

codes. This makes it easy for RITETIME to respond quickly to the

modem.

Next, 'Q0' is a precaution to make sure the modem does send

result codes to RITETIME. This is normally a default condition.

Finally, 'S7=60' tells the modem not to time-out for 60

seconds. The modem default is usually 30 seconds which is not

long enough for the modem to connect in this case.

The second and third strings are as follows:

AT X4

AT W0

The second part of the initialization is to tell the modem

to operate at level 4 of the Hayes 'AT' command protocol. Not all

modems have this capability. Typically, only 2400 baud and higher

modems respond to the X4 command. This allows for single digit

responses but also allows the modem to tell RITETIME that the

line is busy, or that there is no dial tone. In these cases, it

speeds up the operation.

The third part of the initialization is specific to ATI

modems. The command "ATW0" tells the modem not to use protocol

negotiation response codes as they confuse RITETIME.

The reason these strings are sent separately is that some

modems will respond to these commands as an error. For these

strings, the response from the modem is ignored. If there is an

error in any of the command lines, the modem ignores the entire

line, therefore also ignoring all the first part of the initiali-

zation string.

RITETIME  Version 1.5   01/24/94

Page 4

What's Happening

����������������

When you start RITETIME, the first thing the program does is

open the  serial port specified at 300 baud, 8 bits, 1 stop, no

parity. If everything is OK, the next step is to make sure there

is a modem on the port and that it is compatible. If there is an

error anywhere up to this point, the program will exit with error

level 1.

Next, it's time to place the call to the BBS. The program

places the string "ATDT" in front of the phone number on the

command line in order to tell the modem to make the call. There

is no option for anything other than a "Touch-Tone" dialing

method in this program. It is possible to manually patch the

program though in order to make "Pulse" dialing work. It would

then be one or the other (ATDP instead of ATDT).

If the modem is 2400 baud or higher, it will be set to the

X4 Hayes command protocol. This means that if the phone line is

busy or if there is no dial tone, the modem will send a command

back to the program telling it there was no answer at the other

end within a few seconds. If there is no connection, the program

will pause for 15 seconds then try to place the call again.

RITETIME has the ability to call up to five times if necessary,

with a 15 second pause each time in order  to make the connec-

tion. This should be more than ample time to make a connection

with the BBS. If no connection is made, for what ever reason,

RITETIME will display an error message to this effect and will

exit with error level 1.

Once connection is made, the program pauses for one second

just to make sure everyone is settled. The program then sends the

command for the date information to be sent. This information is

then displayed on the screen as it's being received. Next, the

time command is sent and again as the time information is being

received, it is displayed on the screen in the format it's being

received in. Immediately, the system time is set followed by the

system date. This makes sure that little time is lost between the

time data being received and the system time being set in the

computer. There may be a few hundreds of a second lost but it's

hard to calculate system processing and allow for this. In the

program, these two operations are back to back so there is the

least amount of time between operations. If the computer is a AT

class or higher compatible, the CMOS clock will also be set.

If there is an error in the data received, the system time

and/or date, which ever is not correct, will not be set. There is

no return value for these time and date functions so if the data

is corrupt the program will say the date and/or time is set to

some strange value but in fact the old date and/or time will be

the same as before.

RITETIME  Version 1.5   01/24/94

Page 5

Disconnecting

�������������

Now that the date and time info has been captured and the

system date and time set, we have to tell the BBS that we are

done. RITETIME now sends a command to make the remote system hang

up. After this command is sent, there is a one second pause, then

the program checks to make sure that the carrier detect on the

modem has dropped. If there is still carrier detect on the modem,

RITETIME will drop the DTR momentarily to force the modem to

disconnect. It is important that the DTR on your modem is not set

to "forced on" by the modem and that it be controlled by the com-

puter.

Resetting the Modem

�������������������

The last thing RITETIME tells the modem to do is to make it

reset to the standard modem configuration. It does this by send-

ing the command "ATZ" which tells the modem to set itself up to

the power on condition. For most applications this will work. For

people operating a BBS or the like it may be necessary to set the

modem to some special configuration.

The MODEMSET Program

��������������������

Another program is included with RITETIME called Modemset.

It is a stand alone program that will open a port, send a modem

initialization string, then reset the port to its previous set-

ting. Modemset can be used if it is necessary for the modem to be

initialized to a configuration that is different from the power-

on condition. It is very straight forward to use but requires a

knowledge of the Hayes "AT" protocol to be effective. Any command

string can be sent by just entering it on the command line when

the program is called.

For example:

MODEMSET 1 1200 ATE1V1X6 [i5]

The first parameter after the filename is the port #. This

can be any number from 1 to 4. Next is the speed at which the

serial port is opened. This can be from 300 to 19200. Following

that is the initialization string. You must include the "AT"

command in the command line followed by your choice of modem

commands. Make sure the initialization string is in upper case

letters and that there are no breaks in the modem string (just as

illustrated). Some of the newer modems don't require the modem

string to be in upper case but if you're not sure about your

modem, you should use upper case. The string is printed on the

RITETIME  Version 1.5   01/24/94

Page 6

screen to show what information is being sent to the modem, then

the program waits for the message to be returned from the modem.

If the message says "OK", following the command, the string was

accepted by the modem. If the message says "ERROR", the string

was not a proper modem command.

After the string is sent to the modem, the program will

reset the serial port to its previous settings, and return to the

operating system.

XT Multi-Function Clocks

������������������������

If you have an XT type computer as opposed to an AT type

computer, there is a utility called TADXT.COM which will transfer

the computer system time over to the multi-function card's Real-

Time-Clock accurately and automatically. You would run this

utility after successfully running RITETIME. This program may or

may not work depending on your clock type. As well, you can try

your current clock set utility to see if it will store the new

time.

Hayes Compatibility

�������������������

What is meant by a 100% Hayes compatible modem?  It means

that the modem has the ability to accept and respond to the

software commands of the standard Hayes 'AT' protocol. You can

check your modem manual to see if it is compatible... most of the

new type modems do adhere to this standard. Because the Hayes

protocol is becoming so popular, it makes no sense to make RITE-

TIME try to conform to all different types of non-compatible

modems. If your modem doesn't conform to the 'AT' command set,

I'm afraid your going to be out of luck with RITETIME.

Again... check your modem manual if you're not sure about

the compatibility.

In Case of Difficulty

���������������������

In the event you have a problem with RITETIME or Modemset

please do not call Leitch Video. Although they know about this

program, it is not their responsibility to support it. Please

leave a message for the author on CompuServe to 72677,3641.

TADXT.COM is written by Brian Olender.

XT and AT are trademarks of International Business Machines.

Hayes and the 'AT' command set are registered trademarks of Hayes

Micro Computer Inc.

RITETIME  Version 1.5   01/24/94

Page 7

History

�������

Version   Changes - RITETIME

�������   ����������������������������������������������������

1.5     January 24, 1994

RITETIME now in "EXE" format.

Batch file to convert to "pulse" dialing not

included because of "EXE" format.

Fixes added to correct for time zone offsets across

midnight. Correct date and time now set.

Software control added for IRQ2 and IRQ5 hardware

option for all ports.

Serial ports now returned to previous settings

rather than an absolute close.

1.4     Oct. 1, 1991

Added "AT" command "ATW0" to the modem initialization

to disable negotiation result codes for ATI modems.

1.3     May 1, 1990

Corrected bug for release of RTS with DTR in communi-

cation routine.

Added a message to announce forced disconnect if it

was necessary to drop DTR.

1.2     October 8, 1989.

Wait time increased from 45 seconds to 60 seconds.

Countdown time displayed on screen.

Screen write routines directed to console rather

than stdio.

1.1     July 21, 1989.

Time Zone support with offset value entered on

command line.

Program checks to see if carrier lost 1 sec. after

hang up command sent.

1.0     June 15, 1989.

Original release.

Version   Changes - MODEMSET

�������   ����������������������������������������������������

1.3    January 24, 1994.

Modemset now in "EXE format.

Software control added for IRQ2 and IRQ5 hardware

option for all ports.

RITETIME  Version 1.5   01/24/94

Page 8

Serial ports now returned to previous settings

rather than an absolute close.

Minimum baud rate changed to 300.

1.2    May 1, 1990.

Corrected bug for release of RTS with DTR in communi-

cation routine.

Added a message to announce forced disconnect if it

was necessary to drop DTR.

1.1    October 8, 1989.

Screen write routines directed to console rather

than stdio.

Communication routines rewritten to improve

reliability of data returned from modem.

1.0     June 15, 1989.

Original release.

RITETIME  Version 1.5   01/24/94