Maidenhead Grid Square Locator

By Paul Maserang (N5PCA)

The Maidenhead Grid Square system is a method of stating any position on the

globe.  The full 6-character grid square takes the form of two letters, two

numbers, and two more letters (ie. EM02DK is the same as latitude 32�25'N,

longitude 99�41'W).  A grid square co-ordinate is obviously shorter and easier

to state than latitude and longitude.

A 6-character grid square co-ordinate is accurate to 1/12th of a degree (0�05')

of longitude by 1/24th of a degree (0�02'30") of latitude, which works out to

2.5 nautical miles north-to-south by 5.0 nautical miles east-to-west at the

equator.  A 4-character grid square is accurate to 1 degree of longitude by 2

degrees of latitude, and a 2-character grid square is accurate to 10 degrees of

longitude by 20 degrees of latitude.  The origin of the grid square co-ordinate

system is the south pole (latitude 90�S, longitude indeterminate).

This program is designed to convert latitude/longitude positions to Maidenhead

Grid Square co-ordinates, and vise versa.  Usage is fairly straight-forward,

but there are a few things to be aware of:

(1) Latitude and longitude must be entered in degrees and minutes (DD.MM),

not degrees and decimal fractions thereof.  For latitudes, DD can range

from -90 to 90; for longitudes, DDD can range from -180 to 180; and in

either case, MM can be 00 to 59.  The program checks for values outside

the -90 to 90 and -180 to 180 ranges, but does NOT check for invalid

MM entries (MM>59), so be careful with your latitude/longitude entries.

MM values greater than 59 are reduced modulo 60, with the excess carried

into the DDD portion (ie. an entry of 0.90 will become 1.30, which is

equivalent to 1.5�).

(2) SOUTH latitudes and EAST longitudes must be entered as NEGATIVE numbers

(ie. for latitude 32�25'S, your entry should be -32.25).

(3) Grid square entries may consist of the first two characters, first four

characters, or all six characters; short entries will be optimized to

the center of the square before computing the corresponding latitude and

longitude.  Any grid square entry containing an odd number of characters

or is longer than six characters is considered invalid.

(4) In all cases, an invalid entry (other than MM described above) causes

the program to return to the main menu.

(5) When converting a grid square to latitude/longitude, keep in mind that

the resulting latitude can be off by as much as 3 minutes of arc, and

the longitude off by as much as 5 minutes of arc, due to the resolution

limitations of the grid square system (described above).  However, this

translates to a maximum error of only 5 nautical miles in any direction

from the correct position, which is negligible for most purposes,

including amateur radio work.