VERSAMAP (version 1.02)

21 May 1991

Copyright 1991 by Charles H. Culberson

8 Ritter Lane

Newark, Delaware  19711

CONTENTS                                               PAGE

1.  Introduction                                             2

2.  Computer Requirements for Running VERSAMAP               2

3.  Disclaimer                                               2

4.  Registration                                             3

5.  Mapping the World with VERSAMAP                          4

6.  Finding your Way with Latitude and Longitude             5

7.  Keys with Special Functions in VERSAMAP                  6

a. Esc                                                   6

b. Alt                                                   6

c. Exit the program                                      6

d. Using a mouse                                         6

8.  Running VERSAMAP                                         7

a. VERSAMAP Defaults                                     7

b. VERSAMAP Pull-down Menus                              7

c. Text Mode Screen Menus                                8

1. Projection menu                                    8

2. Graticule menu                                    11

3. Input/Output menu                                 14

4. Run menu                                          18

d. Graphics Mode Screen Menu                            18

1. Options menu                                      18

9.  Sources of Digital Map Data for use with VERSAMAP       20

a. World Data Bank I                                    20

b. Micro World Data Bank II                             20

c. The World Digitized                                  21

d. Cartog                                               22

e. User derived digital map data                        22

1. User Derived Data Record Format                   22

10.  Use of VERSAMAP with Graphing Programs                  23

11.  Program Compatibility                                   24

12.  Acknowledgements                                        24

VMAP.DOC                                                 Page  1

1.  Introduction

Welcome to VERSAMAP!  VERSAMAP is a versatile mapping program which

can transform spherical latitude, longitude values into planar x,y

map coordinates and plot them on a map projection of your choice.

VERSAMAP can print the resulting map on the screen, on dot matrix or

laser printers, as a 'PCX' file for use with paint programs, or as

an ASCII text file for use in business and technical graphing

programs.

The VERSAMAP disk(s) should contain the following files:

VMAP.DOC  --  this instruction manual

VMAP.EXE  --  the mapping program

VMAP.MAP  --  a digital map of the world (14,000 points)

VMAPREAD.ME   --  how to get started

VMAP_FIX.MPS  --  a small file used to concatenate World

Digitized binary map files

VMAP_FIX.MP1  --  a small file used to concatenate World

Digitized ASCII map files.

REGISTER.DOC  --  registration form

Please make backup copies of these files before running the program.

2.  Computer Requirements for Running VERSAMAP

MS-DOS 2.1 or higher;

CGA, MCGA, EGA, or VGA video display adapter and

color monitor;

1 disk drive (With a single 360K disk drive your

ability to write files to disk will be limited.);

512K memory;

a dot matrix or laser printer is optional;

a mouse and a math coprocessor are optional.

VERSAMAP will not change your AUTOEXEC.BAT or CONFIG.SYS files, it

will not change your keyboard settings, and it will not write any

files to disk without your permission.

3.  Disclaimer

Users of VERSAMAP must accept this disclaimer of warranty:

"VERSAMAP is supplied as is.  The author disclaims all warranties,

expressed or implied, including, without limitation, the warranties

of merchantability and of fitness for any purpose.  The author

assumes no liability for damages, direct or consequential, which may

result from the use of VERSAMAP."

VMAP.DOC                                                 Page  2

4.  Registration

Shareware distribution gives users a chance to try software before

buying it.  VERSAMAP is a "shareware program" and is provided at no

charge to the user for evaluation.  If you find this program useful

and find that you continue to use VERSAMAP after a reasonable trial

period, you must register the program.  The registration fee will

license one copy for use on any one computer at any one time.  You

must treat this software just like a book.  An example is that this

software may be used by any number of people and may be freely moved

from one computer location to another, so long as there is no

possibility of it being used at one location while it's being used

at another.  Just as a book cannot be read by two different persons

at the same time.

Registration of the program is $15, for which you will receive a

copy of the most recent version of the program plus 2 additional

digital map files.  The additional world maps, MWDB5.ALL (4,000

points) and MWDB3.ALL (26,000 points) allow faster or more detailed

maps to be plotted.  Registration also entitles you to mail and

telephone support concerning problems you encounter with VERSAMAP.

A registration form is included in the file REGISTER.DOC.

The registration form includes space for comments about the program.

I would appreciate your comments about VERSAMAP even if you do not

register your copy.

You are encouraged to share a copy of VERSAMAP with your friends for

evaluation.  Please encourage them to register their copy if they

find it useful.

Anyone distributing VERSAMAP for any kind of remuneration must first

contact Charles Culberson at the address on the registration form

for authorization.  This authorization will be automatically granted

to distributors recognized by the (ASP) as adhering to its

guidelines for shareware distributors, and such distributors may

begin offering VERSAMAP immediately.  However Charles Culberson must

still be advised so that the distributor can be kept up-to-date with

the latest version of VERSAMAP.

This program is produced by a member of the Association of Shareware

Professionals (ASP).  ASP wants to make sure that the shareware

principle works for you.  If you are unable to resolve a

shareware-related problem with an ASP member by contacting the

member directly, ASP may be able to help.  The ASP Ombudsman can

help you resolve a dispute or problem with an ASP member, but does

not provide technical support for members' products.  Please write

to the ASP Ombudsman at 545 Grover Road, Muskegon, MI 49442 or send

a CompuServe message via CompuServe Mail to ASP Ombudsman

70007,3536.

VMAP.DOC                                                 Page  3

5.  Mapping the World with VERSAMAP

Before reading the rest of the manual, let's plot a map of the world

on the screen.  This will familiarize you with the operation of

VERSAMAP.  The rest of the manual should make more sense once you

have actually used VERSAMAP.

VERSAMAP has built in defaults which plot a world map, from 80 N to

80 S latitude, and from 180 W to 180 E longitude, on a Normal

Mercator projection.  The default settings will not print any files

to disk.

To run VERSAMAP with the default settings:

1.  place the VERSAMAP disk containing the files,

VMAP.EXE and VMAP.MAP, in your disk drive

2.  change to that drive

3.  type VMAP

4.  press the Enter key.

A screen with a menu bar across the top will appear once the program

has loaded.  The pull-down menus are selected by pressing the Alt

key, followed by the highlighted letter of the desired menu; or by

pointing and clicking the left mouse button.

To plot the world map:

1.  press the Alt key, to access the menus

2.  press R, to select Run menu

3.  press G, to begin the plot.

To plot the default map, the map file, VMAP.MAP, must be in the

current directory.

When the map begins plotting, the top line of the screen will

display the name of the map input file being read, VMAP.MAP in this

case, and the number of points which have been plotted.

When the map is finished plotting, the top line will display three

items: Options, a pull-down menu; Projection, the map projection

used to plot the map; and a reminder that pressing Alt+O opens the

options menu.

When the map is finished:

1.  press the Alt key, to access the menu

2.  press O to select the Options menu

3.  press M from the Options menu to erase

the map and return to the main menu.

VMAP.DOC                                                 Page  4

To exit the program, you must be in the main menu:

1.  press the Alt key, to access the menus;

2.  press P, to select the Projection menu;

3.  press X from the Projection menu to exit the program.

Pressing the Esc key at any time will halt program execution and

return you to the menu bar.  Esc will not exit the program.

Please read the rest of the instruction manual before attempting to

plot maps which do not use the default parameters.

6.  Finding your Way with Latitude and Longitude

To use this program you should be familiar with the concepts of

latitude and longitude.

Latitude is the angular distance (in degrees) of a point on the

earth's surface north or south of the equator.  The maximum values

of latitude are 90 N (north pole) and 90 S (south pole).  The

equator is 0  latitude.

Longitude is the angular distance (in degrees) of a point on the

earth's surface east or west of the meridian that passes through

Greenwich, England.  The maximum values of longitude are 180 W and

180 E (the international date line).  The meridian passing through

Greenwich is 0  longitude.

Latitude and longitude values in VERSAMAP are entered as degrees,

minutes, seconds.  There are 60 seconds in 1 minute, and 60 minutes

in 1 degree.  One minute of latitude is equal to 1 nautical mile (1

nautical mile = 6076 feet = 1.852 kilometers = 1.1508 statute

miles).

If you are unfamiliar with the use of latitude and longitude to

locate points on the surface of the earth, please consult an

encyclopedia or an atlas.

A easily understandable introduction to the theory of map

projections is the book:

Porter W. McDonnell.  1979.  Introduction to Map Projections.

Marcel Dekker, New York.

A detailed discussion of the map projections used in this program,

showing actual plots, is given in:

John P. Snyder and Philip M. Voxland.  1989.  An Album of Map

Projections.  U.S. Geological Survey Professional Paper 1453.

U.S. Government Printing Office.  249 pages.  $14.

VMAP.DOC                                                 Page  5

7.  Keys with Special Functions in VERSAMAP

Esc

Pressing the Esc key at any time will halt program execution and

return you to the menu bar.  Esc will not exit the program.

If you are in the process of plotting or printing a map, the program

checks the Esc key each time the record number or point, shown in

the upper right corner of the screen, is updated.  Consequently, the

program may take a few seconds to respond after Esc is pressed.

Alt

Pull-down menus are selected by pressing the Alt key, followed by

the highlighted letter of the desired menu.  Individual menu items

are selected by pressing the highlighted letter of each item; by

moving through the menu with the up or down cursor keys, and then

pressing Enter to select the item; or by moving the mouse cursor

over the desired item and pressing the left mouse button.

Exit the program

To exit the program, you must be in the main menu (labeled

Projection in the upper left corner):

1.  if the first item on the menu bar is Options, you are

in the graphics menu; press Alt, then O, then M

to return to the main menu

2.  if the first item on the menu bar is Projection, you

are in the main menu; press Alt, then P, then X

to exit the program.

Using a mouse

Most VERSAMAP features can be accessed by pointing and clicking the

left mouse button.  The only cases in which a mouse cannot be used

are those requiring numerical or alphabetical input: such as the

latitude/longitude boundaries of the map, and filename input.

Pull down menus can be closed by clicking the left mouse button when

the mouse cursor is outside the window.  Windows with an equal (=)

sign in the upper left corner, can be closed by clicking on the

equal sign.

VMAP.DOC                                                 Page  6

8.  Running VERSAMAP

VERSAMAP Defaults

Each map projection in VERSAMAP has built in latitude/longitude

defaults which are active the first time you access the projection

during each session with VERSAMAP.  These defaults allow you to plot

maps using each map projection without getting error messages.  The

defaults are only active the first time you access a particular map

projection.  For instance, if you access the Mercator projection,

then the Equirectangular projection, and finally return to the

Mercator projection, the latitude limits of the grid with be those

of the previous Equirectangular projection, 90 N/S, and you will get

an error message if you attempt to plot a Mercator projection

without changing the graticule.

VERSAMAP Pull-down Menus

The following paragraphs provide a detailed discussion of each of

the VERSAMAP pull-down menus.

For your first experience with VERSAMAP you should run the program

with its default settings.  See Section 5 above.

If your system is limited to a single floppy disk drive and you wish

to print map files to disk, it would be wise to copy your map

boundary input file(s) onto a blank disk.  This will allow larger

map output files to be written to disk.  Once VERSAMAP has loaded in

a single floppy system, remove the disk containing the map program,

VMAP.EXE, and insert the map boundary file(s) disk.

When VERSAMAP is started, the following menu bar appears at the top

of the screen.  This is a text mode screen with a solid, rectangular

mouse cursor.

-----------------------------------------------------------------

Projection      Graticule      Input/Output      Run

-----------------------------------------------------------------

When you begin plotting a map, by selecting the Go option in the Run

menu, the display switches to a graphics mode screen with the

following top line.  This line is not a menu bar; it is for

informational purposes only.  The current input filename is

displayed, and the record number being read is continually updated.

-----------------------------------------------------------------

Input file:          Esc: goto menu          Reading record:

-----------------------------------------------------------------

VMAP.DOC                                                 Page  7

Once the map is completed, the following menu bar appears at the top

of the graphics screen, and the graphics mouse cursor (solid arrow)

is shown.

-----------------------------------------------------------------

Options           Projection:                      Alt+O: menu

-----------------------------------------------------------------

Options is the only menu item on this menu bar; the map projection

used to plot the map is listed after Projection.  Pressing the

letter T toggles between the Alt+O reminder and the time taken to

plot the map.

Text Mode Screen Menus

Projection menu

-----------------------------------------------------------------

Projection      Graticule      Input/Output      Run

-----------------------------------------------------------------

| Mercator                      |

| Equirectangular               |

| Equidistant conic             |

| Lambert conformal conic       |

| Albers equal-area             |

| Gnomonic                      |

| Lambert azimuthal equal-area  |

| Hammer (-Aitoff) equal-area   |

|-------------------------------|

| Exit                          |

|-------------------------------|

This menu lists the map projections on which you can plot your map.

The Gnomonic and Lambert azimuthal equal-area projections allow you

to plot your data in polar or equatorial aspects.

Mercator

On this projection, meridians and parallels are parallel lines

perpendicular to each other.  Meridians are equally spaced,

parallels are unequally spaced.  The north and south poles cannot be

plotted because they project to infinity.

The scale is true along the equator, or along two parallels

equidistant from the equator.  Since the scale increases rapidly

away from the equator, the areas of polar regions are greatly

exaggerated compared to those of equatorial regions.

VMAP.DOC                                                 Page  8

The Mercator projection is commonly used for maps of the world, and

is designed so that straight lines on the map are rhumb lines (lines

of constant compass bearing).  This is a useful property in

navigation and many nautical charts use Mercator projections.

Equirectangular

Meridians and parallels are equally spaced parallel lines

perpendicular to each other.  The scale is true along the two

standard parallels and along all meridians.  The scale along

parallels is too small between the equator and the standard

parallels, and too large between the standard parallels and the

poles.

The map can be visualized as projected on a cylinder which

intersects the globe at two standard parallels equidistant from the

equator.  VERSAMAP allows the user to select the latitude of the

standard parallels.

The fact that meridians and parallels are equally spaced makes

interpolation easy.  The projection is useful for maps of small

regions, since distortion is not large if one of the standard

parallels is chosen to bisect the region under consideration.

Equidistant Conic

Meridians are equally spaced lines converging at a point beyond one

of the poles.  Parallels are equally spaced circular arcs centered

on the point where the meridians converge.  The poles are circular

arcs of infinite scale.  Distortion on the projection is

intermediate between that of equal-area and conformal conic

projections.

The scale is true along meridians and along two standard parallels.

VERSAMAP allows the user to select the latitudes of the standard

parallels.

Lambert Conformal Conic

Meridians are equally spaced lines converging at a point, which is

one of the poles.  Parallels are unequally spaced circular arcs

centered on the point where the meridians converge.  The pole

opposite the point of convergence cannot be shown.

The scale is true along two standard parallels, which VERSAMAP

allows the user to select.  The projection in conformal: the scale

is the same in all directions at a given point.

VMAP.DOC                                                 Page  9

Albers Equal-Area Conic

Meridians are equally spaced lines converging at a point beyond one

of the poles.  Parallels are unequally spaced circular arcs centered

on the point where the meridians converge.  The poles are shown as

circular arcs of infinite scale.

The scale is true along two standard parallels, which VERSAMAP

allows the user to select.  Since the projection is equal area, the

scale at any point on a meridian is the reciprocal of the scale

along a parallel at that point.

Gnomonic

The primary feature of gnomonic projections is that great circles

plot as straight lines.  Thus the shortest distance between two

points on this map is the straight line connecting them.

VERSAMAP allows the user to select either the POLAR or EQUATORIAL

aspect of this projection.

In the polar aspect, meridians are equally spaced straight lines

converging at the pole.  Parallels are unequally spaced circles

centered on the pole.  The equator and the opposite pole cannot be

shown.

In the equatorial aspect, meridians are unequally spaced parallel

lines.  Only meridians within 90  of the central meridian can be

shown.  The equator is a straight line; other parallels are arcs

concave toward the nearest pole.  The poles cannot be shown.

To allow more detail to be shown, VERSAMAP truncates the cusps of

the equatorial gnomonic projection, and plots the map in the form of

a rectangle.  For maps which span the equator, the maximum and

minimum latitudes of the grid are represented by points on the

central meridian at the top and bottom margins of the plot.  For

maps which lie north of the equator, the maximum latitude is

represented by a single point on the central meridian at the top of

the plot.  For maps lying south of the equator, the minimum latitude

is represented by a single point on the central meridian at the

bottom of the plot.

The scale of the projection is only correct where the central

parallel crosses the central meridian.

VMAP.DOC                                                 Page  10

Lambert Azimuthal Equal-Area

VERSAMAP allows the user to select either the POLAR or EQUATORIAL

aspect of this projection.

In the polar aspect, meridians are equally spaced straight lines

converging at one of the poles.  Parallels are unequally spaced

circles centered on the pole.  The opposite pole is a circle.

In the equatorial aspect, the central meridian is a straight line.

Other meridians are curves concave to the central meridian and

intersecting at the poles.  The equator is a straight line; other

parallels are arcs concave toward the nearest pole.  VERSAMAP

requires that the range of longitude for the equatorial aspect of

this projection be no greater than 180  (one hemisphere).

The scale of the projection is only correct at the center of the

projection.

Hammer (-Aitoff) Equal-Area

This projection is useful for equal area maps of the entire world.

The central meridian is a straight line.  Other meridians are curves

concave to the central meridian and intersecting at the poles.  The

equator is a straight line; other parallels are arcs concave toward

the nearest pole.

The scale of the projection is only correct along the equator and

the central meridian.

Graticule menu

-----------------------------------------------------------------

Projection      Graticule      Input/Output      Run

-----------------------------------------------------------------

| Map Boundaries     |

| Standard Parallels |

| Graticule Type     |

|--------------------|

This menu allows you to define the boundaries of the map, select the

standard parallels if any, and to define the type of grid to be

superimposed upon the map.

VMAP.DOC                                                 Page  11

Map Boundaries

After selecting this item, a window appears in which the latitude

and longitude limits of the map projection are entered.  Data

entered on this screen define the area to be plotted, and the

spacing of latitude and longitude grid marks on the finished map.

Once you have completed the entire screen, the program checks for

errors in the latitude/longitude limits of your map projection, and

for errors in grid spacing. If the program detects an error it

prints an error message on the screen and requires you to correct

the mistake.

For instance, the program will not allow a Normal Mercator

projection to be plotted if the Northernmost latitude is 90 N (north

pole) or if the Southernmost latitude is 90 S (south pole).  Try

entering 90 N as the Northernmost latitude limit for a Normal

Mercator projection.  In this case, an error message will appear

telling you to edit the data.

VERSAMAP requires that the grid spacing you select yield equally

spaced divisions between the northern- and southernmost latitudes,

and between the western- and easternmost longitudes.  You will get

an error message, requiring you to edit the data, if the DEGREES

BETWEEN GRID MARKS that you have chosen does not yield equally

spaced divisions along the axes.

If you get an error message, you must correct the error before you

can leave the window!

Standard Parallels

After selecting this item, a window appears in which the latitudes

of the standard parallels are entered.

Conic Map Projections:

The conic projections in this program (equidistant conic, Lambert

conformal conic, Albers equal-area) require that you select 2

standard parallels of latitude.  The scale of the map is true along

the standard parallels; the spacing of the parallels determines the

angle between meridians on the map.

There are 2 constraints on selection of the standard parallels:

1. they must lie between the northern- and southernmost

latitudes of your grid; and

2. they cannot be symmetrical about the equator.  Standard

parallels at 20 N and 20 S are not allowed.

VMAP.DOC                                                 Page  12

The program places default values for the standard parallels at 1/6

and 5/6 of the distance between the northern- and southernmost

latitudes of the grid.

When this screen appears, the cursor will be in the first position

of the degrees column of the northernmost standard parallel.  If the

default values are acceptable, press Esc to continue.

To change the values, enter a new value in the column being edited

and press Enter.  The cursor will skip to the next column.  Enter a

new value if desired and press Enter.  Continue until you have

edited the last value.  When you press enter in the last column (the

seconds column of the southernmost standard parallel), you will have

the option of re-editing the screen.

If you attempt to enter standard parallels which are equally spaced

about the equator, the program will print an error message and force

you to change the latitude of one or both of the parallels.

Equirectangular Map Projection:

This projection requires 2 standard parallels at equal distances

north and south of the equator.  You may only select the northern

standard parallel.  The other parallel will automatically be placed

in the southern hemisphere at an equal distance from the equator.

The program places default values for the standard parallels at 1/2

the distance between the northern- and southernmost latitudes of the

grid if the northern- and southernmost latitudes lie in the same

hemisphere.  If the northern- and southernmost latitudes of the grid

lie in opposite hemispheres, then the standard parallels are placed

at 1/2 the distance between the equator and whichever of the

northern- or southernmost latitudes of the grid lies closest to a

pole.

When this screen appears, the cursor will be in the first position

of the degrees column of the northernmost standard parallel.  If the

default values are acceptable, press Esc to continue.

To change the values, enter a new value in the column being edited

and press Enter.  The cursor will skip to the next column.  Enter a

new value if desired and press Enter.  Continue until you have

edited the last value.  When you press enter in the last column (the

seconds column of the northernmost standard parallel), you will have

the option of re-editing the screen.

VMAP.DOC                                                 Page  13

Graticule Type

This window allows you to select the type of grid that you wish

superimposed on your map.  You have several choices, including

plotting the graticule as points, or with the points connected by

lines.  The last item will plot your map without any latitude and

longitude markings whatsoever.

To get a quick view of the appearance of the different grid choices,

select a grid type and then enter a null string (i.e. no file

specification) for the map input file path name.  In this case, the

program will plot the graticule, but no map, on the screen.

Input/Output menu

-----------------------------------------------------------------

Projection      Graticule      Input/Output      Run

-----------------------------------------------------------------

| Map Input Filenames   |

| Map Output Filenames  |

| Grid Output Filenames |

| Report - screen 1     |

|        - screen 2     |

|        - screen 3     |

|-----------------------|

This menu allows you to define the input map boundary files that you

want to plot, to select pathnames for output files that can be

exported to technical graphics programs, and to display several

report screens.

Map Input Filenames

After selecting this item, a window appears in which the names of

the input boundary files are entered.

Up to 9 map boundary input filenames may be entered.  The map

boundary input files are the files which contain the

latitude/longitude coordinates that you wish plotted on your map.

The default file name for the first input file is VMAP.MAP in the

current directory.  VMAP.MAP is the world map included on the

VERSAMAP disk.  The default file names of the remaining 8 map input

files are null strings (i.e. no file name).

When the window is first displayed, the cursor is located at the

beginning of the first input file name.  Type in the correct path

name and press Enter to accept the path name.  The cursor will jump

to the column labeled style on the same line as the path name.  The

style determines whether your data is plotted as points connected by

lines, or as individual dots.  If you enter the letter L in the

VMAP.DOC                                                 Page  14

style column, all points on a particular line segment will be

connected by straight lines.  If you enter the letter D, the

latitude/longitude data will be plotted as individual points, and

points in the same line segment will not be connected by lines.  The

default style is L.

Once you have entered the style for the first input file, the cursor

will jump to the next line, and will display the current path (the

drive and current directory).  If you wish to input a second map

boundary file, type in its name and press Enter.  Continue until you

have entered all of your map input files.

Once you have entered the style of the last map input file that you

wish plotted, press Enter to move to the next line.  Erase the

current path and then press Enter again.  Entering a blank file name

stops further input of map boundary input files.

The program does not require any input map boundary files.  If you

do not wish to input data, erase the default path name of the first

input file and press Enter.  In this case, only the map graticule

(if any) will be plotted on the screen.

VERSAMAP will accept map boundary input files from the following

public domain, shareware, and commercial digital map databases:

World Data Bank I, Micro World Data Bank II, The World Digitized,

and Cartog.  In addition, VERSAMAP will accept user derived data in

an ASCII text format.  Section 9 contains a detailed discussion of

the various map databases that VERSAMAP recognizes.

VERSAMAP recognizes the various databases by their file name

extensions.  The file names that you enter on this screen must have

one of the extensions given in Section 9.  However, the names that

you enter can have different extensions.  For instance, your first

file name could be from the CARTOG database with the extension BIN,

your second file name could be from the WORLD DIGITIZED database

with the extension MPS, and your third file name could be from the

MICRO WORLD DATA BANK II database, with the extension, ALL.

Map Output Filenames

After selecting this item, a window appears in which the names of

the output boundary files are entered.

If you wish, VERSAMAP will transform the latitude/longitude

coordinates from the above map boundary input files into planar x,y

map coordinates and output them to disk.  These output files can be

imported into graphics programs for the production of publication

quality maps.  The use of the output map files in graphics programs

is discussed in Section 10.

The program will output up to 9 files, one for each of the map

boundary input files.  You may choose any path names for these

files.

VMAP.DOC                                                 Page  15

Output files are not required by VERSAMAP.  If you do not want to

print a particular output file to disk, erase any characters in the

file name, and press Enter.

To enter an output file name, simply type in the path name and press

Enter.  The cursor will then move to the next output file name.

Each output file corresponds to the input file of the same number.

For instance, if you have 5 input files, the fifth output file will

contain the transformed latitude/longitude coordinates from the

fifth input file.

If you decide to print a particular output file, you do not need to

print the output files corresponding to the other input files.  For

instance, if you have 3 input files, you can print the output files

corresponding to the first and third input files, but not the second

input file.  Input a null string for any output file that you do not

want to print to disk (i.e. erase any characters in the file

specification and press Enter).

Data records in the map output files have the following format.

X coordinate | space (ASCII 32) | Y coordinate |

carriage return (ASCII 13) | line feed (ASCII 10)

Data in most digital map databases is organized as line segments;

each line segment corresponds to a series of points which can be

connected by straight lines.  In the map output files, the beginning

of each new line segment is indicated by a dummy point in which the

x coordinate is equal to 5.000000 (5 times the maximum x coordinate

of 1.000000) and the y coordinate is 5 times the maximum y

coordinate.  In a map output file, any point with an x coordinate of

5.000000 is a dummy point which indicates the beginning of a new

line segment, and which will plot outside the boundaries of the map.

Grid Output Filenames

After selecting this item, a window appears in which the names of

the output graticule boundary files are entered.

If you wish, VERSAMAP will transform the latitude/longitude

coordinates of the grid type that you have selected into planar x,y

map coordinates and output them to disk.  These output grid files

can be imported into graphics programs for the production of

publication quality maps.  The use of the output grid files in

graphics programs is discussed in Section 10.

The program will output 1 or 2 grid files, depending on the

graticule type that you chose.  You may choose any path names for

these files.

VMAP.DOC                                                 Page  16

Output grid files are not required by VERSAMAP.  If you do not want

to print a particular grid file to disk, erase any characters in the

file name, and press Enter.

To enter a grid file name, simply type in the path name and press

Enter.  The cursor will then move to the second grid file name (if

there is one), or to the next screen.

In most cases, a single grid file will be written to disk.  The only

exception, the OUTSIDE BOUNDARY PLUS INTERIOR POINTS grid type,

writes two files to disk: the first file contains the interior

points, the second the outside boundary.

Grid files have the same record format as the map boundary files

listed in the previous section.

Report - screens 1, 2, 3

The Versamap Parameter Report is a series of 3 screens listing

various features of your map.

Screen 1

This screen lists general information about the map: the projection

type, the grid type, the latitude/longitude limits of the map, and

the maximum/minimum values of the transformed x,y map coordinates.

The X-axis Length and Y-axis Length, listed under Relative Length of

Axes at the bottom right of the screen, are used to scale maps

produced by exporting the map output and map grid files to technical

and business graphics programs.  The use of these two parameters is

described in Section 10.

Screen 2

This screen lists map input, map output, and grid file names.

Screen 3

This screen is an error report which lists scale and area errors at

selected points on the map.

VMAP.DOC                                                 Page  17

Run menu

-----------------------------------------------------------------

Projection      Graticule      Input/Output      Run

-----------------------------------------------------------------

| Go       |

|----------|

This menu contains only one item, Go.  Your map will begin plotting

once Go is selected.  The program will jump to the graphics menu bar

if Esc is pressed while the map is being drawn.

Graphics Mode Screen Menu

Options menu

-----------------------------------------------------------------

Options           Projection:                      Alt+O: menu

-----------------------------------------------------------------

| Restore Screen |

| Print Map      |

| Define Printer |

| Save as PCX    |

|----------------|

| Main Menu      |

|----------------|

The Options menu bar is displayed after a map has been plotted.

Options is the only menu item on this menu bar.  Projection and the

Alt+O reminder are for your information only.

On CGA and MCGA monitors only two colors are available to plot the

map and to display information.  Since only two colors are

available, menu selections on CGA and MCGA monitors are indicated by

changing the menu item from mixed upper and lower case to all

capitals.  As you use the cursor keys to move through the menu, the

currently selected item is highlighted in upper case.  Pressing the

first letter in each menu item, or clicking the left mouse button on

that item, will also select the item.

Restore Screen

Selecting 'Restore Screen' erases the menu window and displays the

entire map.

VMAP.DOC                                                 Page  18

Print Map

Selecting 'Print Map' allows you to select the dimensions and page

placement of the printed map.  When you select this item, a window

titled 'MAP PLACEMENT ON PAGE IN INCHES' appears.

The default settings place the map in the center of the page, and

set the largest dimension (width or height) at 6 inches.  The width

and height of the printed map are not independent.  If you adjust

one dimension, the program automatically fixes the other.  Maps will

be printed without any descriptive text, such as latitude or

longitude numbers.  The grid spacing, and other information about

the printed map, can be obtained from Report Screens 1, 2, and 3 in

the Input/Output menu on the Main menu bar.  The Options menu bar

will not be shown on the printed map.

Define Printer

Before you print a map, you must define the type of printer which

will print the map.  When you select 'Define Printer', a window

titled 'SELECT PRINTER TYPE AND PORT' appears.  You may select one

of three printer types: Epson 9- or 24-pin dot matrix, or Hewlett-

Packard LaserJet II.  If your printer is not compatible with one of

these printers, select None.

You may also select the printer port.  Lpt1 is the default.

Save as PCX

Before saving the map as a .PCX paint file, you must specify the

filename and path of the file.  After selecting this item, a window

titled 'SELECT .PCX FILENAME' appears.  Type in the correct filename

and press Enter.  The cursor will jump to the '< Save PCX file >'

button after you have entered the filename.  If the filename is

correct, press Enter in the '< Save PCX file >' button.  The program

will prompt you if the file already exists. The menu bar is not

saved as part of the file.

To change the filename, tab to the '< Change filename >' button and

press Enter.  To erase this window and restore the map, tab to the

'< Cancel >' button and press Enter.

Maps on EGA and VGA monitors will be exported as white lines on a

blue background, just as they are displayed on the VERSAMAP screen.

When you print these maps from within a paint program, you may want

to print them as black lines on a white background.  You can do this

by using the color eraser tool of your paint program to change the

white VERSAMAP lines to black, and to change the blue background to

white.

VMAP.DOC                                                 Page  19

Maps will be exported without any descriptive text, such as latitude

or longitude numbers.  The grid spacing, and other information about

the exported map, can be obtained from Report Screens 1, 2, and 3 in

the Input/Output menu on the Main menu bar.

PCX files produced by VERSAMAP have been successfully imported into

Microsoft Paintbrush and PC Paintbrush IV Plus.

Main Menu

Selecting 'Main Menu' returns you to the text mode main menu bar.

The map will be erased and lost when you select this item.

9.  Sources of Digital Map Data for use with VERSAMAP

VERSAMAP will generate maps from the following databases, available

commercially, as shareware, or in the public domain.  In addition,

the program will accept user derived data in a simple ASCII format.

VERSAMAP recognizes the various databases by their file extensions.

The two most complete databases are Micro World Data Bank II and the

World Digitized, both of which are available as shareware.

World Data Bank I

File extension DAT

A database containing 79,000 latitude, longitude points.  Data are

stored as 20 character ASCII records on 5, 5.25 inch, 360K disks

which are available from the National Technical Information Service,

United States Department of Commerce, 5285 Port Royal Road,

Springfield, VA  22161.

Micro World Data Bank II

File extensions PNT, ALL, CCI, MAP

A database in the public domain containing 179,000 latitude,

longitude points.  Latitude and longitude are rounded to the nearest

minute and stored as six byte binary records.  The complete database

is available on 5, 5.25 inch, 360K disks from shareware disk vendors

such as: Big Byte Software, P.O. Box 14008, Arlington, Texas 76094;

Gemini Marketing, Inc., P.O. Box 640, Duvall, Washington 98019; and

The Software Labs, 3767 Overland Ave. #112-115, Los Angeles,

California 90034.

VMAP.MAP, the digital map file on the VERSAMAP disk, and the map

files, MWDB3.ALL and MWDB5.ALL, included with the registered version

of VERSAMAP are taken from Micro World Data Bank II.

VERSAMAP allows you to plot any of the following geographical

features from Micro World Data Bank II files: coastlines, islands,

lakes, rivers, countries, states.

VMAP.DOC                                                 Page  20

The mapping programs, Expert Maps and Finger Maps, contain a digital

map named WORLD.MAP.  This map is a copy of the map file MWDB3.ALL

(26,000 points) from Micro World Data Bank II, and can be used in

VERSAMAP.

The World Digitized

File extensions MPS, MP1

A shareware database containing 100,000 latitude, longitude points

rounded to thousandths of a degree.  The database is available on 3,

5.25 inch, 360K disks (in both binary and ASCII format) from

shareware disk vendors.

The World Digitized database is subdivided into directories, one

directory for each continent.  This useful feature reduces plotting

time since the user can select only those areas he wishes to map.

Each of the World Digitized directories contains 2 or more map

files.  If you wish to plot the entire World Digitized database from

VERSAMAP, you will have to concatenate the individual .MPS(binary)

or .MP1(ASCII) files, since VERSAMAP will not accept more than 9

input files.  The VERSAMAP disks contain 2 files, VMAP_FIX.MPS and

VMAP_FIX.MP1, which must be used to concatenate World Digitized

files for use with VERSAMAP.  These 2 files, VMAP_FIX.MPS and

VMAP_FIX.MP1, force a line break at the end of each World Digitized

file when it is concatenated.

For example, to concatenate the World Digitized binary .MPS files in

the Europe subdirectory for use with VERSAMAP, use the following

form of the DOS copy command:

COPY /B E0.MPS+VMAP_FIX.MPS+E1.MPS+VMAP_FIX.MPS+E2.MPS+

VMAP_FIX.MPS+E3.MPS EUROPE.MPS

where EUROPE.MPS is the concatenated file.

A similar procedure is used to concatenate the World Digitized ASCII

.MP1 files for use with VERSAMAP:

COPY E0.MP1+VMAP_FIX.MP1+E1.MP1+VMAP_FIX.MP1+E2.MP1+

VMAP_FIX.MP1+E3.MP1 EUROPE.MP1

Spurious diagonal lines will be plotted if you do not insert the

VMAP_FIX.MPS or VMAP_FIX.MP1 files between the World Digitized files

when they are concatenated for use with VERSAMAP.

VMAP.DOC                                                 Page  21

Cartog

File extension BIN

The mapping program CARTOG (Byte Magazine, page 329, December 1987)

includes the database WORLDMED.  This database contains 15,000

latitude, longitude points rounded to hundredths of a degree and

stored as 6 byte binary records.  The mapping program and the

WORLDMED database are available on disk from Byte Magazine.

The authors of CARTOG also sell a larger map database, WORLDBIG,

which contains 95,000 points.  Details and price are on the CARTOG

disk.

File structure for user derived digital map data

File extension PRN

In addition to the above file formats, VERSAMAP will also accept

latitude, longitude values in the following ASCII format.  The line

header field is used to indicate the beginning of each new line

segment; -1 indicates the beginning of a new line segment, 0

indicates subsequent points in the line segment.  Latitude and

longitude are given as decimal degrees; negative values represent

South latitude or West longitude.

User Derived Data Record Format

field #1 - line header:  -1 for new line segment

0 for point within line segment

field separator:   one or more spaces (ASCII 32)

field #2 - latitude:   decimal degrees, south latitude is

negative

field separator:   one or more spaces (ASCII 32)

field #3 - longitude:   decimal degrees, west longitude is

negative

end of record:   carriage return (ASCII 13),

line feed (ASCII 10)

Records with this format can be generated by most word processors

and text editors.

VMAP.DOC                                                 Page  22

10.  Use of VERSAMAP with Graphing Programs

VERSAMAP can be used to prepare publication quality maps in

conjunction with suitable graphing programs.  To be useful in

preparing maps, a graphing program should be able to:

a)  handle large data sets;

b)  use more than 1 data set per graph;

c)  physically separate the x and y axes, so that they do

not show on the figure;

d)  connect individual points on the graph with lines; and

e)  exclude data which exceeds the X- and Y-axis limits.

To print a map using a graphing program, import the mapgrid and the

output map boundary files into the program.

If there are two mapgrid files, the points in the first mapgrid file

should be plotted as dots, and the points in the second mapgrid file

should be connected by lines.  If there is only one mapgrid file,

you can connect the grid points by lines or not, depending on how

you want the graticule to appear.

The X- and Y-axes are scaled using the values of X-axis Length and

Y-axis Length listed under Relative Length of Axes on Report Screen

1.  The relative X-axis Length is defined as 1.000000 for all maps;

the relative Y-axis Length may be less than or greater than unity,

depending on the projection and graticule chosen.

Starting values of the X- and Y-axes should be set at 0.  The ending

values of the X- and Y-axes will depend on your graphics program,

but in general they will one or both of the values listed under

Relative Length of Axes on Report Screen 1.

You should configure the graphing program for data clipping (i.e.

exclude points which lie outside the limits of the X- and Y-axes).

This will prevent the dummy points (X = 5.000000), which indicate

new line segments (see Section 8, Map Output Filenames), from being

plotted.

Map and grid output files produced by VERSAMAP have been

successfully imported into the technical graphics programs, Grapher

and SigmaPlot.  For use with SigmaPlot, dummy points (X = 5.000000)

which indicate line breaks in map output files must be converted to

dashes (X = --------), indicating missing data, before the files are

imported into SigmaPlot.  Changing the dummy points from 5.000000 to

-------- can be done using the replace command in Edlin or the

search and replace command in a word processor.

VMAP.DOC                                                 Page  23

11.  Program Compatibility

VERSAMAP has been tested successfully on the following computer

systems.  The last column gives the time required to plot the

default world map, VMAP.MAP, on each system.

Computer         DOS     CPU   Monitor   Disk   Math Co-    Time

Version   MHz             Type   processor   (min)

PC               3.00      5     CGA    Floppy     Yes       4.33

XT               3.30      5     EGA     Hard      Yes       4.24

PS/2 30          3.30      8     MCGA   Floppy     No        9.08

AT               3.30      6     EGA     Hard      No        5.90

PS/2 50          3.30     10     VGA     Hard      Yes       1.02

AT               3.30     12     EGA     Hard      Yes       0.78

386SX            4.01     16     VGA     Hard      Yes       0.85

PS/2 55SX        3.30     16     VGA     Hard      No        2.18

Maps produced by VERSAMAP have been successfully printed on the

following printers.

9-pin dot matrix: Brother M-1709, Okidata u93 Microline,

IBM Proprinter, -Proprinter II.

24-pin dot matrix: Epson LQ-500, LQ-800.

Laser: HP LaserJet-II, -IID, -III.

12.  Acknowledgements

Brother is a registered trademark of Brother Industries, Ltd.  Epson

is a registered trademark of Epson America, Inc.  Expert Maps is a

trademark of Expert Software.  Finger Maps is Copyrighted (c) by

Poisson Technology.  Grapher is a trademark of Golden Software, Inc.

Hewlett-Packard is a registered trademark of Hewlett-Packard

Company.  IBM and PS/2 are registered trademarks of International

Business Machines Corporation.  Microsoft is a registered trademark

of the Microsoft Corporation.  Okidata is a registered trademark of

Oki America Inc.  PC Paintbrush is a registered trademark of ZSoft

Corporation.  SigmaPlot is a trademark of Jandel Corporation.

VERSAMAP uses routines from the PCX Programmer's Toolkit, Copyright

(c) Genus Microprogramming, Inc.  1988-1990.

The boundary file, VMAP.MAP, is the file MWDB4.ALL from the Micro

World Data Bank II, a digital map database placed in the Public

Domain by Fred Pospeschil and Antonio Riveria.

VMAP.DOC                                                 Page  24