FINDTYPE.DOC

Companion document file for FINDTYPE.EXE version 1.1

The FINDTYPE.EXE program is designed to aid in the integration of

a Seagate disc drive into an AT compatible computer system.

This program is designed to benefit those without an extensive

technical background as well as those more experienced systems

integrators.

---------------------------------------

DISCLAIMER

This software is provided, as-is, without warranty of any kind,

either expressed or implied, including, but not limited to, the

implied warranties of merchantability and fitness for a particular

purpose.  In no event shall Seagate Technology, Inc., be held liable

for any loss of profit or any other commercial damage, including but

not limited to special, incidental, or other damages.

Seagate Technology, Inc., retains all rights to the FINDTYPE Program

but does grant the right to make unlimited, unaltered copies of the

FINDTYPE program provided there is no charge to the users.

The three files you should have found within FINDT110.ZIP are:

FINDTYPE.EXE    the executable program

FINDTYPE.DOC    the accompaying document file

DRIVETYP.TXT    a short text file essay on drive types settings

Thank you for your interest in Seagate Technology, Inc.'s FINDTYPE

software.  Please submit your comments and suggestions to the

following address:

Seagate Technology, Inc.

P. O. Box 66360

Att: Technical Support

Scotts Valley, Ca. 95067-0360

For Seagate Technical Support Bulletin Board:

(408)438-8771 [300-9600 HST, MNP 3/5, N-8-1]

---------------------------------------

Swift, Elite, Sabre, MacWren, Wren Runner, Zone Bit Recording,

ZBR, FSD, RSD and FINDTYPE are trademarks of Seagate Technology,

Inc.

Seagate, Seagate Technology, Wren and the Seagate logo are

registered trademarks of Seagate Technology, Inc.

IBM PC/XT/AT, and PC-DOS are registered trademarks of

International Business Machines Corporation.

MS-DOS is a registered trademark of Microsoft Corporation.

DISK MANAGER is a registered trademark of ONTRACK Computer

Systems, Inc.

All registered trademarks and unregistered trademarks are the

sole property of their respective companies.

====================================================================

DESCRIPTION

The purpose of this program is to allow a systems integrator to

install a Seagate disc drive into any AT type computer. The

program reads the system BIOS to determine which drive types are

supported. If an exact match is found, the program will display

the correct type so that this information may be used in the

computer's setup program (SEE EXAMPLE 1).

If an exact match is not found, the program will scan the BIOS

and select a drive type which will most efficiently use the

capacity of the drive being installed. It will also calculate the

percentage of utilization with the selected drive type (see

EXAMPLE 2). The precompensation start track and reduced write

current start track are also compared and if an exact match

cannot be found, the closest one will be selected.

Some BIOS will support a custom or user defined drive type. This

allows the user to input the number of cylinders, heads and

sectors. This CUSTOM drive type may be used when the closest type

supported by the BIOS gives less than 100% utilization of the

drive. The program has the ability to scan the BIOS for support

of CUSTOM drive type. If that support is detected, you will be

prompted to that effect. However, refer to the documentation for

your computer to verify that support and for drive parameter

entry instructions (SEE EXAMPLE 3).

When installing AT interface (IDE) drives with more than 1024

cylinders, using a BIOS that supports a custom drive type, the

program can calculate the optimum standard parameters for the

drive being installed. This is accomplished by typing "CUSTOM" on

the command line after "FINDTYPE" then the Seagate drive model

number (SEE EXAMPLE 4).

If you are installing a drive which is not supported by the BIOS,

and the BIOS does not support the CUSTOM or USER DEFINED drive

type, (SEE EXAMPLE 5) you have four options:

1. Use a controller with a ROM Bios on-board that will support

the interface being used.

2. Replace the BIOS with one that supports the drive or that

supports a CUSTOM drive type.

3. Use a third party software package, such as Disk Manager, to

install the drive.

4. Use the closest drive type which is supported by your BIOS and

achieve only partial utilization of the capacity of the drive.

There are also options which allow the printing of the system

BIOS drive tables (SEE EXAMPLE 6) and a list of parameters for

Seagate drives (SEE EXAMPLE 7).

=======================================================

=======================================================

EXAMPLE 1:

THIS IS AN EXAMPLE WHERE THE DRIVE BEING INSTALLED (ST124) IS

SUPPORTED BY THE BIOS OF THE SYSTEM (TYPE 6).

COMMAND LINE INPUT = FINDTYPE ST124 <RETURN>

FINDTYPE.EXE  ver 1.1

(C)opyright 1990 by Seagate Technology, Inc.

Search Drive Parameters

model    cyls     heads     precomp   LZ     sects    Meg  lba's

ST124      615        4         -1      670      17      20   41820

Type #                       Best Choice

6        615        4         -1      615      17      20   41820

______________________________________________

Total Logical Blocks not accessible =     0

Total bytes not accessible          =     0

NOTE:    100.00% utilized    0.00% not accessible

YOU MUST VERIFY THE DRIVE TYPE NUMBER AND PARAMETERS!

When entering your drive type, double check the parameters, if

they do not match, search the next, or previous type for the

suggested parameters.

EXAMPLE 2:

IN THIS EXAMPLE, THE PROGRAM DOES NOT FIND AN EXACT MATCH FOR THE

ST213 DRIVE BEING INSTALLED. THE PROGRAM SCANS ALL THE DRIVE

TYPES AND DETERMINES THAT THE TYPE 24 IS THE CLOSEST MATCH. BY

USING THIS TYPE ALL BUT 102 LOGICAL BLOCKS (SECTORS) ARE

UTILIZED.

COMMAND LINE INPUT = FINDTYPE ST213 <RETURN>

FINDTYPE.EXE  ver 1.1

(C)opyright 1990 by Seagate Technology, Inc.

Search Drive Parameters

model    cyls     heads     precomp   LZ     sects    Meg  lba's

ST213      615        2        300      670      17      10   20910

Type #                       Best Choice

24       612        2         -1      611      17      10   20808

______________________________________________

Total Logical Blocks not accessible =   102

Total bytes not accessible          = 52224

NOTE:    99.51% utilized    0.49% not accessible

YOU MUST VERIFY THE DRIVE TYPE NUMBER AND PARAMETERS!

When entering your drive type, check the parameters, if they do

not match, search the next, or previous type for the suggested

parameters. If the capacity obtained with the use of this drive

type is not acceptable, you may want to utilize a third party

drive installation program, such as Disk Manager.

EXAMPLE 3:

IN THIS EXAMPLE, THE PROGRAM DOES NOT FIND AN EXACT MATCH FOR THE

ST238 DRIVE BEING INSTALLED. THE PROGRAM SCANS ALL THE DRIVE

TYPES AND DETERMINES THAT THE TYPE 6 IS THE CLOSEST MATCH. BY

USING THIS TYPE 34.62% OF THE DRIVE WILL BE WASTED. BUT, THE

PROGRAM HAS DETECTED THAT THE BIOS SUPPORTS THE CUSTOM OR USER

DEFINED DRIVE TYPE AND HAS PROMPTED THE OPERATOR TO VERIFY THAT

SUPPORT AND USE THAT OPTION. (SEE EXAMPLE 4 FOR USING THE CUSTOM

ARGUMENT ON THE COMMAND LINE).

COMMAND LINE INPUT = FINDTYPE ST238 <RETURN>

FINDTYPE.EXE  ver 1.1

(C)opyright 1990 by Seagate Technology, Inc.

Search Drive Parameters

model    cyls     heads     precomp   LZ     sects    Meg  lba's

ST238      615        4         -1      670      26      31   63960

Type #                       Best Choice

6       615        4         -1      615      17      20   41820

______________________________________________

Total Logical Blocks not accessible =   22140

Total bytes not accessible          =11335680

NOTE:    65.38% utilized   34.62% not accessible

YOU MUST VERIFY THE DRIVE TYPE NUMBER AND PARAMETERS!

When entering your drive type, check the parameters, if they do

not match, search the next, or previous type for the suggested

parameters. If the capacity obtained with the use of this drive

type is not acceptable, you may want to utilize a third party

drive installation program, such as Disk Manager.

***** Custom or User Defined Type Detected ******

** if verified, use of a custom type would yield maximum capacity

** RLL controllers usually provide drive type support for drives

with non-standard physical parameters (e.g. # sectors >17).

Please see your interface card manufacturer's manual for further

info. To use the controller board's BIOS you usually set the

cpu's BIOS to drive type 1.

EXAMPLE 4:

INSTALLING AN IDE DRIVE WITH MORE THAN 1024 CYLINDERS USING THE

CUSTOM OPTION.

COMMAND LINE INPUT = FINDTYPE CUSTOM ST1126a <RETURN>

FINDTYPE.EXE  ver 1.1

(C)opyright 1990 by Seagate Technology, Inc.

Search Drive Parameters

model    cyls     heads     precomp   LZ     sects    Meg  lba's

ST1126a    1072      7          -1      1072      29    106 217616

Custom Type Parameters

custom   536       14          -1      536      29    106 217616

______________________________________________

Total Logical Blocks not accessible =   0

Total bytes not accessible          =   0

NOTE:   100.00% utilized    0.00% not accessible

See your computer's setup instructions to enter the above drive

parameters as a Custom or User Defined drive type.  IDE OR AT DRIVES

DO NOT! REQUIRE A CUSTOMER LOW LEVEL FORMAT!

If LOW LEVEL formatted, bad sector mapping information will be lost!

EXAMPLE 5:

INSTALLING A DRIVE WHICH HAS NO CLOSE MATCH IN THE BIOS TABLE.

COMMAND LINE INPUT = FINDTYPE ST1111e <RETURN>

FINDTYPE.EXE  ver 1.1

(C)opyright 1990 by Seagate Technology, Inc.

Search Drive Parameters

model    cyls     heads     precomp   LZ     sects    Meg  lba's

ST1111e    1072      5          -1      1072      36    94  192960

Type #                     Best Choice

27       1024       5         -1      1023      17     42  87040

______________________________________________________

Total Logical Blocks not accessible = 105920

Total bytes not accessible          = 54231040

NOTE:    45.11% utilized   54.89% not accessible

YOU MUST VERIFY THE DRIVE TYPE NUMBER AND PARAMETERS!

When entering your drive type, check the parameters, if they do

not match, search the next, or previous type for the suggested

parameters. If the capacity obtained with this drive type is not

acceptable, you may want to utilize a third party drive

installation program, such as Disk Manager.

ESDI controllers usually provide logical translation schemes

where total capacity can be obtained with non-standard physical

parameters (e.g. # cylinders > 1024). Please see your interface

card manufacturer's manual! To use the controller board's BIOS

you must set the cpu's BIOS drive type to 1.

EXAMPLE 6:

PRINTING THE LIST OF DRIVE PARAMETERS SUPPORTED BY THE SYSTEM BIOS

FINDTYPE T or t PRINTS TO THE SCREEN

FINDTYPE PT or pt PRINTS TO THE PRINTER

COMMAND LINE INPUT = FINDTYPE PT <RETURN>

FINDTYPE.EXE  ver 1.1

(C)opyright 1990 by Seagate Technology, Inc.

BIOS drive Types

Drive                     Write     Landing                   Total

Type    cyls heads       precomp    Zone    sectors   Meg    Sectors

---------------------------------------------------------------------

1       306     4        128        305       17       10.7    20808

2       615     4        300        615       17       21.4    41820

3       615     6        300        615       17       32.1    62730

4       940     8        512        940       17       65.5    127840

5       940     6        512        940       17       49.1    95880

.       .       .        .          .         .        .       .

.       .       .        .          .         .        .       .

.       .       .        .          .         .        .       .

47      699     7        256        700       17       41      83131

EXAMPLE 7:

PRINTING THE LIST OF SEAGATE DRIVES

FINDTYPE D or d PRINTS TO THE SCREEN

FINDTYPE PD or pd PRINTS TO THE PRINTER

COMMAND LINE INPUT = FINDTYPE PD <RETURN>

FINDTYPE.EXE  ver 1.1

(C)opyright 1990 by Seagate Technology, Inc.

Seagate Drives

Seagate  Imprimis                   Write  Landing               Total

model #  model #      cyls    heads precomp  Zone  sectors Meg  sectors

ST124                  615       4     -1     670     17    21.4  41820

ST125                  615       4     -1     615     17    21.4  41820

ST125-1                615       4     -1     615     17    21.4  41820

ST125a                 404       4     -1     404     26    21.5  42016

ST125n                 407       4     -1     615     26    21.7  42328

.                      .         .     .      .       .       .   .

.                      .         .     .      .       .       .   .

.                      .         .     .      .       .       .   .

ST41200n    94601-12g  1931      15    -1     1931    71* 1052.9  2056515

Note: Some SCSI Drives use Zone Bit Recording, therefore the

Sector per track value is an average and is rounded down to the

next lower integer.

-------------------------------------------------

SEAGATE TECHNOLOGY, INC.

Technical Support Bulletin Board

408/438-8771 [300-9600 HST, MNP 3/5, N-8-1]

GLOSSARY of DRIVE and COMPUTER TERMS

ACCESS

Refers to the process of obtaining data from, or placing data

into a disc storage device, register, or RAM. (i.e. accessing a

memory location).

ACCESS TIME

Time required to perform an ACCESS. Usages, e.g.: 1) seek to

location on a disc, 2) amount of time to read or write to a

memory location, 3) the time to position to the correct location

in a disc drive and carry out a read or write operation. ACCESS

TIME is often defined as the time from the leading edge of the

first step pulse received to SEEK COMPLETE (including settling).

ACTUATOR

See HEAD POSITIONER. The two basic types of actuators are

steppers and voice coils. Open-loop steppers generally cannot

achieve tracks per inch (TPI) as high as the closed-loop system

because of the lack of feedback on track positioning accuracy. In

open-loop stepper drives mechanical tolerances are one of the

most significant factors in limiting TPI enhancement.

ADDRESS

(physical) A specific location in memory where a unit record,

or sector, of data is stored. To return to the same area on the

disc, each area is given a unique address consisting of three

components: cylinder, sector, and head. CYLINDER ADDRESSING is

accomplished by assigning numbers to the disc's surface

concentric circles (cylinders). The cylinder number specifies the

radial address component of the data area. SECTOR ADDRESSING is

accomplished by numbering the data records (sectors) from an

index that defines the reference angular position of the discs.

Index records are then counted by reading their ADDRESS MARKS.

Finally, HEAD ADDRESSING is accomplished by vertically numbering

the disc surfaces, usually starting with the bottom-most disc

data surface. For example, the controller might send the binary

equivalent of the decimal number 610150 to instruct the drive to

access data at cylinder 610, sector 15, and head 0.

ADDRESS MARK

Two byte address at the beginning of both the ID field and

the data field of the track format. The first byte is the "A1"

data pattern, the second byte is used to specify either an ID

field or a data field.

ADJUSTABLE INTERLEAVE

Interleaving permits access to more than one memory module,

e.g., if one memory module contains odd-numbered address and

another even-numbered address, they can both be accessed

simultaneously for storage. If the interleave is adjustable, the

user may select which ranges or areas are to be accessed each

time.

ANSI

American National Standards Institute

APPLICATION PROGRAM

A sequence of programmed instructions that tell the computer

how to perform an end use task (i.e. accounting, word processing

or other work for the computer system user). To use a program, it

must first be loaded into MAIN MEMORY from some AUXILIARY MEMORY

such as a floppy diskette or hard disk.

AREAL DENSITY

Bit density (bits per inch, or BPI) multiplied by track

density (tracks per inch, or TPI), or bits per square inch of the

disc surface. Bit density is measured around a track

(circumferential on the disc), and track density is radially

measured.

ASCII

American Standard for Coded Information Interchange.

ASME

American Society of Mechanical Engineers

ASYNCHRONOUS DATA

Data sent usually in parallel mode without a clock pulse.

Time intervals between transmitted bits may be of unequal

lengths.

AT INTERFACE     DISK DRIVE INTERFACE ON THE IBM PC-AT

COMPUTER AND COMPATIBLES. SOMETIMES CALLED

THE IDE (INTEGRATED DRIVE ELECTRONICS)

INTERFACE

AUTOMATIC BACK UP OF FILES

This gives a user the security to make changes to a file

without worrying about accidently destroying it; there is always

another copy. One weakness of this method is that files take up

twice the room on a disc.

AUXILIARY MEMORY

Memory other than main memory; generally a mass storage

subsystem, it can include disc drives, backup tape drives,

controllers and buffer memory. Typically, AUXILIARY MEMORY is

non-volatile.

AUXILIARY STORAGE DEVICE

Devices, generally magnetic tape and magnetic disk, on which

data can be stored for use by computer programs. Also known as

secondary storage.

AVERAGE ACCESS TIME

The average track access time, calculated from the end of the

CONTROLLER commands to access a drive, to drive "seek complete"

time averaged over all possible track locations at the start of

ACCESS, and over all possible data track ADDRESSES. Typically,

the minimum average access time including carriage settling for

open loop actuators is less than 85 ms and for voice coil disc

drives is less than 40 ms. As technology improves these times

will continue to decrease.

AZIMUTH

The angular distance in the horizontal plane, usually

measured as an angle from true track location.

BACKUP DEVICE

Disc or tape drive used with a fixed Winchester disc drive to

make copies of files or other data for off line storage,

distribution or protection against accidental data deletion from

the Winchester drive, or against drive failure.

BACKUP FILE

File copies made on another removable media device (disc,

tape or sometimes a remote hard dsic system) and kept to ensure

recovery of data lost due to equipment failure, human errors,

updates, disasters and the like.

BAUD RATE

A variable unit of data transmission speed equal to one bit

per second.

BCAI             BYTE COUNT AFTER INDEX. USED IN DEFECT

MAPPING TO INDICATE THE POSITION OF DEFECTS

WITH RELATION TO INDEX.

BDOS

The Basic Disk Operating System (BDOS) controls the

organization of data on a disk. BDOS is usually pronounced

"B-DOS".

BIDIRECTIONAL BUS

A bus that may carry information in either direction but not

in both simultaneously.

BINARY

A number system like the decimal numbers, but using 2 as its

base and having only the two digits 0 (zero) and 1 (one). It is

used in computers because digital logic can only determine one of

two states - "OFF" and "ON." Digital data is equivalent to a

binary number.

BIOS

(BASIC INPUT OUTPUT SYSTEM) A collection of information

(firmware) that controls communication between the Central

Processor and its peripherals.

BIT

The smallest unit of data. Consists of a single binary digit

that can take the value of 0 or 1.

BIT CELL LENGTH

Physical dimension of the bit cell in direction of recording

along the disc circumference of a track.

BIT CELL TIME

The time required to pass one bit of information between the

controller and the drive. Cell time is the inverse of the drive's

data rate; nominally 200 nsec for 5 Mhz drives.

BIT DENSITY

Expressed as "BPI" (for bits per inch), bit density defines

how many bits can be written onto one inch of a track on a disc

surface. It is usually specified for "worst case", which is the

inner track. Data is the densest in the inner tracks where track

circumferences are the smallest.

BIT JITTER

The time difference between the leading edge of read and the

center of the data window.

BIT SHIFT

A data recording effect, which results when adjacent 1's

written on magnetic discs repel each other. The "worst case" is

at the inner cylinder where bits are closest together. BIT SHIFT

is also called pulse crowding.

BLOCK

A group of BYTES handled, stored and accessed as a logical

data unit, such as an individual file record. Typically, one

block of data is stored as one physical sector of data on a disc

drive.

BOOT

(Short for bootstrap). Transfer of a disc operating system

program from storage on diskette or hard disc drive to computer's

working memory.

BUFFER

A temporary data storage area that compensates for a

difference in data transfer rates and/or data processing rates

between sender and receiver.

BUFFERED SEEK

A feature of the ST412 INTERFACE. In buffered mode head

motion is postponed until a string of step pulses can be sent to

the drive. These pulses represent the number of tracks that the

head is to be stepped over and are sent much faster than the

heads can move. The pulses are saved or buffered then the optimum

head movement to the correct track is performed.

BUS

A length of parallel conductors that forms a major

interconnection route between the computer system CPU and its

peripheral subsystems. Depending on its design, a bus may carry

data to and from peripheral's addresses, power, and other related

signals.

BYTE

A sequence of adjacent BINARY digits or BITS considered as a

unit, 8 bits in length. One byte is sufficient to define all the

alphanumeric characters. There are 8 BITS in 1 BYTE. The storage

capacity of a disc drive is commonly measured in MEGABYTES, which

is the total number of bits storable, divided by eight million.

CACHE MEMORY

Cache Memory allows the system to load bytes of data from the

hard disc to memory. The system may then refer to memory for

information instead of going back to the hard disc, thereby

increasing the processing speed.

CAPACITY

Amount of memory (measured in megabytes) which can be stored

in a disc drive. Usually given as formatted (see FORMAT

OPERATION).

CARRIAGE ASSEMBLY

Assembly which holds read/write heads and roller bearings. It

is used to position the heads radially by the actuator, in order

to access a track of data.

CENTRAL PROCESSOR UNIT

(CPU). The heart of the computer system that executes

programmed instructions. It includes the arithmetic logic unit

(ALU) for performing all math and logic operations, a control

section for interpreting and executing instructions, fast main

memory for temporary (VOLATILE) storage of an application program

and its data.

CHARACTER

An information symbol used to denote a number, letter, symbol

or punctuation mark stored by a computer. In a computer a

character can be represented in one (1) byte or eight (8) bits of

data. There are 256 different one-byte binary numbers, sufficient

for 26 lower case alphas, 26 upper case alphas, 10 decimal

digits, control codes and error checks.

CHIP

An integrated circuit fabricated on a chip of silicon or

other semiconductor material, e.g., a CHIP is an integrated

circuit, a microprocessor, memory device, or a digital logic

device.

CLOCK RATE

The rate at which bits or words are transferred between

internal elements of a computer or to another computer.

CLOSED LOOP

A control system consisting of one or more feedback control

loops in which functions of the controlled signals are combined

with functions of the command to maintain prescribed

relationships between the commands and the controlled signals.

This control technique allows the head actuator system to

detect and correct off-track errors. The actual head position is

monitored and compared to the ideal track position, by reference

information either recorded on a dedicated servo surface, or

embedded in the inter-sector gaps. A position error is used to

produce a correction signal (FEEDBACK) to the actuator to correct

the error. See TRACK FOLLOWING SERVO.

CLUSTER SIZE

Purely an operating system function or term describing the

number of sectors that the operating system allocates each time

disc space is needed.

CODE

A set of unambiguous rules specifying the way which digital

data is represented physically, as magnetized bits, on a disc

drive. One of the objectives of coding is to add timing data for

use in data reading. See DATA SEPARATOR, MFM and RLL.

COERCIVITY

A measurement in units of orsteads of the amount of magnetic

energy to switch or "coerce" the flux change (di-pole) in the

magnetic recording media.

COMMAND

1) An instruction sent by the central processor unit (CPU) to

a     controller for execution. 2) English-like commands entered

by users to select computer programs or functions. 3) A CPU

command, which is a single instruction such as "add two binary

numbers" or "output a byte to the display screen."

CONSOLE

(also called CRT or Terminal) A device from which a computer

can be operated; often includes a monitor and keyboard.

CONTROLLER

A controller is a printed circuit board required to interpret

data access commands from host computer (via a BUS), and send

track seeking, read/write, and other control signals to a disc

drive. The computer is free to perform other tasks until the

controller signals DATA READY for transfer via the CPU BUS.

CORE

Originally a computer's main memory was made of ferrite rings

(CORES) that could be magnetized to contain one bit of data each.

CORE MEMORY is synonymous with MAIN MEMORY. Main memory today is

fabricated from CHIPS.

CPU

See CENTRAL PROCESSOR UNIT

CRASH

A malfunction in the computer hardware or software, usually

causing loss of data.

CYCLIC-REDUNDANCY-CHECK

(CRC). Used to verify data block integrity. In a typical

scheme, 2 CRC bytes are added to each user data block. The 2

bytes are computed from the user data, by digital logical chips.

The mathematical model is polynomials with binary coefficients.

When reading back data, the CRC bytes are read and compared to

new CRC bytes computed from the read back block to detect a read

error. The read back error check process is mathematically

equivalent to dividing the read block, including its CRC, by a

binomial polynomial. If the division remainder is zero, the data

is error free.

CYLINDER

The cylindrical surface formed by identical track numbers on

vertically stacked discs. At any location of the head positioning

arm, all tracks under all heads are the cylinder. Cylinder number

is one of the three address components required to find a

specific ADDRESS, the other two being head number and sector

number.

DAISY CHAIN

A way of connecting multiple drives to one controller. The

controller drive select signal is routed serially through the

drives, and is intercepted by the drive whose number matches. The

disc drives have switches or jumpers on them which allow the user

to select the drive number desired.

DATA

Information processed by a computer, stored in memory, or fed

into a computer.

DATA ACCESS

When the controller has specified all three components of the

sector address to the drive, the ID field of the sector brought

under the head by the drive is read and compared with the address

of the target sector. A match enables access to the data field of

the sector.

DATA ADDRESS

To return to the same area on the disc, each area is given a

unique address consisting of the three components: cylinder, head

and sector. HORIZONTAL: accomplished by assigning numbers to the

concentric circles (cylinders) mapped out by the heads as the

positioning arm is stepped radially across the surface, starting

with 0 for the outermost circle. By specifying the cylinder

number the controller specifies a horizontal or radial address

component of the data area. ROTATIONAL: once a head and cylinder

have been addressed, the desired sector around the selected track

of the selected surface is found by counting address marks from

the index pulse of the track. Remember that each track starts

with an index pulse and each sector starts with an address mark.

VERTICAL: assume a disc pack with six surfaces, each with its own

read/write head, vertical addressing is accomplished by assigning

the numbers 00 through XX to the heads, in consecutive order. By

specifying the head number, the controller specifies the vertical

address component of the data area.

DATA BASE

An organized collection of data stored in DISC FILES, often

shared by multiple users., e.g., the Official Airline Guide,

which contains up-to-date schedules for all airlines.

DATA BASE MANAGEMENT SYSTEM

(DBMS) Application program used to manage, access and update

files in a data base.

DATA ENCODING

To use a code such as GCR, MFM, RLL, NZR, etc. to represent

characters for memory storage.

DATA FIELD

The portion of a sector used to store the user's DIGITAL

data. Other fields in each sector include ID, SYNC and CRC which

are used to locate the correct data field.

DATA SEPARATOR

Controller circuitry takes the CODED playback pulses and uses

the timing information added by the CODE during the write process

to reconstruct the original user data record. See NRZ, MFM, and

RLL.

DATA TRACK

Any of the circular tracks magnetized by the recording head

during data storage.

DATA TRANSFER RATE

(DTR). Speed at which bits are sent: In a disc storage

system, the communication is between CPU and controller, plus

controller and the disc drive. Typical units are bits per second

(BPS), or bytes per second, e.g., ST506/412 INTERFACE allows 5

Mbits/sec. transfer rate.

DECREASE THE FLYING HEIGHT

Since the head core is closer to the media surface, the lines

of flux magnetize a smaller area. Thus, more bits can be recorded

in a given distance, and higher BPI (bits per inch) is

achievable.

DEDICATED SERVO SYSTEM

A complete disc surface is dedicated for servo data.

DEFAULT

A particular value of a variable which is used by a computer

unless specifically changed, usually via an entry made through a

software program.

DENSITY

Generally, bit recording density. SEE AREAL, BIT and STORAGE

DENSITY.

DIGITAL

Any system that processes digital binary signals having only

the values of a 1 or 0. An example of a non-digital signal is an

analog signal which continuously varies, e.g., TV or audio.

DIGITAL MAGNETIC RECORDING

See MAGNETIC RECORDING

DIRECT ACCESS

Generally refers to an AUXILIARY MEMORY device, having all

data on-line. E.G., a tape drive without a tape mounted is not

direct access, but a WINCHESTER DRIVE is direct access.

DIRECTORY

A special disc storage area (usually cylinder zero) that is

read by a computer operating system to determine the ADDRESSES of

the data records that form a DISC FILE.

DISC FILE

A file of user data, e.g. the company employee list, with all

names and information. The data in the file is stored in a set of

disc SECTORS (records).

DISC OPERATING SYSTEM

(DOS). A computer program which continuously runs and

mediates between the computer user and the APPLICATION PROGRAM,

and allows access to disc data by DISC FILE names.

DISC PACK

A number of metal discs packaged in a canister for removal

from the disc drive. WINCHESTER DRIVES do not have disc packs.

DISC/PLATTER

For rigid discs, a flat, circular aluminum disc substrate,

coated on both sides with a magnetic substance (iron oxide or

thin film metal media) for non-VOLATILE data storage. The

substrate may consist of metal, plastic, or even glass. Surfaces

of discs are usually lubricated to minimize wear during drive

start-up or power down.

DISC STORAGE

Auxiliary memory system containing disc drives.

DISKETTE

A floppy disc. A plastic (mylar) substrate, coated with

magnetic iron oxide, enclosed in a protective jacket.

DOS              DISC OPERATING SYSTEM. A COMPUTER PROGRAM

WHICH RUNS CONTINUOUSLY AND MEDIATES BETWEEN

THE COMPUTER USER AND THE APPLICATION

PROGRAM AND ALLOWS ACCESS TO THE DISC DATA

BY DISC FILE NAMES.

DRIVE

A computer memory device with moving storage MEDIA (disc or

tape).

DRIVE SELECT

An ADDRESS component that selects among a string of drives

attached to a disc controller. In the ST 506/412 interface

standard, a drive's select code is physically set in the drive to

a value between 0 and 3. When the controller activates one of the

four drive select code lines in the J1 cable, the selected drive

is enabled to respond to access commands from the controller.

DRIVE TYPE       A NUMBER REPRESENTING A STANDARD

CONFIGURATION OF PHYSICAL PARAMETERS

(CYLINDERS, HEADS, AND SECTORS) OF A

PARTICULAR TYPE OF DISC DRIVE.  EACH AT SYSTEM

BIOS CONTAINS A LIST OF DRIVE TYPES THAT THE

SYSTEM CONSIDERS "STANDARD TYPES".  THESE

TYPES ARE NOT NECESSARILY THE SAME FROM ONE

BIOS TO THE NEXT.  THAT IS, DRIVE TYPE 25 ON

ONE BIOS MAY REPRESENT A DRIVE THAT HAS 615

CYLINDERS, 4 DATA HEADS, AND 17 SECTORS PER

TRACK, WHILE TYPE 25 ON ANOTHER BIOS COULD

BE TOTALLY DIFFERENT.

DROP-IN/DROP-OUT

Types of disc media defects usually caused by a pin-hole in

the disc coating. If the coating is interrupted, the magnetic

flux between medium and head is zero. A large interruption will

induce two extraneous pulses, one at the beginning and one at the

end of the pin-hole (2 DROP-INs). A small coating interruption

will result in no playback from a recorded bit (a DROP-OUT).

DRUM

An early form of rotating magnetic storage, utilizing a

rotating cylindrical drum and a multiplicity of heads (one per

track). Discs stack more compactly than drums.

ECC

ERROR CORRECTION CODE: The ECC hardware in the controller

used to interface the drive to the system can typically correct a

single burst error of 11 bits or less. This maximum error burst

correction length is function of the controller. With some

controllers the user is allowed to the select this length. The

most common selection is 11.

ELECTRO-STATIC DISCHARGE

(ESD) An integrated circuit (CHIP) failure mechanism. Since

the circuitry of CHIPs are microscopic in size, they can be

damaged or destroyed by small static discharges. People handling

electronic equipment should always ground themselves before

touching the equipment. Electronic equipment should always be

handled by the chassis or frame. Components, printed circuit

board edge connectors should never be touched.

EMBEDDED SERVO SYSTEM

Servo data is embedded or superimposed along with data on

every cylinder.

ERASE

To remove previously recorded data from magnetic storage

media.

ERROR

See HARD ERROR and SOFT ERROR.

ESDI

ENHANCED SMALL DEVICE INTERFACE. A set of specifications for

the drives. See also SCSI.

EXECUTE

To perform a data processing operation described by an

instruction or a program in a computer.

FCI

(FLUX CHANGES PER INCH): Synonymous with FRPI (flux reversals

per inch). In MFM recording 1 FCI equals 1 BPI (bit per inch). In

RLL encoding schemes, 1 FCI generally equals 1.5 BPI.

FEEDBACK

A closed-loop control system, using the head-to-track

positioning signal (from the servo head) to modify the HEAD

POSITIONER signal (to correctly position the head on the track).

FETCH

A CPU read operation from MAIN MEMORY and its related data

transfer operations.

FIELDS

1. SOFTWARE: Storage units grouped together to make a record

are considered to be a field; e.g., a record might be a company's

address; a field in the record might be the company's ZIP code.

FILE

See DISC FILE. 1. SOFTWARE: A file consists of a group of

logically related records that, in turn, are made up of groups of

logically related fields.

FILE ALLOCATION TABLE

FAT: What the operating systems uses to keep track of which

clusters are allocated to which files and which are available for

use. FAT is usually stored on Track-0.

FILE NAME

Each file has a name, just like the name on the tab of a file

folder. When you want DOS to find a file, you give DOS the file

name.

FIRMWARE

A computer program written into a storage medium which cannot

be accidentally erased, e.g., ROM. It can also refer to devices

containing such programs.

FIXED DISC

A disc drive with discs that cannot be removed from the drive

by the user, e.g., WINCHESTER DISC DRIVE.

FLOPPY DISC

A flexible plastic disc coated with magnetic media and

packaged in a stiff envelope. Comes in 8-inch, 5-1/4-inch, and

various sub-4 inch sizes. FLOPPY DISCS generally exhibit slow

ACCESS TIME and smaller CAPACITY compared to WINCHESTER DRIVES,

but feature removable diskettes.

FLUX CHANGE

Location on the data track, where the direction of

magnetization reverses in order to define a 1 or 0 bit.

FLUX CHANGES PER INCH

(FCI). Linear recording density defined as the number of flux

changes per inch of data track.

FM

Frequency modulation CODE scheme, superceded by MFM, which is

being superceded by RLL.

FORMAT

The purpose of a format is to record "header" data that

organize the tracks into sequential sectors on the disc surfaces.

This information is never altered during normal read/write

operations. Header information identifies the sector number and

also contains the head and cylinder ADDRESS in order to detect an

ADDRESS ACCESS error.

FORMATTED CAPACITY

Actual capacity available to store user data. The formatted

capacity is the gross capacity, less the capacity taken up by the

overhead data used in formatting the discs. While the unformatted

size may be 24 M bytes, only 20 M bytes of storage may actually

be available to the user after formatting.

FPI

(flux changes per inch), also FRPI, the number of Flux

Reversals per inch.

FRICTION

Resistance to relative motion between two bodies in contact;

e.g., there is sliding friction between head and disc during

drive power up/down.

FULL HEIGHT DRIVE

Winchester 5-1/4" drive which fits in the same space as full

height mini-floppy drive (called the full-height form factor).

G

A G is a unit of force applied to a body at rest equal to the

force exerted on it by gravity. Hard disc drive shock

specifications are usually called out in Gs. A shock

specification of 40 Gs non-operating means that a drive will not

suffer any permanent damage if subjected to a 40 G shock. This is

roughly equivalent to a drop of the drive to a hard surface from

a distance of 1 inch.

GAP

1. FORMAT: Part of the disc format. Allows mechanical

compensations (e.g. spindle motor rotational speed variations)

without the last sector on a track overwriting the first sector.

2. HEAD: An interruption in the permeable head material, usually

a glass bonding material with high permeability, allowing the

flux fields to exit the head structure to write / read data bits

in the form of flux changes on the recording media.

GAP LENGTH

Narrowing the head gap length achieves higher bit density

because the lines of force magnetize a smaller area where writing

data in the form of flux changes on the recording media.

GAP WIDTH

The narrower the gap width, the closer the tracks can be

placed. Closer track placement results in higher TPI.

GCR

GROUP CODE ENCODING. Data encoding method.

GUARD BAND

1. Non-recorded band between adjacent data tracks, 2. For

closed loop servo drives, extra servo tracks outside the data

band preventing the Carriage Assembly from running into the crash

stop.

HALF HIGH DRIVE

A Winchester drive which fits in one half of the space of a

full height mini-floppy drive.

HARD DISC DRIVE

Commonly called rigid disc drives, or Winchester disc drives.

An electromechanical device that can read rigid discs. Though

similar to floppy disc drives, the hard discs have higher bit

density and multiple read/write surfaces.

HARD ERROR

An error that occurs repeatedly at the same location on a

disc surface. Hard errors are caused by imperfections in the disc

surface, called media defects. When formatting hard disc drives,

hard error locations, if known, should be spared out so that data

ia not written to these locations. Most drives come with a hard

error map listing the locations of any hard errors by head,

cylinder and BFI (bytes from index - or how many bytes from the

beginning of the cylinder).

HARD ERROR MAP

Also called defect map, bad spot map, media map. Media

defects are avoided by deleting the defective sectors from system

use, or assigning an alternative track (accomplished during

format operation). The defects are found during formatting, and

their locations are stored on a special DOS file on the disc,

usually on cylinder 0.

HARD SECTOR MODE

A HARDWARE CONTROLLED CONVENTION DEFINING A FIXED

NUMBER OF SECTORS PER TRACK IN ANY SPECIFIED ZONE

HARDWARE

Computer equipment (as opposed to the computer progrms and

software).

HDA

HEAD/DISK ASSEMBLY: A sealed Winchester assembly including

discs, heads, filter and actuator assembly.

HEAD

An electromagnetic device that can write (record), read

(playback), or erase data on magnetic media. There are three

types: Head Type BPI TPI Areal density Monolithic 8000 450 3.6 X

10 to 6th Composition 12000 1000 12 X 10 to 6th Thin-film 25000

1500 37.5 X 10 to 6th

HEAD CRASH

A head landing occurs when the disc drive is turned on or

off. This function normally does not damage the disc as the disc

has a very thin lubricant on it. A head crash occurs when the

head and disc damage each other during landing, handling or

because a contaminant particle gets between them. Head crash is a

catastrophic failure condition and causes permanent damage and

loss of data.

HEAD LANDING AND TAKEOFF

In Winchester drives, the head is in contact with the platter

when the drive is not powered. During the power up cycle, the

disc begins rotation and an "air bearing" is established as the

disc spins up to full RPM (rotations per minute). This air

bearing prevents any mechanical contact between head and disc.

HEAD LANDING ZONE

An area of the disc set aside for takeoff and landing of the

Winchester heads when the drive is turned on and off.

HEAD POSITIONER

Also known as the ACTUATOR, a mechanism that moves the

CARRIAGE ASSEMBLY to the cylinder being accessed.

HEAD SLAP

Similar to a head crash but occurs while the drive is turned

off. It usually occurs during mishandling or shipping. Head slap

can cause permanent damage to a hard disc drive. See HEAD CRASH.

HEXIDECIMAL

(HEX) A number system based on sixteen, using digits 0

through 9 and letters A through F to represent each digit of the

number. (A = 10, B = 11, C = 12, D = 13, E = 14, F = 15).

ID FIELD

The address portion of a sector. The ID field is written

during the Format operation. It includes the cylinder, head, and

sector number of the current sector. This address information is

compared by the disc controller with the desired head, cylinder,

and sector number before a read or write operation is allowed.

IMAGE-BACKUP MODE

Used with streaming tape, image-backup mode records an exact

copy of the disc, including unused sectors and bad tracks.

INDEX

(PULSE): The Index Pulse is the starting point for each disc

track. The index pulse provides initial synchronization for

sector addressing on each individual track.

INDEX TIME

The time interval between similar edges of the index pulse,

which measures the time for the disc to make one revolution. This

information is used by a disc drive to verify correct rotational

speed of the media.

INPUT

1. Data entered into the computer to be processed. 2. User

commands or queries.

INPUT/OUTPUT

The process of entering data into or removing data from a

computer system.

INTELLIGENT PERIPHERAL

A peripheral device that contains a processor or

microprocessor to enable it to interpret and execute commands,

thus relieving the computer for other tasks.

INTERFACE

The protocol data transmitters, data receivers, logic and

wiring that link one piece of computer equipment to another, such

as a disc drive to a controller or a controller to a system bus.

Protocol means a set of rules for operating the physical

interface, e.g., don't read or write before SEEK COMPLETE is

true.

INTERFACE STANDARD

The interface specifications agreed to by various

manufacturers to promote industry-wide interchangeability of

products such as disc drives and controllers. An interface

standard generally reduces product costs, allows buyers to

purchase from more than one source, and allows faster market

acceptance of new products. (See ST-506/412, SCSI, ESDI)

INTERLEAVE FACTOR

The ratio of physical disc sectors skipped for every sector

actually written.

INTERLEAVING

The interleave value tells the controller where the next

logical sector is located in relation to the current sector. For

example, an interleave value of one (1) specifies that the next

logical sector is physically the next sector on the track.

Interleave of two (2) specifies every other physical sector,

three (3) every third sector and so on. Interleaving is used to

improve the system throughout based on overhead time of the host

software, the disc drive and the controller; e.g., if an

APPLICATION PROGRAM is processing sequential logical records of a

DISC FILE in a CPU time of more than one second but less than

two, then an interleave factor of 3 will prevent wasting an

entire disc revolution between ACCESSES.

INTERRUPT

A signal, usually from a peripheral device to a CPU, to

signify that a commanded operation has been completed or cannot

be completed.

I/O PROCESSOR

Intelligent processor or controller that handles the

input/output operations of a computer.

KILOBYTE

(KBYTE). 1) 1024 bytes (two to the tenth power); 2) 1000

bytes; 1024 bytes is the normal definition.

LAN

Local Area Network

LANDING ZONE

The landing zone is where the read/write head sits when it is

not active. If the system features a dedicated landing zone, the

head will rest on the same track each time.

LATENCY

(ROTATIONAL) The time for the disc to rotate the accessed

sector under the head for read or write. On the average, latency

is the time for half of a disc revolution.

LOGIC

Electronic circuitry that switches on and off ("1" and "0")

to perform digital operations.

LOOKUP

The action of obtaining and displaying data in a file.

LOW LEVEL FORMAT

The first step in preparing a drive to store information

after physical installation is complete. The process sets up the

"handshake" between the drive and the controller. In an XT

system, the low level format is usually done using DOS's debug

utility. In an AT system, AT advanced diagnostics is typically

used. Other third party software may also be used to do low level

format on both XTs and ATs.

LUN

Logical Unit Number

MAGNETIC MEDIA

A disc or tape with a surface layer containing particles of

metal, or metallic oxides that can be magnetized in different

directions to represent bits of data, sounds or other

information.

MAGNETIC RECORDING

The use of a head, recording head, recording media (tape or

disc), and associated electronic circuitry for storing data or

sound or video.

MAINFRAME COMPUTER

A large computer generally found in data processing centers.

See MINICOMPUTER AND MICROCOMPUTER.

MAIN MEMORY

Random-access memory used by the CPU for storing program

instructions and data currently being processed by those

instructions. See RANDOM-ACCESS MEMORY.

MEAN TIME BEFORE FAILURE

(MTBF). The average time before a failure will occur. This

is not a warranty measurement. MTBF is a calculation taking into

consideration the MTBF of each component in a system and is the

statistical average operation time between the start of a unit's

lifetime and its time of a failure. After a product has been in

the field for a few years, the MTBF can become a field proven

statistic.

MEAN TIME TO REPAIR

(MTTR) The average time to repair a given unit. Limited to a

qualified technician with proper equipment.

MEDIA

The magnetic layers of a disc or tape. See DISC/PLATTER.

MEDIA DEFECT

A media defect can cause a considerable reduction of the read

signal (missing pulse or DROP-OUT), or create an extra pulse

(DROP-IN). See HARD ERROR MAP.

MEGABYTE

One million bytes (exactly 1,000,000 bytes). Abbreviation: MB

or Mbyte.

MEMORY

Any device or storage system capable of storing and

retrieving information. See also STORAGE DEFINITIONS.

MICROCOMPUTER

A computer whose central processor unit (CPU) is manufactured

as a chip or a small number of chips. Personal computers are

examples of microcomputers.

MICROINCH

One-millionth of an inch.

MICROSECOND

One-millionth of a second.

MILLISECOND

(Msec)  One-thousandth of a second.

MINICOMPUTER

A computer midway in size and processing power between a

MICROCOMPUTER and a MAINFRAME COMPUTER.

MINI-SLIDER HEADS

Manganese/Zinc Ferrite Winchester heads. Smaller, lighter

heads with stiffer load arms than standard Winchester heads. They

allow smaller flying heights, and therefore higher bit and track

density, if they are made with smaller and narrower gaps.

MINI WINCHESTER

A Winchester disc drive with 5-1/4 or 3-1/2 inch diameter

discs.

MNEUMONIC

A shortened code for a longer term.

MODIFIED FREQUENCY MODULATION

(MFM). A method of recording digital data, using a particular

CODE to get the flux reversal times from the data pattern. MFM

recording is self-clocking because the CODE guarantees timing

information for the playback process. The controller is thus able

to synchronize directly from the data. This method has a maximum

of one bit of data with each flux reversal. (See NRZ, RLL).

MULTIPROCESSOR

A computer containing two or more processors.

MULTITASKING

The ability of a computer system to execute more than one

program or program task at a time.

MULTIUSER

The ability of a computer system to execute programs for more

than one user at a time.

NOISE

Extraneous electronic signals that interfere with information

signals (similar to radio static or TV interference). Sources of

noise in computers can be power supplies, ground loops, radio

interference, cable routing, etc.

NRZ

NON-RETURN TO ZERO 1) User digital data bits; 2) A method of

magnetic recording of digital data in which a flux reversal

denotes a one bit, and no flux reversal a zero bit, NRZ recording

requires an accompanying synchronization clock to define each

cell time unlike MFM or RLL recording). No Seagate drives use NRZ

recording methods.

OFF LINE

Processing or peripheral operations performed while not

connected to the system CPU via the system BUS.

OPEN COLLECTOR

A type of output structure found in certain bipolar logic

families. The device has a transistor that enables it to output

to a low voltage level only. When the device is inactive, an

external resistor holds the device output at a high voltage

level.

OPERATING SYSTEM

An operating system is a program which acts as an interface

between the user of a computer and the computer hardware. The

purpose of the operating system is to provide an environment in

which a user may run programs. The goal of the operating system

is to enable the user to conveniently use the computer's

resources such as the CPU, memory, storage devices and printers.

OUTPUT

Processing data being transferred out of the computer system

to peripherals (i.e. disc, printer, etc.). This includes

responses to user commands or queries.

PARITY

A computer data checking method using an extra bit in which

the total number of binary 1's (or 0's) in a byte is always odd

or always even; thus, in an odd parity scheme, every byte has

eight bits of data and one parity bit. If using odd parity and

the number of 1 bits comprising the byte of data is not odd, the

9th or parity bit is set to 1 to create the odd parity. In this

way, a byte of data can be checked for accurate transmission by

simply counting the bits for an odd parity indication. If the

count is ever even, an error is indicated.

PARKING

Parking the disc drive heads means the recording heads are

moved so that they are not over the platter's data area. Many

drives have an auto-park feature where the heads are

automatically parked when power to the drive is shut off. Other

drives require the user to run some kind of parking software to

park the heads.

PARTITIONING

Method for dividing an area on disc drive for use by more

than one disc operating system or for dividing large disc drives

into areas which the File Allocation Table (FAT) can deal with

when in use. The current IBM DOS maximum partition size is 32 MB

for the XT and AT. This limit can be overridden using

partitioning software written expressly for this purpose.

PATH

The DOS term "path" has three definitions and each definition

involves directories. A PATH may be defined as: 1) the names of

the chain of directories leading to a file; 2) the complete file

or directory name; 3) a DOS command.

PERIPHERAL EQUIPMENT

Auxiliary memory, displays, printers, disc drives, and other

equipment usually attached to computer systems' CPU by

controllers and cables (they are often packaged together in a

desktop computer).

PLATED THIN FILM DISCS

Magnetic disc memory media having its surface plated with a

thin coating of a metallic alloy instead of being coated with

oxide.

PLATTER

The round magnetic disc surfaces used for read/write

operations in a hard disc system.

POLLING

A technique that discerns which of several devices on a

connection is trying to get the processor's attention.

PRECOMPENSATION

Applied to write data by the controller in order to partially

alleviate bit shift which causes adjacent 1's written on magnetic

media physically to move apart. When adjacent 1's are sensed by

the controller, precompensation is used to write them closer

together on the disc, thus fighting the repelling effect caused

by the recording. Precompensation is only required on some oxide

media drives.

PREVENTIVE MAINTENANCE

A method of doing a scheduled routine observation or

exchanging a part, prior to a breakdown of a piece of equipment.

PRINTED CIRCUIT BOARD

(PCB) The circuit board with the chips attached to a drive.

PROCESSING

(DATA PROCESSING) The process of computer handling,

manipulating, and modifying data such as arithmetic calculation,

file lookup and updating, or word processing.

PROGRAM

A sequence of instructions stored in memory and executed by a

processor or microprocessor. See also APPLICATIONS PROGRAMS.

PROTOCOL

A set of conventions governing the format of messages to be

exchanged within a communications system.

RADIAL

A way of connecting multiple drives to one controller. In

radial operation, all output signals are active even if the drive

is not selected. Also see DAISY CHAIN.

RAM DISC

A DOS operation, where part of the computer's random access

memory is used to simulate a disk drive. The RAM disc and its

contents will disappear if power is lost or DOS MAIN MEMORY is

restarted. RAM is far faster (microseconds ACCESS TIME) than

discs (milliseconds), so APPLICATIONS PROGRAMS which access the

disk run faster.

RANDOM ACCESS MEMORY

(RAM) Memory where any location can be read from or written

to in a random order. Random access memory usually refers to

volatile memory where the contents are lost when power is

removed. The user addressable memory of a computer is random

access memory.

READ

To access a storage location and obtain previously recorded

data.

RECALIBRATE

Return to Track Zero. A common disc drive function in which

the heads are returned to track 0 (outermost track).

RECORD

1. Software. A record is a single unit made up of logically

related fields.

REDUCED WRITE CURRENT

A signal input (to some older drives) which decreases the

amplitude of the write current at the actual drive head. Normally

this signal is specified to be used during inner track write

operations to lessen the effect of adjacent bit "crowding." Most

drives today provide this internally and do not require

controller intervention.

REDUCED WRITECURRENT

TO MINIMIZE THE EFFECTS OF PEAK SHIFT, ON

SOME DRIVES, THE MAGNITUDE OF THE WRITE

CURRENT IS REDUCED ON SOME OF THE INNERMOST

TRACKS.  WHEN INSTALLING A DRIVE IN A

SYSTEM, THE NUMBER REQUESTED IS THE FIRST

TRACK NUMBER TO BEGIN THE AREA OF REDUCED

WRITE CURRENT. THAT TRACK AND ALL SUBSEQUENT

TRACKS WILL BE WRITTEN WITH REDUCED WRITE

CURRENT.

RESOLUTION

With regards to magnetic recording, the band width (or

frequency response) of the recording heads.

RLL

(RUN LENGTH LIMITED CODE). 1) A method of recording digital

data, whereby the combinations of flux reversals are

coded/decoded to allow greater than one (1) bit of information

per flux reversal. This compaction of information increases data

capacity by approximately 50 percent; 2) a scheme of encoding

designed to operate with the ST412 interface at a dial transfer

rate of 7.5 megabit/sec. The technical name of the specific RLL

CODE used is "two, seven".

ROM

(READ ONLY MEMORY) A chip that can be programmed once with

bits of information. This chip retains this information even if

the power is turned off. When this information is programmed into

the ROM, it is called burning the ROM.

ROTATIONAL SPEED

The speed at which the media spins. On a 5-1/4 or 3-1/2"

Winchester drive it is usually 3600 rpm.

SCSI

Small Computer Systems Interface. The current "high end"

CPU-to-drive interface.

SECTOR

A sector is a section of a track whose size is determined by

formatting. When used as an address component, sector and

location refer to the sequence number of the sector around the

track. Typically, one sector stores one user record of data.

Drives typically are formatted from 17 to 26 sectors per track.

Determining how many sectors per track to use depends on the

system type, the controller capabilities and the drive encoding

method and interface.

SECTOR-SLIP

Sector-slip allows any sector with a defect to be mapped and

bypassed. The next contiguous sector is given that sector

address.

SEEK

The radial movement of the heads to a specified track

address.

SEEK COMPLETE

An ST506 interface signal from drive to controller which

indicates that read/write heads have settled on the desired track

and completed the seek.

SEQUENTIAL ACCESS

Writing or reading data in a sequential order, such as

reading data blocks stored one after the other on magnetic tape

(the opposite of random access).

SERVO TRACK

A prerecorded reference track on the dedicated servo surface

of a closed-loop disc drive. All data track positions are

compared to their corresponding servo track to determine

"off-track/on-track" position.

SERVO TRACK      INFORMATION WRITTEN ON THE SERVO SURFACE

THAT THE ELECTRONICS OF THE DRIVE USES TO

POSITION THE HEADS OVER THE CORRECT DATA

TRACK. THIS INFORMATION IS WRITTEN ON THE

DRIVE BY THE SERVO TRACK WRITER.

SETUP            PROGRAM USED BY AT TYPE COMPUTERS TO STORE

CONFIGURATION IN CMOS.  THIS PROGRAM IS

SOMETIMES FOUND IN THE SYSTEM BIOS AND CAN

BE ACCESSED FROM THE KEYBOARD.  ON OTHER

SYSTEMS, THE PROGRAM IS ON A DISKETTE.

SILICON

Semiconductor substrate material generally used to

manufacture micro- processors and other integrated circuit chips.

SKEWING

Some low-level formatting routines may ask for a Head and/or

Cylinder Skew value. The value will represent the number of

sectors being skewed to compensate for head switching time of the

drive and/or track-to-track seek time allowing continuous

read/write operation without losing disk revolutions.

SMD

SURFACE MOUNTED DEVICE. A CHIP in a smaller integrated

surface package, without connection leads.

SOFT ERROR

A bit error during playback which can be corrected by

repeated attempts to read.

SOFT SECTOR MODE

A CONVENTION, DEFINED BY SOFTWARE, OF SETTING A

VARIABLE NUMBERS OF SECTORS PER TRACK IN DIRECT

RELATIONSHIP TO THE DRIVE'S FCI RATING IN REGARDS

TO THE AREA OF MEDIA THAT PASSES BENEATH THE

HEAD. THIS SCHEMA TAKES ADVANTAGE OF THE FACT

THAT, IN ACTUAL SURFACE AREA, THE OUTERMOST

TRACKS ARE LONGER THAN THE INNERMOST.

SOFTWARE

APPLICATION PROGRAMS, disc operating systems and other

programs (as opposed to HARDWARE). The instructions or programs,

usually stored on floppy or hard discs, which are used to direct

the operations of a computer, or other hardware.

SOFTWARE PATCH

Software modification which allows or adds functions not

otherwise available using the standard software program.

SPINDLE

The rotating hub structure to which the discs are attached.

SPINDLE MOTOR

The spindle motor is the electro-mechanical part of the disc

drive that rotates the platters.

ST-506/ST-412 INTERFACE

One of several industry standard interfaces between a hard

disc and hard disc controller. In the ST-506/ST-412 interface,

the "intellegence" is on the controller rather than the drive.

SEE INTERFACE STANDARD, ESDI AND SCSI.

STEP

An increment or decrement of the head positioning arm to move

the heads in or out, respectively, one track from their current

position. In buffered mode (open loop drives), the head motion is

postponed until the last of a string of step pulses has been

received.

STEPPER MOTOR

The stepper motor is the electro-mechanical part of the disc

drive that positions the heads by step pulse on the tracks of the

disc to read and write data.

STEP PULSE

The pulse sent from the controller to the stepper motor on

the step interface signal line to initiate a step operation.

STEP TIME

The time required by the drive to step the heads from the

current cylinder position to a target cylinder.

STORAGE CAPACITY

Amount of data that can be stored in a memory, usually

specified in kilobytes (KB) for main memory and floppy disc

drives and megabytes (MB) for hard disc and tape drives.

STORAGE DENSITY

Usually refers to recording density (BPI, TPI, or their

product, AREAL DENSITY).

STORAGE LOCATION

A memory location, identified by an ADDRESS, where

information is to be read or written.

STORAGE MODULE DRIVE

(SMD). Storage module drive interface. An interface, used in

larger disc drives, e.g., 14" drives.

SYNCHRONOUS DATA

Data sent, usually in serial mode, with a clock pulse.

TAPE DRIVE

A sequential access memory device whose magnetic media is

tape in a cassette, reel or continuous loop.

THIN FILM HEADS

A read/write head whose read/write element is deposited using

integrated circuit techniques rather than being manually

fabricated by grinding ferrite and hand winding coils.

TPI

Tracks per inch.

TRACK

The radial position of the heads over the disc surface. A

track is the circular ring traced over the disc surface by a head

as the disc rotates under the heads.

TRACK ACCESS TIME

See AVERAGE ACCESS TIME.

TRACK DENSITY

See TPI.

TRACK FOLLOWING SERVO

A closed-loop positioner control system that continuously

corrects the position of the disc drive's heads by utilizing a

reference track and a feedback loop in the head positioning

system. See also CLOSED LOOP.

TRACK PITCH

Distance from centerline to centerline of adjacent tracks

(TPI divided into 1.0).

TRACKS PER INCH

Track density, number of tracks per inch.

TRACK WIDTH

Width of data track. Also called core width of Read/Write

Head.

TRACK ZERO

Track zero is the outermost data track on a disc drive. In

the ST 506 INTERFACE, the interface signal denotes that the heads

are positioned at the outermost cylinder.

TRACK ZERO DETECTOR

An obsolete technology that RECALIBRATES by sensing when

infrared beams between a LED and infrared sensitive

photo-transistor are blocked by the track zero interrupter (TZI).

TUNNEL ERASE

An erase scheme where both sides of the recorded data is

erased when writing data to eliminate track to track

interference. This is primarily used on floppy disk drives.

UNFORMATTED

(Capacity) Drive byte capacity before formatting. Maximum

capacity of a disc drive before formatting = (bits per track) x

number of heads x # of cylinders. See MEGABYTE.

UPGRADE PATH

Generally, with disc products, a family having multiple

products with varying capacities such that the system storage

capacity can increase with changing application requirements

simply using a different disc drive within the product family.

VERIFICATION

This feature lets the computer go back and read what it just

wrote to disc to ensure the data was written correctly.

VOICE COIL MOTOR

An electro-magnetic positioning motor in the rigid disk drive

similar to that used in audio speakers. A wire coil is placed in

a stationary magnetic field. When current is passed through the

coil, the resultant flux causes the coil to move. In a disc

drive, the CARRIAGE ASSEMBLY is attached to the voice coil motor.

Either a straight line (linear) or circular (rotary) design may

be employed to position the heads on the disc's surface.

VOLATILE

Memory that will be erased if power is lost. Typically, MAIN

MEMORY is volatile, and AUXILIARY MEMORY is non-volatile and can

be used for permanent (but changeable at will) storage of

programs and data.

WAN

Wide Area Network

WEDGE SERVO SYSTEM

A certain part of each CYLINDER contains servo positioning

data. Gap spacing between each sector contains servo data to

maintain position on that cylinder.

WINCHESTER DRIVE

A disc drive with a Winchester head and non-removable (fixed)

discs sealed in a contaminant-free housing.

WORD

Number of bits processed in parallel (in a single operation)

by a CPU. Standard word lengths are 8, 16, 32, and 64 (1, 2, 4 or

8 bytes).

WRITE

To access a storage location and store data on the magnetic

surface.

WRITE CURRENT

The optimum HEAD write current necessary to saturate the

magnetic media in a cell location.

WRITE FAULT

Disc drive interface signal to the controller used to inhibit

further writing when a condition exists in the drive which, if

not detected, would cause improper writing on the disc.

XSMD

Extended storage module drive interface.

ZBR (Zone Bit Recording)

Trademark of Seagate Technology. A media optimization

technique where the number of sectors per track is dependent upon

the cylinder circumference. E.G. tracks on the outside cylinders

have more sectors per track than the inside cylinders. The ZBR

format is only done at the factory. These drives should not be

low-level formatted by the end-user.

****    End-of-File   FINDTYPE.DOC    ****