�������������������������������������������������������������Ŀ

� Trademarked names are the property of the respective owners �

���������������������������������������������������������������

Copyright (c) 1989-1991, FM de Monasterio

Licensed Material - All rights reserved

VIz version 4.16

----------------

SUMMARY

VIz is a small resident program that accelerates BIOS (and DOS) video input

and output in text mode; it can be used with IBM-compatible MDA, EGA or VGA

adapter cards and Hercules HGC and HGC+ cards.  In its present version, VIz

produces video interference ("snow") in old CGA cards; for such systems you

can use VIz-CGA, a snow-free, 8088-CPU oriented version.

In addition, VIz includes an independent, resident component for displaying

a block cursor, either continously or alternating with a line cursor (at an

adjustable rate of alternation). The latter display is well suited for easy

localization of the cursor in some laptop screens.

Finally, VIz also contains an independent, resident ANSI-filter driver that

mediates a subset of the ANSI escape sequences to control the console. This

filter can be used, instead of ANSI.SYS, to implement ANSI escape sequences

to control video display, with the advantage that can be turned off and on,

and removed from memory without rebooting.

All 3 resident modules occupy a total of less than about 1700 bytes of RAM.

Depending on the system configuration and BIOS version, video accelerations

by a factor of between 2 and 10 may be expected.

------------------------------------------------------------------------------

GENERAL INFORMATION

There are 3 basic ways of producing character-oriented, text video displays

in DOS computers: (1) using direct video memory mapping, which is extremely

fast but dependent on the video hardware and has a limited portability; (2)

via BIOS services, provided by the first "hidden" DOS boot file and usually

augmented by the video adapter, which are not as fast as direct mapping but

more portable; and (3) via DOS services, provided by the other "hidden" DOS

boot file and sometimes augmented via installable device drivers, which are

much slower than the other two approaches, but have greater portability and

permit additional operations such as redirection.

Many of the DOS and BIOS video services can be accelerated using a resident

program that intercepts the calls made to such services, and implements the

video operation with faster instructions.  Because of the nature of DOS and

of BIOS services, the interception must occur at the input site of the BIOS

services (interrupt 10h), whereas it can occur at the input site (interrupt

21h) or output site (interrupt 29h), or both, of the DOS services.

VIz is a utility that accelerates many of the basic text-video functions and

permits changing the video mode, page and font of the display as well as the

raw or normal mode of DOS video ouput.  When used with a VGA/EGA system, VIz

also permits control of the red, green, and blue mixture that determines the

color of each of the 16 registers of the EGA/VGA color palette, allowing for

the remapping of the RGB mixture to suit user preferences. Finally, VIz also

permits the control of the frequency (pitch) of the bell's tone.

VIz releases require MS-DOS or PC-IBM DOS version 2.0 or later.

A 32-bit CRC calculation is performed each time VIz is invoked. If it fails,

the program will abort to DOS; CRC failures indicate corruption of the code,

and the program therefore should not be used.

----------------

HARDWARE COMMANDS

Cursor position is updated via direct hardware commands.  Starting with VIz

release 3.00, such commands can be sent out as WORD (double-byte) values or

as two successive BYTE values. The WORD-out method of addressing, also used

by other programs (e.g. Microsoft Windows), yields faster accelerations but

it may not work with some older clone systems.

|  To test which method of addressing you can use, enter the command 'VIZ /?H'

|  to see if the cursor appears at the indicated position. If it does so, your

|  system can handle WORD-out commands.

CPU VERSIONS

|  VIz is entirely written in assembly language.  Each VIz version consists of

|  4 processor-type releases (086, 268, 386, and 486), where the code has been

|  optimized for the specific type of 80x86 processor.  This is needed because

|  the processors currently available differ markedly in instruction execution

|  times, and the availability of memory caches affects those times further by

|  influencing both speed of instruction fetching and memory access.  The type

|  and degree of code optimization vary with the processor model.

|

|  In addition to standard (086) assembly language instructions, the 286, 386,

|  and 486 releases of VIz also use 286-, 386- or 486-specific instructions in

|  real mode only (which is the native mode for 086 processors).

|

|  These releases differ slightly in the size of the residents.  Sizes for the

|  3 main installation switches are listed below, with bytes rounded up to the

|  nearest 16-byte multiple (which is how DOS allocates memory blocks):

|

|                   VIz 4.16    /Z+       /V+       /A+

|                   -----------------------------------

|                     086:      928      1024      1696

|                     286:      912       992      1696

|                     386:      944      1024      1728

|                     486:      960      1040      1856

Average accelerations and comparisons with other accelerators based on four

benchmarks are described in the accompanying file VIZ.DAT for 086, 286, 386

and 486 VIz versions.

------------------------------------------------------------------------------

1.  VIDEO ACCELERATION MODULE

VIz installs itself as a resident program that can be invoked repeatedly for

modification of its parameters, without producing multiple copies on memory;

the accelerator module of the resident uses less than 1,000 bytes of RAM. To

maximize speed of code, but minimize resident size, acceleration only occurs

for video page 0 in a text mode (1-3, color; 7, monochrome). In other pages,

or graphic modes, VIz becomes inactive and is automatically reactivated by a

change that resets the video to page 0 in a text mode.  Viz accelerates BIOS

video functions, and it can also accelerate DOS-mediated screen output.

The acceleration of VIz can be turned off either by deallocating the program

with the switch /U (if no other resident has been installed after VIz) or by

making it quiescent with switch /Q (i.e. it remains resident but not active)

until reactivation (any switch but /QU) or deallocation by a further call to

VIz.

The relative acceleration depends on several factors, including BIOS version

and date, 80x86 Intel CPU model and speed, and whether or not ROM video BIOS

is shadowed or remapped to faster RAM. Another factor, often disregarded, is

the use of other residents also intercepting the interrupt 10h. In addition,

some memory managers, even when RAM-shadowing video BIOS, have been found to

produce a not insignificant degradation of the video speed.  Whereas smaller

acceleration factors are to be expected in machines performing a faster BIOS

operation (i.e. tighter ROM code, faster CPU, shadowed or remapped ROM), the

relative weight of individual factors may yield unexpected results.  Another

factor affecting acceleration is the area of upper conventional memory (UMB)

in which VIz is loaded when switch /H is selected.  This is free memory made

available through an extended memory specification (XMS) manager (see below)

that reclaims unused memory above 640 kb but below 1 Mb, i.e., memory filled

in above the video adapters.  Contingent upon the hardware, there are marked

differences in memory access time in this region, which can affect the video

acceleration of a UMB-loaded VIz resident (see switch /H).

VIz updates relevant parameters in the BIOS data area (segment 40h), such as

cursor position (page 0 only) and type, video mode, page, and font, that are

also processed by VIz; while such updating degrades video acceleration time,

it also decreases the possibility of conflict with foreground utilities that

use BIOS data but bypass BIOS calls.

----------------

2.  BLOCK-AND-LINE CURSOR ALTERNATION MODULE

To help cursor visualization in screens where the cursor is not easily seen,

as in some laptop displays, VIz provides a small resident module that allows

for the display of a (full-)block cursor, either continuously or alternating

with a line cursor. The alternation rate can be adjusted in steps of 54.9 ms

between about 0.055 s (/V1) and 5.5 s (/V99).  A zero alternation rate (/V0)

produces a continously present block cursor.  For the module to be loaded in

memory, VIz must be installed with any /V switch (see below), and it adds an

additional 100 bytes to the resident size of VIz.

Very short or very long alternation rates are distracting or hard to follow.

When selecting a rate for laptop displays, start with a value of n=9 (.50 s)

or so.

Block cursor alternation can be turned off by making this resident quiescent

with the switch /V+ (until its reactivation with /Vn). All cursor operations

use direct commands to the hardware, and assume register-level compatibility

with the Motorola 6845 CRT controller. This chip is present in MDA, CGA, and

HGC cards, while a 6845-compatible CRTC is a custom LSI chip in the IBM-EGA,

part of the Memory Controller Gate Array on the MCGA, or a part of the Video

Graphics Array on the VGA.

Notice that the hardware-controlled blinking of the cursor is not eliminated

by this service. Hence, depending on the alternation rate selected, a 2-beat

frequency may be noticeable.

----------------

3.  ANSI-COMPATIBLE VIDEO FILTER MODULE

MS-DOS and IBM PC-DOS files include ANSI.SYS, a console filter driver that

mediates a subset of American National Standards Institute (ANSI) standard

escape sequences for video functions as well as a supplementary subset for

keyboard functions (such as key reprogramming).

VIz has a second, independently installable resident module that implements

standard ANSI escape sequences to control the console video functions. This

ANSI-compatible filter uses less than some 800 additional bytes of RAM when

installed. It can be turned off either by uninstalling all the VIz resident

modules or by inactivating the filter with switch /A-.  The inactive filter

can be reactivated with switch /A+.  All ANSI operations use BIOS calls.

The filter does not provide by itself much acceleration of DOS video I/O as

far as ANSI instructions are concerned.  Its advantages are small size, the

capability of being switched on or off, and its harmonious working with the

acceleration module of VIz.

COEXISTENCE WITH OTHER ANSI FILTERS

To avoid installing the ANSI filter of VIz if ANSI.SYS or another ANSI-like

utility intercepting interrupt 29h, such as ANSI.COM (Copyright 1989, Ziff-

Davis Corp.), is already resident, request installation with switch /A only

(as opposed to /A+).  To force installation of the ANSI filter when an ANSI

like utility is already resident, request installation with switch /A+.

The presence of an installed ANSI-like utility can be ascertained using the

Status/Usage/Help panel displayed when the command 'VIZ ?#�' is issued; the

Status line at the bottom of the screen displays one of the following:

ANSI.SYS -- Indicates that interrupt 29h (see below) points to a

resident code containing a device driver header with

the attributes of a character device servicing calls

to int 29h and the device name 'CON.' This is likely

to be ANSI.SYS or an ANSI-compatible device driver.

ANSI.COM -- Indicates that interrupt 29h (see below) points to a

resident code containing the string 'CON    ' at the

place where the device driver name should be, but it

does not contain other elements of the device driver

header.  This is an ANSI-compatible resident utility

of the type of ANSI.COM.

ANSI: No -- Indicates absence of the above conditions.

ANSI IMPLEMENTATIONS

The ANSI-compatible filter implements the following subset of ANSI 3.64-1979

standard escape sequences to control console video functions (see your MS/PC

DOS documentation of ANSI.SYS commands):

A. Selection of the foreground and background colors on a color display or

the text attributes on a monochrome display system. The sequence syntax

is <Esc>[#;...;#m, where # is 30-37 to select a foreground color, 40-47

47 to select a background color, and 0-1, 4-5 or 7-8 to select the text

attributes.

B. Selection of the video mode (similar to using the MODE command of DOS).

The sequence syntax is <Esc>[=#h or <Esc>[=#l, where # is 0-6 or higher

(depending on the adapter card in use).

C. Selection of word wrapping, i.e. whether lines longer than the width of

the screen are truncated. The sequence syntax is <Esc>[?7h or <Esc>[?7l

to turn word wrapping ON or OFF, respectively.

D. Cursor positioning for the following sequences:

<Esc>[r#;c#H  -  Move to specified row (r#) and column (c#)

<Esc>[r#;c#f  -  Same a previous sequence

<Esc>[r#A     -  Move cursor up by specified number of rows

<Esc>[r#B     -  Move cursor down by specified number of rows

<Esc>[c#C     -  Move forward the specified number of columns

<Esc>[c#D     -  Move backward the specified number of columns

<Esc>[s       -  Store current position of the cursor

<Esc>[u       -  Restore to the position stored by <Esc>[s

<Esc>[K       -  Erase from the cursor to end of the line

<Esc>[2J      -  Clear all rows and home cursor (this sequence

preserves current color attribute)

Not supported by the filter are the cursor-position-reporting ANSI sequence

(<Esc>[r#;c#R), and the supplementary set of sequences used by the ANSI.SYS

driver to permit reprogramming of the keyboard (i.e., <Esc>[#;'string';3p),

a set which is not part of the 1979 ANSI standard.

Thus, you should consider using the ANSI filter of VIz, instead of ANSI.SYS

or other ANSI-like utility, when interested in using video escape sequences

alone, without any keyboard remapping.

------------------------------------------------------------------------------

4.  USAGE

Syntax:  VIZ [/Switches] [;Comments]

The switches can be entered from:

(1) the DOS command line (COMMAND.COM shell prompt)

(2) a batch file

(3) a DOS environment string with the format VIZ=/SWITCH1.../SWITCHn

Switches can be given in any order.  Any character between space and slash

[/] can be used as a switch delimiter.  Switches are not case sensitive.

Switches /A, /B, /C, /D, /I, /M, /P, /R, and /S are "sticky" parameters in

the sense that the video change they produce will remain in effect even if

VIz is made quiescent or is uninstalled.

--------------------------------------------------------------------------

Switches marked with asterisks are available only in registered copies

--------------------------------------------------------------------------

- [/?]  Status/Usage/Help panels.  Switch /? displays a status/usage panel,

which shows the status of video I/O services, StdOut stream re-direction,

ANSI filter, DOS character processing and the current value of video page

and video mode.  When VIz is installed, a brief explanation of the status

of each highlighted service can be obtained by pressing the left or right

(cursor) arrow keys or, when a Microsoft Mouse driver 6.0 or a compatible

driver is already installed, by clicking the mouse over the screen button

marked 'Status.' Such descriptions can be erased by pressing key <F10>.

When video modes higher than n+128 (which do not clear the display buffer

when selected [AT and PS/2]) are detected, the mode number is followed by

an asterisk; thus video mode 131 is displayed as '3*' in the Status line.

In addition to the status-line panels, the status of some on/off switches

is also shown by a '+' or '-' on the switches menu before a given switch.

When switch /? is requested and a EGA or VGA adapter is present, VIz will

attempt to read (and save) the contents of the palette registers prior to

asserting its own attribute palette for the display. For the EGA, this is

attempted by setting up a 'Parameter Save Area' that the EGA BIOS can use

to store the contents of the palette registers, and the border (overscan)

register, so that these contents can be restored upon exiting the Syntax/

Status panel. If the EGA BIOS fails to store the data, VIz then loads the

default palette described below in switch /C; the original palette can be

restored by clearing the screen (if VIz had been installed with switch /C

enabled) or by resetting the video mode.

The Usage panel lists all program switches; to cycle between the menus of

main and auxiliary switches, press <F2> or <M> (or click one mouse button

on the [Menu] pad).  The Help panel provides a summary description of the

program and its switches: Press <F1> or <H> (or click one mouse button on

the [Help] pad) to show the help panel.  To return to DOS from any panel,

press <Esc> or <X> (or click one mouse button over the [eXit] pad).

--------

* [/A�]  Installs a resident filter to implement (the video subset of) ANSI

escape sequences; installation is aborted if presence of the DOS ANSI.SYS

is detected. Although this filter can take advantage of VIz acceleration,

it is independent of the accelerator resident component, and its activity

can be controlled independently with the following switches:

* /A     If VIz is not yet installed, adds the ANSI filter module

to the resident code to be installed, provided that ANSI

or another logical device named 'CON     ' mediating int

29h (see #4 above) is not installed, and inactivates the

remapping of the EGA/VGA palette via /C+, /Cn:n, or /Cm.

When VIz is already installed, /A is interpreted as /A+.

* /A+    If VIz is not yet installed, adds the ANSI filter module

to the resident code to be installed, even when ANSI.SYS

or a similar CON utility has already been installed, and

inactivates the remapping of the EGA/VGA palette via /C+

/Cn:n, or /Cm.

If VIz is already installed, and the resident filter was

made inactive with /A-, /A+ reactivates the ANSI filter.

* /A-    Inactivates the filter, if installed and active. It also

reactivates a previously active remapping of the EGA/VGA

palette.

Defaults: /A = /A+ when VIz is installed.

NOTE: The ANSI filter module is not resident unless VIz is installed with

the switch /A or /A+ and cannot be loaded after installation of the

VIz resident.

--------

* [/Bn]  Selects the color for the screen border in text mode and the back-

ground and border color for graphic modes. This color is specified by the

Red-Green-Blue mixture value given as 1 or 2-digit number in octal format

in the range from 0 to 77o (see Switch /C below).

Since the border is notoriously poor in most EGA systems (due to overscan

properties), switch /B is not likely to produce an acceptable border with

such systems.  No defaults.

--------

* [/C]   Allows changes in the EGA/VGA color palette.  This palette, which

consists of 16 registers, is used map the video memory data into colors.

In EGA-compatible modes of the VGA and in the EGA, where 16 simultaneous

colors can be displayed out of a possible 64-color palette, the value in

each one of such registers determines the amount of Red, Green, and Blue

mixture in the displayed color.

This amount can be represented by the data bits <RGBrgb>, where <000rgb>

shows the color at 33%, <RGB000> at 66%, and <RGBrgb> at 100% intensity.

These bits, and their resultant color, are shown below using both binary

(base 1) and octal (base 8) number formats:

�������������������������������Ŀ

� binary  � octal �    Color    �    Since the binary format takes up a

�������������������������������Ĵ    lot of space it is more convenient

� 000 000 �  00   � 100% black  �    to use the 2-digit octal format to

� 000 001 �  01   �  33% blue   �    represent an RGB value. The binary

� 000 010 �  02   �  33% green  �    equivalence of octal numbers is:

� 000 011 �  03   �  33% cyan   �

� 000 100 �  04   �  33% red    �

� 000 101 �  05   �  33% magenta�              0o = 000000b

� 000 110 �  06   �  33% yellow �              1o = 000001b

� 000 111 �  07   �  33% white  �              2o = 000010b

�������������������������������Ĵ              3o = 000011b

� 001 000 �  10   �  66% blue   �              4o = 000100b

� 010 000 �  20   �  66% green  �              5o = 000101b

� 011 000 �  30   �  66% cyan   �              6o = 000110b

� 100 000 �  40   �  66% red    �              7o = 000111b

� 101 000 �  50   �  66% magenta�             10o = 001000b

� 110 000 �  60   �  66% yellow �             11o = 001001b

� 111 000 �  70   �  66% white  �             12o = 001010b

�������������������������������Ĵ             13o = 001011b

� 001 001 �  11   � 100% blue   �             14o = 001100b

� 010 010 �  22   � 100% green  �             15o = 001101b

� 011 011 �  33   � 100% cyan   �             16o = 001110b

� 100 100 �  44   � 100% red    �             17o = 001111b

� 101 101 �  55   � 100% magenta�             20o = 010000b

� 110 110 �  66   � 100% yellow �                etc

� 111 111 �  77   � 100% white  �

���������������������������������

(�) IBM-EGA default color palette

Of course, other colors can also be generated when using a different RGB

mixture value.  For instance, a desaturated (i.e., whitish) RGB sequence

is obtained in the octal range 71o through 76o.  Those using the utility

SPECTRUM.COM (Copyright 1987 Ziff-Davis Publishing Corp.) will recognize

the <RGBrgb> octal format as the so-called 'SPECTRUM code.'

This palette control is also enabled (VIz 2.42+) for the monochrome mode

of VGA and MCGA adapters to remap color codes into gray-scale values.

The following services are available:

* /Cn:n  To modify the EGA/VGA color palette from the DOS command

line, type a 16-number sequence in which numbers must be

separated by colons (:) as in the following example:

/C03:00:20:30:40:50:01:77:70:11:22:33:44:55:66:76

that represents the default palette colors loaded by VIz

when switch /C? is used.  When an incomplete sequence is

provided, the RGB values are written into the respective

palette registers starting from register 0.  When a non-

numeric value is provided, the palette is not changed.

|            * /Cm    Unlike the EGA, when the MCGA and VGA emulate a 16-color

|                     text mode for a monochrome display, the palette consists

|                     of four groups of 4 shades of gray. In monochrome modes,

|                     switch /Cm loads a palette in which the gray-scale value

|                     increase uniformly with increasing attribute values. The

|                     monochrome palette corresponds to the command:

/C00:50:01:31:61:12:42:03:43:04:44:05:55:26:07:77

Switch /Cm can also be used with CGA monochrome displays

(e.g., laptops) to translate color attributes into gray-

scale values.

|                     In a color mode, switch /Cm� enables [+] or disables [-]

|                     the BIOS gray-scale summing of the RGB mixture, by which

|                     the gray-scale equivalent of a given color is 30% of the

|                     Red, 50% of the Green, and 11% of the Blue values in the

|                     video digital-to-analog registers (these percentages are

|                     close to the relative brightness of the displayed 'pure'

|                     red, green, and blue. (On MCGA/VGA gray-scale summing is

|                     used by default when a monochrome display is attached.)

/C?    This service provides a display of the palette registers

and the corresponding RGB mixture in octal format. It is

possible to alter the RGB mixture of any register simply

by pressing <R>, <G>, or <B> and then <#> or <#>.  These

arrows cycle through 0%, 33%, 66%, and 100% intensity of

the respective RGB component.  The actual RGB mixture of

the selection is also shown on a graph.  Press the right

or left arrow to cycle through the registers, and <HOME>

to go to register 0 or <END> to go to register 15.

Press <ENTER> to save the current palette, <ESC> to quit

the service without any palette changes, <DEL> to reject

any change and to restart the service, and the indicated

function keys to load several available palettes.

* /C-    Use to inactivate the implementation of EGA/VGA palette

changes via VIz.  The video attributes remain unchanged

until the a video operation reloads the palette, e.g. a

video mode change.  Switch /A+ also turns off remapping

of the palette.

* /C+    Use to reactivate the implementation of EGA/VGA palette

changes via VIz; it reloads the last color palette that

was selected via VIz.  Switch /A- reactivates remapping

if remapping was in effect prior to an /A+ request.

Defaults: /C = /C+.

The selected EGA/VGA palette changes are asserted (i.e. the palette is re-

loaded with the selected RGB values) each time that a video mode change or

a video font change (int 10h functions 5h and 11h) is requested and VIz is

active (i.e., no /Q switch, graphics mode, or video page other than 0).

To preserve the small size of Viz, there is no hotkey pop-up service. If a

program changes the palette when executed, the RGB mix values selected via

VIz cannot be asserted except by shelling to DOS and invoking VIz with the

switch /C+ to reload the palette or triggering a video mode/font change.

--------

* [/Dn]  Adjusts brightness of screen (VGA card only); the brightness change

step is specified by the sign and value of number <n> between -63 and +63.

Use of a high negative or positive number results in an unreadable screen.

Requesting /D0 results in restoration of the screen to its default setting

(as dictated by the manual controls of the video monitor).  Defaults: /D =

= /D+1; /D1 = /D+1.

--------

* [/E]  Creates the DOS environment string VIZ=/SWITCH1.../SWITCHn, in which

SWITCH1...SWITCHn are the switches specified when VIz is executed from the

DOS command line.  When the string VIZ= already exists, the switch updates

the string contents.  (Although the switch is incorporated into the string

contents, its presence is ignored when VIz uses such string as input.)

Switch /E modifes the global environment whether the resident shell is the

initial copy of COMMAND.COM (global environment) or a secondary one (local

environment).  See section 5.1 for more details.  No defaults.

--------

|  * [/FHx]  If VIz detects a HERCULES card adapter (HGC or HGC+), switch /FHt

|    changes the card to text mode (80x25) and switch /FHg changes the card to

|    the default (page 0) 720x348 graphics mode.

|  * [/Fn�]  If VIz detects an EGA or VGA adapter, switch /Fn allows selection

|    of the number of displayed rows on the screen.  The following video fonts

|    are supported:  12 (EGA), 14 (VGA), 25 (EGA/VGA), 28 (VGA), 35 (EGA/VGA),

|    43 (EGA), and 50 (VGA) displayed rows.  Except for the 12-/14- and 35-row

|    displays, a selected font is implemented until switch /F- (which disables

|    font implementation) or another font is selected; font implementation can

|    be reenabled with /F+.

|    If a number <n> is not specified, switch /F toggles between 25-row and 43

|    (EGA) or 50-row (VGA) displays. (If an EGA/VGA adapter is found, VIz also

|    selects the alternate BIOS print-screen service to avoid the default BIOS

|    print-screen routine that works with 25 rows only.)  For 25-row displays,

|    VIz uses a 8x14 font for EGA or a 9x16 font for VGA. Fonts with more than

|    25 rows can be loaded only if the current video page is between 0 and 3.

|    Note that by using video mode 0 (40-column screen display) in combination

|    with the various fonts, considerable screen magnification can be achieved

|    for low-visual acuity users. To avoid screen overrun during this display,

|    use the XMORE.EXE utility.  No defaults.

--------

* [/Gn]  Selects frequency in Hertz (cycles per second) of 55-ms bell tones.

Values between 25 Hz and 5,000 Hz are accepted.  Values below 25 become 25

Hz and above 5,000 become 5,000 Hz; use of more than 4 digits results in a

reading of the 4 least significant digits (i.e. 50,000 Hz is read as 0000,

triggering the use of 25 Hz).  Defaults: /G = /G440.

(Most computer speakers work properly only in the range between 100 Hz and

3,000 Hz.  Frequencies for the octave starting at middle C are: C = 523.3,

D = 587.3, E = 659.3, F = 698.5, A = 880.0, and B = 987.7 Hz.  Frequencies

at higher [lower] octaves increase [decrease] approximately by a factor of

2 per octave change from the above values.)

The tones always last 55 ms (one tick of the 8253/8254 timer chip).  Since

the tones are forced to begin at the start of a 55-ms tick period, stacked

bell calls will result in a distinctive, staccato burst due to intervening

silent 55-ms tick periods.

--------

* [/Hn]  Instructs the program to load its resident module(s) in high memory

between 640 kb and 1,024 kb, i.e., below the 1 Mb boundary of conventional

RAM but above the RAM area normally controlled by DOS.  Switch /H requires

that an XMS manager (XMM) complying with the specifications of XMS 2.0 has

been previously installed, and that an upper memory block (UMB) of about 2

kb is available. The XMM must support functions 10h (Allocate UMB) and 11h

|    (Free UMB).  To test support for these functions, use the program UMAX.EXE

|    (see UMAX.DOC).

Unlike the high loading provided by 386 memory managers, the required size

of the free UMB is that of the resident(s) and not that of the entire code

of the program; a few additional bytes are used by the XMM when allocating

the UMB.  (See High Memory Installation section.)

A lower high-memory limit for the UMB can optionally be specified via <n>,

a (hexadecimal) number between A000 (640 kb) and FFFF (1,024 kb), so as to

avoid specific areas of high memory with a slow access time. Memory access

times are provided by some memory managers, e.g. QEMM386 (Quarterdeck) and

386MAX (Qualitas), but not by others, e.g. HIMEM.SYS (Microsoft).  To time

memory access in more detail, use the program TIMEM.EXE (see TIMEM.DOC).

Defaults:  Low-memory installation, if any errors are found during the UMB

loading. High-memory installation below lower limit if no UMB is available

above such limit.

--------

* [/I�]  Toggles blinking (/I-) and intense (/I+) background colors whenever

the background attribute corresponds to a value between 8 and F (bit 7 on)

in the selection of screen foreground/background color.  No defaults.

--------

* [/Mn]   Selects the video mode specified by a number <n> in the range of 0

through 99; note that there is no checking that the selected mode is valid

for the adapter.  No defaults.

--------

- [/N�]   Messages dealing with nonfatal operations are either not displayed

(/N+), as if they had been sent to the NUL device, or (normally) displayed

via the StdOut device (/N-), which can be redirected if switch /Z+ has not

been selected.

Since VIz gives a beep when a command error is detected, switch /N- should

need to be used only after such a beep to display the error message again.

Note that the installing, uninstalling and fatal-operation messages of VIz

cannot be redirected and are always displayed. The status of operation can

be monitored in batch files with ERRORLEVEL commands.  Defaults: /N = /N+.

--------

* [/Pn]   Selects the video page specified by a number (n) in the range of 0

to 7 for EGA/VGA with 25-row font, or 0 to 3 for CGA and 43/50-row EGA/VGA

adapter.  No defaults.

--------

- [/Q]   Quiescent.  Switch /Q forces VIz to a 'quiet' mode; although still

resident, VIz will not provide video acceleration.  The overhead time for

having VIz in quiescent mode is 2% or less of that without VIz. Note that

even when quiescent or during graphics video mode, VIz still monitors the

BIOS video interrupt to check for page or mode change requests. Switch /Q

disables video BIOS and StdOut (/Z+) acceleration, but does no affect the

operation of the ANSI filter (/A+).

--------

* [/R�]   Forces the StdOut character device to normal text output mode [R-]

or 'raw' mode [R+].  In normal mode output, the DOS kernel builds a device

request header for a single character output, makes a Control-C check, and

passes the request to the device; if there is no error, the buffer pointer

and character count are updated, and this process is repeated until either

an end-of-file character (ASCII 26, Ctrl-Z) is found or all characters are

processed.  Tabs (ASCII 9) are expanded to 8 spaces.

In raw or binary mode, however, DOS does not filter the stream for control

characters, but a single request header is passed to the device. Depending

on the DOS version, the DOS-mediated video output is faster in raw mode by

factor of about 1.40 (a much larger factor is provided by switch /Z+).  No

defaults.

NOTE:   When using 'raw' DOS mode as the default condition, be sure to set

BREAK=ON in the CONFIG.SYS file to facilitate Ctrl-Break detection

Some utilities set the DOS mode to 'raw' when executed and then change the

mode to normal when they finish, irrespective of the mode encountered when

they were executed.  Hence, repeated use of switch /R+ may be necessary to

maintain the raw character processing mode.

--------

|  * [/Sn�]  Enables [Sn+] or disables [Sn-] special services triggered by some

|    keys, which can have untoward effects under some circumstances, especially

|    in LAN servers.

|

|            * /S1�   Use to enable [+] or disable [-] the Print Screen service

|                     triggered by the <PrtSc> key.  Switch /S1- is useful when

|                     the keys of the number pad are used often, and the chance

|                     of triggering unwanted prints is commensurably high. In a

|                     machine not connected to an on-line printer, <PrtSc> will

|                     freeze the system.

|

|            * /S2�   Use to enable [+] or disable [-] the <Pause> key service

|                     (in PS/2s and COMPAQs) or the <Ctrl><NumLock> combination

|                     (in PCs) which produce a dynamic halt of processing until

|                     another key is pressed. Switch /S2� is active only if the

|                     alternating cursor module (see switch /V) is also loaded.

|

|    When /S1� (or /S�, or simply /S) is requested at the time of installation,

VIz selects via a BIOS call the alternate print-screen service in the EGA/

VGA card BIOS extension that works properly if the screen length is not 25

lines (when the card is an EGA or VGA adapter).  This alternate service is

needed when other fonts are to be used, since the ROM BIOS default service

only prints 25 lines.

|    NOTE:  Some EGA cards do not work properly when the alternate print-screen

|    service is enabled. If selection of switch /S1� during installation yields

|    a print-screen malfunction, first install VIz without selecting /S1�, /S�,

|    or /S�, and then select switch /S1 in a subsequent execution.

|

|    Defaults: /S = /S1+ (enable BIOS alternate print-screen service).

--------

- [/U]   Uninstall.  Switch /U uninstalls the resident components of VIz if

the original interrupt vector 10h (and 29h, if the ANSI resident has also

been installed) has not been changed.  As with any other resident, do not

uninstall VIz after another resident has been installed since the machine

will crash if such resident had intercepted any of the interrupts used by

VIz. (Otherwise, VIz can be removed safely, although the resulting memory

'hole' contributes to the memory fragmentation of the system.)

Starting with version 2.20, VIz can be identified by name in low-memory-

usage maps. Although VIz removes its own copy of the DOS' environment at

at loading time (to minimize resident size), 'VIzxxx' will appear in the

in the listings of these maps irrespective of the actual contents of the

DOS command line at the time of VIz installation.

For example, MAPMEM.COM (TurboPower Software) should display

the following identification for a normal, lower-memory load

of the 386+ release of VIz (accelerator module only):

PSP   blks bytes owner    command line        hooked vectors

----- ---- ----- -------  ------------------  --------------

hex#   1   944   N/A      VIZ416              10 21

With residents loaded in high memory via switch /H, however, some memory

mapping utilities fail to list the allocated UMB  (or lump it with other

allocated blocks.  When XMS services are implemented by a 286/386 memory

manager, use the mapping services of the manager to examine high memory.

For example, 386MAX (Qualitas), version 4.09, should display

the following listing for an upper-memory loading of the 286

release of VIz (accelerator module only):

High DOS Memory

------------------------------------------------------------

Hex    Hex    Hex     Decimal

Name        Start  End    Owner   Length   Interrupt Numbers

------------------------------------------------------------

hex#   hex#   -UMB-       48

hex#   hex#   -UMB-      912   10 21

------------------------------------------------------------

Alternatively, MANIFEST (Quarterdeck) version 1.00, provides

the following listing for the same high-memory loading:

hex# - hex#   0.1K   (yyyy)

hex# - hex#   0.9K   (zzzz)

where name 'zzzz' is also listed as owning the interrupts:

hex#: (zzzz)      10 21

--------

- [/V]   Forces a display of an alternating full-block-and-thin-line cursor

[Vn, n=1 to 99] or of a continuous full block [Vn, n=0], makes the cursor

invisible [V-], or allows display of the normal cursor [V+]. Switches /Vn

and /V- produce changes of cursor type that can be reversed with /V+. Due

to a potential BIOS conflict, switch /V- is ignored at installation time;

to make an invisible cursor, first use /V+ or /Vn and then /V-.

Defaults: /V = /V+ when VIz is installed.

NOTE:  The cursor module is not resident unless VIz is installed with the

switch /V+ or /Vn,  and cannot be loaded after installation of the

VIz resident.

--------

- [/Z�]  Directs video output mediated via interrupt 21h, fn. 40h (write to

device [StdOut]) to the console driver [Z-], or to the accelerator module

of VIz [Z+].  Defaults: Null switch or /Z = /Z-.

Switch /Z+ provides a significant acceleration of video output for DOS-2+

services and many other utilities using the write-to-StdOut function.

See below for conflicts with StdOut redirection. Z+ processing uses an 8-

|    space expansion for tabs (which is the standard expansion used by DOS) in

|    text modes with 80 columns or more.  This, however, conflicts with the 4-

|    space expansion used by the DOS command 'DIR' for its switch /W.  In text

|    modes with 40 columns, Z+ processing uses a 4-space tab expansion.

--------

Comments may be added in the DOS command line invoking VIz after the desired

switches.  The comments, which may be useful in clarifying batch files, must

be preceded by a semicolon (;), and are ignored by VIz.

NOTE:   Do not use the DOS redirection (<>) and pipe (|) characters in these

comments as DOS will attempt to implement the implied redirection or

pipe request.

------------------------------------------------------------------------------

5.0  INSTALLATION

It is recommended that VIz be installed before other resident programs also

using the BIOS video interrupt 10h.  Due to the nature of some of the video

functions served, VIz often will not daisy chain the interrupt-10h requests

to residents located down the intercepted interrupt-10h chain. In order for

these residents to receive interrupt-10h data, they must be installed after

VIz has been installed; thus, it is convenient to install VIz early in your

AUTOEXEC.BAT file.

Keep in mind that the larger the number of additional residents, the slower

the absolute video acceleration times provided by VIz.  This is because the

int-10h request has to be processed by each element in the interrupt chain.

One way to minimize the problem is to install a minimum number of residents

via the AUTOEXEC.BAT and, before executing a given application, install any

additional resident via a batch file that also executes the application and

removes the residents upon the completion of the task. (VIz itself may fall

in this transient category.)

If you intend to make occasional use of the ANSI-compatible filter, without

removing the resident ANSI component from memory, VIz may be installed just

as decribed above.  However, if you want to remove the ANSI component after

a given ANSI-oriented task, then VIz must be the last resident to have been

installed prior to such a removal.

ANSI.SYS or ANSI.COM should not be loaded in order to use the ANSI resident

of VIz.  Notice that if an ANSI filter is active and VIz is switched to its

'fast DOS' mode (/Z+), this filter is likely to be unable to process escape

sequences, unless the sequences are sent directly to the CONsole device via

interrupt 29h instead of via DOS interrupt 21h.

5.1  HIGH-MEMORY RESIDENT INSTALLATION

DIRECT UMB LOADING

Starting with version 4.02 VIz can install itself directly into the upper

conventional memory (RAM addresses between 640 kb and 1024 kb) via switch

/H.  The loading into an upper-memory block (UMB) is arbitrated by an XMS

manager (XMM) version 2.0 or higher, which must be installed prior to the

installation of the resident.  UMB loading also requires the remapping of

upper memory addresses by a UMB provider, if this task is not implemented

by the XMM; the XMS manager HIMEM.SYS and the UMB-provider EMM386.EXE are

supplied with DOS 5.  Other 386-specific memory managers, such as 386MAX.

SYS or QEMM386.SYS, provide both XMS support and UMB remapping.  All XMMs

require DOS version 3.0 or later.

For the loading to be successful, an upper memory block (UMB) of the size

of the resident, plus an additional 48-byte block used by the XMM must be

available.

Switch /Hn, where A000 � n � FFFF (hexadecimal), allows the specification

of a lower high-memory bound for the UMB to avoid upper-memory areas with

a very slow access time (or to preserve areas with a fast access time for

other utilities that require faster times, such as a disk cache or screen

accelerator).

Hence, compared to the high-memory installation provided by some (but not

all) memory managers, direct UMB loading has two major advantages: (1) it

does not need an initial free block of high memory the size of the entire

program, as opposed to the resident size, to be available, and (2) allows

some user control on the selection of the high-memory block.

LOADING VIA 386-MEMORY MANAGERS

VIz can also be loaded into high memory via 386-specific memory managers,

provided that there is enough high-memory for the actual size of VIz plus

the additional memory that is required by the loading utility.

VIz is a compressed file that expands when loaded. The difference between

the loaded and the directory size of VIz may produce conflicts with high-

memory loading utilities that fail to measure the actual size of the file

being loaded. If there is not enough high-DOS memory to permit expansion,

but there is enough memory for the nominal directory size, such utilities

will load the compressed program, whose subsequent expansion is likely to

overwrite adjacent code and crash the computer.  Note that when switch /?

|  or /C? has been selected, an additional 4,800 bytes of RAM are needed.

Further, even when the proper size is specified and available, some high-

memory loading programs for 386 systems may become corrupted when loading

into high memory compressed files of the type of VIz.

5.2  HIGH-MEMORY RESIDENT UPDATING

Starting with version 3.40, a high-memory loading utility is not required

to update the resident component of VIz, when this resident was installed

in the first megabyte of memory (286+ CPU).  Such updates can be made via

normal calls from the DOS command line or batch-file calls. This obviates

a major restriction in using high memory installation, namely the need to

keep enough free high memory to load an entire program only to update its

small resident component.

5.3  HIGH-MEMORY RESIDENT REMOVAL

When VIz has been installed in high memory directly via switch /L, it can

be removed directly from memory via switch /U from the DOS command line.

When a high-memory installation was mediated by a memory manager, Viz can

be removed with switch /U directly from the command line or after opening

the high memory with the memory manager. When the resident is removed via

such a manager, notice that some of these utilities warn that a "resident

installation has failed."  Such warnings are harmless and may be ignored.

------------------------------------------------------------------------------

6.   TECHNICAL INFORMATION

This section describes technical information that may help in the selection

of the above VIz switches, and in any potential conflicts that may be found

when other residents are loaded.

6.1  INTERCEPTED INTERRUPTS

VIz versions 2.40 and higher do not monitor interrupt 1Ah any longer before

attempting to uninstall the residents.  The following hardware and software

interrupts are intercepted according to the loading configuration:

INTERRUPT 8h (system timer):

Channel 0 of the 8253/8254 timer chip produces a hardware interrupt (IRQ0),

interrupt 8h, about 18.2 times a second that is used by the system time-of-

day clock. When the cursor module is installed, switch /Vn (n=0 or 1 to 99)

forces the display of the block cursor, either in continuous or alternating

manner. Cursor shape is implemented via commands using either 16-bit (word)

or 8-bit (byte) output, depending on the VIz version that was selected.

INTERRUPT 10h (video BIOS):

The accelerator module of VIz intercepts the ROM BIOS video interrupt, 10h.

When the system is in page 0 and a text mode, video acceleration occurs for

subfunctions 0h through Eh, except for subfunctions 4h, 7h, Bh, and Ch.

INTERRUPT 21h (DOS services):

Starting with version 2.0, DOS allows the redirection of the Standard Input

(StdIn) and Standard Output (StdOut) console device, permitting programs to

send input to or to receive output from other programs.  Unless redirected,

the StdIn comes from the keyboard and the StdOut goes to the screen.  These

and some other devices can be addressed by predefined DOS 'file handles' as

if they were disk files.  A common DOS-mediated technique to send output to

the screen is to write to the handle 1 (StdOut) via interrupt 21h, function

40h; in fact, this is the preferred method of DOS-mediated video output for

DOS version 2.0 or higher.

The accelerator module of VIz also intercepts interrupt 21h. Switch /Z+ re-

directs the 21h/40h/1-console stream (StdOut) to the accelerator, resulting

in significant accelerations of the DOS video services. The acceleration is

not increased by setting the DOS mode to 'raw' (see below).  Switch /Z- (or

default installation) permits the 21h/40h/1-video output to be processed by

the 'CON' device.

The redirection of StdOut to VIz does not conflict with the CLS command (as

the redirection does not affect interrupt 29h [see section 6.2]). Selection

of switch /Z+, however, voids any redirection of the StdOut, including that

to the 'NUL' device (e.g. to avoid some messages), since the 21h/40h-stream

is intercepted by VIz before the output can be redirected by DOS.

Switch /Z requests are not disregarded when ANSI.SYS, ANSI.COM, or VIz-ANSI

filter is active. Notice, however, that the redirection of StdOut by switch

/Z+ will conflict with any ANSI command sent to the CON device via standard

output calls to interrupt 21h/40h/1.  ANSI commands using interrupt 29h are

not affected by switch /Z+ (see below).

INTERRUPT 29h (fast character video output of CON)

The ANSI-compatible module of VIz intercepts interrupt 29h, an undocumented

feature present in all recent versions of DOS. Int 29h is normally provided

by the built-in console device driver of DOS, and consists of a call to the

BIOS interrupt 10h.  DOS uses int 29h instead of the normal request-passing

mechanism to accelerate output to the current 'CON' device. The ANSI module

filters this output.

If a request to inactivate the installed ANSI filter is encountered (switch

/A-), VIz checks whether the int 29h can be restored to its original memory

address. If the revectoring is not possible (i.e. when another resident has

also intercepted int 29h), this filter remains partially active in order to

handle an ANSI escape sequence generated by COMMAND.COM to clear the screen

when a CLS command is detected (see below); other ANSI escape sequences are

ignored by the filter and passed to the screen.  Hence, when revectoring is

possible, switch /A- has the same effect as installing VIz without a switch

/A, except that the memory occupied by the ANSI filter is not released; the

filter is reactivated by a subsequent call to execute VIz with switch /A+.

VIz versions 3.64 and higher do not redirect interrupt 29h to the teletype

service of VIz any longer (old switch /X�), since a much faster DOS output

can be obtained with switch /Z.

6.2  THE CLS COMMAND AND INTERRUPT 29h

The internal console device driver installed during booting handles the CLS

as well as int 29h service. The CLS procedure of COMMAND.COM first tests to

see if (1) the standard-output handle is associated with a character device

driver (bit 7=1), and (2) the driver services int 29h (bit 4=1).  If one or

both tests fail, or if both tests are successful but the vector for int 29h

does not point to the memory segment of the DOS kernel, COMMAND.COM outputs

the ANSI escape sequence <Esc>[2J to int 29h; otherwise, it makes a call to

int 10h, subfunction 6 of the BIOS to clear the screen.

When int 29h cannot be revectored on /A-, the partial driver activity slows

down slightly the DOS video output whenever the ASCII character 27 (Esc) is

found in the character stream. This partial activity is needed, however, as

DOS issues an ANSI escape sequence to clear the screen whenever int 29h has

been revectored.  Thus, if VIz were not to retain its ANSI driver partially

active when revectoring of int 29h is not possible, the DOS escape sequence

would be displayed (without clearing the screen) whenever a CLS command was

issued.

6.3  ENVIRONMENT VARIABLE 'VIZ'

Starting with VIz version 3.63, when VIz is executed without any arguments

from either the DOS command line or a batch file, the program switches can

be specified via a DOS environment variable 'VIZ' having the format:

VIZ=/SWITCH1/SWITCH2..../SWITCHn

Up to 128 characters are allowed after the equal sign.  There should not be

spaces between switches; use slash as a separator. The string can be placed

in the DOS environment block via the SET command or via switch /E.

While the SET command modifies the default environment block, whether it is

the global or a local one, switch /E always modifies the global environment

block (even when VIz is executed via a secondary copy of COMMAND.COM, as in

the case of 'shelling to DOS' from inside an application).  Note that local

environment blocks (i.e. a secondary COMMAND) are only a copy of the master

global block and lack sufficient room to increase the length of a variable;

hence, when the default block is a local one, switch /E does not modify it.

In contrast, when reading the environment variable, the program always uses

the default environment block.

# NOTE:  See the file DOS_ENV.TXT, distributed in this release, for a more

detailed description of the DOS environment block.

A recommended use of the environment string is to specify the default /Cn:n

configuration (e.g. /C03:00:20:30:40:50:01:77:70:11:22:33:44:55:66:76) that

can be restored simply by executing VIz without any arguments after the RBG

EGA/VGA palette has been changed.

6.4  DOS ERRORLEVEL

Starting with version 3.10, VIz passes to DOS execution status values that

can be tested in batch files via the ERRORLEVEL command; the following are

values for version 4.02 or higher:

Value   Condition

-----   --------------------------------------

255   Cyclical Redundancy Check failure

254   CPU type does not support the 386 or 286 version

|                 253   Not enough memory for /C? or /? or to install

252   Unable to remove resident from memory

251   Invalid cursor (switch /Vn) request

250   Invalid ANSI (switch /A) request

128   XMM not installed or XMS error in UMB load

64   Incomplete color palette data list

32   Invalid color palette datum

16   Invalid video font for adapter and/or page

8   Invalid video mode request

4   Invalid video page for adapter and/or font

2   Invalid video adapter for the request

1   Unknown or invalid switch request

0   Succesful execution

Note that some errorlevel conditions in the range between 1 and 128 may be

additive, and the errorlevel output may represent the sum of their values.

------------------------------------------------------------------------------

SINGLE USER (BBS-DISTRIBUTED) LICENSE AGREEMENT

7.  REGISTRATION

These programs and the documentations (the "software") are the copyrighted

property of FM de Monasterio (the "author"), who provides the software and

licenses its use.  Neither the title to nor ownership of this software are

transferred to you by such a license, and both are retained by the author.

All rights are reserved.

The software is user-supported; the enclosed release, although lacking the

options marked by asterisks, is a fully usable program.  You may test this

release for (in)compatibilities with your system, but after the testing is

completed you are requested to order a registered release of this software

from the address below.

Alternatively, to continue use of this unregistered copy, send to the same

address a US$5.00 check marked "For Deposit Only," and payable to "PATIENT

CARE FUND, CHILDREN'S HOSPITAL" of Washington, DC.  Unregistered users who

donate to this fund may legally keep such a copy under the injunctions and

limitations described below.

A form needed for the registration or upgrade of this software is provided

in the file VIZ.REG.  Refer all inquiries about the software to:

FM de Monasterio

P.O. Box 219

Cabin John, MD 20818-0219

LICENSE

Upon registration you receive a non-exclusive, non-transferable license to

use the software. Two types of license are available:

Single User License:

Upon registration/upgrade you are granted a nontransferable license to use

the software in a single computer at a time. You may transfer the software

physically to another computer provided that the software is then not used

on more than one computer at any time.

Site/10-Computer License:

Upon registration/upgrade you are granted a nontransferable license to use

the software in a single site or a set of sites provided that the software

is not used in more than ten computers at any time and that such computers

are located exclusively within the registered site(s).

UPGRADES

Within the same major version:  Registered software users can upgrade free

of charge within the same major version (e.g., 2.00 through 2.99).

Outside the same major version:  Registered users can upgrade for half the

the license fee for a different version of the software.

To upgrade send a self-addressed stamped diskette mailer with the original

360 kb diskette to the address above, along with the upgrade form.

SUPPORT

Limited technical support is provided to registered users. If you discover

a problem, please try to replicate it under the same conditions and, then,

in the absence of other resident programs. (Changes in the order residents

are installed sometimes solve conflicts.) If the problem can be replicated

please write at the address above providing:

- Your name and address.

- Program version number (see the Syntax/Help panel).

- Computer model and configuration, and hardware used.

- A sufficiently detailed description of the problem.

- A list of resident and nonresident programs in use,

and their sequence of loading into memory.

Messages may be left c/o Sysop at:    The DC Information Exchange BBS

~~~              202-433-6639, Sysop: Bill Walsh

(2400+ baud).

INJUNCTIONS

You may distribute this software via magnetic and/or electronic means, but

you are specifically prohibited from:

- Charging fees or asking donations in exchange of or payment

for copies of the software.

- Distributing this software with commercial products without

the written permission from the author.

- Distributing this software via a for-profit organization or

group, either alone or with other software.

The unauthorized copying, decompiling or disassembling of this software is

prohibited; any other use of this software is prohibited without a written

permission in advance from the author.

LIMITED WARRANTY

The author-copyright owner warrants for a period of thirty (30) days from

the date of delivery that, under normal use and without any modification,

the programs substantially conform to the accompanying specifications and

that the documentation is substantially complete and contains information

the author deems necessary to use the programs. If, during the thirty-day

period, a demonstrable defect in the programs, magnetic medium upon which

they are distributed, or documentation should appear, you may return this

software for repair or replacement, at the author's option. If the defect

cannot be repaired, or the software replaced with functionally equivalent

software, within sixty (60) days of the author's receipt of the defective

software, then you shall be entitled to a full refund of the license fee,

but not of the mailing/handling costs described in the registration form.

All other warranties of any kind, either express or implied,including but

not limited to the implied warranties of  merchantability and fitness for

for as a particular purpose, are expressly excluded.

The author cannot and does not warrant that any function contained in the

programs will meet your requirements, or that the operation will be error

free. The entire risk as to the software performance or quality, or both,

is solely with the user, and not with the author.

LIMITATION OF REMEDIES

You assume responsibility for the selection of the program to achieve your

intended results, and for the installation, use, and results obtained from

the software.

The author makes no warranty, either implied or expressed, including with-

out limitation any warranty with respect to this software documented here,

its quality, performance, or fitness for a particular purpose. In no event

shall the author be liable to you for damages, whether direct or indirect,

incidental, special, or consequential arising out the use of or any defect

in the software, even if the author has been advised of the possibility of

such damages, or for any claim by any other party.

The author's entire liability, and your exclusive remedy shall be: (1) the

replacement of an original software diskette not meeting the above Limited

Warranty and which is returned to the author along with proof of purchase,

or (2), if the author is unable to deliver a replacement diskette which is

free of defects, you may terminate the License Agreement by returning this

software and the corresponding license fee will be returned.

GENERAL

By using the software, you acknowledge (1) to have read and understood all

parts of this document and (2) to have agreed with and accepted all of its

provisions without any reservation.

------------------------------------------------------------------------------

[END]