ATSEND V1.6

(Formerly AT.EXE)

By Joseph Sheppard, Sysop, The Ledge PCBoard - (818) 896-4015

�����������

DESCRIPTION

�����������

ATSEND.EXE allows users to send "AT" commands to Hayes compatible modems

from the DOS command prompt, or from batch files.

ATSEND.EXE does not need a DTR utility to make it work.  DTR is handled

directly by ATSEND.

ATSEND.EXE follows the DSZPORT= setting if it's found in the DOS

environment.

ATSEND.EXE works with non-standard COM ports via environment

variables COMBASE and COMIRQ.

ATSEND.EXE is written with Borland's Turbo Pascal  using communication

routines from Prokit, (c) Samuel Smith.

������������

HOW IT WORKS

������������

To send a command to the modem, the user runs ATSEND.EXE by

typing ATSEND at the DOS command prompt followed by the command to be sent

to the modem, and the port the modem is connected to.

The default port is COM1 at standard address 3F8/IRQ4, so if your modem is

attached to COM1 then you don't need to specify a port on the command line,

or put it in the DOS environment using the DSZPORT variable.

If your modem is connected to COM2 at standard address 2F8/IRQ3, then you

will always need to follow the command to be sent with the number 2, or

have DSZPORT=2 in your DOS environment.  Non-Standard COM Ports are

supported and explained later in this document.

��������

EXAMPLES

��������

ATSEND H1               (Take modem connected to COM1 off the hook)

ATSEND H1M0 2           (Take modem connected to COM2 off hook and turn-off

speaker)

ATSEND M1DT18188964015  (Tell modem connected to COM1 to dial

1-818-896-4015)

ATSEND Z 2              (Perform a reset on the modem connected to COM2)

To use the DSZPORT variable, and avoid having to type the port

number on the command line, put the following line in your

AUTOEXEC.BAT file:  SET DSZPORT=<PORT>.  For example, if you are

using COM2:

SET DSZPORT=2

���������������������������

NON-STANDARD PORT SETTINGS:

���������������������������

ATSEND works with standard and non-standard serial port configurations.

Here are the defaults:

Port    Base Address    IRQ

COM1    3F8             IRQ4

COM2    2F8             IRQ3

COM3    3E8             IRQ4

COM4    2E8             IRQ3

COM5    Undefined       Undefined

COM6    Undefined       Undefined

COM7    2E8             IRQ5

COM8    3E8             IRQ5

If you use these setups, you can simply specify the port number in your

ATSEND command line (Example: ATSEND Z 8 to reset a modem on COM8:):

If your port is setup differently, then ATSEND will look for two

environment variables called COMBASE and COMIRQ.

COMBASE must be converted from hex to decimal in order to work properly

(Example 744 for 2E8).  A utility called HEX2DEC.EXE is included to assist

in changing your hex port address to decimal.

COMIRQ is simply the IRQ number being used.

For example, if you were going to run COM7 with a Base Address of 2F8 on

IRQ2, at the DOS prompt, or in a batch file you would type:

SET COMBASE=760

SET COMIRQ=2

This setting will override any DSZPORT, or command line commands.

I have included a program called HEX2DEC.EXE that can be used to

convert your hex port address into decimal.  If COMBASE and COMIRQ are in

the environment, there is no need to put the port number on the command

line.

�����������

ATBATCH.EXE

�����������

ATBatch is the easy way out.  It will create batch files for you

that send the proper commands to ATSEND, including COMIRQ and

COMBASE, including automatic hex to decimal conversion.  Use

ATBATCH to create a batch file called ATH1.BAT.  Then you can

just type ATH1 from DOS just like you were inside your favorite

terminal program!

����

USES

����

I wrote this as a utility to take my modem off the hook during BBS mail

runs so that callers wouldn't ring the line during the event.  I'm sure

there are other uses for it, and would be interested in hearing from

users who find new ways to use this program.

�������

SUPPORT

�������

The latest version of this program along with other software I have written

are always available for download by first time callers to my BBS, The

Ledge PCBoard, 818-896-4015 (HST/V32.bis). First time callers may download

my programs on the first call when they use the "SUPPORT DOOR" which can be

found by typing the word OPEN at the main menu.  You may pay for

registration online with MASTERCARD or VISA when you call The Ledge and

type REGISTER at the main menu command prompt.

Please report any problems, or suggestions to me at the following

addresses:

US Mail:

--------

Joseph Sheppard

The Ledge PCBoard

6858 Acco Street

Los Angeles, CA 90040

Modem:

------

The Ledge PCBoard

818-896-4015

USRobotics Dual Standard HST (V.32bis)

Echo Mail:

----------

U'NI-net/US (All Conferences)

Smartnet (Sysop Conference)

Compuserve:

-----------

73175,623

������������

REGISTRATION

������������

ATSEND.EXE is fully functional.  this is not crippled or limited software.

You can support this method of distribution, and encourage it to continue

by sending a shareware registration.  I will accept whatever you feel this

program is worth to you, I would suggest $10.00.

For registrations by Visa or Mastercard, please call The Ledge PCBoard at

(818) 896-4015 and enter the word REGISTER at the main menu command prompt.

Checks and Money Orders may be made payable to THE LEDGE PCBOARD and mailed

to:

JOSEPH SHEPPARD

THE LEDGE PCBOARD

6858 ACCO STREET

LOS ANGELES, CA 90040

�������

WARNING

�������

I make no warranty of any kind, express or implied, including without

limitation, any warranties of merchantability and/or fitness for a

particular purpose.  I shall not be liable for any damages, whether

direct, indirect, special or consequential arising from a failure of

this program to operate in the manner desired by the user.  I shall

not be liable for any damage to data or property which may be caused

directly or indirectly by the use of this program.

IN NO EVENT WILL I BE LIABLE TO YOU FOR ANY DAMAGES, INCLUDING ANY

LOST PROFITS, LOST SAVINGS OR OTHER INCIDENTAL OR CONSEQUENTIAL

DAMAGES ARISING OUT OF YOUR USE OR INABILITY TO USE THE PROGRAM, OR

FOR ANY CLAIM BY ANY OTHER PARTY.