TriBBS 4.02 Interim Update Documentation

----------------------------------------

TriBBS 4.02  provides a  couple of new  features that  require further

explanation.

Nonstandard IRQs

----------------

Because  a number  of sysops  have  expressed interest  in having  the

ability  to run  TriBBS with nonstandard  IRQs, I have  decided to put

support for them into 4.02 and not wait for 4.1.

First of all, please note that  many doors (read that as most)  do NOT

support  nonstandard IRQs.   Also, some external  protocol drivers may

not support nonstandard IRQs.

By default, ISA PCs use the following IRQs for COM1 through COM4:

COM1        IRQ4

COM2        IRQ3

COM3        IRQ4

COM4        IRQ3

Therefore, it  is possible  to have four  serial devices  connected to

your PC.  If they require interrupts to  function, then you are really

limited to only  two serial ports because  COM1 and COM3 use  the same

IRQ and  COM2  and COM4  use  the same  IRQ  and the  way  the PC  was

originally designed does not allow for sharing of interrupts.

You can  get around this  by assigning  a nonstandard  IRQ setting  to

serial devices on COM3 and COM4.

XT  compatible machines  have 8 available  interrupt vectors  (IRQ0 to

IRQ7) and are used as follows:

IRQ0        System timer

IRQ1        Keyboard

IRQ2        Available

IRQ3        COM2

IRQ4        COM1

IRQ5        Hard disk controller

IRQ6        Floppy disk controller

IRQ7        LPT1

As you can see,  you are pretty much limited  to using either IRQ2  or

IRQ7 on an XT compatible machine.

AT compatible machines  have 16 available  interrupt vectors (IRQ0  to

IRQ15) and are used as follows:

IRQ0        System timer

IRQ1        Keyboard

IRQ2        Cascade input for second interrupt controller

IRQ3        COM2

IRQ4        COM1

IRQ5        LPT2

IRQ6        Floppy disk controller

IRQ7        LPT1

IRQ8        Realtime clock

IRQ9        Redirected as IRQ2

IRQ10       Available

IRQ11       Available

IRQ12       PS/2 mouse

IRQ13       Math coprocessor

IRQ14       Hard disk controller

IRQ15       Available

As  you can see, an  AT compatible machine has a  lot more options for

using a nonstandard IRQ (IRQ5, IRQ7, IRQ10, IRQ11, and IRQ15).

If you  have any perhiperials  that use  interrupts, be sure  that you

don't use an interrupt that is already in use by another device.

Configuring for a Nonstandard IRQ

---------------------------------

You can  configure your system  for a nonstandard IRQs  through either

TriMan's Node  Configuration option or  through a new  BBS.EXE command

line  switch.  The  command line  switch is /Qn  where "n" is  the IRQ

number  to be used.  For example if  you wanted to use IRQ5 for a node

that is already setup for COM3,  you would enter the following command

line:

BBS /Q5

DOWNLOAD.BAT

------------

TriBBS 4.02  now supports  a DOWNLOAD.BAT file.   This  optional batch

file will  be called, if it exists in  the node's main directory, just

after a file is moved into the TRANSFER directory.   It is called with

parameters  that  are  similar  to   the  ones  TriBBS  uses  to  call

UPLOAD.BAT:

%1 - Is the file's complete pathname.

%2 - Is the file's extension, if any.