��� � � �ͻ     �ͻ �ͻ �ͻ ��� �˻ �ͻ ��� �ͻ ��� �ͻ �ͻ

�  �͹ ��      �ͼ � � �˼  �  ��� ��   �  �ͻ  �  ��  �˼

�  � � ���     �   �ͼ ���  �  � � ��� ��� �ͼ  �  ��� ���

Fast Port Lock and Unlock Utility for Spitfire

The PortMeister 1.00

September 8, 1992

Copyright (C) 1992 Mike Robinson

The Soft Parade

Shelton, Connecticut, USA

203-924-5603

The Portmeister is a PC communications port lock and unlock utility

designed for use on a Spitfire BBS. It adjusts the computer-to-modem

(DTE/DCE) data communications speed on the fly. It does not alter the

modem-to-modem link speed. It works well with the US Robotics Dual

Standard modem, including the new 16800 HST model. It may also work

with other modems which use the Hayes AT command set, but this has

not yet been tested.

Many BBSs run with what is sometimes called a "locked" comm port.

This means the comm port is set to a fixed speed, usually either

19200 bps or 38400 bps. This is fine for most operations, but there

are some doors (external programs) which will not operate properly at

such high speeds. These are usually older programs that are limited

to a maximum speed of 9600 bps. One possible solution would be to run

the BBS full time at the slower speed. But this slows the entire BBS

down. A better solution for many is to change the port speed on the

fly as required.

*** SETUP ***

The PortMeister requires these modem settings:

1. The escape code response must be set to NOT DROP CARRIER when the

modem goes to command mode. On a US Robotics Dual Standard this is

done by setting DIP SWITCH 9 DOWN.

2. The modem must also have the following command settings which are

usually the same as those used by Spitfire. Note that the actual

command may be different if you are not using a USR Dual Standard.

But the command function and effect must be the same.

&B1  Fixed DTE Rate. A fixed DTE Rate (port speed) must be set.

Since this is a modem command, the computer port speed can

still be changed and the modem made to follow the DTE rate.

The PortMeister will not work correctly for link rates higher

than 9600 bps if the port speed is set to follow the link

rate.

Q0   Result Codes Displayed. (With a US Robotics Dual Standard

this can also be done by setting DIP switch 3 down.)

V1   Verbal Result Codes.    (With a US Robotics Dual Standard

this can also be done by setting DIP switch 2 up.)

S2=1  Escape character is ASCII character number 1. (smiley face)

In it's simplest form, The PortMeister is copied to the Spitfire home

directory and executed in a batch file like this:

PMEISTER UNLOCK

SFMNUEXT         ( SFMNUEXT shown, but any program can be run here. )

PMEISTER LOCK

This is the way I run it on my BBS. For most people, this will be all

that is required. It's that simple. The PortMeister will search the

current directory and find the data file SFDOORS.DAT, SFFILE.DAT,

SFMAIN.DAT or SFMESS.DAT.

But if you like to get complicated about things, there are more

options. You can execute it from any directory and specify the path

and name of the data file like this:

PMEISTER UNLOCK C:\SF\SFDOORS.DAT

SFMNUEXT

PMEISTER LOCK   C:\SF\SFDOORS.DAT

And if you really want get fancy, you can do this:

PMEISTER UNLOCK C:\SF\SFDOORS.DAT C:\SF\PMEISTER.CFG

SFMNUEXT

PMEISTER LOCK   C:\SF\SFDOORS.DAT C:\SF\PMEISTER.CFG

In this case, you are specifying the path to the configuration file

too. The configuration file is OPTIONAL. The PortMeister will run

fine without it. It is only required if you want to change the timing

of the commands to the modem or you want to change the escape

character used. If the path and name to the configuration file is not

specified, then it will search the current directory for

PMEISTER.CFG. If the config file is not found, then the default

configuration values will be used.

*** OPTIONAL CONFIGURATION FILE ***

The PortMeister reads each line in this file. If the line begins with

a keyword then the configuration is modified according to the

information that follows the keyword. If a line begins with anything

other than a letter, it is ignored. The order of the lines is not

important. The keywords are NOT case sensitive, but they must be

spelled correctly.

EscapeChar 1

; This defines command mode escape character sent to the modem.

; I doubt you will ever have to change this since Spitfire requires

; that the escape character be ASCII character 1 (smiley face).

; The default is 1.

PreTics 27

; This is the pause in clock tics BEFORE the command mode escape code

; is sent to the modem. There are approximately 18 clock tics in 1

; second. This setting is critical to proper operation. Most modems

; require at least a one second pause with no data before sending

; the command mode escape code.

; The default is 27 tics (which equals 1.5 seconds)

; This seems to work best here. Experiment if you like, but be

; careful. There is a wide variation of responses. If a certain

; timing works for a 9600 bps caller, it may not work for a 1200.

PostTics 18

; This is the pause in clock tics AFTER the command mode escape code

; is sent to the modem. Most modems require at least a one second

; pause with no data after sending the command mode escape code.

; The default is 18 tics (which equals 1 second)

*** HOW IT WORKS ***

1. The PortMeister is executed in unlock mode. It reads the caller's

connect speed on line 5 of SFDOORS.DAT, SFFILE.DAT, SFMAIN.DAT or

SFMESS.DAT. It unlocks and sets the port to this speed. If the

connect speed is higher than 9600 bps, then the port speed will be

set to 9600. It then modifies and saves SFDOORS.DAT (or

equivalent) with both the connect speed (line 5) and maximum speed

(line 20) set to the new lower port speed. It saves the original

values in a temporary file named PMEISTER.DAT.

2. The door or external program is then run. This is often an

extended menu type of program such as SFMNUEXT or SFEXTEND.

3. The PortMeister is executed in lock mode. It reads and then erases

PMEISTER.DAT. It then restores the port and SFDOORS.DAT

(or equivalent) to their original settings.

*** REGISTRATION ***

This is SHAREWARE. You may use this program for a free 30 day trial

period. After that, you must register it or stop using it.

You may give away free copies of this entire unmodified package,

but you may not sell it. I retain all rights to this program.

In the registered version the * EVALUATION COPY *  message on the

logo is replaced by your name.

Registration is $10 (TEN) dollars in US funds.

Personal check, bank check or money orders are acceptable.

Registered Sysops are entitled to free program support, free

upgrades to all future versions, and free access to all files and

features on my BBS. Support is available from me on my BBS, in

CircuitNet and in FidoNet. Suggestions for future improvements from

registered Sysops will receive careful consideration.

In order to register the program, you will have to call my BBS and

download your personal registration key and instruction file. This

key will work for all future versions, so there will be no need to

download any specially compiled versions of the executable program

itself.

If you do not yet have access to my BBS, please be sure to send

along a password and birthdate with your registration. Then I can

set up your logon account in advance. Please be sure to specify

your name and your BBS name exactly as you would like them to

appear on the first door screen.

-------------------------------------------------------

-------------------------------------------------------

-------------------------------------------------------

The PortMeister 1.00     REGISTRATION FORM

Your Name:

(40 characters maximum)

BBS Name:

BBS Phone Number:

Password to use on  Mike Robinson's BBS:

(14 characters maximum)

BirthDate to use on Mike Robinson's BBS:

(mm-dd-yy)

Your Address:

Send this form and $10 (TEN) dollars in US funds to:

Michael Robinson

9 Richard Boulevard

Shelton, CT. 06484

Personal check, bank check or money orders are acceptable.

-------------------------------------------------------

-------------------------------------------------------

-------------------------------------------------------

�������������������ͻ ����������������������ͻ �������������������ͻ

�  The Soft Parade  � � Sysop: Mike Robinson � � 1200 to 16800 bps �

�  Shelton, CT. USA � � Spitfire version 3.2 � �   HST   V.32bis   �

�   203-924-5603    � � Open 24 hours 7 days � �  MNP 5   V.42bis  �

� FidoNet 1:141/485 � �  CircuitNet  203004  � � USR Dual Standard �

�������������������ͼ ����������������������ͼ �������������������ͼ

Legal Stuff:

Use this program at your own risk. No guarantees of any sort.

"Spitfire" is a registered trademark of Buffalo Creek Software.

The PortMeister Revision History

Version 1.00 - Initial Public Release - Sept 8, 1992

ZIP archive security notice for this archive should read:

Authentic files verified!   # ACO784   The Soft Parade