SHADOW RAM MEMORY CONTROLLERS

supported by

THE LAST BYTE MEMORY MANAGER (tm)

Version 2.11

Copyright (C) 1990-92

All Rights Reserved

by

KEY SOFTWARE PRODUCTS

440 Ninth Avenue

Menlo Park, California 94025

415-364-9847

The Last Byte Memory Manager is a trademark of Key Software Products.

TABLE OF CONTENTS

CHAPTER 1 - INTRODUCTION        ...........................    1

1.1  Hardware Requirements       .......................    1

1.2  Supported Chipsets       ..........................    1

1.2.1  Chips and Technologies      ..................    1

1.2.2  ETEQ Microelectronics       ..................    2

1.2.3  Forex Computer Corporation      ..............    2

1.2.4  Headland Technology (G2)      ................    2

1.2.5  Intel Corporation       ......................    3

1.2.6  OPTi Electronics       .......................    3

1.2.7  Sun Electronics (SUNTAC)      ................    4

1.2.8  Symphony Laboratories       ..................    4

1.2.9  Texas Instruments       ......................    4

1.2.10  United Microelectronics Corporation (UMC)        4

1.2.11  VLSI Technology       .......................    5

1.3  Unsupported Chipsets       ........................    5

1.3.1  No Shadow Ram Support     ....................    6

1.3.2  Shadow Ram is not Read/Write    ..............    6

1.3.3  Overly Complex Control      ..................    6

1.3.4  Locked Configuration Registers      ..........    6

1.4  Checking Compatibility Using CHIPSET     ..........    7

Aug 11, 1992      SHADOW RAM MEMORY CONTROLLERS                 1

CHAPTER 1 - INTRODUCTION

1.1 Hardware Requirements

To  install The Last Byte Memory Manager, your computer must have

a  means  of providing read-write memory in the Upper Memory Area

between  640k  and  1 megabyte.  This is possible if you have any

of the following:

o  Fixed  RAM memory already resident in the upper area

(such  as  the 32-bit "Built In Memory" installed in

some Compaq 386 machines), or

o  An  expanded memory board (EEMS, LIM 3.2 EMS, or LIM

4.0 EMS), or

�������������������������������������������������Ŀ

� NOTE: Using a LIM 3.2 EMS board is not advised  �

� as a method of providing upper memory.  It pro- �

� vides only 64k of upper memory and prevents ap- �

� plication software from using expanded memory.  �

���������������������������������������������������

o  A  386sx,  386,  486,  or  486sx  processor  and the

Microsoft  Windows  3.0/3.1  or  DOS  5  versions of

EMM386, or

o  One  of  the  shadow  ram  memory  controller  chips

listed in the next section.

1.2 Supported Chipsets

Key  Software  Products  is  continually  adding  support for new

chipsets  as  they  become  known to us and technical information

can  be  obtained.  Those that are currently supported in version

2.11  are  listed in the paragraphs below.  Some that we know of,

but which cannot be supported are listed in the next section.

1.2.1 Chips and Technologies

The  following  chips  all  provide 384k of memory in twenty-four

16k blocks between A0000-FFFFF.

Copyright (C) 1990-92, Key Software Products. All Rights Reserved

Aug 11, 1992      SHADOW RAM MEMORY CONTROLLERS                 2

82C212 - From the CS8221 NEAT (tm) chipset

82C222 - From the CHIPS/250 PS/2 50/60 chipset

82C230 - The PS/2 30 Single Chip Controller

82C235 - The SCAT (tm) Single Chip AT VLSI chip

82C241 - From the CS8223 LeAPset (tm) chipset

82C302 - From the CS8230 386/AT CHIPSet (tm)

82C307 - From the CS8231 386/AT CHIPSet (tm)

82C311 - From the CS8233 PEAK 386/AT CHIPSet (tm)

82C351 - From the CS82310 PEAK DM/386 AT CHIPSet (tm)

82C812 - From the CS8281 NEATsx (tm) chipset

82C836 - The SCATsx (tm) Single Chip AT VLSI chip

82C841 - From the CS8283 LeAPset-sx (tm) chipset

(The  82C212  is  identical  to the UMC82C212 made by United

Microelectronics Corporation.)

The  following chip provides 384k of memory in sixteen 16k blocks

between A0000-DFFFF and one 128k block between E0000-FFFFF.

82C322 - From the CHIPS/280 PS/2 70/80 chipset

1.2.2 ETEQ Microelectronics

These  chips  all  provide  192k  of  memory in twelve 16k blocks

between  C0000-EFFFF.   If you have a VGA or EGA adapter, you may

get  additional  memory  by  using  the  MOVE=OVERLAY  option  of

LASTBYTE.SYS.

ET82C391  - From the Bobcat 386DX/486DX chipset

ET82C491  - From the Cougar 386DX/486DX chipset

ET82C4901 - From the Bengal 386DX/486DX chipset

(The  ET82C391  and  '491  are  indistinguishable;  both are

reported as a '491.)

1.2.3 Forex Computer Corporation

The  following  chip  provides  256k  of memory in six 32k blocks

between C0000-EFFFF and one 64k block between F0000-FFFFF.

FRX36C300 - From the FRX36C300/200 Cache chipset

1.2.4 Headland Technology (G2)

The  following  chip provides 64k of memory by using the four 16k

blocks of the EMS page frame.

Copyright (C) 1990-92, Key Software Products. All Rights Reserved

Aug 11, 1992      SHADOW RAM MEMORY CONTROLLERS                 3

HT12  - Single 286 AT Chip

The  following  chips all provide 128k of memory by using the EMS

mapping area between C0000-DFFFF.

HT18  - 286/386SX Advanced Memory Manager Chip

HT21  - 286/386SX Advanced Memory Manager Chip

HT22  - 286/386SX Advanced Memory Manager Chip

HT103 - 286/386SX Advanced Memory Manager Chip

HT113 - 286/386SX Advanced Memory Manager Chip

1.2.5 Intel Corporation

The  following  chips provide 192k of memory in twelve 16k blocks

between  C0000-EFFFF.   If you have a VGA or EGA adapter, you may

get  additional  memory  by  using  the  MOVE=OVERLAY  option  of

LASTBYTE.SYS.

82343 - From the 82340SX Chip Set

82346 - From the 82340DX Chip Set

(The  82343  and  82346  are  identical  to the VL82C320 and

VL82C330 made by VLSI Technology.)

The  following  chip  provides  384k of memory in twenty-four 16k

blocks between A0000-FFFFF.

82359 - From the 82350DT EISA Chip Set

1.2.6 OPTi Electronics

The  following  chips  all  provide  192k of memory in twelve 16k

blocks  between  C0000-EFFFF.   If you have a VGA or EGA adapter,

you  may  get  additional memory by using the MOVE=OVERLAY option

of LASTBYTE.SYS.

82C281 - From the Cache SX/AT chipset (w/posted write)

82C282 - From the Cache SX/AT chipset (w/o posted write)

82C283 - From the Block Interleave SX/AT chipset

82C382 - From the HiD/386 AT chipset

82C391 - From the 386WB PC/AT chipset

82C461 - From the Notebook PC/AT chipset

82C482 - From the HiB/486 AT chipset

82C491 - From the 486SXWB PC/AT chipset

82C493 - From the 486SXWB PC/AT chipset

82C495 - From the 486SXWB PC/AT chipset

82C496 - From the DXBB PC/AT chipset

(The  '281,  '282, and '283 are software compatible; all are

Copyright (C) 1990-92, Key Software Products. All Rights Reserved

Aug 11, 1992      SHADOW RAM MEMORY CONTROLLERS                 4

reported  as an 82C281.  Similarly, the '491, '493, and '495

are both reported as an 82C493.)

The  following  chip  provides 256k of memory in eight 16k blocks

between C0000-DFFFF and two 64k blocks between E0000-FFFFF.

82C682 - From the 386/486WB EISA chipset

1.2.7 Sun Electronics (SUNTAC)

All  SunTac  chips  provide  128k  of  memory  in four 32k blocks

between  E0000-FFFFF.   An  additional  128k  in eight 16k blocks

between  C0000-DFFFF  is available at the expense of all Expanded

Memory  by adding the word NOEMS to the SUNTAC Physical option as

in: Physical=ST62C202,NOEMS.

ST62C202 - From the chipset 20 for 286

ST62C241 - From the chipset 24 for 386

ST62C251 - From the chipset 25 for 286 and 386sx

1.2.8 Symphony Laboratories

The  following  chips  provide 256k of memory in eight 16k blocks

between   C0000-DFFFF   andanother   two   64k   blocks   between

E0000-FFFFF.

SL82C361 - From the SL82C360 80386DX/SX chipset

SL82C461 - From the SL82C460 80486 chipset

1.2.9 Texas Instruments

The  following  chips  provide  384k of memory in twenty-four 16k

blocks between A0000-FFFFF.

TACT82411  - The Snake Single-Chip AT Controller

TACT82S411 - The Snake+ Single-Chip AT Controller

1.2.10 United Microelectronics Corporation (UMC)

The  following  chip  provides  384k of memory in twenty-four 16k

blocks between A0000-FFFFF.

UM82C212 - From the UM82C210 386SX/286 AT Chip Set

(The  UMC82C212 is identical to the 82C212 made by Chips and

Technologies.)

Copyright (C) 1990-92, Key Software Products. All Rights Reserved

Aug 11, 1992      SHADOW RAM MEMORY CONTROLLERS                 5

The  following  chip  provides 256k of memory in eight 16k blocks

between C0000-DFFFF and two 64k blocks between E0000-FFFFF.

UM82C481 - From the UM82C480 50Mhz 80486 Chip Set

1.2.11 VLSI Technology

The  following  chip  provides  384k  of memory in six 64k blocks

between A0000-FFFFF.

VL82C202 - PC/AT-Compatible High Speed Memory Controller

�������������������������������������������������Ŀ

� Pin 69 on th VL82C202 controls whether its      �

� shadow ram is enabled or relocated.  This pin   �

� is usually tied to a jumper on the motherboard  �

� which must be set properly to work.             �

���������������������������������������������������

The  following  chips  provide  384k of memory in four 32k blocks

between  A0000-BFFFF,  eight  16k blocks between C0000-DFFFF, and

four 32k blocks between E0000-FFFFF.

VL82C310 - From the SCAMP 286/386sx Chip Set

VL82C311 - From the SCAMP 286/386sx Chip Set

The  following  chips provide 192k of memory in twelve 16k blocks

between  C0000-EFFFF.   If you have a VGA or EGA adapter, you may

get  additional  memory  by  using  the  MOVE=OVERLAY  option  of

LASTBYTE.SYS.

VL82C320 - From the TOPCAT 286/386sx Chip Set

VL82C330 - From the TOPCAT 386 DX Chip Set

(The  VL82C320  and  VL82C330 are identical to the 82343 and

82346 made by Intel Corporation.)

The  following  chip  provides  384k of memory in twenty-four 16k

blocks between A0000-FFFFF.

VL82C486 - Single Chip, High Performance 486 PC/AT Controller

1.3 Unsupported Chipsets

If  your computer has shadow ram capability, then it probably has

a  shadow ram memory controller.  However, some of these chipsets

cannot  be  supported by The Last Byte Memory Manager for reasons

shown below.

Copyright (C) 1990-92, Key Software Products. All Rights Reserved

Aug 11, 1992      SHADOW RAM MEMORY CONTROLLERS                 6

Some  of  these  chips, however, do have hardware EMS support and

thus  many  of  their motherboards come with an EMS device driver

designed  specifically  to take advantage of this capability.  So

in  many  cases,  you can install this driver first, and then The

Last  Byte  Memory  Manager can use it to manage the upper memory

area.

1.3.1 No Shadow Ram Support

Some   of   the   older   chipsets  do  not  provide  shadow  ram

capability.

Examples: VLSI Technologies 82C100 series

Western Digital chipsets

Texas Instruments TACT82300 series

Early Suntac chipsets (ST62BC001-6)

United Microelectronics Corp. UM82C231

1.3.2 Shadow Ram is not Read/Write

Some  chipsets  have  shadow ram, but can only put it into either

write-only  mode  (for copying rom contents to ram), or read-only

mode  (to  prevent accidental writes into what's supposed to be a

copy of rom).

Examples: ACC Microelectronics chipsets

Headland/G2 Technologies chipsets

CONTAQ chipsets

1.3.3 Overly Complex Control

The  Last Byte Memory Manager software is table-driven, making it

relatively   easy   to   adapt  to  different  memory  controller

architectures, but there are limits!

Examples: Zymos chipsets

United Micro. Corp. UM82C384, '388, '389

Forex Computer Corp. FRX36C100

Silicon Integrated System Corp. (SIS)

chipsets

1.3.4 Locked Configuration Registers

Some  memory  controllers  are  designed  so  that  they  can  be

"locked"  at  the  end  of  software  initialization, requiring a

hardware  reset  to  unlock,  and  thus  preventing The Last Byte

Memory Manager from accessing the controller.

Copyright (C) 1990-92, Key Software Products. All Rights Reserved

Aug 11, 1992      SHADOW RAM MEMORY CONTROLLERS                 7

Examples: Intel 82335, 82335SX

1.4 Checking Compatibility Using CHIPSET

CHIPSET  is a program that attempts to determine if your computer

has  one  of the memory controllers required to use The Last Byte

Memory  Manager.   To  run  CHIPSET, simply enter its name on the

command line:

A>CHIPSET

and  follow  the  prompts.   CHIPSET  will  try  to  identify the

particular  memory controller you have, if any.  If it can't find

a  memory  controller, it will look for fixed memory in the upper

area  or  expanded  memory as a means of installing The Last Byte

Memory  Manager.   If  it  is  successful,  it  will give you the

option  of  automatically linking to the INSTALL program to setup

for use of The Last Byte Memory Manager.

CHIPSET   automatically  senses  whether  you  have  a  color  or

monochrome  display  to  determine  how  to  present  data on the

screen.   However,  you  may  force  the mode by using one of the

command  line  options:  /MONO,  /COLOR,  or /LCD (for monochrome

liquid crystal displays).

If  your  computer  has  one of the listed devices, then The Last

Byte Memory Manager should work on your computer.

Copyright (C) 1990-92, Key Software Products. All Rights Reserved