Encoding of Effective Addresses

Unless you are concerned with the nitty-gritty details of 86

instruction encoding, you don't need to read this file.

Every instruction with an effective address has an encoded byte,

known as the effective address byte, following the 1-byte opcode

for the instruction. (For obscure reasons, Intel calls this byte

the ModRM byte.)  If the effective address is a memory variable,

or an indexed memory location with a non-zero constant offset,

then the effective address byte will be immediately followed by

the offset amount.  Amounts in the range -128 to +127 are given

by a single signed byte, denoted by "d8" in the table below.

Amounts requiring a 2-byte representation are denoted by "d16" in

the table below.  As with all 16-bit memory quantities in the 86

family, the word is stored with the least significant byte FIRST.

The following table of effective address byte values is organized

into 32 rows and 8 columns.  The 32 rows give the possible values

for the effective address operand: 8 registers and 24 memory

indexing modes.  A 25th indexing mode, [BP] with zero

displacement, has been pre-empted by the simple-memory-variable

case.  If you code [BP] with no displacement, you will get

[BP]+d8, with a d8-value of zero.

The 8 columns of the table reflect further information given by

the effective address byte.  Usually, this is the identity of the

other (always a register) operand of a 2-operand instruction.

Those instructions are identified by a "/r" following the opcode

byte in the instruction list.  Sometimes, the information given

supplements the opcode byte in identifying the instruction

itself.  Those instructions are identified by a "/" followed by a

digit from 0 through 7.  The digit tells which of the 8 columns

you should use to find the effective address byte.

For example, suppose you have a perverse wish to know the precise

bytes encoded by the instruction SUB B[BX+17],100.  This

instruction subtracts an immediate quantity, 100, from an

effective address quantity, B[BX+17].  By consulting the

instruction list, you find the general form SUB eb,ib.  The

opcode bytes given there are 80 /5 ib.  The "/5" denotes an

effective address byte, whose value will be taken from column 5

of the following table.  The offset 17 decimal, which is 11 hex,

will fit in a single "d8" byte, so we take our value from the

"[BX] + d8" row.  The table tells us that the effective address

byte is 6F.  Immediately following the 6F is the offset, 11 hex.

Following that is the ib-value of 100 decimal, which is 64 hex.

So the bytes generated by SUB B[BX+17],100 are 80 6F 11 64.

Table of Effective Address byte values

s  =     ES   CS   SS   DS

rb =     AL   CL   DL   BL   AH   CH   DH   BH

rw =     AX   CX   DX   BX   SP   BP   SI   DI

digit=    0    1    2    3    4    5    6    7

Effective

EA byte                                           address:

values:  00   08   10   18   20   28   30   38    [BX + SI]

01   09   11   19   21   29   31   39    [BX + DI]

02   0A   12   1A   22   2A   32   3A    [BP + SI]

03   0B   13   1B   23   2B   33   3B    [BP + DI]

04   0C   14   1C   24   2C   34   3C    [SI]

05   0D   15   1D   25   2D   35   3D    [DI]

06   0E   16   1E   26   2E   36   3E    d16 (simple var)

07   0F   17   1F   27   2F   37   3F    [BX]

40   48   50   58   60   68   70   78    [BX + SI] + d8

41   49   51   59   61   69   71   79    [BX + DI] + d8

42   4A   52   5A   62   6A   72   7A    [BP + SI] + d8

43   4B   53   5B   63   6B   73   7B    [BP + DI] + d8

44   4C   54   5C   64   6C   74   7C    [SI] + d8

45   4D   55   5D   65   6D   75   7D    [DI] + d8

46   4E   56   5E   66   6E   76   7E    [BP] + d8

47   4F   57   5F   67   6F   77   7F    [BX] + d8

80   88   90   98   A0   A8   B0   B8    [BX + SI] + d16

81   89   91   99   A1   A9   B1   B9    [BX + DI] + d16

82   8A   92   9A   A2   AA   B2   BA    [BP + SI] + d16

83   8B   93   9B   A3   AB   B3   BB    [BP + DI] + d16

84   8C   94   9C   A4   AC   B4   BC    [SI] + d16

85   8D   95   9D   A5   AD   B5   BD    [DI] + d16

86   8E   96   9E   A6   AE   B6   BE    [BP] + d16

87   8F   97   9F   A7   AF   B7   BF    [BX] + d16

C0   C8   D0   D8   E0   E8   F0   F8    ew=AX   eb=AL

C1   C9   D1   D9   E1   E9   F1   F9    ew=CX   eb=CL

C2   CA   D2   DA   E2   EA   F2   FA    ew=DX   eb=DL

C3   CB   D3   DB   E3   EB   F3   FB    ew=BX   eb=BL

C4   CC   D4   DC   E4   EC   F4   FC    ew=SP   eb=AH

C5   CD   D5   DD   E5   ED   F5   FD    ew=BP   eb=CH

C6   CE   D6   DE   E6   EE   F6   FE    ew=SI   eb=DH

C7   CF   D7   DF   E7   EF   F7   FF    ew=DI   eb=BH

d8 denotes an 8-bit displacement following the EA byte, to be

sign-extended and added to the index.

d16 denotes a 16-bit displacement following the EA byte, to be

added to the index.

Default segment register is SS for effective addresses containing

a BP index; DS for other memory effective addresses.