HRTIMER.SYS is a high resolution timer implemented as a device driver. The

source code for this device driver was obtained from the Fall 1991 issue of

IBM Personal Systems Developer magazine. For insightful information on the

way this device driver works, please read that article. It's very interesting...

HRTIMER.SYS runs under OS/2 1.x and 2.0. It has a resolution of 840 nanoseconds.

HRTEST.EXE is a sample C Set/2 program that shows how to use the device driver

to calculate elapsed times. It demonstrates how to open the device driver, read

timestamps from it and close it. It factors in the overhead of the read and

has a function that is used to calculate elapsed time from a start and stop

timestamp.

To install the device driver, put the following statement in your config.sys:

DEVICE=HRTIMER.SYS

To run the test program, use the following command-line:

HRTEST [ milliseconds ]

HRTEST.EXE will issue a DosSleep for the amount of milliseconds specified or

will use a default if no command-line parameter is given. It will get a

timestamp from the device driver before and after the DosSleep and will

calculate the elapsed time of that sleep and display the results. It will do

this continuously until Ctrl-C or Ctrl-Break is pressed.

Keep in mind that DosSleep has a granularity of 32 milliseconds. Any

discrepency between the number of milliseconds used for the DosSleep and the

elapsed time results from the timer are the fault of this granularity, not a

problem with the timer. DosSleep is used solely as a convenient method of

displaying the capabilities of the driver.

*******************************************************************************

IMPORTANT NOTE: There is a known bug with running HRTIMER.SYS while running

a full-screen Windows application as WinOS2 hooks the same timer that is used

by the device driver. I have not seen the bug myself but it was pointed out

to me. It is recommended therefore that if you use full-screen Windows apps

that you rem out the device statement in your config.sys when not using the

timer. The bug does not exist with seamless Windows apps.

*******************************************************************************

This package is distributed on an as-is basis solely for the benefit of other

developers either in need of a high-resolution timer or some sample device

driver code. The author makes no promises to provide support for the contents.

Any comments can be sent to my CIS id 72251,750.

Hope this proves useful...

Rick Fishman

Code Blazers, Inc.

4113 Apricot

Irvine, CA 92720